| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0064-v1 |
| --- | --- |

| Title: | AHG9: On the MAI, DRI, and ACI SEI messages and their interactions with the SDI SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Yang Wang2, Li Zhang1 18910 University Center Lane, San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com wangyang.cs@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes on the multiview acquisition information (MAI), depth representation information (DRI), and alpha channel information (ACI) SEI messages and their interactions with the scalability dimension information (SDI) SEI message:

- It is specified that the MAI SEI message persists in decoding order from the current AU until the next AU containing an MAI SEI message for which the content differs from the current MAI SEI message or the end of the bitstream.

The following constraints are further specified:

  - When an MAI SEI message is present in any AU of a CVS, an MAI SEI message shall be present for the first AU of the CVS.

  - All MAI SEI messages that apply to the same CVS shall have the same content.

  - An MAI SEI message shall not be scalable-nested.

- Add following constraints on the presence of the MAI, DRI, and ACI SEI messages depending on the SDI SEI message:

  - When a CVS does not contain an SDI SEI message, the CVS shall not contain an MAI SEI message.

  - When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to 2 for at least one value of i, no picture in the CVS shall be associated with a DRI SEI message.

  - When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to 1 for at least one value of i, no picture in the CVS shall be associated with an ACI SEI message.

- Add following constraints on the decoding order of the MAI, DRI, and ACI SEI messages relative to the SDI SEI message:

  - When an AU contains both an SDI SEI message and an MAI SEI message, the SDI SEI message shall precede the MAI SEI message in decoding order.

  - When an AU contains both an SDI SEI message with sdi_aux_id[ i ] equal to 2 for at least one value of i and a DRI SEI message, the SDI SEI message shall precede the DRI SEI message in decoding order.

  - When an AU contains both an SDI SEI message with sdi_aux_id[ i ] equal to 1 for at least one value of i and an ACI SEI message, the SDI SEI message shall precede the ACI SEI message in decoding order.

- The variable NumViews is derived in the semantics of the SDI SEI message and used in the MAI SEI message syntax and semantics, and the use of the array nestingLayerIdList is removed from the semantics of the MAI SEI message.

- The associated primary layers for an auxiliary layer are signalled in the SDI SEI message syntax and specified in the SDI SEI semantics, and the related texts in the semantics of the DRI and ACI SEI messages are changed accordingly based on the established association.

The text changes for the proposals in JVET-V0064 (this contribution) are included in the attachment to JVET-V0063.

# Introduction

The current syntax and semantics of the MAI, DRI, and ACI SEI messages have the following issues:

- The MAI SEI message carries information for all views in a multiview bitstream, thus it should not be specified as layer-specific (as it is the case now). Rather, the scope should be for the current CVS instead of the current CLVS. Furthermore, since an MAI SEI message contains information for all layers, it would make more sense to disallow it to be contained in a scalable nesting SEI message.

- It does not make sense to have a multiview acquisition information (MAI), DRI, or ACI SEI message present in the bitstream but there is no SDI SEI message.

- Currently, when an AU contains both an SDI SEI message and an MAI SEI message, the MAI SEI message may precede the SDI SEI message in decoding order. However, the presence and the interpretation of the MAI SEI message should depend on the SDI SEI message. Therefore, it makes more sense to require that an SDI SEI message precedes an MAI SEI message in the same AU in decoding order. A similar issue applies for the DRI SEI message and the ACI SEI message.

- The variable numViewsMinus1 and the array nestingLayerIdList used in the MAI SEI message syntax and/or semantics are not properly specified, e.g., as pointed out by the following editor's note:

[Ed. (JB): How is nestingLayerIdList[ 0 ][ i ] determined?  It exists in HEVC, but not in HEVC. For VVC, is this supposed to be a reference to NestingLayerId[ i ] ? ]

- When some auxiliary information is present in the bitstream, e.g., as indicated by the scalability dimension information (SDI) SEI message, and the depth representation information (DRI) SEI message or the alpha channel information (ACI) SEI message, it’s unknown to which non-auxiliary or primary layers the auxiliary information applies.

# Proposal

The proposal for addressing the above-mentioned issues is summarized in the abstract. The proposed text changes are as follows:

## 8.19 Scalability dimension information SEI message

8.19.1 Scalability dimension information SEI message syntax

| scalability_dimension_info( payloadSize ) { | Descriptor |
| --- | --- |
| sdi_max_layers_minus1 | u(6) |
| sdi_multiview_info_flag | u(1) |
| sdi_auxiliary_info_flag | u(1) |
| if( sdi_multiview_info_flag ¦ ¦ sdi_auxiliary_info_flag ) { |  |
| if( sdi_multiview_info_flag ) |  |
| sdi_view_id_len | u(4) |
| for( i = 0; i <= sdi_max_layers_minus1; i++ ) { |  |
| if( sdi_multiview_info_flag ) |  |
| sdi_view_id_val[ i ] | u(v) |
| if( sdi_auxiliary_info_flag ) |  |
| sdi_aux_id[ i ] | u(8) |
| if( sdi_aux_id[ i ] > 0 ) { |  |
| sdi_num_associated_primary_layers_minus1[ i ] | u(6) |
| for( j = 0; j <= sdi_num_associated_primary_layers_minus1[ i ]; j++ ) |  |
| sdi_associated_primary_layer_idx[ i ][ j ] | u(6) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

8.19.2 Scalability dimension information SEI message semantics

The scalability dimension information (SDI) SEI message provides the scalability dimension informationSDI for each layer in bitstreamInScope (defined below), such as 1) when bitstreamInScope may be a multiview bitstream, the view ID of each layer; and 2) when there may be auxiliary information (such as depth or alpha) carried by one or more layers in bitstreamInScope, the auxiliary ID of each layer.

The bitstreamInScope is the sequence of AUs that consists, in decoding order, of the AU containing the current scalability dimension informationSDI SEI message, followed by zero or more AUs, including all subsequent AUs up to but not including any subsequent AU that contains a scalability dimension information SEI message.

[Ed. (JB): Should add some wording here and in VVC to describe how the i-th layer is defined.]

sdi_max_layers_minus1 plus 1 indicates the maximum number of layers in bitstreamInScope.

sdi_multiview_info_flag equal to 1 indicates that bitstreamInScope may be a multiview bitstream and the sdi_view_id_val[ ] syntax elements are present in the scalability dimension information SEI message. sdi_multiview_flag equal to 0 indicates that bitstreamInScope is not a multiview bitstream and the sdi_view_id_val[ ] syntax elements are not present in the scalability dimension information SEI message.

sdi_auxiliary_info_flag equal to 1 indicates that there may be auxiliary information carried by one or more layers in bitstreamInScope and the sdi_aux_id[ ] syntax elements are present in the scalability dimension information SEI message. sdi_auxiliary_info_flag equal to 0 indicates that there is no auxiliary information carried by one or more layers in bitstreamInScope and the sdi_aux_id[ ] syntax elements are not present in the scalability dimension information SEI message.

sdi_view_id_len specifies the length, in bits, of the sdi_view_id_val[ i ] syntax element.

sdi_view_id_val[ i ] specifies the view ID of the i-th layer in bitstreamInScope. The length of the sdi_view_id_val[ i ] syntax element is sdi_view_id_len bits. When present, the value of sdi_view_id_val[ i ] is inferred to be equal to 0.

The variable NumViews is derived as follows:

NumViews = 1
if( sdi_multiview_info_flag ) {
	for( i = 1; i  <=  sdi_max_layers_minus1; i++ ) {
		newViewFlag = 1
		for( j = 0; j < i; j++ )		(X)
			if( sdi_view_id_val[ i ]  = =  sdi_view_id_val[ j ] )
				newViewFlag = 0
		if( newViewFlag )
			NumViews++
	}
}

sdi_aux_id[ i ] equal to 0 indicates that the i-th layer in bitstreamInScope does not contain auxiliary pictures. sdi_aux_id[ i ] greater than 0 indicates the type of auxiliary pictures in the i-th layer in bitstreamInScope as specified in Table X.

Table X – Mapping of sdi_aux_id[ i ] to the type of auxiliary pictures

| sdi_aux_id[ i ] | Name | Type of auxiliary pictures |
| --- | --- | --- |
| 1 | AUX_ALPHA | Alpha plane |
| 2 | AUX_DEPTH | Depth picture |
| 3..127 |  | Reserved |
| 128..159 |  | Unspecified |
| 160..255 |  | Reserved |

NOTE 1 – The interpretation of auxiliary pictures associated with sdi_aux_id in the range of 128 to 159, inclusive, is specified through means other than the sdi_aux_id value.

sdi_aux_id[ i ] shall be in the range of 0 to 2, inclusive, or 128 to 159, inclusive, for bitstreams conforming to this version of this Specification. Although the value of sdi_aux_id[ i ] shall be in the range of 0 to 2, inclusive, or 128 to 159, inclusive, in this version of this Specification, decoders shall allow values of sdi_aux_id[ i ] in the range of 0 to 255, inclusive.

If sdi_aux_id[ i ] is equal to 0, the i-th layer is referred to as a primary layer. Otherwise, the i-th layer is referred to as an auxiliary layer. When sdi_aux_id[ i ] is equal to 1, the i-th layer is also referred to as an alpha auxiliary layer. When sdi_aux_id[ i ] is equal to 2, the i-th layer is also referred to as a depth auxiliary layer.

sdi_num_associated_primary_layers_minus1[ i ] plus 1 specifies the number of associated primary layers of i-th layer, which is an auxiliary layer. The value of sdi_num_associated_primary_layers_minus1[ i ] shall be less than the total number of primary layers.

sdi_associated_primary_layer_idx[ i ][ j ] specifies the layer index of the j-th associated primary layer of the i-th layer, which is an auxiliary layer. The value of sdi_aux_id[ sdi_associated_primary_layer_idx[ i ][ j ] ] shall be equal to 0.

NOTE 2 – An auxiliary layer describes a property of and applies to its associated primary layers.

## 8.20 Multiview acquisition information SEI message

8.20.1 Multiview acquisition information SEI message syntax

| multiview_acquisition_info( payloadSize ) { | Descriptor |
| --- | --- |
| intrinsic_param_flag | u(1) |
| extrinsic_param_flag | u(1) |
| if( intrinsic_param_flag ) { |  |
| intrinsic_params_equal_flag | u(1) |
| prec_focal_length | ue(v) |
| prec_principal_point | ue(v) |
| prec_skew_factor | ue(v) |
| for( i = 0; i <= intrinsic_params_equal_flag ? 0 : numViewsMinus1; i++ ) { |  |
| for( i = 0; i <= intrinsic_params_equal_flag ? 0 : ( NumViews − 1 ); i++ ) { |  |
| sign_focal_length_x[ i ] | u(1) |
| exponent_focal_length_x[ i ] | u(6) |
| mantissa_focal_length_x[ i ] | u(v) |
| sign_focal_length_y[ i ] | u(1) |
| exponent_focal_length_y[ i ] | u(6) |
| mantissa_focal_length_y[ i ] | u(v) |
| sign_principal_point_x[ i ] | u(1) |
| exponent_principal_point_x[ i ] | u(6) |
| mantissa_principal_point_x[ i ] | u(v) |
| sign_principal_point_y[ i ] | u(1) |
| exponent_principal_point_y[ i ] | u(6) |
| mantissa_principal_point_y[ i ] | u(v) |
| sign_skew_factor[ i ] | u(1) |
| exponent_skew_factor[ i ] | u(6) |
| mantissa_skew_factor[ i ] | u(v) |
| } |  |
| } |  |
| if( extrinsic_param_flag ) { |  |
| prec_rotation_param | ue(v) |
| prec_translation_param | ue(v) |
| for( i = 0; i <= numViewsMinus1; i++ ) |  |
| for( i = 0; i < NumViews; i++ ) |  |
| for( j = 0; j < 3; j++ ) { /* row */ |  |
| for( k = 0; k < 3; k++ ) { /* column */ |  |
| sign_r[ i ][ j ][ k ] | u(1) |
| exponent_r[ i ][ j ][ k ] | u(6) |
| mantissa_r[ i ][ j ][ k ] | u(v) |
| } |  |
| sign_t[ i ][ j ] | u(1) |
| exponent_t[ i ][ j ] | u(6) |
| mantissa_t[ i ][ j ] | u(v) |
| } |  |
| } |  |
| } |  |

8.20.2 Multiview acquisition information SEI message semantics

The multiview acquisition information (MAI) SEI message specifies various parameters of the acquisition environment. Specifically, intrinsic and extrinsic camera parameters are specified. These parameters could be used for processing the decoded views prior to rendering on a 3D display.

The MAI SEI message persists in decoding order from the current AU until the next AU containing an MAI SEI message for which the content differs from the current MAI SEI message or the end of the bitstream. When an MAI SEI message is present in any AU of a CVS, an MAI SEI message shall be present for the first AU of the CVS. All MAI SEI messages that apply to the same CVS shall have the same content.

An MAI SEI message shall not be contained in a scalable nesting SEI message.

Let the current AU be the AU containing the current MAI SEI message, and the current CVS be the CVS containing the current AU.

When a CVS does not contain an SDI SEI message, the CVS shall not contain an MAI SEI message.

When an AU contains both an SDI SEI message and an MAI SEI message, the SDI SEI message shall precede the MAI SEI message in decoding order.

The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the multiview acquisition information SEI message applies.

When present, the multiview acquisition information SEI message that applies to the current layer shall be included in an access unit that contains an IRAP picture that is the first picture of a CLVS of the current layer. The information signalled in the SEI message applies to the CLVS.

[Ed. (YK): This SEI message carries information for all views in a multiview bitstream, thus it should not be specified as layer-specific. Rather, the scope should be for the current CVS instead of the current CLVS, and add two constraints: a) When a CVS contains an MAI SEI message, at least the first AU of the CVS shall contain an MAI SEI message. b) All MAI messages in a CVS shall have the same payload content.

At the next JVET meeting, discuss and decide whether an errata item for this should be made for HEVC. The semantics of this SEI message in AVC seems OK.]

When the multiview acquisition information SEI message is included in a scalable nesting SEI message, the syntax elements bitstream_subset_flag, nesting_op_flag and all_layers_flag in the scalable nesting SEI message shall be equal to 0.

The variable numViewsMinus1 is derived as follows:

- If the multiview acquisition information SEI message is not included in a scalable nesting SEI message, numViewsMinus1 is set equal to 0.

- Otherwise (the multiview acquisition information SEI message is included in a scalable nesting SEI message), numViewsMinus1 is set equal to nesting_num_layers_minus1.

Some of the views for which the multiview acquisition information is included in a multiview acquisition information SEI message may not be present.

In the semantics below, index i refers to the syntax elements and variables that apply to the layer with nuh_layer_id equal to nestingLayerIdList[ 0 ][ i ]. [Ed. (JB): How is nestingLayerIdList[ 0 ][ i ] determined?  It exists in HEVC, but not in HEVC. For VVC, is this supposed to be a reference to NestingLayerId[ i ] ? ]

...

## 8.21 Depth representation information SEI message

8.21.1 Depth representation information SEI message syntax

...

8.21.1 Depth representation information SEI message semantics

The syntax elements in the depth representation information (DRI) SEI message specify various parameters for auxiliary pictures of type AUX_DEPTH for the purpose of processing decoded primary and auxiliary pictures prior to rendering on a 3D display, such as view synthesis. Specifically, depth or disparity ranges for depth pictures are specified.

When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to 2 for at least one value of i, no picture in the CVS shall be associated with a DRI SEI message.

When an AU contains both an SDI SEI message with sdi_aux_id[ i ] equal to 2 for at least one value of i and a DRI SEI message, the SDI SEI message shall precede the DRI SEI message in decoding order.

When present, the depth representation information SEI message shall be associated with one or more layers with AuxId value equal to AUX_DEPTH that are indicated as depth auxiliary layers by an SDI	 SEI message. The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the depth representation information SEI message applies.

When present, the depth representation information SEI message may be included in any access unit. It is recommended that, when present, the SEI message is included for the purpose of random access in an access unit in which the coded picture with nuh_layer_id equal to targetLayerId is an IRAP picture.

For an auxiliary picture with AuxId[ targetLayerId ] equal to AUX_DEPTH, an associated primary picture, if any, is a picture in the same access unit having AuxId[ nuhLayerIdB ] equal to 0 such that ScalabilityId[ LayerIdxInVps[ targetLayerId ] ][ j ] is equal to ScalabilityId[ LayerIdxInVps[ nuhLayerIdB ] ][ j ] for all values of j in the range of 0 to 2, inclusive, and 4 to 15, inclusive.

The information indicated in the SEI message applies to all the pictures with nuh_layer_id equal to targetLayerId from the access unit containing the SEI message up to but excluding the next picture, in decoding order, associated with a depth representation information SEI message applicable to targetLayerId or to the end of the CLVS of the nuh_layer_id equal to targetLayerId, whichever is earlier in decoding order.

...

## 8.22 Alpha channel information SEI message

8.22.1 Alpha channel information SEI message syntax

...

8.22.1 Alpha channel information SEI message semantics

The alpha channel information (ACI) SEI message provides information about alpha channel sample values and post-processing applied to the decoded alpha planes coded in auxiliary pictures of type AUX_ALPHA and one or more associated primary pictures.

When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to 1 for at least one value of i, no picture in the CVS shall be associated with an ACI SEI message.

When an AU contains both an SDI SEI message with sdi_aux_id[ i ] equal to 1 for at least one value of i and an ACI SEI message, the SDI SEI message shall precede the ACI SEI message in decoding order.

When an access unit contains an auxiliary picture picA with nuh_layer_id equal to nuhLayerIdA and AuxId equal to AUX_ALPHA in a layer that is indicated as an alpha auxiliary layer by an SDI SEI message, the alpha channel sample values of picA persist in output order until one or more of the following conditions are true:

–	The next picture, in output order, with nuh_layer_id equal to nuhLayerIdA is output.

–	A CLVS containing the auxiliary picture picA ends.

–	The bitstream ends.

–	A CLVS of any associated primary layer of the auxiliary picture layer with nuh_layer_id equal to nuhLayerIdA ends.

[Ed. (YK): Check the consistency between these four items and the three items below in the semantics of alpha_channel_cancel_flag.]

The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the alpha channel information SEI message applies.

alpha_channel_cancel_flag equal to 1 indicates that the alpha channel information SEI message cancels the persistence of any previous alpha channel information SEI message in output order that applies to the current layer. alpha_channel_cancel_flag equal to 0 indicates that alpha channel information follows.

Let currPic be the picture that the alpha channel information SEI message is associated with. The semantics of alpha channel information SEI message persist for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB with nuh_layer_id equal to targetLayerId in an access unit containing an alpha channel information SEI message with nuh_layer_id equal to targetLayerId is output having PicOrderCnt( picB ) greater than PicOrderCnt( currPic ), where PicOrderCnt( picB ) and PicOrderCnt( currPic ) are the PicOrderCntVal values of picB and currPic, respectively, immediately after the invocation of the decoding process for picture order count for picB. A picture in the current layer in an AU associated with an alpha channel information SEI message is output that follows the current picture in output order.

...

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).