| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–30 Apr. 2021 | Document: JVET-V0074 |
| --- | --- |

| Title: | AHG11: Separate density attention network for loop filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Zhao Wang, Changyue Ma, Ru-Ling Liao, Yan Ye | Email: | baixiu.wz@alibaba-inc.com |
| Source: | Alibaba |  |  |

_____________________________

- Abstract

To achieve better trade-off between the model performance and the complexity, this contribution presents a separate density attention network (SDAN) for loop filtering. The SDAN has less than 1M parameters, a reduction of 70% compared to the multi-density network (MDN) presented in JVET-U0054. The experimental results show that our method can achieve 5.01%, 14.02% and 13.71% coding gain over VVC for Y, U and V components under RA configuration.

Introduction

In actual applications, complexity is the crucial factor of neural network applying to image/video compression and processing, in terms of not only computational time but also consumed hardware resources. Therefore, this contribution presents a novel SDAN to achieve better trade-off between the model performance and the complexity. In total, there are just 1.0 million parameters in this model and achieves 70% parameter reduction compared to MDN in JVET-U0054 [1].

Network

The overall architecture of the proposed network is shown in Figure 1. The network includes the main branch, one density branch and three attention branches, which are illustrated in Figure 2. The main branch consists of 26 resblocks and plays the key role of feature extraction and processing. The density branch consists of one down-sampling block, three resblocks and one up-sampling block, aiming to utilize multi-scale features in different sampling densities. With respect to the attention branch, spatial attention mechanism is adopted to reduce the complexity. Specifically, channel-wise down-sampling is first applied and then spatial-wise down-sampling is successively conducted. After reaching up the target smaller resolution, spatial-wise up-sampling is further applied and the sigmoid function is finally used to generate the weight map. The weight map is fused to the feature map of main branch and density branch to softly weight the features. More design principles and details can be referred to the paper [3].

In our implementation, 3x3 kernel size and 40 channels are used. There are about 30k parameters in each attention branch, 140k parameters in the density branch and 760k parameters in the main branch. In total, there are 0.998 million parameters in this model.

Figure 1. Illustration of the proposed separate density attention network.

Figure 2. Illustration of the main branch, density branch and attention branch

Training and inference

The training conditions specified in JVET-T2006 [4] are closely followed on the dataset BVI-DVC. Five models are trained according to the frame QP (0-31, 32-36, 37-41, 42-46, 47-max), respectively. During inference, the reconstructed frame to be fed into network will first be converted into YUV444 by copying the neighboring samples directly. For the output, only the top-left sample of each 2x2 chroma block is saved. The neural network based loop filtering locates between deblocking and SAO.

For the output frame of network, a frame-level scaling factor and CTU-level on/off flag will be further signaled. The scaling process is applied as:

P= PNN-Prec*factor+Prec

where PNN and Prec are the output and input of the network respectively.

The details of training and inference are shown in Table 1.

Table 1 Training information

| Training and inference information |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | 4x Telsa-V100-32G |
|  | CPU Type | Intel(R) Xeon(R) Platinum 8163 CPU @ 2.50GHz |
|  | Number of GPUs per Task | 4 (training), 1 (inference) |
|  | Framework: | Pytorch 1.2.0 |
|  | Iterations: | 0.64 million |
|  | Batch size: | 32 |
|  | Training time | 40 hours |
|  | Total Parameter Number | 1.0 M/model |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Training set | BVI-DVC |
| Optional | Patch size | 128x128x3 (training), frame (inference) |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Loss function: | L2 |

Simulation results

The common test conditions defined in JVET-T2006 are closely followed. The performance on VTM-10.0 under RA configurations are shown.

Table 2 Experimental results on VTM-10.0 without scaling

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -6.16% | -10.60% | -11.27% |  |  |
| Class A2 | -4.69% | -14.97% | -12.21% |  |  |
| Class B | -5.08% | -13.41% | -14.69% |  |  |
| Class C | -4.29% | -16.62% | -15.45% |  |  |
| Overall | -5.01% | -14.02% | -13.71% |  |  |
| Class D | -5.88% | -15.76% | -18.28% |  |  |

Table 3 Experimental results on VTM-10.0 with scaling

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |

Reference

- Z. Wang, R.-L. Liao, C.Y. Ma, Y. Ye (Alibaba), “AHG11: Multi-density network for in-loop filtering”, JVET-U0054, 21th Meeting: by teleconference, Jan 2021.

- E. Alshina, S. Liu, W. Chen, Y. Li, R.-L. Liao, Z. Ma and H. Wang, “Description of Exploration Experiments on NN-based video coding”, JVET-T2023, 20th Meeting: by teleconference, 7 – 16 October 2020.

- Z. Wang, C.Y. Ma, R.-L. Liao, Y. Ye, Multi-Density Convolutional Neural Network for In-Loop Filter in Video Coding, DCC 2021.

- S. Liu, A. Segall, E. Alshina, R.-L. Liao, “JVET common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-T2006, 20th Meeting: by teleconference, 7 – 16 October 2020.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).