| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0078 |
| --- | --- |

| Title: | AHG 10: QP control for very smooth blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, J. Enhorn, J. Ström | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

Large transforms are excellent in reducing spatial redundancy. However, when source samples are very smooth the subjective quality can drop very quickly in such areas with too coarse quantization. It is therefore suggested to reduce the QP according to a linear model for very smooth regions to prolong the breakdown of the subjective quality. Since smooth regions are relatively easy to code it is claimed that the overhead in bitrate can be manageable.

The impact on BDR has been tested on HDR:

VTM-12.0 HDR RA CTC (luma/Cb/Cr):              0.14%/0.14%/0.02%

HM-16.22 (TF on) HDR RA CTC (luma/Cb/Cr): 0.49%/-0.36%/-0.28%

It is asserted that the proposal can improve subjective quality of smooth regions for HDR and it is suggested to update VTM and HM with this functionality.

# Introduction

It has been observed that HM and VTM can drop in quality at relatively low QPs in very smooth regions due to imprecise coding of transform coefficients. This contribution aims to circumvent that by reducing the QP for smooth blocks.

# Proposal

Smooth blocks are detected by fitting luma source samples to a low order polynomial model in regions of 64x64 samples. Then the distortion from a prediction according to the model is computed (SAD) and compared with a threshold. When the distortion is below the threshold the luma QP is reduced according to a linear model. The granularity of the QP adjustment is 64x64.

Configurable parameters are added to enable control of amount of strength of the QP reduction and controlling which frames it is applied for (every, every second, every fourth, etc):

smoothQPoffset = max(m_pcEncCfg->getSmoothQPReductionLimit(),min(0, (int)(m_pcEncCfg->getSmoothQPReductionModelScale()*(double)baseQP+m_pcEncCfg->getSmoothQPReductionModelOffset())));

The setting used in this contribution are as follows:

m_pcEncCfg->getSmoothQPReductionLimit() = -16

m_pcEncCfg->getSmoothQPReductionModelScale()=-1

m_pcEncCfg->getSmoothQPReductionModelOffset()=27

m_pcEncCfg->getSmoothQPReductionFrames() = 1 (all frames)

m_pcEncCfg->getSmoothQPReductionThreshold() = 3 (times the number of samples)

# Results

The method is implemented both on top of HM-16.22 with temporal filtering turned on [1] and VTM-12.0 [2].

## Objective results

The method has been tested for HDR RA CTC [3]. It can be noted that there is a drop in BDR by the proposed method and that the drop is slightly larger for HM than for VTM. The encoding and decoding times are not reliable.

Table 1: VTM-12.0 for CTC HDR RA

|  | PSNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class H1 | 0,17% | 0,19% | 0,07% | 100% | 102% |
| Class H2 | 0,09% | 0,04% | -0,08% | 104% | 122% |
| Overall | 0,14% | 0,14% | 0,02% | 101% | 109% |

Table 2: HM-16.22 (TF on) for CTC HDR RA

|  | PSNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class H1 | 0,71% | -0,48% | -0,23% | 85% | 85% |
| Class H2 | 0,11% | -0,15% | -0,38% | 84% | 84% |
| Overall | 0,49% | -0,36% | -0,28% | 85% | 85% |

## Subjective results

Subjective video quality is best evaluated by playout on an HDR capable TV. To illustrate the subjective quality improvement by the proposed method we provide a snapshot from the 4K HDR PQ Netflix sequence SparksTruckPack for VTM-12.0 in Figure 1 and for HM-16.22 (TF on) in Figure 3. Even due to imperfect presentation in this document resulting in banding effects it is evident that the sky is enhanced by the proposed method on top of VTM in Figure 2 and on top of HM in Figure 4, especially to the left in the sky and above the middle spruce/pine tree. We also provide example for 4K HDR HLG for VTM-12.0 in Figure 5 and for HM-16.22 (TF on) in Figure 7. The proposed method on top of VTM is show in Figure 6 and on top of HM in Figure 8.

Figure 1: VTM-12.0: SparksTruckPack at QP 38 (483 kbps).

Figure 2: Proposed on top of VTM-12.0: SparksTruckPack at QP 38 (492 kbps).

Figure 3: HM-16.22 (TF on): SparksTruckPack at QP 35 (840 kbps).

Figure 4: Proposed on top of HM-16.22 (TF on): SparksTruckPack at QP 35 (862 kbps).

Figure 5: VTM-12.0: SunsetBeach at QP 37 (4071 kbps).

Figure 6: Proposed on top of VTM-12.0: SunsetBeach at QP 37 (4077 kbps).

Figure 7: HM-16.22 (TF on): SunsetBeach at QP 37 (4318 kbps).

Figure 8: Proposed on top of HM-16.22 (TF on): SunsetBeach at QP 37 (4329 kbps).

# Conclusions

It has been showed that by accepting a minor objective BDR loss by reducing the QP for smooth blocks the subjective quality can be improved in smooth areas. It is therefore suggested to include this functionality into both HM and VTM reference software.

# References

[1] HM-16.22, available at https://vcgit.hhi.fraunhofer.de/jvet/HM/-/releases/HM-16.22

[2] VTM-12.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-12.0

[3] A. Segall, E. François, W. Husak, S. Iwamura and D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-T2011, Meeting by Teleconference, October 2020.

# Patent rights declaration

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).