| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 April 2021 | Document: JVET-V0080 |
| --- | --- |

| Title: | AHG12: Extensions to CCALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Steve Keating Adrian Browne Karl Sharman | Email: | steve.keating@sony.com adrian.browne@sony.com karl.sharman@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This contribution proposes extensions to CCALF in both the size of the filters and number of filters available.  Experiments have shown that using a different filter shape for Type 0 and Type 2 chroma can give some further improvement in coding efficiency.  BD-rate of the proposed Type 0 filter is 0.09%, -1.65%, -1.75% (AI) when applied over JVET-U0100 (JVET-M1010 CTC).

# Introduction

At the previous meeting, JVET-U0100 proposed various tool modifications to improve coding efficiency including extensions to ALF.  It is proposed that extensions to CCALF should also be considered.

# Number of Filters

Currently CCALF has a choice of 4 filters per colour component, increasing this to a choice of 8 gives a BD-rate of  0.05%, -0.69%, -0.61% over JVET-U0100 (JVET-M1010 CTC).

# Filter Size and Shape

Increasing the number of filter taps from 7 to 11 can further improve the gain to  0.09%, -1.65%, -1.75% over JVET-U0100 (JVET-M1010 CTC).  The 7-tap and 11-tap filter shapes are shown below:

|  |  | T0 |  |  |
| --- | --- | --- | --- | --- |
|  | T1 | C | T2 |  |
|  | T3 | T4 | T5 |  |
|  |  | T6 |  |  |

7-tap Filter

|  |  | T0 |  |  |
| --- | --- | --- | --- | --- |
| T7 | T1 | C | T2 | T8 |
| T9 | T3 | T4 | T5 | T10 |
|  |  | T6 |  |  |

11-tap Filter

It can be seen that this 11-tap arrangement should be well suited to type 0 chroma but may not be ideal for type 2 chroma.  An alternative 11-tap filter shape is proposed for type 2 chroma content:

|  | T9 | T0 | T10 |  |
| --- | --- | --- | --- | --- |
| T7 | T1 | C | T2 | T8 |
|  | T3 | T4 | T5 |  |
|  |  | T6 |  |  |

11-tap Filter (Type 2 chroma)

The two 11-tap filters were tested with the M1010 CTC, the U2018-STDQP CTC and with 4:2:0 Type 0 and Type 2 content converted from 4:4:4 source sequences.  Only 4:2:0 content has been tested but for 4:2:2 and 4:4:4 content the Type 2 filter might be expected to perform better than the Type 0 filter.

# Results

All results below use JVET-U0100 as anchor.

## JVET-M1010 CTC (AI)

|  | 8sets Over U0100 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A1 | 0.04% | -0.43% | -0.55% |
| Class A2 | 0.05% | -0.65% | -1.76% |
| Class B | 0.06% | -1.54% | -0.91% |
| Class C | 0.05% | -0.18% | -0.17% |
| Class E | 0.03% | -0.26% | 0.42% |
| Overall | 0.05% | -0.69% | -0.61% |
| Class D | 0.01% | -0.14% | -0.08% |
| Class F | 0.04% | -0.34% | -0.30% |
| Class TGM | 0.02% | -0.15% | -0.18% |

|  | 11tapType0 8sets Over U0100 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A1 | 0.07% | -0.65% | -1.98% |
| Class A2 | 0.06% | -1.73% | -2.89% |
| Class B | 0.09% | -2.77% | -2.38% |
| Class C | 0.11% | -0.75% | -0.66% |
| Class E | 0.13% | -1.90% | -0.78% |
| Overall | 0.09% | -1.65% | -1.75% |
| Class D | 0.02% | -0.76% | -0.31% |
| Class F | 0.10% | -1.20% | -0.96% |
| Class TGM | 0.06% | -0.97% | -0.61% |

|  | 11tapType2 8sets Over U0100 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A1 | 0.07% | -0.62% | -2.05% |
| Class A2 | 0.06% | -1.75% | -3.02% |
| Class B | 0.09% | -2.70% | -2.26% |
| Class C | 0.11% | -0.70% | -0.60% |
| Class E | 0.13% | -1.50% | -0.74% |
| Overall | 0.09% | -1.55% | -1.73% |
| Class D | 0.02% | -0.69% | -0.20% |
| Class F | 0.11% | -1.26% | -1.00% |
| Class TGM | 0.06% | -1.13% | -0.82% |

## JEVT-U2018-STDQP CTC (AI)

|  | 8sets over U0100 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | wPSNR |  |  | PSNR |  |  |
|  | Y | U | V | Y | U | V |
| Class H1 | 0.06% | -0.98% | -2.66% | 0.06% | -0.97% | -2.57% |
| Class H2 |  |  |  | 0.04% | -1.39% | -0.61% |
| Overall | 0.06% | -0.98% | -2.66% | 0.05% | -1.18% | -1.59% |

|  | 11tapType0 8sets over U0100 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | wPSNR |  |  | PSNR |  |  |
|  | Y | U | V | Y | U | V |
| Class H1 | 0.10% | -1.24% | -3.19% | 0.10% | -1.34% | -3.06% |
| Class H2 |  |  |  | 0.06% | -1.79% | -0.94% |
| Overall | 0.10% | -1.24% | -3.19% | 0.08% | -1.57% | -2.00% |

|  | 11tapType2 8sets over U0100 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | wPSNR |  |  | PSNR |  |  |
|  | Y | U | V | Y | U | V |
| Class H1 | 0.10% | -1.42% | -3.67% | 0.10% | -1.51% | -3.55% |
| Class H2 |  |  |  | 0.06% | -1.82% | -0.89% |
| Overall | 0.10% | -1.42% | -3.67% | 0.08% | -1.66% | -2.22% |

## Direct Type 0 vs. Type 2 comparison (AI)

The sequences BalloonFestival, EBU_04_Hurdles, FireEater, Market3 and EBU_06_Starting were converted from 4:4:4 original to 4:2:0 Type 0 chroma and Type 2 chroma using HDRTools with OutputChromaLocationTop set appropriately.  This gives a direct comparison of the two filter shapes with Type 0 and Type 2 content.

|  | 11tapType0 8sets over U0100 |  |  |
| --- | --- | --- | --- |
|  | PSNR |  |  |
|  | Y | U | V |
| Type0 | 0.06% | -0.64% | -0.80% |
| Type2 | 0.06% | -0.55% | -1.12% |

|  | 11tapType2 8sets over U0100 |  |  |
| --- | --- | --- | --- |
|  | PSNR |  |  |
|  | Y | U | V |
| Type0 | 0.06% | -0.66% | -0.80% |
| Type2 | 0.06% | -0.64% | -1.35% |

# Conclusion

It is proposed to extend CCALF by both increasing the maximum number of filters and increasing the number of filter taps.  Experiments have shown that using a different filter shape for Type 0 and Type 2 chroma can give some further improvement in coding efficiency.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] Y-J. Chang, C-C. Chen, J. Chen, J. Dong, H. Egilmez, N. Hu, H. Huang, M. Karczewicz, J. Li, B. Ray, K. Reuze, V. Seregin, N. Shlyakhov, L. Pham Van, H. Wang, Y. Zhang, Z. Zhang, “Compression efficiency methods beyond VVC”, JVET-U0100, by teleconference, 6–15 Jan 2021