| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0081 |
| --- | --- |

| Title: | AHG9: On the decoder initialization information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry, SangHeon Lee, Seung Hwan Kim LG Electronics Ye-Kui Wang, Kai Zhang, Li Zhang, Yang Wang, Jizheng Xu, Zhipin Deng Bytedance Inc. | Email: | dr.hendry@lge.com sangheon1.lee@lge.com seunghwan3.kim@lge.com yekui.wang@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com wangyang.cs@bytedance.com xujizheng@bytedance.com zhipin.deng@bytedance.com |
| Source: | LG Electronics and Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposed modifications to the decoder initialization information that was proposed by JVET-U0083 at the 21st JVET meeting. It was asserted that the proposed signalling in JVET-U0083 helps for decoder to avoid re-initialization; however, it might not be complete.

The proposed modification for the previously proposed decoder initialization information is as follows:

- Instead of just signalling one set of decoder initialization information, which comprises of maximum values of parameters (i.e., max DPB size, max picture size, max bit depth, and max chroma format), allow signalling multiple sets of decoder initialization information.

- For each set of decoder initialization information, include the level information.

The proposed decoder initialization information may be signalled as an extension to the DCI NAL unit or as a new SEI message as suggested by JVET-U0083, although the authors of this contribution prefer that the signalling is conveyed in a new SEI message.

# Introduction

Contribution JVET-U0083 described an issue related to decoder re-initialization that may take relatively long time and disrupt playback continuity during a streaming session. Re-initialization may be needed when the streaming client switches to a different representation of the same content with a different spatial resolution or fidelity quality, for adapting to the changing network conditions, e.g., bandwidth. It was quoted that although at best decoder initialization or re-initialization can be done in just 16 ms, typically, it may take between 100 to 200 ms depending on various factors in the streaming session including memory allocation time, whether the session is secure / non-secure one, mapping time, memory fragmentation of double data rate (DDR), internal fragmentation of system memory management unit (SMMU), etc. Such long time for re-initialization in typical cases may make it difficult to achieve a smooth playback.

To overcome the asserted issue, it was proposed to signal decoder initialization information which can be used by decoder during initialization process. The idea is that the decoder initialization information provides the maximum memory required to decode the given bitstreams so that decoder can initialize its memory allocation according to this maximum requirement (i.e., max number of picture buffer in DPB, max picture size, max bit depth, and max chroma format). Provided this information, it was asserted that when the stream changes (e.g., changing of representation in DASH session) there is no need for decoder to invoke re-initialization since the memory initialization / allocation that was done earlier would still be sufficient to handle the new stream / representation.

Figure 1 illustrates how the proposed solution can help decoder invoking re-initialization process at switching point. For the given content, three representations were made available. Each representation has the same DPB size, bit depth, and chroma format; however, the picture size and required level is different. Decoder initialization information would show the maximum values of those parameters, i.e., max DPB 8, max picture width 3860, max picture height 1920, max bit depth 10, and max chroma format 1 (i.e., 420). Given the information, in the beginning of the streaming, decoder would be initialized so that it has DPB size of 8 of which each picture buffer can store 3860x1920 420 @10 bits picture. With this initialization, decoder would have enough memory to handle coded picture from any of the three representations.

Figure 1 – Example how decoder initialization info may help in decoder initialization process.

# Problem descriptions

## Problem 1

The decoder initialization information proposed by JVET-U0083 is only useful for decoder that can afford decoding up to the highest representation for the streaming session. Decoder with less capability (e.g., a level 4.1 decoder) cannot use the information.

Figure 2 illustrates the asserted problem. Given the same content for the streaming session, a level 4.1 decoder can handle the content from the first two representations; however, it would not be able to take advantage of the decoder initialization information as this information is the maximum parameters based on all the representations.

Figure 2 – Decoder conforming to lower level cannot use proposed signalling in JVET-U0083

## Problem 2

When only the maximum values is provided for each parameter in decoder initialization information, the total memory requirement may exceed what a particular level allowed which consequently makes the information not useful for decoder. Figure 3 illustrates an example of this problem. Instead of having the HD resolution stream to have level 3.1 with DPB size 8, encoder decides to encode the representation with more reference pictures (i.e., needs DPB size of 16) so that the stream is of level 4.1, instead of level 3.1. Such change affects the max DPB in decoder initialization information such that now its max DPB is 16 instead of 8. Note that while a level 5.1 decoder can handle the content up to the representation with the largest resolution, it cannot be initialized according to the information from the decoder initialization info since level 5.1 does not support DPB size 16 for which each picture buffer is capable of storing 3860x1920 10 bits 420 picture.

Figure 3 – The proposed signalling in JVET-U0083 resulted in requirement 
that is higher than required by the given level.

# Proposed solutions

## General solution

To overcome the asserted problems, the following modification to the solution proposed in JVET-U0083 is proposed:

- Instead of just signalling one set of decoder initialization information which comprises of maximum values of parameters (i.e., max DPB size, max picture size, max bit depth, and max chroma format), allow signalling multiple sets of decoder initialization information.

- For each set of decoder initialization information, include the level information.

By having multiple sets of decoder initialization information, the problems described above can be solved. For problem 1, content provider (or encoder) may provide multiple sets of decoder initialization information, i.e., one set of each level of decoders that are anticipated to handle the bitstream. For problem 2 as illustrated in Figure 3, the content provider / encoder may include a set of decoder initialization information with Level 6.1. This way, although the highest level that the bitstream conforms to is Level 5.1, decoder that conforms to Level 6.1 may be able to decoder the bitstream with doing decoder initialization once at the beginning and never have to worry about re-initialization later.

The information can be signalled in an extension to the DCI NAL unit or in an SEI message as shown in option 1 and option 2 below. The text is implemented on top of the text provided by JVET-U0083 in the next sub sections.

## Option 1 – Signalling in DCI parameter set extension

##### 7.3.2.1.1 DCI extension syntax

| dci_extension( ) { | Descriptor |
| --- | --- |
| dci_num_initialization_set_minus1 | u(4) |
| for( i = 0; i <= dci_num_initialization_set_minus1; i++ ) |  |
| dci_level_idc[ i ] | u(8) |
| dci_max_num_dec_pics_in_dpb_minus1[ i ] | u(4) |
| dci_max_bitdepth_minus8[ i ] | u(4) |
| dci_max_pic_width[ i ] | u(16) |
| dci_max_pic_height[ i ] | u(16) |
| dci_max_chroma_format[ i ] | u(2) |
| dci_ext_reserved_zero_6bits[ i ] | u(6) |
| } |  |
| } |  |

##### 7.4.3.1.1 DCI extension semantics

NOTE – When the dci_extension( ) syntax structure is present for a bitstream and the decoder is intialized according to the information associated with one of the initialization set provided in the DCI NAL unit, it is expected that no decoder reinitialization is needed for decoding CVS with general_level_idc equal to or less than the level associated with the initialization set.

dci_num_initialization_set_minus1 plus 1 specifies the number of initialization set that are present in the DCI extension.

dci_level_idc[ i ] specifies the level associated with the i-th initialization set.

dci_max_num_dec_pics_in_dpb_minus1[ i ] plus 1 specifies the maximum number of decoded pictures needed in the DPB at any moment when decoding pictures in the bitstream conforming to the level that is equal. to or less than dci_level_idc[ i ].

dci_max_bitdepth_minus8[ i ] plus 8 indicates the maximum bit depth, in units of bits, of each colour component of a decoded picture corresponds to pictures in the bitstream conforming to the level that is equal. to or less than dci_level_idc[ i ]. The value of dci_max_bitdepth_minus8[ i ] shall be in the range of 0 to 8, inclusive.

dci_max_pic_width[ i ] specifies the maximum width, in units of luma samples, of a decoded picture corresponds to pictures in the bitstream conforming to the level that is equal. to or less than dci_level_idc[ i ].

dci_max_pic_height[ i ] specifies the maximum height, in units of luma samples, of a decoded picture corresponds to pictures in the bitstream conforming to the level that is equal. to or less than dci_level_idc[ i ].

dci_max_chroma_format[ i ] indicates the maximum value of sps_chroma_format_idc of the SPS referenced by a picture corresponds to pictures in the bitstream conforming to the level that is equal. to or less than dci_level_idc[ i ].

dci_ext_reserved_zero_6bits[ i ] shall be equal to 0 in bitstreams conforming to this version of this Specification. The values greater than 0 for dci_ext_reserved_zero_6bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall allow values greater than 0 for dci_ext_reserved_zero_6bits[ i ] to appear in the bitstream and shall ignore the value of dci_ext_reserved_zero_6bits[ i ].

## Option 2 – Signalling in decoder initialization information SEI message

#### 8.x Decoder initialization information SEI message

##### 8.x.1 Decoder initialization information SEI message syntax

| decoding_init_info( payloadSize ) { | Descriptor |
| --- | --- |
| dii_num_initialization_set_minus1 | u(4) |
| for( i = 0; i <= dii_num_initialization_set_minus1; i++ ) |  |
| dii_level_idc[ i ] | u(8) |
| dii_max_num_dec_pics_in_dpb_minus1[ i ] | u(4) |
| dii_max_bitdepth_minus8[ i ] | u(4) |
| dii_max_pic_width[ i ] | u(16) |
| dii_max_pic_height[ i ] | u(16) |
| dii_max_chroma_format[ i ] | u(2) |
| dci_ext_reserved_zero_6bits[ i ] | u(6) |
| } |  |
| } |  |

##### 8.x.2 Decoding initialization information SEI message semantics

When associated with an AU, the decoder initialization information (DII) SEI message provides information that applies to the entire bitstream. A DII SEI message could be made available to the decoder, through either being present in the bitstream, included in at least the first AU of the bitstream, or provided through external means.

When present, all DII SEI messages in a bitstream shall have the same payload content.

When any two CVSs in the bistream are indicated to conform to different profiles, there shall be no DII SEI message associated with an AU of the bitstream.

NOTE – When the DII SEI message associated with an AU of a bitstream and the decoder may be intialized according to the profile and one of the initialization information set provided in the DII SEI message, it is expected that no decoder reinitialization is needed for decoding the CVS within the bitstream with level that is equal to or less than the level associated with the initialization information set that is used.

dii_num_initialization_set_minus1 plus 1 specifies the number of initialization set that are present in the SEI message.

dii_max_level_idc[ i ] specifies the level associated with the i-th initialization set..

dii_max_num_dec_pics_in_dpb_minus1[ i ] plus 1 specifies the maximum number of decoded pictures needed in the DPB at any moment when decoding pictures in the bitstream conforming to the level that is equal. to or less than dii_level_idc[ i ].

dii_max_bitdepth_minus8[ i ] plus 8 indicates the maximum bit depth, in units of bits, of each colour component of a decoded picture corresponds to pictures in the bitstream conforming to the level that is equal. to or less than dii_level_idc[ i ]. The value of dci_max_bitdepth_minus8[ i ] shall be in the range of 0 to 8, inclusive.

dii_max_pic_width[ i ] specifies the maximum width, in units of luma samples, of a decoded picture corresponds to pictures in the bitstream conforming to the level that is equal. to or less than dii_level_idc[ i ].

dii_max_pic_height[ i ] specifies the maximum height, in units of luma samples, of a decoded picture corresponds to pictures in the bitstream conforming to the level that is equal. to or less than dii_level_idc[ i ].

dii_max_chroma_format[ i ] indicates the maximum value of sps_chroma_format_idc of the SPS referenced by a picture corresponds to pictures in the bitstream conforming to the level that is equal. to or less than dci_level_iic[ i ].

dii_ext_reserved_zero_6bits[ i ] shall be equal to 0 in bitstreams conforming to this version of this Specification. The values greater than 0 for dci_ext_reserved_zero_6bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall allow values greater than 0 for dii_ext_reserved_zero_6bits[ i ] to appear in the bitstream and shall ignore the value of dii_ext_reserved_zero_6bits[ i ].

# Patent rights declaration(s)

LG Electronics and Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).