| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0090 |
| --- | --- |

| Title: | AHG11: Neural network based temporal processing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi, Zeqiang Li, Wei Wang, Wei Jiang, Xiaozhong Xu, Shan Liu | Email: | {bdchoi, zeqiangli} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This informative proposal reports the test results of a neural network (NN) based temporal filtering for both detail enhancement and inter-prediction. The proposed NN processing is composed of two steps, NN-based reconstruction process and NN-based prediction process. Both processes has the same NN model utilizing temporal features, but the parameters of each model was individually trained for improving the quality of the reconstruction or minimizing error of the motion compensated prediction. The test results show coding gains 4.69% and 6,87%  respectively, for classes C & D in luma for RA.

# Introduction

JVET-T0058 [1] and JVET-U0087 [2] reported the results of virtual reference picture generation with a neural network (NN) temporal processing for better motion-compensated prediction. Even if those proposal shows promising results, there were some concerns about increment of the number of reference pictures. In order to address the concerns, we have researched on how to apply NN-based temporal processing without inserting a generated reference picture. As one of our successive research results, this informative report introduces a combination of neural network (NN) based temporal processes The proposed NN-based temporal processing updates the reconstructed picture for improvement of the details and the prediction accuracy. It consists of two modules; the first module filters the decoded picture before output, and the second module updates the reference picture. The details are introduced in the following sections.

# Framework of temporal filtering with neural networks

Figure 1. Proposed neural network based temporal filtering.

The NN-based temporal processing consists of two stages; NN-based reconstruction processing and NN-based prediction processing. The first NN-based reconstruction filter is processed right after deblocking process to improve the details of the decoded picture for better visual quality of output picture.  A forward reference picture and a backward one are utilized for the network inference process. After the reconstructed picture is stored in decoded picture buffer (DPB), the second NN-based filter modifies to the reference pixel values for better prediction of the following pictures in decoding order.

Both NN-based reconstruction processing and prediction processing employ the same network model, but with differently trained parameters. The network model consists of two parts, PCD (Pyramid, Cascading and Deformable) alignment module for spatial and temporal optimization and TSA (Temporal and Spatial Attention) fusion module to apply attention to emphasize important features for subsequent restoration. The NN-based reconstruction filter has been trained by minimizing the error between the reconstruction pixel values and the original pixel values of the current picture. Whereas the prediction filter has been trained to minimize the error between the reference pixel values and the pixel values of the following pictures in decoding order. Figure 2 illustrates the comparison of two NN inference processes.

(a)                                                                                (b)

Figure 2. Proposed neural network model structures; (a) NN-based reconstruction filter (b) NN-based prediction filter.

As for details of the model, during PCD module, all three frames are fed as input, to first generate the aligned features, then to calculate the pixels offset map of deformable convolution for center frame, which provides the new position of DNN convolution kernel. During TSA module, temporal attention is to compute frame similarity in an embedding space. Intuitively, in an embedding space, a neighboring frame that is more similar to the reference one, should be paid more attention, and the spatial attention to emphasize important features for subsequent restoration. When appropriate neighboring reference pictures are not available in decoded picture buffer, alternatively the image-based detail enhancement model is applied. In that case, the residual block is modified to use even smaller kernel size (1 and 3), and weight normalization layer is also applied to speed up the convergence and improve robustness of model training.

# Simulation results

Simulations were conducted based on [3]. The results of classes A&B will be updated. The en/decoding times in the below table are not reliable at all, because the cloud servers for tests were congested irregularly.

Table 1. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | Device Type | CPU: Intel(R) Xeon(R) CPU E5-2698 v4 @ 2.20GHz GPU: Tesla V100-DGXS-32GB |
|  | Framework: | Pytorch 1.15.1 + CUDA 10.1 + cuDNN 7.6.4 |
|  | Number of GPUs per Task | 4 |
|  | Epoch: | 60 |
|  | Batch size: | 4 |
|  | Training time: | 6 days |
|  | Training data information: | BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | Same as VTM CTC QP=22, 27, 32, 37, 42 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 480x480, 64*64 |
|  | Loss Functions | L1Loss |
|  | Optimizer: | ADAM |
|  | Other information: |  |

Table 2. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
|  | Device Type | CPU: Intel Core i9-9900x CPU @ 3.60GHz, 32GB Memory |
|  | Framework: | Pytorch 1.15.1 + CUDA 10.1 + cuDNN 7.6.4 |
|  | Number of GPUs per Task | 0 |
|  | Number of model | 5 x (Image-based model) + 5 x (video-based model) |
|  | - | - |
|  | Memory Parameter (MB) | 5 x 80.9MB |
|  | MAC (Giga) | 2.03e-3 Giga/pixel |
| Optional |  |  |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Other information: | - |

Table 3. BD-rate and Coding Time for NN-based Video Coding Tool Testing in Inference Stage 
(Tested 33frames with GOP=32 and patch size 256x256)

|  | Random Access Over VTM-10.0 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | Encoding Time (CPU) | Decoding Time (CPU) | Encoding Time (GPU) | Decoding Time (GPU) |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -4.69% | -8.07% | -6.54% | 3804.11% | 4466192.03% |  |  |
| Class D | -6.87% | -5.82% | -6.09% | 3611.28% | 3361108.43% |  |  |

# Discussion and future works

The proposed NN-based temporal processing show some promising results, even though its complexity is high. This proposal aims to inspire people to study the possibilities of NN-based temporal processing that utilizes the temporal features. We will further study on how much additional gains can be achieved by utilizing NN-based temporal processing, in addition to NN-based spatial processing like inloop filtering in EE.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] B. Choi, Z. Li, W. Wang, W. Jiang, X. Xu, S. Liu, “AHG11: Information on inter-prediction coding tool with deep neural network”, ISO/IEC JTC1/SC29 JVET-T0058.

[2] Z. Li, B. Choi, W. Wang, W. Jiang, X. Xu, S. Liu, “AHG11: Updated information on inter-prediction coding tool with deep neural network.

[3] S. Liu, A. Segall, E. Alshina, R.-L. Liao, “Common Test Conditions and evaluation procedures for neural network-based video coding technology”, ISO/IEC JTC1/SC29 JVET-U2016.