| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0105 |
| --- | --- |

| Title: | AHG11: neural network-based intra prediction: updated signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | T.Dumas, F.Galpin, P.Bordes P.Nikitin | Email: | thierry.dumas@interdigital.com philippe.bordes@interdigital.com franck.galpin@interdigital.com pavel.nikitin@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution suggests several updates of the signaling of the single additional neural network-based intra prediction mode in VVC introduced in “AHG11: neural network-based intra prediction with transform selection in VVC” [1]. Moreover, it compares the trade-off between rate-distortion performance and encoder/decoder running times variations when the neural networks run inside VTM-11.0 using two different C++ frameworks and with/without neural network quantization.

For VTM-11.0 including the single additional neural network-based intra prediction mode, the parameters of its neural networks being quantized on 16-bit, -3.46%, -3.21%, -3.29% and -1.69%, -0.71%, -1.01% of mean BD-rate reduction are reported in AI and RA for Y, Cb, Cr respectively. The average encoder and decoder running times for AI and RA are 390%, 3560% and 168%, 551% respectively.

# Updated signaling

This section describes the two main updates of the signaling of the single additional neural network-based intra prediction mode in VVC.

## Derivation of the list of MPMs

As the single additional neural network-based intra prediction mode acts as a generic intra prediction mode, inside VVC, it overshadows PLANAR in terms of frequency of selection, see Figure 1 and Figure 2. Therefore, according to the principle of entropy coding, the priority of PLANAR in the derivation of the list of MPMs should be decreased, see Algorithm 1.

Figure 1: frequency ν8|8 of selection of each intra prediction for 8×8 blocks when the images in the Holidays dataset converted into luminance are encoded via VTM-11.0 at QP=22. The yellow bar cumulates the frequencies of selection of the MIP modes. The leftmost blue bar is associated to PLANAR. The rightmost blue bar is associated to the intra prediction mode of index 66.

Figure 2: frequency ν8|8 of selection of each intra prediction for 8×8 blocks when the images in the Holidays dataset converted into luminance are encoded via VTM-11.0 including the single additional neural network-based mode at QP=22. The yellow bar cumulates the frequencies of selection of the MIP modes. The leftmost blue bar is associated to PLANAR. The rightmost blue bar is associated to the intra prediction mode of index 66. The red bar is associated to the single additional neural network-based mode.

Algorithm 1: derivation of the list of MPMs in VTM-11.0 with the single additional neural network-based mode. NN_IDX denotes the index of the single additional neural network-based mode.

L ≡ intra mode of the left CU whose index belongs to 0, 66 ∪ {NN_IDX}

A ≡ intra mode of the above CU whose index value belongs to 0, 66 ∪ {NN_IDX}

offset = 61

mod = 64

Initialization of the MPM list:

MPM[0] = PLANAR_IDX

MPM[1] = DC_IDX

MPM[2] = VER_IDX

MPM[3] = HOR_IDX

MPM[4] = VER_IDX – 4

MPM[5] = VER_IDX + 4

if (L == A)

if ((L > DC_IDX) && (L != NN_IDX))

if(multiRefIdx > 0)

MPM[0] = PLANAR_IDX

MPM[1] = L

MPM[2] = ((L + offset) % mod) + 2

MPM[3] = ((L – 1) % mod) + 2

MPM[4] = ((L + offset – 1) % mod) + 2

MPM[5] = (L % mod) + 2

else

MPM[0] = L

MPM[1] = ((L + offset) % mod) + 2

MPM[2] = ((L – 1) % mod) + 2

MPM[3] = PLANAR_IDX

MPM[4] = ((L + offset – 1) % mod) + 2

MPM[5] = (L % mod) + 2

else

use initialized values

else

if ((L == NN_IDX) || (A == NN_IDX))

if((L == NN_IDX) && (A > DC_IDX))

if(multiRefIdx > 0)

MPM[0] = PLANAR_IDX

MPM[1] = A

MPM[2] = ((A + offset) % mod) + 2

MPM[3] = ((A – 1) % mod) + 2

MPM[4] = ((A + offset – 1) % mod) + 2

MPM[5] = (A % mod) + 2

else

MPM[0] = A

MPM[1] = ((A + offset) % mod) + 2

MPM[2] = ((A – 1) % mod) + 2

MPM[3] = PLANAR_IDX

MPM[4] = ((A + offset – 1) % mod) + 2

MPM[5] = (A % mod) + 2

else if ((A == NN_IDX) && (L > DC_IDX))

if(multiRefIdx > 0)

MPM[0] = PLANAR_IDX

MPM[1] = L

MPM[2] = ((L + offset) % mod) + 2

MPM[3] = ((L – 1) % mod) + 2

MPM[4] = ((L + offset – 1) % mod) + 2

MPM[5] = (L % mod) + 2

else

MPM[0] = L

MPM[1] = ((L + offset) % mod) + 2

MPM[2] = ((L – 1) % mod) + 2

MPM[3] = PLANAR_IDX

MPM[4] = ((L + offset – 1) % mod) + 2

MPM[5] = (L % mod) + 2

else

use initialized values

else

if ((L > DC_IDX) && (A > DC_IDX))

if(multiRefIdx > 0)

MPM[0] = PLANAR_IDX

MPM[1] = L

MPM[2] = A

else

MPM[0] = L

MPM[1] = A

MPM[2] = PLANAR_IDX

MPM[3] =     ((min(L, A) + offset) % mod) + 2 if  *

((min(L, A) – 1) % mod) + 2 if **

((min(L, A) – 1) % mod) + 2 if ***

((min(L, A) + offset) % mod) + 2 if ****

MPM[4] =     ((max(L, A) – 1) % mod) + 2 if *

((max(L, A) + offset) % mod) + 2 if **

((min(L, A) + offset) % mod) + 2 if ***

((min(L, A) – 1) % mod) + 22 if ****

MPM[5] =     ((min(L, A) + offset – 1) % mod) + 2 if *

(min(L, A) % mod) + 2 if **

((max(L, A) – 1) % mod) + 2 if ***

((max(L, A) + offset) % mod) + 2 if ****

else if (L + A >= 2)

if(multiRefIdx > 0)

MPM[0] = PLANAR_IDX

MPM[1] = max(L, A)

MPM[2] = ((max(L, A) + offset) % mod) + 2

MPM[3] = ((max(L,A) – 1) % mod) + 2

MPM[4] = ((max(L, A) + offset – 1) % mod) + 2

MPM[5] = (max(L, A) % mod) + 2

else

MPM[0] = max(L, A)

MPM[1] = ((max(L, A) + offset) % mod) + 2

MPM[2] = ((max(L,A) – 1) % mod) + 2

MPM[3] = PLANAR_IDX

MPM[4] = ((max(L, A) + offset – 1) % mod) + 2

MPM[5] = (max(L, A) % mod) + 2

else

use initialized values

with

* max(L, A) – min(L, A) == 1

** max(L, A) – min(L, A) >= 62

*** max(L, A) – min(L, A) == 2

**** all other cases

## Signaling of LFNST for a chrominance CB predicted by CCLM

In VVC, for a CU in a separate tree, for the current chrominance CBs, if CCLM is selected to predict the current chrominance CBs and lfnstIdx∈1, 2, the implicit signaling of LFNST uses the index of the intra prediction mode selected to predict the luminance CB that is collocated with the chrominance CBs. In the same vein, in VVC including the single additional neural network-based intra prediction mode, for a CU in separate tree, for the current chrominance CBs, if CCLM is selected to predict the current chrominance CBs, lfnstIdx∈1, 2, and the collocated luminance CB is predicted by the single additional neural network-based mode, the implicit signaling of LFNST uses the LFNST group indices returned by the neural network-based mode predicting the collocated luminance CB, see Table 1 and Figure 3.

Table 1: definition of grpIdxi, i∈1, 2, for the residue of the neural network prediction.

| grpIdxlfnstIdx, lfnstIdx∈1, 2 | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| transposition of the primary transform coefficients resulting from the application of the DCT-2 horizontally and the DCT-2 vertically to the residue of the neural network prediction | no | no | no | no | yes | yes | yes |
| LFNST kernel index | 0 | 1 | 2 | 3 | 3 | 2 | 1 |

Figure 3: implicit signaling of LFNST for the current chrominance CBs in the case of a CU in separate tree in VVC with the single additional neural network-based mode. NN_IDX denotes the index of the single additional neural network-based mode. idxModeLuma denotes the index of the intra prediction mode selected to predict the luminance CB collocated with the current chrominance CBs.

# Information on the neural networks in the experiments

| Network information in training stage |  |  |
| --- | --- | --- |
| Mandatory | GPU type | GPU: Tesla-P100-16GB |
|  | Framework | Tensorflow 1.9.0 |
|  | Number of GPUs per task | 1 |
|  | Epoch | 15 |
|  | Batch size | 100 |
|  | Training time | 8 hours |
|  | Training data information | The set of images used to create the training sets combines frames extracted from the BVI-DVC dataset and the RGB images in the CLIC2020 training set for low-rate image compression, converted into YCbCr. |
|  | Training configurations for generating compressed training data | VTM-8.0; QP {22, 27, 32, 37} |
|  | Patch size | The neural network predicting w×h blocks is trained on pairs of a w×h block and its context. |
|  | Learning rate | 1e-4 |
|  | Optimizer | ADAM |
|  | Loss function | Mean-squared error |
|  | Preprocessing | See [1] |

The single additional neural network-based intra prediction mode contains 8 neural networks, each predicting blocks of a different size in 4×4, 8×4, 16×4, 32×4, 8×8, 16×8, 16×16, 32×32. The neural network predicting blocks of size w×h is denoted fh, w . , θh,w where θh,w gathers its parameters, see [1].

| Network information in the inference stage |  |
| --- | --- |
| Parameter number | θ4,4 contains 1574430 parameters. θ4,8 contains 1632046 parameters. θ4,16 contains 1747278 parameters. θ4,32 contains 1977742 parameters. θ8,8 contains 1920078 parameters. θ8,16 contains 2150542 parameters. θ16,16 contains1256282 parameters. θ32,32 contains 3904782 parameters. |
| Parameter precision | 32-bit float for Test-A and 16-bit signed integer for Test-B. |
| MAC per pixel | 1.6 Mmac: 4×4 block. 1.6 Mmac: 8×4 block. 1.7 Mmac: 16×4 block. 2.0 Mmac: 32×4 block. 1.9 Mmac: 8×8 block. 2.1 Mmac: 16×8 block. 4.3 Mmax: 16×16 block. 11.5 Mmax: 32×32 block. |
| Total conv. layers | See Figure 3, Table 2, Table 3, Table 5, Table 6 in [1] |
| Total FC layers | See Table 1, Table 4, Table 7 in [1] |
| Mem.T (MB) | 1.44 MB (1200×1200) |
| GPU Type | No GPU |
| CPU Type | Xeon 2.4GHz |
| Framework | C++ with a single thread in both Test-1 and Test-2 |

# Trade-off between rate-distortion performance and running times variations

## Comparison of two C++ frameworks for running the neural networks

In Test-1, the Tensorflow C++ API version 1.9.0 runs the neural networks of the single additional neural network-based intra prediction mode inside VTM-11.0. In Test-2, a custom C++ library runs these neural networks. The custom framework implements some SIMD optimization for some layers. In Test-1 and Test-2, the neural network parameters and the neural network representations are in 32-bit floats. In Test-1 and Test-2, a single thread and AVX2 are used. Finally, in Test-1 and Test-2, the single additional neural network-based intra prediction mode predicts the LFNST group indices, see [1].

The conditions of the experiments follow [2]. In Table 2 and Table 3, the rate-distortion performances in AI look very similar and the running times differ only by at most a factor 0.2. In Table 4 and Table 5, the same goes for the rate-distortion performances in RA and the running times. Therefore, running times may be fairly compared as long as a single thread and a given acceleration (e.g. AVX2) are used.

Table 2: AI results, Test-1 (float NNs using TF framework) versus VTM 11.0.

|  | BD-rate Over VTM-11.0 (All Intra Main 10) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT | bit DIFF |
| Class A1 | -4.36% | -3.80% | -4.03% | -4.55% | -3.81% | -3.66% | 703% | 5297% | 4% |
| Class A2 | -2.54% | -2.63% | -3.00% | -2.56% | -1.98% | -2.42% | 777% | 6171% | 2% |
| Class B | -2.96% | -2.64% | -2.80% | -2.69% | -2.14% | -1.82% | 799% | 6634% | 2% |
| Class C | -3.13% | -2.80% | -3.07% | -3.13% | -2.03% | -2.21% | 673% | 6535% | 3% |
| Class E | -4.84% | -4.78% | -3.80% | -4.96% | -4.20% | -2.52% | 681% | 6420% | 4% |
| Overall | -3.48% | -3.22% | -3.26% | -3.45% | -2.71% | -2.43% | 730% | 6258% | 3% |
| Class D | -3.13% | -2.84% | -3.10% | -2.99% | -1.89% | -2.08% | 625% | 7606% | 3% |
| Class F | -2.30% | -1.82% | -2.00% | -2.35% | -1.33% | -1.40% | 358% | 3534% | 2% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! |

Table 3: AI results, Test-2 (float NNs using the custom framework) versus VTM 11.0.

|  | BD-rate Over VTM-11.0 (All Intra Main 10) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT | bit DIFF |
| Class A1 | -4.36% | -3.81% | -4.02% | -4.55% | -3.78% | -3.61% | 548% | 4541% | 4% |
| Class A2 | -2.54% | -2.68% | -2.95% | -2.56% | -1.98% | -2.39% | 648% | 5068% | 2% |
| Class B | -2.96% | -2.67% | -2.82% | -2.69% | -2.23% | -1.84% | 606% | 5490% | 3% |
| Class C | -3.12% | -2.81% | -3.05% | -3.17% | -2.12% | -2.24% | 595% | 6980% | 3% |
| Class E | -4.84% | -4.77% | -3.77% | -4.97% | -4.18% | -2.54% | 570% | 6055% | 4% |
| Overall | -3.47% | -3.24% | -3.25% | -3.46% | -2.74% | -2.43% | 594% | 5627% | 3% |
| Class D | -3.13% | -2.88% | -3.07% | -3.00% | -1.99% | -1.99% | 593% | 7607% | 3% |
| Class F | -2.31% | -1.82% | -1.98% | -2.39% | -1.36% | -1.53% | 328% | 3558% | 2% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! |

Table 4: RA results, Test-1 (float NNs using TF framework) versus VTM-11.0.

|  | BD-rate Over VTM-11.0 (Random Access Main 10) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT | bit DIFF |
| Class A1 | -2.34% | -0.83% | -1.94% | -2.29% | -0.46% | -1.48% | 213% | 839% | 2% |
| Class A2 | -1.28% | -0.61% | -0.98% | -1.22% | 0.16% | -0.31% | 199% | 562% | 1% |
| Class B | -1.60% | -0.61% | -0.67% | -1.41% | 0.02% | 0.32% | 207% | 808% | 1% |
| Class C | -1.64% | -0.78% | -0.82% | -1.45% | 0.24% | 0.66% | 217% | 1214% | 1% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -1.69% | -0.70% | -1.02% | -1.56% | 0.01% | -0.08% | 209% | 844% | 1% |
| Class D | -1.44% | -1.02% | -1.17% | -1.37% | -1.12% | -0.60% | 196% | 966% | 1% |
| Class F | -1.11% | -0.68% | -0.62% | -1.18% | -0.23% | 0.59% | 193% | 599% | 1% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! |

Table 5: RA results, Test-2 (float NNs using the custom framework) versus VTM-11.0.

|  | BD-rate Over VTM-11.0 (Random Access Main 10) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT | bit DIFF |
| Class A1 | -2.32% | -0.66% | -1.91% | -2.25% | -0.23% | -1.50% | 198% | 786% | 2% |
| Class A2 | -1.27% | -0.70% | -0.98% | -1.27% | -0.08% | -0.37% | 179% | 453% | 1% |
| Class B | -1.62% | -0.47% | -0.59% | -1.45% | 0.30% | 0.57% | 193% | 761% | 1% |
| Class C | -1.66% | -0.73% | -0.69% | -1.57% | 0.45% | 0.84% | 200% | 946% | 1% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -1.70% | -0.62% | -0.96% | -1.61% | 0.16% | 0.04% | 193% | 732% | 1% |
| Class D | -1.42% | -1.14% | -1.30% | -1.36% | -1.15% | -0.36% | 181% | 749% | 1% |
| Class F | -1.09% | -0.60% | -0.74% | -1.16% | -0.13% | 0.51% | 180% | 475% | 1% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! |

## Impact of the quantization

In Test-2B, the neural network parameters and the neural network representations are in 16-bit signed integers. In Test-2B, the standalone C++ library runs these neural networks with a single thread and AVX2. Finally, in Test-2B, the single additional neural network-based intra prediction mode predicts the LFNST group indices, see [1].

Looking at Table 6 and Table 7, the neural network quantization incurs no loss in mean BD-rate reduction and accelerates the encoder and the decoder (by a factor of 1.4 approximatively).

Table 6: AI results, Test-2B (16 bits NNs using the custom framework) versus VTM-11.0.

|  | BD-rate Over VTM-11.0 (All Intra Main 10) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT | bit DIFF |
| Class A1 | -4.35% | -3.77% | -4.00% | -4.52% | -3.78% | -3.62% | 370% | 3206% | 4% |
| Class A2 | -2.54% | -2.67% | -2.98% | -2.56% | -2.01% | -2.44% | 428% | 3602% | 2% |
| Class B | -2.95% | -2.62% | -2.85% | -2.69% | -2.17% | -1.83% | 420% | 3513% | 3% |
| Class C | -3.12% | -2.74% | -3.09% | -3.18% | -1.94% | -2.17% | 369% | 3861% | 3% |
| Class E | -4.79% | -4.79% | -3.91% | -4.90% | -4.16% | -2.48% | 353% | 3586% | 4% |
| Overall | -3.46% | -3.21% | -3.29% | -3.45% | -2.69% | -2.41% | 390% | 3560% | 3% |
| Class D | -3.12% | -2.73% | -3.09% | -3.00% | -1.91% | -1.88% | 352% | 3926% | 3% |
| Class F | -2.28% | -1.81% | -1.88% | -2.38% | -1.60% | -1.32% | 225% | 1962% | 2% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! |

Table 7: RA results, Test-2B (16 bits NNs using the custom framework) versus VTM-11.0.

|  | BD-rate Over VTM-11.0 (Random Access Main 10) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT | bit DIFF |  |
| Class A1 | -2.31% | -0.83% | -1.80% | -2.26% | -0.28% | -1.30% | 169% | 561% | 2% |  |
| Class A2 | -1.25% | -0.54% | -1.04% | -1.22% | 0.32% | -0.36% | 159% | 390% | 1% |  |
| Class B | -1.60% | -0.68% | -0.79% | -1.37% | 0.06% | 0.27% | 166% | 536% | 1% |  |
| Class C | -1.66% | -0.80% | -0.66% | -1.64% | 0.25% | 0.90% | 177% | 730% | 1% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.69% | -0.71% | -1.01% | -1.59% | 0.10% | 0.00% | 168% | 551% | 1% |  |
| Class D | -1.42% | -1.07% | -1.08% | -1.49% | -0.51% | -0.10% | 162% | 571% | 1% |  |
| Class F | -1.09% | -0.54% | -0.71% | -1.26% | 0.07% | 0.13% | 162% | 377% | 1% |  |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! |  |

Table 8: summary of the experiments.

|  | AI | RA |
| --- | --- | --- |
|  | Y gains ¦ enc. time ¦ dec. time | Y gains ¦ enc. time ¦ dec. time |
| TF, float NNs | -3.48% ¦ 730% ¦ 6258% | -1.69% ¦ 209% ¦ 844% |
| Custom, float NNs | -3.47% ¦ 594% ¦ 5627% | -1.70% ¦ 193% ¦ 732% |
| Custom, integer 16 bits NNs | -3.46% ¦ 390% ¦ 3560% | -1.69% ¦ 168% ¦ 551% |

# Conclusion

This contribution has presented an updated signaling of a single additional neural network-based intra prediction mode in VVC. Besides, it is shown that the running times of VTM-11.0 including a neural network-based tool using two different frameworks for running the neural networks can be fairly compared as long as a single thread and a given acceleration are used. Finally, it is reported that, in the context of the neural network-based intra prediction for VVC, the neural network quantization incurs no loss in mean BD-rate reduction and slightly accelerates the encoder and the decoder. The main advantage of the neural network quantization is to get systematically decodable bitstreams.

# References

- AHG11: neural network-based intra prediction with transform selection in VVC, JVET-T0073, T. Dumas, F.Galpin, P. Bordes, F. Le Léannec.

- JVET common test conditions and evaluation procedures for neural network-based video coding technology, JVET-U2016, S. Liu, A.Segall, E.Alshina, R.Liao.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).