| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20-28th Apr. 2021 | Document: JVET-V0106 |
| --- | --- |

| Title: | CE-related: On history-enhanced method of Rice parameter derivation for regular residual coding (RRC) at high bit depths |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy Marta Karczewicz Luong Pham Van Muhammed Coban | Email: | dmytror@qti.qualcomm.com martak@qti.qualcomm.com lphamvan@qti.qualcomm.com mcoban@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution describes a history-based extension of Rice parameters derivation for regular residual coding (RRC), for VVCv2 development. It is advocated that a local (template based) methods of Rice parameter derivation in RRC, can be further improved by enabling history (across TUs) utilization during the template computation. One example of such history-based extension is currently being studied in CE1.4, where local method of CE1.2 is being extended with a history usage. This proposal present additional example of history-based extension for local CE1.1 derivation and present improvement provided by the said extension compare to the local methods.

It is asserted by the proponent, that proposed history-enhancement for local RRC derivation provides a complexity (or dependency) scalable RRC parameter derivation method with each component (of the method) providing a gain in all tested class. Method is proposed for adoption to VVCv2.

The following results (against the CE anchor) are reported for local CE1.1 and CE1.2 RRC methods and their enhanced versions, respectively:

|  | Test | HDR PQ |  |  | HDR HLG |  |  | SVT12 RGB | SVT16 RGB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | wY | wU | wV | Y | U | V | Aver.GBR | Aver.GBR |
| AI | CE1.1 | -0.08% | -0.07% | -0.08% | -0.07% | -0.02% | -0.02% | -0.27% | -0.71% |
|  | CE1.2 | -0.09% | -0.06% | -0.07% | -0.10% | -0.03% | -0.04% | -0.41% | -0.79% |
|  | CE1.1+Hist | -0.31% | -0.21% | -0.22% |  |  |  | -1.33% | -2.36% |
|  | CE1.2+Hist | -0.33% | -0.23% | -0.25% | -0.50% | -0.30% | -0.32% | -1.43% | -2.27% |
| LDB | CE1.1 | -0.03% | -0.03% | -0.03% | -0.01% | 0.00% | 0.01% |  |  |
|  | CE1.2 | 0.03% | 0.03% | 0.02% | 0.02% | 0.05% | 0.06% | -0.19% | -0.66% |
|  | CE1.1+Hist | -0.09% | -0.10% | -0.12% | -0.05% | -0.04% | -0.05% |  |  |
|  | CE1.2+Hist | -0.06% | -0.08% | -0.09% | -0.06% | -0.01% | 0.00% | -0.66% | -1.53% |
| RA | CE1.1 | -0.02% | -0.03% | -0.03% |  |  |  |  |  |
|  | CE1.2 | -0.01% | -0.01% | -0.02% | 0.01% | 0.03% | 0.03% | -0.17% | -0.60% |
|  | CE1.1+Hist |  |  |  |  |  |  |  |  |
|  | CE1.2+Hist | -0.08% | -0.08% | -0.09% | -0.04% | -0.02% | -0.02% | -0.57% | -1.45% |

# Introduction

In the current VVC [1], rice parameter for RRC is derived using look-up table with considering of coefficient value of the neighboring transform coefficients in a template. The template of the neighboring coefficients is presented in Figure 1.

First, locSumAbs which is the sum of absolute of five available neighbouring coefficients in the template is calculated. The locSumAbs is then normalized (subtraction and clip) as following:

locSumAbs = Clip3( 0, 31, locSumAbs - baseLevel * 5 )

The rice parameter is derived using this locSumAbs with look-up table (Table 1). With the current design, the rice parameter range is constrained from 0 to 3.

Figure 1 Neighboring coefficients which are using in calculation of the localSumAbs of the current  coefficient

Table 1. Look-up table for Rice Parameter based on locSumAbs in current spec

| locSumAbs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 |
| locSumAbs | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| cRiceParam | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |

When the bitdepth of the input video is increased, or the extended precision is enabled, or the quantization parameter is set very low, the range of the coefficients and therefore locSumAbs value may significantly increase. In such cases, the current VVC range of allowed rice parameter may not be sufficient. This limitation causes a coding performance drop of VVC in high bit-depth coding.

Several proposals to modify Rice derivation method in VVC were proposed in JVET-T and U meetings, and CE was conducted on them. Current contribution present simulation results conducted under the CE CTC and compared to the CE anchor. CE anchor has in turn the following performance vs. VTM12.0.

Table 1. Reported bd-rate gain of CE Anchor vs. VTM12.0 under CE CTC, low QP configuration.

| Test | HDR PQ |  |  | HDR HLG |  |  | SVT12 RGB | SVT16 RGB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | wY | wU | wV | Y | U | V | Aver.GBR | Aver.GBR |
| AI | -0.49% | -0.49% | -0.52% | -0.44% | -0.35% | -0.36% | -2.68% | -24.70% |
| LDB | -0.07% | -0.06% | -0.05% | 0.02% | -0.02% | -0.06% | -0.50% | -16.38% |
| RA | -0.13% | -0.13% | -0.14% | -0.01% | -0.04% | -0.06% | -0.54% | -15.56% |

# Tested local-based RRC methods

## Description of CE-1.1

CE1.1 method was originally proposed in JVET-T0105. To extend the range of allowed Rice parameters, it was proposed that output of the template derivation locSumAbs is compared against a set of thresholds {Tx} and apply normalization right shift value {Rx} corresponding to a met thresold. Such that scaled value would fit in the allowed range of the Table 128 of the VVC specficiation without clipping. Following this, an output of the Table 128 is being adjusted by incrementing with a shift value if scaling process was taken a place for a given locSumAbs value.

Local (template based) method CE1.1 was studied and optimized in 2 CEs following T- meeting, in this contribution, it is tested with following parameters:

- Humber of thresholds in Tx and number of shift parameters Rx is set equal to 4 and 5, respectively, as following:

Tx[4] = { 32,128, 512, 2048 }, Rx[5] = { 0, 2, 4, 6, 8}

- Removal of slice-level signaling.

- baseLevel dependent offset derivation is modified as following:

# baseLevelOffset = (bitDepth > 12) ? (isIntra() ? 5: 10) : (isIntra() ? 10: 15)

Proposed changes to the specification are limited to the clause 9.3.3.2, and its invoking process to derivate baseLevel parameter.

#### 9.3.3.2 Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ] (CE1.1) Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight. Output of this process is the Rice parameter cRiceParam. Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo-code process: locSumAbs = 0 if( xC < ( 1 << log2TbWidth ) − 1 ) { locSumAbs += AbsLevel[ xC + 1 ][ yC ] if( xC < ( 1 << log2TbWidth ) − 2 ) locSumAbs += AbsLevel[ xC + 2 ][ yC ] if( yC < ( 1 << log2TbHeight ) − 1 ) locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ] (1517) } if( yC < ( 1 << log2TbHeight ) − 1 ) { locSumAbs += AbsLevel[ xC ][ yC + 1 ] if( yC < ( 1 << log2TbHeight ) − 2 ) locSumAbs += AbsLevel[ xC ][ yC + 2 ] } Variable shift is derived as following for idx set equal to 0, if baseLevel is equal to 0, and set equal to 1, otherwise: shift = (localSumAbs < Tx[0]) ? Rx[0] : ( (localSumAbs < Tx[1]) ? Rx[1]: ((localSumAbs < Tx[2]) ? Rx[2]: ((localSumAbs < Tx[3]) ? Rx[3]: Rx[4]))). locSumAbs = locSumAbs >> shift Variable localSumAbs is updated as follows: locSumAbs = Clip3( 0, 31, locSumAbs − baseLevelOffset) Given the variable locSumAbs, the Rice parameter cRiceParam is derived as specified in Table 128. cRiceParam is then is refined as: cRiceParam = cRiceParam + shift

## Description of CE-1.2

Initial design tested in CE-1.2 was presented in the JVET-T0105, and JVET-U0065. Similarly, to the Test CE-1.1, to extend the range of the Rice parameters, it is proposed to scale values locSumAbs with a shift, if locSumAbs exceed certain threshold, such that scaled value would fit in the allowed range of the Table 128 of the VVC specficiation without clipping. Following this, an output of the Table 128 is being adjusted by incrementing with shift value if scaling process was taken a place for a given locSumAbs value.

The difference with CE-1.1 is limmited to the way, the shift parameter for locSumAbs scaling and cRiceParam increment is derived. Instead of comparing value against the tabulate threshold, CE1.2 employs floorLog2(x) computation (can be implemented through MSB find operation).  In CE-1.2, it is tested as follows, with syntax element ScaleRice taking values 1 or 2.

scalledCoef = (localSumAbs * ScaleRice) >> 3

shift = (scalledCoef == 0) ? 0 : Floor( Log2( scalledCoef) )

Utilized variable ScaleRice can be either derived at the encoder side and signalled as syntax element in the slice header, or it can be derived at the decoder side.  Omitting slice level signaling and/or ScaleRice derivation, proposed changes to the specification are limited to the clause 9.3.3.2.

#### 9.3.3.2 Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ] (CE1.2) Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight. Output of this process is the Rice parameter cRiceParam. Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo-code process: locSumAbs = 0 if( xC < ( 1 << log2TbWidth ) − 1 ) { locSumAbs += AbsLevel[ xC + 1 ][ yC ] if( xC < ( 1 << log2TbWidth ) − 2 ) locSumAbs += AbsLevel[ xC + 2 ][ yC ] if( yC < ( 1 << log2TbHeight ) − 1 ) locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ] (1517) } if( yC < ( 1 << log2TbHeight ) − 1 ) { locSumAbs += AbsLevel[ xC ][ yC + 1 ] if( yC < ( 1 << log2TbHeight ) − 2 ) locSumAbs += AbsLevel[ xC ][ yC + 2 ] } Variable shift is derived as following: scalledCoef = (localSumAbs * sh_scale_rice) >> 3 shift = (scallPreCoef == 0) ? 0 : Floor( Log2( scalledCoef) ) locSumAbs = locSumAbs >> shift Variable localSumAbs is updated as follows: locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 ) Given the variable locSumAbs, the Rice parameter cRiceParam is derived as specified in Table 128. cRiceParam is then is refined as: cRiceParam = cRiceParam + shift

# History-enhanced RRC methods

## History-based extension of Template

History-enhancement of the local based RRC method was tested and evaluated in CE3.5/CE3.6 for the the JVET-U meeting. It was assessed that template computation employed for the Rice parameter derivation may produce inaccurate Rice estimates of coefficients, if they are located at TU boundary, or being first decoded with Rice method. This happen due to fact, that for those coefficients, Rice template computation is being biased toward 0, due to fact that some template position are being located outside of the TU, and therefore interpreted as 0 value. Visualization of such template pattern for localSumAbs being located at the TU boundary is shown in Figure 2.

TU

Figure 2. Visualization of the template pattern for localSumAbs being located at the TU boundary.

To improve the accuracy of Rice estimate from the computed template, it is proposed for template positions being outside of the current TU to update localSumAbs counter with a history derived value, instead of 0 initialization. Implementation of this method is shown as VVC specification text extract for clause 9.3.3.2, with proposed text being highlighted in yellow.

To maintain a history, a single counter per color component is utilized, StatCoeff[3]. It is being updated once per TU from the first, non-zero, Golomb-Rice coded transform coefficient (abs_remainder[ ] or dec_abs_level[ ]) through a process of exponential moving average.

When first, non-zero, Golomb-Rice coded transform coefficient in TU is coded as abs_remainder, the history counter for color component cIdx is updated as following:

StatCoeff[cIdx] = ( StatCoeff[cIdx] + Floor(Log2(abs_remainder[ ])) + 2 ) >> 1

When first, non-zero, Golomb-Rice coded transform coefficient in TU is coded as abs_remainder, the history counter for color component cIdx is updated as following:

StatCoeff[cIdx] = ( StatCoeff[cIdx] + Floor(Log2(dec_abs_level[ ])) ) >> 1

Prior to TU decoding and history counter update, the variable histValue is being initialized as:

histValue = 1 << StatCoeff[cIdx].

Several method of history counter StatCoeff[3] initialization were studied, including resetting the counter at the CTU level with a default value, or history counter maintenance (propagation) between different CTUs.

#### 9.3.3.2 Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ] Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight. Output of this process is the Rice parameter cRiceParam. Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo-code process: locSumAbs = 0 if( xC < ( 1 << log2TbWidth ) − 1 ) { locSumAbs += AbsLevel[ xC + 1 ][ yC ] if( xC < ( 1 << log2TbWidth ) − 2 ) locSumAbs += AbsLevel[ xC + 2 ][ yC ] else locSumAbs += histValue if( yC < ( 1 << log2TbHeight ) − 1 ) locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ] (1517) else locSumAbs += histValue } else locSumAbs += 2 * histValue if( yC < ( 1 << log2TbHeight ) − 1 ) { locSumAbs += AbsLevel[ xC ][ yC + 1 ] if( yC < ( 1 << log2TbHeight ) − 2 ) locSumAbs += AbsLevel[ xC ][ yC + 2 ] else locSumAbs += histValue } else locSumAbs += histValue Variable localSumAbs is updated as follows: locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 ) Given the variable locSumAbs, the Rice parameter cRiceParam is derived as specified in Table 128.

## CE1.1 + Hist

History enhancement of the local based CE1.1 was implemented as a direct integration of the CE1.1. history based template extension described above. No slice level signaling was tested in this configuration. Cross-CTU history propagation was enabled.

## CE1.2 + Hist

History enhancement of the local based CE1.2 discussed in this document is identical to the CE test CE1.4-B.  No slice level signaling was tested in this configuration. Cross-CTU history propagation was enabled.

# Results

Simulation for described methods have been conducted following the CE CTC and are being reported against the CE anchor and follow the CE reporting requirements. Complete simulation results are provided along this document for lowQP, normal QP and lossless configurations. Due to size of the presented data and variety of configuration and bitdepthes, this section provides a summary tables for tested method, following CE report practice. Missing QP points and run-time estimates will be updated.

Table 2. Simulation results for described RRC tests, CE CTC, LowQP test configuration.

|  | Test | HDR PQ |  |  | HDR HLG |  |  | SVT12 RGB | SVT16 RGB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | wY | wU | wV | Y | U | V | Aver.GBR | Aver.GBR |
| AI | CE1.1 | -0.08% | -0.07% | -0.08% | -0.07% | -0.02% | -0.02% | -0.24% | -0.71% |
|  | CE1.2 | -0.09% | -0.06% | -0.07% | -0.10% | -0.03% | -0.04% | -0.41% | -0.79% |
|  | CE1.1+Hist | -0.31% | -0.21% | -0.22% |  |  |  | -1.33% | -2.36% |
|  | CE1.2+Hist | -0.33% | -0.23% | -0.25% | -0.50% | -0.30% | -0.32% | -1.43% | -2.27% |
| LDB | CE1.1 | -0.03% | -0.03% | -0.03% | -0.01% | 0.00% | 0.01% | -0.05% | -0.58% |
|  | CE1.2 | 0.03% | 0.03% | 0.02% | 0.02% | 0.05% | 0.06% | -0.19% | -0.66% |
|  | CE1.1+Hist | -0.09% | -0.10% | -0.12% | -0.05% | -0.04% | -0.05% |  |  |
|  | CE1.2+Hist | -0.06% | -0.08% | -0.09% | -0.06% | -0.01% | 0.00% | -0.66% | -1.53% |
| RA | CE1.1 | -0.02% | -0.03% | -0.03% |  |  |  | -0.06% | -0.51% |
|  | CE1.2 | -0.01% | -0.01% | -0.02% | 0.01% | 0.03% | 0.03% | -0.17% | -0.60% |
|  | CE1.1+Hist |  |  |  |  |  |  |  |  |
|  | CE1.2+Hist | -0.08% | -0.08% | -0.09% | -0.04% | -0.02% | -0.02% | -0.57% | -1.45% |

Table 3. Simulation results for described RRC tests, CE CTC, NormQP test configuration.

|  | Test | HDR PQ |  |  |  |  | HDR HLG |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DE100 | PSNR-L100 | wY | wU | wV | Y | U | V |
| AI | CE1.1 |  |  |  |  |  |  |  |  |
|  | CE1.2 | 0.01% | -0.03% | -0.03% | 0.09% | -0.08% | -0.01% | -0.10% | 0.05% |
|  | CE1.1+Hist |  |  |  |  |  |  |  |  |
|  | CE1.2+Hist | 0.00% | -0.02% | -0.02% | 0.06% | -0.07% | -0.01% | 0.00% | -0.04% |
| RA | CE1.1 |  |  |  |  |  |  |  |  |
|  | CE1.2 | 0.09% | 0.03% | 0.02% | -0.14% | -0.22% | -0.01% | 0.11% | -0.30% |
|  | CE1.1+Hist |  |  |  |  |  |  |  |  |
|  | CE1.2+Hist | -0.01% | -0.03% | -0.05% | -0.29% | -0.25% | 0.00% | -0.02% | -0.09% |

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1]	Benjamin Bross, Jianle Chen, Shan Liu and Ye-Kui Wang, “Versatile Video Coding (Draft 10)” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET- S2001, 19th Meeting: Teleconference, 22 June – 01 July 2020.

[2]	JVET-U2018, "Common test conditions for high bit depth and high bit rate video coding", A. Browne, T. Ikai, D. Rusanovskyy, X. Xiu.

[3]	JVET-U2022, "CE on Entropy Coding for High Bit Depth and High Bit Rate Coding", A. Browne, T. Hashimoto, H.-J. Jhu, D. Rusanovskyy.