| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0108 |
| --- | --- |

| Title: | AHG9: Colour Transform Information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard François, Miloš Radosavljević, Philippe de Lagrange, Fabrice Le Léannec | Tel: Email: | {edouard.francois,milos.radosavljevic}@interdigital.com |  |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution describes additional study of a new SEI message named Colour Transform Information (CTI) SEI message, proposed in JVET-U0078. The SEI message signals mapping parameters that can apply to the decoded pictures. The mapping can operate intra-component (for luma and chroma) or inter-component (for chroma). In this contribution, the CTI has been tested as a mandatory process for out-of-loop mapping for HDR-PQ content. BD-rate variations compared to the VTM12.0, for the CTCs HDR PQ content, are reported in the table below, for AI, RA and LB configurations.

| Config. | DE100 | PSNRL100 | wPSNRY | wPSNRU | wPSNRV | PSNRY | PSNRU | PSNRV | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AI | -1.15% | -0.76% | -0.50% | -5.43% | -7.24% | -0.31% | -2.52% | -3.33% | 99% | 98% |
| RA | -2.41% | -1.72% | -1.17% | -5.37% | -8.06% | -1.00% | -1.39% | -2.79% | 98% | 92% |
| LB | -5.53% | -2.49% | -1.56% | -10.77% | -15.14% | -1.46% | -1.27% | -5.01% | 98% | 91% |

# Introduction

This contribution is a follow-up of JVET-U0078, where a new SEI message, named Colour Transform Information (CTI) SEI message, was proposed. The CTI SEI message signals mapping parameters that can apply to the reconstructed pictures after the decoding process. CTI can be used as a tool for out-of-loop content mapping, in order to get improved coding performance. This feature can be of interest for some use cases or content types. For HDR PQ cases, it is observed that the CTI using the same fixed mapping function as LMCS leads to compression gains compared to VTM anchors.

# Proposal

The proposed SEI conveys metadata corresponding to a post-decoding colour transform, hereunder named mapping process. For the luma component (component 0), the mapping process operates as an intra-component mapping.  For the chroma components (components 1 and 2), the mapping process can operate as an intra-component mapping or as a cross-component mapping. When cross-component mapping is used, the chroma cross-component mapping functions can be inferred from the luma mapping function, or explicitly signalled. When cross-component mapping is not used, the chroma intra-component mapping functions are explicitly signalled. The mapping functions are based on piecewise linear models whose parameters are signalled in the CTI. The horizontal axis of the piecewise linear functions is uniformly spaced.

The signalling is summarized below.

For colour component 0:

- LUT0 coded

For colour components 1 and 2

- crossCompFlag coded

- If crossCompFlag = 1, chromaInferredFlag coded

- If crossCompFlag = 0 or chromaInferredFlag = 0, LUT1/LUT2 coded

- Else chromaOffset coded, LUT1/LUT2 inferred from LUT0

The colour transform process is summarized below.

For colour component 0:

- outC0 = LUT0[inC0]

For colour component x=1, 2

- crossCompFlag = 0	outCx = LUTx[inCx]

- crossCompFlag = 1	outCx = ( (inCx–2^(B–1)) * LUTx[inC0] + 2^(B–1) * 2^11 ) >> 11

## Syntax

The proposed syntax is described in the table below. The first part of the table optionally indicates the format of the pictures resulting from the colour transform process. The second part signals parameters to derive the mapping functions of the 3 color components. Piecewise linear models are used to define the mapping functions.

| colour_transform_info( payloadSize ) { | Descriptor |
| --- | --- |
| colour_transform_cancel_flag | u(1) |
| if( !colour_transform_cancel_flag ) { |  |
| colour_transform_persistence_flag | u(1) |
| colour_transform_video_signal_info_present_flag | u(1) |
| if( colour_transform_video_signal_info_present_flag ) { |  |
| colour_transform_full_range_flag | u(1) |
| colour_transform_primaries | u(8) |
| colour_transform_transfer_function | u(8) |
| colour_transform_matrix_coefficients | u(8) |
| } |  |
| colour_transform_input_bit_depth_minus8 | u(3) |
| colour_transform_output_bit_depth_minus8 | u(3) |
| colour_transform_log2_number_of_points_per_lut | u(3) |
| colour_transform_cross_component_flag | u(1) |
| if ( colour_transform _cross_component_flag ) |  |
| colour_transform_cross_comp_inferred_flag | u(1) |
| for( i = 0; i < colourTransformSize; i++ ) |  |
| colour_transform_lut0[ i ] | u(v) |
| if( colour_transform_cross_component_flag = = 0 ¦ ¦ colour_transform_cross_comp_inferred_flag = = 0 ) { |  |
| colour_transform_number_chroma_lut_minus1 | u(1) |
| for( i = 0; i < colourTransformSize; i++ ) |  |
| colour_transform_lut1[ i ] | u(v) |
| if ( colour_transform_number_chroma_lut_minus1 ) |  |
| for( i = 0; i < colourTransformSize; i++ ) |  |
| colour_transform_lut2[ i ] | u(v) |
| } |  |
| else |  |
| colour_transform_chroma_offset | u(v) |
| } |  |
| } |  |

colourTransformSize is set equal to (1 << colour_transform_log2_number_of_points_per_lut).

# Complexity analysis

The mapping functions are based on piecewise linear models, defined by pivot points and one slope value per piece/segment. The horizontal axis of the piecewise linear functions is uniformly spaced, which greatly eases the identification of the index which a sample belongs to (simple right shift).

When the mapping is intra-component, the following process applies for a given decoded sample Yin of bitdepth B, using the pivot points pivotPoint and slope Luts of the intra mapping function:

- idx = Yin >> K1

- tmp = pivotPoint[idx] + ( (slope[idx] * ( Yin – ( idx << K1 ) ) + 1024 ) >> 11 )

- Yout = Clip3(0, 2^B – 1, tmp )

K1 being set to ( B – colour_transform_log2_number_of_points_per_lut ).

The same process applies to components C1 and C2 when their mapping is intra-component.

The number of operations per sample is therefore:

- 1 mult, 2 adds, 2 right shifts, 1 left shift

- 1 clipping

When the mapping is cross-component, the following process applies for a given decoded sample Cin of bitdepth B, using the pivot points pivotPoint and slope Luts of the cross-component mapping function:

- Ycoloc is the collocated decoded sample of the sample Cin

- idx = Ycoloc >> K1

- scale = pivotPoint[idx] + ( (slope[idx] * ( Ycoloc – ( idx << K1 ) ) ) >> K1 )

- tmp = ( (2^B – 1)<<11 + ( Cin – 2^B – 1 ) * scale + 1024 ) >> 11

- Cout = Clip3(0, 2^B – 1, tmp )

The number of operations per sample is therefore:

- 2 mult, 3 adds, 1 clipping, 3 right shifts, 1 left shift

# Simulation results

The proposal is implemented in VTM12.0. The corresponding patch is attached to the contribution. Simulations have been performed for the AI, RA configurations as specified in the HDR CTCs (JVET-T2011). LB configuration has also been tested. In the simulations the piecewise linear functions are made of 16 segments. In the results below, CTI is enabled, LMCS is disabled. The CTI luma mapping function is built from the LMCS luma mapping function used in VTM for HDR-PQ content. Cross-component scaling is applied, with cross-component scaling LUT derived from the luma mapping function.

Results are summarized in the table below. Detailed results are reported in the attached excel file. Gains are observed for all metrics and all configurations.

|  |  | wPSNR |  |  | PSNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| All Intra |  |  |  |  |  |  |  |  |  |
| -1.15% | -0.76% | -0.50% | -5.43% | -7.24% | -0.31% | -2.52% | -3.33% | 99% | 98% |
| Random Access Gop 32 |  |  |  |  |  |  |  |  |  |
| -2.41% | -1.72% | -1.17% | -5.37% | -8.06% | -1.00% | -1.39% | -2.79% | 98% | 92% |
| Low Delay B |  |  |  |  |  |  |  |  |  |
| -5.53% | -2.49% | -1.56% | -10.77% | -15.14% | -1.46% | -1.27% | -5.01% | 98% | 91% |

The authors thank Dolby for the discussions and for the cross-checking of the provided results.

Acknowledgement: this work has been achieved in the context of the 3EMS project funded by the “Region Bretagne”.

# Patent rights declaration

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).