| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0115-v3 |
| --- | --- |

| Title: | EE1-1.4: Test on Neural Network-based In-Loop Filter with Large Activation Layer |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongtao Wang Jianle Chen Anand Meher Kotra Kevin Reuze Marta Karczewicz | Email: | hongtaow@qti.qualcomm.com cjianle@qti.qualcomm.com akotra@qti.qualcomm.com kreuz@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports the EE test results of combining JVET-U0104 and JVET-U0115. In the proposed Neural Network-based method, one NN-based filter can be selected out of four candidates as an in-loop filter for each color channel type (luma/chroma) of a picture. The deblocking boundary strength information of VVC is also used as additional input to the NN filter but the actual deblocking filtering process is bypassed. Simulation results reportedly show 8.23 %, 19.88 % and 20.41 % BD rate saving for RA and 7.46 %, 17.94 %, 19.72 % BD rate saving for AI, for Y, Cb and Cr components respectively.

# Introduction

The NN filter tested in this contribution is a combination of the filters proposed in JVET-U0104 [1] and that in JVET-U0115[2], the network structure is illustrated in Figure 1. It contains 14 filter blocks, including 12 filter blocks used as hidden layers. Each filter block consists of two 1x1 convolution layers with a leaky ReLU as activation layer between them, and followed by a 3x3 convolution layer, where the value of M is set to be larger than the value of K.

Figure 1 Proposed network architecture

The input size of the NN filtering process is 144x144, including the current CTU and 8 neighboring samples to each side of the current CTU. The luma samples are interleaved into four 72x72 sized blocks before being used as inputs of the filtering process. The output tensor corresponds to the filtered CTU samples (64x64x6), including 4 luma blocks and 2 chroma blocks.

Additionally, the QP and boundary strength (BS) information is used as additional input planes to the network.

## Scaling of NN filter residues

When the proposed NN filter is being applied to reconstructed pictures, a scaling factor is signaled for each color component in picture headers. The difference between the input samples and the NN filtered samples (residues) are scaled by the scaling factors before being added to input samples.

# Simulation results

Simulations are performed based on the common test conditions for neural network-based video coding technology[5]. The results reportedly show 8.23 %, 19.88 % and 20.41 % BD rate saving for RA, 6.91%, 20.39%, 22.21% for LB and 7.46 %, 17.94 %, 19.72 % BD rate saving for AI, for Y, Cb and Cr components respectively.

Table 1 Simulation results

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -8.17% | -15.62% | -17.77% | 235% | 33114% | 0% |
| Class A2 | -8.58% | -19.41% | -18.96% | 224% | 30428% | 1% |
| Class B | -7.82% | -21.38% | -21.21% | 212% | 35954% | 1% |
| Class C | -8.53% | -21.56% | -22.49% | 176% | 34147% | 1% |
| Class E |  |  |  |  |  |  |
| Overall | -8.23% | -19.88% | -20.41% | 208% | 33739% | 1% |
| Class D | -9.87% | -23.72% | -24.56% | 170% | 34660% | 1% |
| Class F | -4.62% | -16.32% | -16.19% | 267% | 45901% | 0% |
| Class H | -7.39% | -15.53% | -20.06% | 339% | 46185% | 0% |
|  | Low delay B Main10 |  |  |  |  |  |
|  | BD-rate Over VTM-11.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -7.14% | -16.96% | -17.59% | 228% | 31287% | 1% |
| Class A2 | -6.69% | -19.74% | -19.38% | 219% | 27829% | 2% |
| Class B | -6.50% | -19.30% | -21.77% | 208% | 34468% | 2% |
| Class C | -7.53% | -21.75% | -22.76% | 178% | 35134% | 2% |
| Class E | -6.75% | -16.84% | -15.11% | 349% | 64209% | 2% |
| Overall | -6.91% | -20.39% | -22.21% | 224% | 36459% | 2% |
| Class D | -8.45% | -24.16% | -25.62% | 172% | 34464% | 2% |
| Class F | -4.64% | -17.44% | -17.96% | 262% | 49252% | 1% |
| Class H | -6.27% | -18.96% | -23.37% | 302% | 50326% | 1% |
|  | All Intra Main10 |  |  |  |  |  |
|  | BD-rate Over VTM-11.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -6.69% | -15.87% | -18.46% | 246% | 24685% | 0% |
| Class A2 | -6.76% | -18.87% | -17.83% | 186% | 19887% | 0% |
| Class B | -6.81% | -17.26% | -19.82% | 163% | 20314% | 0% |
| Class C | -7.64% | -18.37% | -21.62% | 132% | 15212% | 1% |
| Class E | -9.76% | -19.65% | -20.14% | 168% | 23788% | 0% |
| Overall | -7.46% | -17.94% | -19.72% | 171% | 20101% | 0% |
| Class D | -7.59% | -18.55% | -22.70% | 123% | 14532% | 1% |
| Class F | -5.69% | -17.36% | -17.16% | 124% | 17430% | 0% |
| Class H | -7.28% | -13.73% | -17.14% | 213% | 17900% | 0% |

## Supplemental results

In this section, supplemental results are provided for the models with refined training process. The results reportedly show that additional gain can be achieved by refining the training process of the proposed models. The refined model achieved 8.49 %, 22.05 % and 22.29 % BD rate saving for RA, 7.84%, 24.55 %, 23.69 % for LB and 7.61 %, 17.87 %, 19.99 % BD rate saving for AI, for Y, Cb and Cr components respectively.

Table 2 Supplemental results of models with refined training

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -8.23% | -18.04% | -20.22% | 238% | 33483% | 1% |
| Class A2 | -8.77% | -20.88% | -20.83% | 223% | 31169% | 1% |
| Class B | -8.11% | -23.79% | -22.57% | 215% | 36845% | 1% |
| Class C | -8.94% | -23.75% | -24.60% | 179% | 34832% | 1% |
| Class E |  |  |  |  |  |  |
| Overall | -8.49% | -22.05% | -22.29% | 210% | 34437% | 1% |
| Class D | -10.28% | -25.41% | -27.37% | 172% | 35096% | 1% |
| Class F | -4.79% | -16.92% | -16.49% | 267% | 45375% | 0% |
| Class H | -7.71% | -16.53% | -20.46% | 334% | 45223% | 0% |
|  | Low delay B Main10 |  |  |  |  |  |
|  | BD-rate Over VTM-11.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -7.66% | -20.65% | -20.43% | 239% | 35663% | 2% |
| Class A2 | -7.53% | -22.55% | -21.80% | 231% | 32816% | 2% |
| Class B | -7.13% | -23.85% | -22.98% | 205% | 35485% | 2% |
| Class C | -8.51% | -25.43% | -24.58% | 177% | 35992% | 2% |
| Class E | -8.12% | -19.53% | -17.22% | 346% | 62056% | 3% |
| Overall | -7.84% | -24.55% | -23.69% | 227% | 38599% | 2% |
| Class D | -9.46% | -27.25% | -28.17% | 174% | 36066% | 2% |
| Class F | -5.18% | -19.90% | -19.41% | 267% | 51873% | 2% |
| Class H | -6.87% | -21.54% | -24.79% | 324% | 50942% | 1% |
|  | All Intra Main10 |  |  |  |  |  |
|  | BD-rate Over VTM-11.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -6.99% | -16.47% | -18.94% | 233% | 22174% | 0% |
| Class A2 | -6.86% | -19.25% | -18.78% | 178% | 18570% | 0% |
| Class B | -6.92% | -15.99% | -19.20% | 158% | 19458% | 0% |
| Class C | -7.75% | -18.36% | -21.68% | 133% | 14859% | 1% |
| Class E | -9.97% | -20.38% | -21.28% | 164% | 23339% | 0% |
| Overall | -7.61% | -17.87% | -19.99% | 167% | 19196% | 0% |
| Class D | -7.67% | -18.05% | -22.40% | 131% | 15778% | 1% |
| Class F | -5.71% | -17.91% | -17.65% | 127% | 18005% | 0% |
| Class H | -7.47% | -13.95% | -17.71% | 215% | 18351% | 0% |

# Training and inference information

The models reported in this EE are trained based on DIV2K[4] and BVI-DVC data set[3]. The raw image/video data was compressed by VTM-11.0 using AI and RA (BVI-DVC only) configurations. In this test, separate models are used for luma and chroma components, 4 (2 luma + 2 chroma) models are trained using the AI compressed data and 4 additional models are trained using the RA compressed data. Figure 2 shows the training and validation loss curve.

Figure 2 Training/Validation losses of model 1

More information related to the training & inference of models can be found in the tables below.

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: NVIDIA Tesla V100-SXM2-32GB |
|  | Framework: | Tensorflow 2.2.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | ~110 |
|  | Batch size: | 82Kx16 |
|  | Training time: | ~220h |
|  | Training data information: | BVI-DVC, DIV2K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP=20,25,30,35,40,45 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size |  |
|  | Learning rate: | 2.00E-04 |
|  | Optimizer: | ADAM |
|  | Loss function: | Weighted L2 |
|  | Preprocessing: | Convert 144x144 YUV420 signal to 6 72x72 blocks. Normalize to 0~1 |
|  | Other information: |  |

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | Tensorflow 2.2.0 |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 990796x4 + 989498x4 |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | 3.78x4 + 3.77x4 |
|  | MAC (Giga) | 6.24e-4 Giga/pixel |
| Optional |  |  |
|  | Total Conv. Layers | 38 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: |  |
|  | Patch size |  |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage | 4.27MB (processing unit size 128x128) |
|  | Other information: |  |

# References

- AHG11: In-loop filtering with convolutional neural network and large activation, JVET-U0104, J. Chen, H. Wang, A. M. Kotra, M. Karczewicz (Qualcomm)

- AHG11: Neural Network-based In-Loop Filter Performance with No Deblocking Filtering stage, JVET-U0115, 	H. Wang, J. Chen, A. M. Kotra, M. Karczewicz (Qualcomm)

- BVI-DVC: A Training Database for Deep Video Compression (https://data.bris.ac.uk/data/dataset/3hj4t64fkbrgn2ghwp9en4vhtn ), Di Ma, Fan Zhang, David R. Bull

- https://data.vision.ee.ethz.ch/cvl/DIV2K/ R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool

- Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-U2016, S. Liu, A. Segall, E. Alshina, R.-L. Liao

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).