| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0117 |
| --- | --- |

| Title: | EE2-related: Combination of GPM and template matching |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao Yan Ye Xinwei Li Jie Chen | Email: | {ruling.lrl, yan.ye, sid.lxw, jiechen.cj}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

In this contribution, geometric partition mode with template matching is proposed. For a CU coded in GPM, each partition can decide whether the motion is refined using template matching or not. When TM is applied, motion is refined in the same way as TM applied to merge mode. It is reported that on top of EE2 common software, the overall coding performance impact for {Y, U, V, EncT, DecT} is {xx%, xx%, xx%, xx%, xx%} for RA and {xx%, xx%, xx%, xx%, xx%} for LDB.

Introduction

Template matching (TM) which refines the motion at the decoder side is proposed in JVET-U0100 [1] and is studied in EE2 [2]. In TM mode, motion is refined by constructing a template from left and above neighboring samples and finding the closest match between the template in the current picture and a corresponding area in a reference picture. In the current EE2 design, the TM is applied to AMVR mode and merge mode with MMVD, GPM, CIIP and subblock merge mode disabled. The motion refinement process depends on the coding mode and the MV precision, as shown in Table 1.

Table 1. Search patterns of AMVR and merge mode.

| Search pattern | AMVR mode |  |  |  | Merge mode |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 4-pel | Full-pel | Half-pel | Quarter-pel | AltIF=0 | AltIF=1 |
| 4-pel diamond | v |  |  |  |  |  |
| 4-pel cross | v |  |  |  |  |  |
| Full-pel diamond |  | v | v | v | v | v |
| Full-pel cross |  | v | v | v | v | v |
| Half-pel cross |  |  | v | v | v | v |
| Quarter-pel cross |  |  |  | v | v |  |
| 1/8-pel cross |  |  |  |  | v |  |

Proposed method

In this contribution, it is proposed to apply template matching to GPM. When a CU is coded in GPM, two motion for two geometric partitions are selected from a merge candidate list which is derived in the same way as that in VVC standard. Each motion for the geometric partition can decide whether to be refined using TM. When TM is chosen, a template is constructed using left and above neighboring samples. Then, the motion is refined by finding the minimum difference between the current template and a reference area using the same search pattern of merge mode with half-pel interpolation filter disabled. The refined motion is used to perform motion compensation for the geometric partition and is stored in the motion field.

For the syntax design, when a CU coded in GPM, two additional flags are signaled to indicate whether motion is refined for the two geometric partitions, respectively. Then, the geometric partition mode and two merge indices are further signaled.

Simulation results

The proposed method is implemented on top of EE2 common software. The simulation results are tested under EE2 test condition with random access and low delay B configurations.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EE2 common software |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.24% | -0.45% | -0.52% | 110% | 99% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.15% | -0.52% | -0.47% | 109% | 100% |
| Class F |  |  |  |  |  |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EE2 common software |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.16% | -0.42% | -0.21% | 114% | 98% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.09% | 0.04% | -0.26% | 112% | 98% |
| Class F |  |  |  |  |  |

References

- Y.-J. Chang, et. al., “Compression efficiency methods beyond VVC,” JVET-U0100, Jan. 2021.

- V. Seregin, et. al., “Exploration Experiment on Enhanced Compression beyond VVC capability,” JVET-U2024, Jan. 2021.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).