| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0120-v2 |
| --- | --- |

| Title: | EE2: Tests of compression efficiency methods beyond VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang Chun-Chi Chen Jie Dong Nan Hu Han Huang Marta Karczewicz Bappaditya Ray Vadim Seregin Yan Zhang Zhi Zhang | Tel: Email: | yjchang@qti.qualcomm.com chunchic@qti.qualcomm.com jiedong@qti.qualcomm.com nanh@qti.qualcomm.com hanhuang@qti.qualcomm.com martak@qti.qualcomm.com bray@qti.qualcomm.com vseregin@qti.qualcomm.com yzh@qti.qualcomm.com zhizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents tools test results included in EE on Enhanced Compression beyond VVC capability. The implementation was done based on VTM-10.0 with the added fixes related to MCTF. VTM-11.0 is used as anchor in the tests. All tools enabled demonstrates -12.14%, -13.31%, -13.62% BD rate reduction for luma and chroma components, respectively with 265% encoder and 459% decoder run times in RA configuration using VTM CTC.

# Introduction

In JVET-U0100, several tools and modifications were proposed to enhance compression efficiency on top of VVC standard.

The modifications are split into the following categories: base modifications, intra prediction, inter prediction, transform and coefficient coding, in-loop filtering, and entropy coding. Each category is described below in detail and is separately tested in EE as described in JVET-U2024.

# Base modification

In this section base modifications, simplifications removal, and encoder changes are discussed and tested.

## Non-normative base (test 1.1)

The following non-normative changes are introduced in the EE base software:

- Lossy encoder speedups for affine, AMVR and merge, macros: AFFINE_ENC_OPT, AMVR_ENC_OPT, MERGE_ENC_OPT

- Use original samples for SAO and ALF with macro ALF_SAO_TRUE_ORG

- Old PCM code removal for speedup, macro: REMOVE_PCM

- More SIMD, macros: MCIF_SIMD_NEW, DIST_SSE_ENABLE, TRANSFORM_SIMD_OPT

In config files, e.g. cfg_encoder_randomaccess_vtm.cfg

- GOP32 as in VTM-11.0 (but without the reference frame fix in VTM-11.1)

- MCTF enabled

- MinQTNonISlice is changed from 8 to 16

EE software is configured as BASE_ENCODER = 1, BASE_NORMATIVE = 0, TOOLS = 0, config files provided in cfg/tool-on/ are used with the change “MinQTNonISlice : 16”. Anchor is VTM-11.0.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 94% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 96% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 104% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 104% | 106% |
| Class E | 0.00% | 0.00% | 0.00% | 104% | 106% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class D | 0.00% | 0.00% | 0.00% | 105% | 110% |
| Class F | 0.00% | 0.00% | 0.00% | 105% | 109% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.07% | -0.01% | 92% | 99% |
| Class A2 | 0.31% | 0.17% | 0.33% | 91% | 98% |
| Class B | 0.04% | 0.10% | 0.09% | 92% | 99% |
| Class C | -0.57% | -0.51% | -0.45% | 95% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | -0.05% | -0.03% | 93% | 100% |
| Class D | -0.74% | -0.59% | -0.65% | 96% | 105% |
| Class F | 0.04% | 0.02% | -0.05% | 96% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -0.16% | -0.05% | 92% | 99% |
| Class C | -0.01% | 0.17% | -0.21% | 93% | 97% |
| Class E | -0.09% | 0.19% | -0.24% | 89% | 99% |
| Overall | 0.00% | 0.04% | -0.15% | 92% | 99% |
| Class D | -0.14% | 0.23% | -0.03% | 96% | 107% |
| Class F | 0.03% | -0.13% | 0.37% | 93% | 100% |

## Normative base (test 1.2)

The following normative changes are introduced in the EE base software. They either extend VVC capability or remove restrictions:

- Increase CTU and TU sizes to 256

- Remove MTS zero out

- Remove VPDU

- Remove constraints on small intra (SCIPU), inter, and CIIP blocks; CCLM latency restriction, use LMS linear model for CCLM, affine and BDOF constraints.

- Adjust deblocking parameters based on temporal ID

EE software is configured as BASE_ENCODER = 0, BASE_NORMATIVE = 1, TOOLS = 0, config files provided in cfg/base/ are used with classA.cfg applied to classes A1 and A2. Anchor is VTM-11.0.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.57% | -5.87% | -2.68% | 122% | 155% |
| Class A2 | -1.77% | -4.76% | -2.17% | 114% | 134% |
| Class B | -1.18% | -2.41% | -2.21% | 110% | 118% |
| Class C | -0.77% | -2.48% | -2.16% | 107% | 115% |
| Class E | -1.03% | -2.37% | -2.00% | 110% | 144% |
| Overall | -1.23% | -3.39% | -2.23% | 112% | 130% |
| Class D | -0.70% | -2.45% | -2.06% | 107% | 115% |
| Class F | -4.40% | -6.39% | -6.52% | 111% | 116% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.44% | -5.53% | -3.47% | 145% | 172% |
| Class A2 | -2.71% | -4.48% | -3.24% | 131% | 133% |
| Class B | -1.59% | -2.64% | -1.86% | 117% | 127% |
| Class C | -1.69% | -2.93% | -2.35% | 117% | 126% |
| Class E |  |  |  |  |  |
| Overall | -2.01% | -3.66% | -2.59% | 125% | 136% |
| Class D | -2.36% | -3.67% | -3.48% | 118% | 126% |
| Class F | -3.99% | -6.15% | -6.20% | 119% | 121% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.60% | -2.33% | -2.33% | 114% | 144% |
| Class C | -1.91% | -1.75% | -2.00% | 110% | 127% |
| Class E | -2.52% | -4.13% | -3.46% | 110% | 132% |
| Overall | -1.93% | -2.59% | -2.50% | 111% | 135% |
| Class D | -2.79% | -3.23% | -3.07% | 113% | 116% |
| Class F | -3.74% | -4.53% | -4.15% | 113% | 131% |

## Normative and non-normative base changes (test 1.3)

This is a combinational test of non-normative and normative changes described in the previous two sections.

EE software is configured as BASE_ENCODER = 1, BASE_NORMATIVE = 1, TOOLS = 0, config files provided in cfg/base/ are used with classA.cfg applied to classes A1 and A2.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.57% | -5.87% | -2.68% | 108% | 115% |
| Class A2 | -1.77% | -4.76% | -2.17% | 107% | 110% |
| Class B | -1.18% | -2.41% | -2.21% | 110% | 109% |
| Class C | -0.77% | -2.48% | -2.16% | 107% | 113% |
| Class E | -1.03% | -2.37% | -2.00% | 111% | 120% |
| Overall | -1.23% | -3.39% | -2.23% | 109% | 113% |
| Class D | -0.70% | -2.45% | -2.06% | 108% | 112% |
| Class F | -4.40% | -6.39% | -6.52% | 111% | 113% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.62% | -5.56% | -3.56% | 117% | 110% |
| Class A2 | -2.58% | -4.40% | -2.93% | 111% | 109% |
| Class B | -1.63% | -2.32% | -1.95% | 102% | 108% |
| Class C | -2.21% | -3.13% | -2.79% | 107% | 117% |
| Class E |  |  |  |  |  |
| Overall | -2.17% | -3.60% | -2.69% | 108% | 111% |
| Class D | -3.18% | -4.44% | -4.12% | 108% | 121% |
| Class F | -3.90% | -6.12% | -6.12% | 108% | 111% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.48% | -2.12% | -2.29% | 101% | 113% |
| Class C | -1.93% | -2.03% | -1.94% | 102% | 117% |
| Class E | -2.32% | -3.92% | -4.21% | 91% | 109% |
| Overall | -1.84% | -2.54% | -2.65% | 99% | 113% |
| Class D | -2.85% | -3.73% | -3.05% | 105% | 114% |
| Class F | -3.53% | -4.40% | -4.19% | 103% | 114% |

# Intra prediction

## Multi-model LM (test 2.2)

CCLM included in VVC is extended by adding three Multi-model LM (MMLM) modes, left and top, top-only, and left-only, as proposed in JVET-D0110. In each MMLM mode, the reconstructed neighboring samples are classified into two classes using a threshold which is the average of the luma reconstructed neighboring samples. The linear model of each class is derived by using the Least-Mean-Square (LMS) method, if enabled, or min/max method of VVC.

MMLM modes are added to the CCLM modes and are signaled with an index using truncated unary binarization.

EE software is configured as BASE_ENCODER = 0, BASE_NORMATIVE = 0, TOOLS = 0, MMLM = 1, config files are provided in cfg/tool-on/. Anchor is VTM-11.0.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.56% | -3.01% | -5.28% | 101% | 101% |
| Class A2 | -0.10% | -1.01% | -0.82% | 101% | 101% |
| Class B | -0.15% | -1.85% | -2.03% | 101% | 101% |
| Class C | -0.22% | -1.47% | -1.79% | 101% | 96% |
| Class E | -0.02% | -0.37% | -0.41% | 100% | 95% |
| Overall | -0.21% | -1.57% | -2.05% | 101% | 99% |
| Class D | -0.07% | -0.99% | -0.71% | 101% | 99% |
| Class F | -0.38% | -1.78% | -2.19% | 101% | 99% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.44% | -1.71% | -4.12% | 101% | 101% |
| Class A2 | -0.05% | -0.62% | -0.51% | 102% | 101% |
| Class B | -0.03% | -1.50% | -1.22% | 103% | 102% |
| Class C | -0.07% | -1.01% | -0.69% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -1.24% | -1.52% | 101% | 100% |
| Class D | -0.01% | -0.71% | -0.56% | 101% | 101% |
| Class F | -0.18% | -0.99% | -1.19% | 99% | 98% |

## Gradient PDPC (test 2.3)

In VVC, for a few scenarios, PDPC may not be applied due to the unavailability of the secondary reference samples. As proposed in JVET-Q0391, in these cases, a gradient based PDPC, extended from horizontal/vertical mode, is applied. The PDPC weights (wT / wL) and nScale parameter for determining the decay in PDPC weights with respect to the distance from left/top boundary are set equal to corresponding parameters in horizontal/vertical mode, respectively. When the secondary reference sample is at a fractional sample position, bilinear interpolation is applied.

EE software is configured as BASE_ENCODER = 0, BASE_NORMATIVE = 0, TOOLS = 0, GRAD_PDPC = 1, config files are provided in cfg/tool-on/. Anchor is VTM-11.0.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.09% | 0.00% | 101% | 100% |
| Class A2 | -0.09% | -0.05% | -0.13% | 100% | 98% |
| Class B | -0.10% | -0.11% | -0.16% | 102% | 100% |
| Class C | -0.03% | -0.06% | -0.04% | 101% | 104% |
| Class E | -0.13% | -0.20% | -0.26% | 103% | 105% |
| Overall | -0.08% | -0.10% | -0.12% | 101% | 102% |
| Class D | 0.01% | -0.08% | 0.04% | 101% | 97% |
| Class F | 0.02% | 0.02% | 0.00% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | -0.10% | 100% | 100% |
| Class A2 | -0.03% | -0.15% | -0.13% | 100% | 100% |
| Class B | -0.05% | -0.21% | 0.19% | 100% | 100% |
| Class C | -0.04% | 0.08% | 0.13% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.08% | 0.05% | 100% | 100% |
| Class D | 0.02% | 0.14% | 0.05% | 104% | 108% |
| Class F | 0.00% | -0.14% | -0.17% | 100% | 101% |

## Secondary MPM (test 2.4)

Secondary MPM list is introduced as described in JVET-D0114.The existing primary MPM (PMPM) list consists of 6 entries and the secondary MPM (SMPM) list includes 16 entries. A general MPM list with 22 entries is constructed first, and then the first 6 entries in this general MPM list are included into the PMPM list, and the rest of entries form the SMPM list. The first entry in the general MPM list is the Planar mode. The remaining entries are composed of the intra modes of the left (L), above (A), below-left (BL), above-right (AR), and above-left (AL) neighbouring blocks as shown in Figure 1, the directional modes with added offset from the first two available directional modes of neighbouring blocks, and the default modes.

If a CU block is vertically oriented, the order of neighbouring blocks is A, L, BL, AR, AL; otherwise, it is L, A, BL, AR, AL.

Figure 1. Neighbouring blocks (L, A, BL, AR, AL) used in the derivation of a general MPM list.

A PMPM flag is parsed first, if equal to 1 then a PMPM index is parsed to determine which entry of the PMPM list is selected, otherwise an SMPM flag is parsed to determine whether to parse the SMPM index or the remaining modes.

EE software is configured as BASE_ENCODER = 0, BASE_NORMATIVE = 0, TOOLS = 0, SECONDARY_MPM = 1, config files are provided in cfg/tool-on/. Anchor is VTM-11.0.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | -0.25% | -0.19% | 100% | 102% |
| Class A2 | -0.25% | -0.22% | -0.35% | 101% | 102% |
| Class B | -0.25% | -0.16% | -0.23% | 101% | 103% |
| Class C | -0.27% | -0.19% | -0.26% | 101% | 98% |
| Class E | -0.33% | -0.45% | -0.34% | 99% | 97% |
| Overall | -0.26% | -0.24% | -0.27% | 101% | 101% |
| Class D | -0.14% | -0.30% | -0.12% | 101% | 103% |
| Class F | -0.26% | -0.29% | -0.29% | 100% | 98% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.08% | -0.09% | 100% | 100% |
| Class A2 | -0.14% | 0.03% | -0.21% | 102% | 102% |
| Class B | -0.14% | -0.23% | 0.05% | 102% | 102% |
| Class C | -0.14% | 0.16% | 0.10% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.14% | -0.04% | -0.02% | 101% | 101% |
| Class D | -0.03% | 0.08% | 0.15% | 100% | 101% |
| Class F | -0.10% | -0.02% | 0.02% | 100% | 99% |

## Reference sample interpolation and smoothing for intra-prediction (test 2.5)

Three modifications are proposed. First, the 4-tap cubic interpolation is replaced with a 6-tap cubic interpolation filter, as described in JVET-D0119, to derive fractional samples from the reference samples. Second, for the coding blocks where a gaussian interpolation filter is applied to filter the reference samples, a 6-tap gaussian filter is applied for larger blocks (W >= 32 and H >=32) and the existing VVC 4-tap gaussian interpolation filter is used otherwise. Third, to derive the extended intra reference samples, the 4-tap interpolation filter is used instead of the nearest neighbor rounding.

EE software is configured as BASE_ENCODER = 0, BASE_NORMATIVE = 0, TOOLS = 0, INTRA_6TAP = 1, config files are provided in cfg/tool-on/. Anchor is VTM-11.0.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.21% | -0.03% | 104% | 101% |
| Class A2 | -0.03% | 0.02% | -0.05% | 102% | 98% |
| Class B | -0.12% | -0.08% | -0.13% | 104% | 102% |
| Class C | -0.24% | -0.28% | -0.29% | 103% | 103% |
| Class E | -0.26% | -0.35% | -0.45% | 105% | 104% |
| Overall | -0.14% | -0.17% | -0.19% | 104% | 102% |
| Class D | -0.22% | -0.20% | -0.05% | 103% | 102% |
| Class F | -0.10% | -0.10% | -0.07% | 102% | 101% |
|  | Random access Main10 |  |  |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.01% | -0.14% | 101% | 100% |
| Class A2 | -0.05% | -0.06% | -0.10% | 100% | 100% |
| Class B | -0.07% | -0.16% | 0.17% | 101% | 101% |
| Class C | -0.13% | 0.08% | 0.16% | 102% | 106% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | -0.04% | 0.05% | 101% | 102% |
| Class D | -0.12% | 0.04% | 0.08% | 105% | 112% |
| Class F | -0.04% | 0.07% | 0.08% | 101% | 100% |

## Decoder side intra mode derivation (test 2.6)

When decoder side intra mode derivation (DIMD) is applied, two intra modes are derived from the reconstructed neighbor samples, and those two predictors are combined with the planar mode predictor with the weights derived from the gradients as described in JVET-O0449.

Derived intra modes are included into the primary list of the intra most probable modes (MPM), so the DIMD process is performed before the MPM list is constructed. The primary derived intra mode of a DIMD block is stored with a block and is used for MPM list construction of the neighboring blocks.

EE software is configured as BASE_ENCODER = 0, BASE_NORMATIVE = 0, TOOLS = 0, ENABLE_DIMD = 1, config files are provided in cfg/tool-on/. Anchor is VTM-11.0.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.71% | -0.56% | -0.66% | 113% | 108% |
| Class A2 | -0.35% | -0.16% | -0.24% | 114% | 110% |
| Class B | -0.40% | -0.20% | -0.19% | 113% | 109% |
| Class C | -0.37% | -0.17% | -0.19% | 111% | 110% |
| Class E | -0.47% | -0.30% | -0.33% | 112% | 112% |
| Overall | -0.45% | -0.26% | -0.30% | 113% | 110% |
| Class D | -0.33% | -0.22% | -0.13% | 111% | 110% |
| Class F | -0.25% | -0.06% | -0.07% | 106% | 108% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.39% | -0.17% | -0.38% | 103% | 107% |
| Class A2 | -0.21% | -0.31% | -0.29% | 103% | 105% |
| Class B | -0.23% | -0.32% | -0.11% | 101% | 107% |
| Class C | -0.23% | -0.04% | -0.08% | 100% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.26% | -0.21% | -0.19% | 102% | 106% |
| Class D | -0.13% | 0.04% | -0.09% | 102% | 107% |
| Class F | -0.18% | -0.13% | -0.06% | 103% | 110% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | -0.17% | -0.28% | 104% | 120% |
| Class C | -0.01% | -0.23% | -0.29% | 102% | 119% |
| Class E | -0.18% | -0.77% | -0.02% | 103% | 118% |
| Overall | -0.07% | -0.34% | -0.22% | 103% | 119% |
| Class D | 0.03% | 0.53% | 0.35% | 102% | 116% |
| Class F | -0.05% | 0.04% | 0.48% | 104% | 123% |

# Inter prediction

## Local illumination compensation (test 3.1)

Local illumination compensation (LIC) is an inter prediction technique to model local illumination variation between a current block and its prediction block as a function of the current block and the reference block templates. The parameters of the function can be denoted by a scale α and an offset β, which forms a linear equation, that is, α*p[x]+β to compensate illumination changes, where p[x] is a reference sample pointed to by MV at a location x in the reference picture. Since α and β can be derived based on the current block and the reference block templates, no signaling overhead is required for them, except that an LIC flag is signaled for AMVP mode to indicate the use of LIC.

The local illumination compensation that was previously proposed in JVET-O0066 was adopted in this contribution for uni-prediction inter CUs with the following modifications.

- Intra neighbor samples can be used in LIC parameter derivation;

- LIC is disabled for blocks with less than 32 luma samples;

- For both non-subblock and affine modes, LIC parameter derivation is performed based on the template block samples corresponding to the current CU, instead of the partial template block samples corresponding to the first top-left 16x16 unit;

- Samples of the reference block template are generated by using MC with the block MV without rounding it to the integer-pel precision.

EE software is configured as BASE_ENCODER = 0, BASE_NORMATIVE = 0, TOOLS = 0, INTER_LIC = 1, config files are provided in cfg/tool-on/. Anchor is VTM-11.0.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.30% | -0.54% | -0.94% | 121% | 99% |
| Class A2 | -1.00% | -0.64% | -0.81% | 124% | 101% |
| Class B | -0.84% | -0.48% | -0.43% | 123% | 100% |
| Class C | -0.77% | -0.41% | -0.26% | 122% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.94% | -0.51% | -0.56% | 122% | 100% |
| Class D | -0.43% | -0.04% | -0.17% | 119% | 100% |
| Class F | -1.37% | -0.75% | -0.92% | 109% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.90% | -0.20% | -0.78% | 127% | 101% |
| Class C | -0.82% | -0.39% | -0.60% | 128% | 98% |
| Class E | -0.18% | -0.75% | 0.12% | 104% | 103% |
| Overall | -0.69% | -0.40% | -0.50% | 121% | 101% |
| Class D | -0.36% | -0.09% | 0.43% | 124% | 101% |
| Class F | -3.13% | -2.46% | -1.88% | 116% | 105% |

## Non-adjacent spatial candidate (test 3.2)

The non-adjacent spatial merge candidates as described in JVET-L0399 are inserted after the TMVP in the regular merge candidate list. The pattern of the spatial merge candidates is shown on Figure 2. The distances between the non-adjacent spatial candidates and the current coding block are based on the width and height of the current coding block.

Figure 2. Spatial neighboring blocks used to derive the spatial merge candidates

EE software is configured as BASE_ENCODER = 0, BASE_NORMATIVE = 0, TOOLS = 0, NON_ADJACENT_MRG_CAND = 1, config files are provided in cfg/tool-on/ and the parameter “MaxNumMergeCand : 10” is added. Anchor is VTM-11.0.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | -0.42% | -0.43% | 100% | 98% |
| Class A2 | -0.49% | -0.48% | -0.55% | 104% | 91% |
| Class B | -0.48% | -0.54% | -0.39% | 100% | 103% |
| Class C | -0.34% | -0.27% | -0.28% | 104% | 113% |
| Class E |  |  |  |  |  |
| Overall | -0.41% | -0.44% | -0.40% | 102% | 102% |
| Class D | -0.13% | -0.12% | -0.15% | 104% | 112% |
| Class F | -0.92% | -1.05% | -1.06% | 106% | 122% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.64% | -0.62% | -0.85% | 98% | 93% |
| Class C | -0.54% | -0.72% | -0.54% | 104% | 99% |
| Class E | -0.55% | -0.75% | -0.49% | 101% | 92% |
| Overall | -0.58% | -0.69% | -0.65% | 101% | 95% |
| Class D | -0.29% | -0.04% | -0.78% | 104% | 97% |
| Class F | -0.88% | -0.70% | -0.47% | 109% | 126% |

## Template matching (test 3.3)

Template matching (TM) is a decoder-side MV derivation method to refine the motion information of the current CU by finding the closest match between a template (i.e., top and/or left neighbouring blocks of the current CU) in the current picture and a block (i.e., same size to the template) in a reference picture. As illustrated in Figure 3, a better MV is to be searched around the initial motion of the current CU within a [– 8, +8]-pel search range. The template matching that was previously proposed in JVET-J0021 is adopted in this contribution with two modifications: search step size is determined based on AMVR mode and TM can be cascaded with bilateral matching process in merge modes.

Figure 3. Template matching performs on a search area around initial MV.

In AMVP mode, an MVP candidate is determined based on the template matching error to pick up the one which reaches the minimum difference between the current block and the reference block templates, and then TM performs only for this particular MVP candidate for MV refinement. TM refines this MVP candidate, starting from full-pel MVD precision (or 4-pel for 4-pel AMVR mode) within a [–8, +8]-pel search range by using iterative diamond search. The AMVP candidate may be further refined by using cross search with full-pel MVD precision (or 4-pel for 4-pel AMVR mode), followed sequentially by half-pel and quarter-pel ones depending on AMVR mode as specified in Table 1. This search process ensures that the MVP candidate still keeps the same MV precision as indicated by AMVR mode after TM process.

Table 1. Search patterns of AMVR and merge mode with AMVR.

| Search pattern | AMVR mode |  |  |  | Merge mode |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 4-pel | Full-pel | Half-pel | Quarter-pel | AltIF=0 | AltIF=1 |
| 4-pel diamond | v |  |  |  |  |  |
| 4-pel cross | v |  |  |  |  |  |
| Full-pel diamond |  | v | v | v | v | v |
| Full-pel cross |  | v | v | v | v | v |
| Half-pel cross |  |  | v | v | v | v |
| Quarter-pel cross |  |  |  | v | v |  |
| 1/8-pel cross |  |  |  |  | v |  |

In the merge mode, similar search method is applied to the merge candidate indicated by the merge index. As Table 1 shows, TM may perform all the way down to 1/8-pel MVD precision or skipping those beyond half-pel MVD precision, depending on whether the alternative interpolation filter (that is used when AMVR is of half-pel mode) is used according to merged motion information. Besides, when TM mode is enabled, template matching may work as an independent process or an extra MV refinement process between block-based and subblock-based bilateral matching (BM) methods, depending on whether BM can be enabled or not according to its enabling condition check. When BM and TM are both enabled for a CU, the search process of TM stops at half-pel MVD precision and the resulted MVs are further refined by using the same model-based MVD derivation method as in DMVR.

EE software is configured as BASE_ENCODER = 0, BASE_NORMATIVE = 0, TOOLS = 0, TM_AMVP = 1, TM_MRG = 1, config files are provided in cfg/tool-on/. Anchor is VTM-11.0.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.12% | -2.41% | -2.88% | 115% | 112% |
| Class A2 | -2.96% | -2.98% | -3.00% | 115% | 114% |
| Class B | -2.03% | -2.29% | -2.23% | 115% | 113% |
| Class C | -2.11% | -2.28% | -2.48% | 117% | 119% |
| Class E |  |  |  |  |  |
| Overall | -2.26% | -2.45% | -2.58% | 116% | 115% |
| Class D | -1.99% | -2.05% | -2.08% | 115% | 121% |
| Class F | -0.91% | -0.98% | -1.15% | 109% | 106% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.62% | -2.35% | -2.46% | 129% | 118% |
| Class C | -1.78% | -1.87% | -2.46% | 130% | 118% |
| Class E | -1.57% | -1.67% | -0.94% | 116% | 115% |
| Overall | -1.66% | -2.02% | -2.08% | 126% | 117% |
| Class D | -1.59% | -1.21% | -1.90% | 130% | 128% |
| Class F | -1.22% | -1.28% | -1.51% | 118% | 115% |

## Multi-pass decoder-side motion vector refinement (test 3.4)

In this contribution, a multi-pass decoder-side motion vector refinement is applied instead of DMVR. In the first pass, bilateral matching (BM) is applied to a coding block. In the second pass, BM is applied to each 16x16 subblock within the coding block. In the third pass, MV in each 8x8 subblock is refined by applying bi-directional optical flow (BDOF). The refined MVs are stored for both spatial and temporal motion vector prediction.

### First pass – Block based bilateral matching MV refinement

In the first pass, a refined MV is derived by applying BM to a coding block. Similar to decoder-side motion vector refinement (DMVR), the refined MV is searched around the two initial MVs (MV0 and MV1) in the reference picture lists L0 and L1. The refined MVs (MV0_pass1 and MV1_pass1) are derived around the initiate MVs based on the minimum bilateral matching cost between the two reference blocks in L0 and L1.

BM performs local search to derive integer sample precision intDeltaMV and half-pel sample precision halfDeltaMv. The local search applies a 3×3 square search pattern to loop through the search range [–sHor, sHor] in a horizontal direction and [–sVer, sVer] in a vertical direction, wherein, the values of sHor and sVer are determined by the block dimension, and the maximum value of sHor and sVer is 8.

The bilateral matching cost is calculated as: bilCost = mvDistanceCost + sadCost. When the block size cbW * cbH is greater than 64, MRSAD cost function is applied to remove the DC effect of the distortion between the reference blocks. When the bilCost at the center point of the 3×3 search pattern has the minimum cost, the intDeltaMV or halfDeltaMV local search is terminated. Otherwise, the current minimum cost search point becomes the new center point of the 3×3 search pattern and the search for the minimum cost continues, until it reaches the end of the search range.

The existing fractional sample refinement is further applied to derive the final deltaMV. The refined MVs after the first pass are then derived as:

- MV0_pass1 = MV0 + deltaMV

- MV1_pass1 = MV1 – deltaMV

### Second pass – Subblock based bilateral matching MV refinement

In the second pass, a refined MV is derived by applying BM to a 16×16 grid subblock. For each subblock, the refined MV is searched around the two MVs (MV0_pass1 and MV1_pass1), obtained on the first pass for the reference picture list L0 and L1. The refined MVs (MV0_pass2(sbIdx2) and MV1_pass2(sbIdx2)) are derived based on the minimum bilateral matching cost between the two reference subblocks in L0 and L1.

For each subblock, BM performs full search to derive integer sample precision intDeltaMV. The full search has a search range [–sHor, sHor] in a horizontal direction and [–sVer, sVer] in a vertical direction, wherein, the values of sHor and sVer are determined by the block dimension, and the maximum value of sHor and sVer is 8.

The bilateral matching cost is calculated by applying a cost factor to the SATD cost between the two reference subblocks, as: bilCost = satdCost * costFactor. The search area (2*sHor + 1) * (2*sVer + 1) is divided up to 5 diamond shape search regions shown on Figure 4. Each search region is assigned a costFactor, which is determined by the distance (intDeltaMV) between each search point and the starting MV, and each diamond region is processed in the order starting from the center of the search area. In each region, the search points are processed in the raster scan order starting from the top left going to the bottom right corner of the region.  When the minimum bilCost within the current search region is less than a threshold equal to sbW * sbH, the int-pel full search is terminated, otherwise, the int-pel full search continues to the next search region until all search points are examined.

Figure 4. Diamond regions in the search area.

BM performs local search to derive half sample precision halfDeltaMv. The search pattern and cost function are the same as defined in 4.4.1.

The existing VVC DMVR fractional sample refinement is further applied to derive the final deltaMV(sbIdx2). The refined MVs at second pass is then derived as:

- MV0_pass2(sbIdx2) = MV0_pass1 + deltaMV(sbIdx2)

- MV1_pass2(sbIdx2) = MV1_pass1 – deltaMV(sbIdx2)

### Third pass – Subblock based bi-directional optical flow MV refinement

In the third pass, a refined MV is derived by applying BDOF to an 8×8 grid subblock. For each 8×8 subblock, BDOF refinement is applied to derive scaled Vx and Vy without clipping starting from the refined MV of the parent subblock of the second pass. The derived bioMv(Vx, Vy) is rounded to 1/16 sample precision and clipped between -32 and 32.

The refined MVs (MV0_pass3(sbIdx3) and MV1_pass3(sbIdx3)) at third pass are derived as:

- MV0_pass3(sbIdx3) = MV0_pass2(sbIdx2) + bioMv

- MV1_pass3(sbIdx3) = MV0_pass2(sbIdx2) – bioMv

EE software is configured as BASE_ENCODER = 0, BASE_NORMATIVE = 0, TOOLS = 0, MULTI_PASS_DMVR = 1, config files are provided in cfg/tool-on/. Anchor is VTM-11.0.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.75% | -1.78% | -1.76% | 113% | 182% |
| Class A2 | -2.91% | -2.69% | -2.84% | 112% | 233% |
| Class B | -1.41% | -1.31% | -1.25% | 112% | 200% |
| Class C | -1.44% | -1.36% | -1.33% | 110% | 199% |
| Class E |  |  |  |  |  |
| Overall | -1.79% | -1.69% | -1.69% | 112% | 202% |
| Class D | -1.57% | -1.15% | -1.22% | 108% | 206% |
| Class F | -0.56% | -0.76% | -0.73% | 107% | 156% |

## Sample-based BDOF (test 3.5)

In the sample-based BDOF, instead of deriving motion refinement (Vx, Vy) on a block basis, it is performed per sample.

The coding block is divided into 8×8 subblocks. For each subblock, whether to apply BDOF or not is determined by checking the SAD between the two reference subblocks against a threshold. If decided to apply BDOF to a subblock, for every sample in the subblock, a sliding 5×5 window is used and the existing BDOF process is applied for every sliding window to derive Vx and Vy. The derived motion refinement (Vx, Vy) is applied to adjust the bi-predicted sample value for the center sample of the window.

This test is performed together with multi-pass DMVR. EE software is configured as BASE_ENCODER = 0, BASE_NORMATIVE = 0, TOOLS = 0, MULTI_PASS_DMVR = 1, SAMPLE_BASED_BDOF = 1, config files are provided in cfg/tool-on/. Anchor is VTM-11.0.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.75% | -1.78% | -1.84% | 114% | 185% |
| Class A2 | -2.94% | -2.66% | -2.75% | 113% | 237% |
| Class B | -1.50% | -1.36% | -1.29% | 113% | 203% |
| Class C | -1.57% | -1.38% | -1.45% | 111% | 199% |
| Class E |  |  |  |  |  |
| Overall | -1.85% | -1.71% | -1.74% | 113% | 204% |
| Class D | -1.79% | -1.39% | -1.32% | 109% | 206% |
| Class F | -0.60% | -0.62% | -0.68% | 109% | 158% |

## Overlapped block motion compensation (test 3.6)

When overlapped block motion compensation (OBMC) is applied, top and left boundary pixels of a CU are refined using neighboring block’s motion information with a weighted prediction as described in JVET-L0101. An OBMC flag is inherited to be true for merge mode and is signaled to indicate the mode otherwise.

Conditions of not applying OBMC are as follows:

- When OBMC is disabled at SPS level

- When current block has intra mode or IBC mode

- When current block applies LIC

- When current luma block area is smaller or equal to 32

- OBMC flag is false

A subblock-boundary OBMC is further proposed to apply the same blending to the top, left, bottom, and right subblock boundary pixels using neighboring subblock’s motion information. It is enabled for the subblock based coding tools:

- Affine AMVP modes;

- Affine merge modes and subblock-based temporal motion vector prediction (SbTMVP);

- Subblock-based bilateral matching.

EE software is configured as BASE_ENCODER = 0, BASE_NORMATIVE = 0, TOOLS = 0, ENABLE_OBMC = 1, config files are provided in cfg/tool-on/. Anchor is VTM-11.0.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | -0.65% | -0.82% | 110% | 153% |
| Class A2 | -0.35% | -1.47% | -1.57% | 112% | 179% |
| Class B | -0.44% | -1.25% | -0.99% | 110% | 157% |
| Class C | -0.82% | -1.64% | -1.55% | 110% | 153% |
| Class E |  |  |  |  |  |
| Overall | -0.47% | -1.28% | -1.22% | 110% | 159% |
| Class D | -0.68% | -1.66% | -1.80% | 112% | 166% |
| Class F | -0.10% | -0.82% | -0.84% | 108% | 159% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.77% | -1.43% | -1.55% | 113% | 162% |
| Class C | -1.19% | -1.70% | -1.98% | 112% | 158% |
| Class E | -0.99% | -1.53% | -1.25% | 112% | 159% |
| Overall | -0.96% | -1.54% | -1.62% | 113% | 160% |
| Class D | -1.54% | -1.90% | -2.96% | 113% | 160% |
| Class F | 0.08% | -0.74% | 0.26% | 110% | 136% |

## 12-tap interpolation (test 3.7)

The 8-tap interpolation filter used in VVC is replaced with a 12-tap filter in this proposal. The proposed interpolation filter is derived from the sinc function of which the frequency response is cut off at Nyquist frequency, and cropped by a cosine window function. Table 2 gives the filter coefficients of all 16 phases. Figure 5. compares the frequency responses of the proposed interpolation filter with the VVC interpolation filter, both at half-pel phase.

Table 2. Filter coefficients of the proposed 12-tap interpolation filter

| 1/16 | -1 | 2 | -3 | 6 | -14 | 254 | 16 | -7 | 4 | -2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2/16 | -1 | 3 | -7 | 12 | -26 | 249 | 35 | -15 | 8 | -4 | 2 | 0 |
| 3/16 | -2 | 5 | -9 | 17 | -36 | 241 | 54 | -22 | 12 | -6 | 3 | -1 |
| 4/16 | -2 | 5 | -11 | 21 | -43 | 230 | 75 | -29 | 15 | -8 | 4 | -1 |
| 5/16 | -2 | 6 | -13 | 24 | -48 | 216 | 97 | -36 | 19 | -10 | 4 | -1 |
| 6/16 | -2 | 7 | -14 | 25 | -51 | 200 | 119 | -42 | 22 | -12 | 5 | -1 |
| 7/16 | -2 | 7 | -14 | 26 | -51 | 181 | 140 | -46 | 24 | -13 | 6 | -2 |
| 8/16 | -2 | 6 | -13 | 25 | -50 | 162 | 162 | -50 | 25 | -13 | 6 | -2 |
| 9/16 | -2 | 6 | -13 | 24 | -46 | 140 | 181 | -51 | 26 | -14 | 7 | -2 |
| 10/16 | -1 | 5 | -12 | 22 | -42 | 119 | 200 | -51 | 25 | -14 | 7 | -2 |
| 11/16 | -1 | 4 | -10 | 19 | -36 | 97 | 216 | -48 | 24 | -13 | 6 | -2 |
| 12/16 | -1 | 4 | -8 | 15 | -29 | 75 | 230 | -43 | 21 | -11 | 5 | -2 |
| 13/16 | -1 | 3 | -6 | 12 | -22 | 54 | 241 | -36 | 17 | -9 | 5 | -2 |
| 14/16 | 0 | 2 | -4 | 8 | -15 | 35 | 249 | -26 | 12 | -7 | 3 | -1 |
| 15/16 | 0 | 1 | -2 | 4 | -7 | 16 | 254 | -14 | 6 | -3 | 2 | -1 |

Figure 5. Frequency responses of the proposed interpolation filter and the VVC interpolation filter at half-pel phase

EE software is configured as BASE_ENCODER = 0, BASE_NORMATIVE = 0, TOOLS = 0, IF_12TAP = 1, IF_12TAP_SIMD = 1, SIMD_4x4_12 = 1, config files are provided in cfg/tool-on/. Anchor is VTM-11.0.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.03% | -0.09% | 104% | 103% |
| Class A2 | -0.23% | -0.02% | -0.20% | 104% | 104% |
| Class B | -0.37% | -0.23% | -0.02% | 105% | 104% |
| Class C | -1.99% | -0.89% | -0.56% | 103% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.71% | -0.31% | -0.21% | 104% | 103% |
| Class D | -3.45% | -1.52% | -1.29% | 101% | 105% |
| Class F | -0.15% | 0.15% | 0.12% | 102% | 102% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.37% | -0.11% | -0.05% | 104% | 103% |
| Class C | -2.19% | -1.46% | -1.26% | 103% | 103% |
| Class E | -0.61% | -1.00% | -0.31% | 104% | 102% |
| Overall |  |  |  |  |  |
| Class D | -4.62% | -2.86% | -1.98% | 100% | 102% |
| Class F | -0.34% | 0.27% | 0.21% | 102% | 102% |

## Multi-hypothesis prediction (test 3.8)

The multi-hypothesis prediction (MHP) previously proposed in JVET-M0425 is adopted in this contribution. Up to two additional predictors are signalled on top of inter AMVP mode, regular merge mode, affine merge and MMVD mode. The resulting overall prediction signal is accumulated iteratively with each additional prediction signal.

pn+1=1-αn+1pn+αn+1hn+1

The weighting factor α is specified according to the following table:

| add_hyp_weight_idx | α |
| --- | --- |
| 0 | 1/4 |
| 1 | -1/8 |

For inter AMVP mode, MHP is only applied if non-equal weight in BCW is selected in bi-prediction mode.

The additional hypothesis can be either merge or AMVP mode. In the case of merge mode, the motion information is indicated by a merge index, and the merge candidate list is the same as in the Geometric Partition Mode. In the case of AMVP mode, the reference index, MVP index, and MVD are signaled.

EE software is configured as BASE_ENCODER = 0, BASE_NORMATIVE = 0, TOOLS = 0, MULTI_HYP_PRED = 1, config files provided in cfg/tool-on/. Anchor is VTM-11.0.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.19% | -0.30% | 104% | 103% |
| Class A2 | -0.06% | -0.05% | -0.06% | 108% | 95% |
| Class B | -0.59% | -0.60% | -0.43% | 108% | 113% |
| Class C | -0.32% | -0.20% | 0.01% | 110% | 117% |
| Class E |  |  |  |  |  |
| Overall | -0.31% | -0.30% | -0.21% | 108% | 108% |
| Class D | -0.24% | 0.16% | 0.27% | 112% | 124% |
| Class F | -0.95% | -0.77% | -0.77% | 112% | 133% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.71% | -0.56% | -0.22% | 105% | 98% |
| Class C | -0.36% | -0.20% | -0.29% | 112% | 108% |
| Class E | -0.29% | 0.60% | 0.91% | 111% | 109% |
| Overall | -0.49% | -0.15% | 0.04% | 109% | 104% |
| Class D | -0.34% | 0.62% | -0.03% | 108% | 103% |
| Class F | -1.43% | -1.02% | -0.74% | 110% | 135% |

## Affine MMVD (test 3.9)

The affine MMVD mode that was previously proposed in JVET-N0378 is adopted in this contribution. The notion of MMVD that applies a distance offset to a base candidate motion is extended to the CPMVs of affine merge mode. In Affine MMVD mode, there is only one base vector candidate which is the first affine candidate in the subblock merge candidate list, and thus there is no base vector candidate flag to be signaled. There are three sets of distance offsets to be selected based on the sequence resolution, as follows:

- {1/16, 1/8, 1/4, 1/2, 1} for sequences smaller than 1280x720;

- {1/4, 1/2, 1, 2, 4} for sequences smaller than 2560x1600;

- {1/2, 1, 2, 4, 8} for sequences larger than or equal to 2560x1600.

Same as MMVD, affine MMVD also supports 4 directions (i.e., (+,0), (-,0), (0,+), (0,-)) for each of its distance offsets. When the base candidate is of uni-prediction, the offset vector (i.e., distance offset times direction) is added up equally to each CPMV. For the bi-prediction candidate, the offset vector is added up equally to L0 CPMVs, while the offset vector is firstly mirrored based on POC distance and is then added up to L1 CPMVs.

EE software is configured as BASE_ENCODER = 0, BASE_NORMATIVE = 0, TOOLS = 0, AFFINE_MMVD = 1, config files are provided in cfg/tool-on/. Anchor is VTM-11.0.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -0.09% | -0.13% | 109% | 100% |
| Class A2 | -0.26% | -0.17% | -0.18% | 110% | 99% |
| Class B | -0.15% | -0.13% | 0.08% | 110% | 98% |
| Class C | -0.14% | -0.15% | 0.05% | 110% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.17% | -0.13% | -0.02% | 110% | 101% |
| Class D | -0.11% | 0.03% | -0.09% | 111% | 106% |
| Class F | -0.14% | -0.12% | -0.17% | 109% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.10% | 0.03% | -0.29% | 112% | 101% |
| Class C | -0.18% | 0.12% | -0.14% | 110% | 102% |
| Class E | -0.14% | -0.40% | 0.02% | 115% | 104% |
| Overall | -0.13% | -0.05% | -0.16% | 112% | 102% |
| Class D | -0.38% | -0.22% | -0.25% | 111% | 105% |
| Class F | -0.24% | -0.60% | 0.27% | 110% | 103% |

## CIIP with PDPC blending (test 3.10)

The CIIP mode is extended as described in JVET-O0537. In this extended mode (CIIP_PDPC), the prediction of the regular merge mode is refined using the above (Rx, -1) and left (R-1, y) reconstructed samples. This refinement inherits the position dependent prediction combination (PDPC) scheme. The flowchart of the prediction of the CIIP_PDPC mode can be depicted as in Figure 6, where WT and WL are the weighted values which depend on the sample position in the block as defined in PDPC.

The CIIP_PDPC mode is signaled together with CIIP mode. When CIIP flag is true, another flag, namely CIIP_PDPC flag, is further signaled to indicate whether to use CIIP_PDPC.

Figure 6. CIIP_PDPC flowchart of the extended CIIP mode using PDPC

EE software is configured as BASE_ENCODER = 0, BASE_NORMATIVE = 0, TOOLS = 0, CIIP_PDPC = 1, config files are provided in cfg/tool-on/. Anchor is VTM-11.0.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.03% | -0.03% | 101% | 100% |
| Class A2 | -0.10% | -0.07% | 0.01% | 100% | 100% |
| Class B | -0.11% | -0.09% | 0.06% | 101% | 102% |
| Class C | -0.14% | -0.14% | 0.02% | 102% | 107% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.09% | 0.02% | 101% | 102% |
| Class D | -0.02% | 0.14% | 0.06% | 101% | 103% |
| Class F | -0.03% | 0.03% | 0.02% | 101% | 102% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.20% | -0.40% | -0.71% | 100% | 99% |
| Class C | -0.21% | -0.11% | -0.20% | 98% | 96% |
| Class E | -0.12% | -0.37% | 0.05% | 100% | 97% |
| Overall | -0.18% | -0.30% | -0.35% | 99% | 97% |
| Class D | -0.11% | 0.25% | -0.13% | 103% | 107% |
| Class F | -0.23% | -0.25% | 0.12% | 100% | 99% |

# Transform and coefficient coding

Both CTU size and maximum transform size (i.e., all MTS transform kernels) are extended to 256, where the maximum intra coded block can have a size of 128x128. The maximum CTU size is set to 256 for UHD sequences and it is set to 128, otherwise. In the primary transformation process, there is no normative zeroing out operation applied on transform coefficients. However, if LFNST is applied, the primary transform coefficients outside the LFNST region are normatively zeroed-out, yet the coefficients within the LFNST region (i.e., secondary transform coefficients) are not zeroed-out.

## 8-states DQ (test 4.1)

The 8-state dependent quantization proposed in JVET-Q0243 is employed in this contribution.

EE software is configured as BASE_ENCODER = 0, BASE_NORMATIVE = 0, TOOLS = 0, TCQ_8STATES = 1, config files are provided in cfg/tool-on/ and with additionally added “DepQuant : 2”. Anchor is VTM-11.0.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.55% | 0.23% | 0.26% | 110% | 98% |
| Class A2 | -0.74% | 0.14% | 0.01% | 113% | 98% |
| Class B | -0.44% | 0.23% | 0.31% | 114% | 100% |
| Class C | -0.26% | 0.01% | 0.10% | 123% | 104% |
| Class E | -0.26% | 0.14% | 0.03% | 117% | 100% |
| Overall | -0.44% | 0.15% | 0.16% | 115% | 100% |
| Class D | -0.26% | -0.02% | 0.16% | 121% | 100% |
| Class F | -0.10% | -0.18% | -0.03% | 114% | 104% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.36% | 0.08% | 0.14% | 105% | 100% |
| Class A2 | -0.36% | -0.02% | -0.11% | 106% | 100% |
| Class B | -0.37% | -0.04% | 0.42% | 108% | 101% |
| Class C | -0.24% | 0.06% | 0.14% | 111% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.33% | 0.02% | 0.18% | 108% | 101% |
| Class D | -0.15% | 0.24% | 0.34% | 109% | 98% |
| Class F | -0.11% | -0.05% | 0.04% | 112% | 103% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.55% | -0.15% | 0.14% | 106% | 100% |
| Class C | -0.41% | 0.04% | 0.23% | 109% | 104% |
| Class E | -0.18% | -0.43% | 0.63% | 103% | 98% |
| Overall | -0.41% | -0.16% | 0.29% | 106% | 101% |
| Class D | -0.35% | 0.14% | 0.39% | 105% | 94% |
| Class F | -0.27% | -0.38% | 0.54% | 108% | 103% |

## Extended LFNST (test 4.2)

The LFNST design in VVC is extended as follows:

- The number of LFNST sets (S) and candidates (C) are extended to S=35 and C=3, and the LFNST set (lfnstTrSetIdx) for a given intra mode (predModeIntra) is derived according to the following formula:

  - For predModeIntra < 2, lfnstTrSetIdx is equal to 2

  - lfnstTrSetIdx = predModeIntra, for predModeIntra in [0,34]

  - lfnstTrSetIdx = 68 – predModeIntra, for predModeIntra in [35,66]

- The 8x8 LFNST matrix size (i.e., the transform support) in VVC is extended to full 64x64, such that it is not reduced as no zeroing-out process is applied to secondary transform coefficients.

EE software is configured as BASE_ENCODER = 0, BASE_NORMATIVE = 0, TOOLS = 0, EXTENDED_LFNST = 1, config files are provided in cfg/tool-on. Anchor is VTM-11.0.

LFNST is not used in LB test configuration in CTC.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.55% | -0.01% | -0.04% | 122% | 102% |
| Class A2 | -0.47% | -0.51% | -0.79% | 123% | 103% |
| Class B | -0.68% | -0.63% | -0.55% | 122% | 99% |
| Class C | -1.33% | -0.99% | -0.93% | 123% | 102% |
| Class E | -1.01% | -0.88% | -0.56% | 123% | 104% |
| Overall | -0.82% | -0.63% | -0.59% | 122% | 102% |
| Class D | -0.80% | -0.75% | -0.93% | 123% | 100% |
| Class F | -1.02% | -0.71% | -0.65% | 110% | 101% |
|  | Random access Main10 |  |  |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.40% | -0.15% | -0.21% | 108% | 100% |
| Class A2 | -0.31% | -0.32% | -0.21% | 106% | 99% |
| Class B | -0.42% | -0.57% | -0.28% | 106% | 100% |
| Class C | -0.70% | -0.46% | -0.39% | 107% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.47% | -0.41% | -0.28% | 106% | 101% |
| Class D | -0.40% | -0.30% | -0.41% | 107% | 107% |
| Class F | -0.49% | -0.40% | -0.40% | 107% | 104% |

## Transform sign prediction (test 4.3)

Following JVET-D0031 and JVET-J0021, basic idea of the coefficient sign prediction method is for applicable transform coefficients to calculate reconstructed residual for both negative and positive sign combinations and select the hypothesis that minimizes a cost function.

To derive the best sign, the cost function is defined as discontinuity measure across block boundary shown on Figure 7. It is measured for all hypotheses, and the one with the smallest cost is selected as a predictor for coefficient signs.

Figure 7. Discontinuity measure.

The cost function is defined as a sum of absolute second derivatives in the residual domain for the above row and left column as follows

cost=x=ow-Rx,-1+2Rx,0-Px,1-rx,1 +y=oh-R-1,y+2R0,y-P1,y-r1,y

where R is reconstructed neighbors, P is prediction of the current block, and r is the residual hypothesis. The term -R-1+2R0-P1 can be calculated only once per block and only residual hypothesis is subtracted.

Up to 8 signs are predicted and for those coefficients, instead of the coefficient sign, a bin indicating whether the sign is equal to the selected hypothesis is signaled. This sing prediction bin is context coded, where context is derived whether a coefficient is DC or not, and contexts are separated for intra and inter blocks, for luma and chroma components.

EE software is configured as BASE_ENCODER = 0, BASE_NORMATIVE = 0, TOOLS = 0, SIGN_PREDICTION = 1, ENABLE_SIMD_SIGN_PREDICTION = 1, config files are provided in cfg/tool-on/. Anchor is VTM-11.0.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | -0.66% | -0.60% | 115% | 119% |
| Class A2 | -0.28% | -0.47% | -0.44% | 116% | 120% |
| Class B | -0.27% | -0.39% | -0.59% | 118% | 121% |
| Class C | -0.25% | -0.45% | -0.45% | 122% | 127% |
| Class E | -0.41% | -0.62% | -0.67% | 120% | 123% |
| Overall | -0.29% | -0.50% | -0.55% | 118% | 122% |
| Class D | -0.22% | -0.56% | -0.44% | 124% | 136% |
| Class F | -0.30% | -0.43% | -0.45% | 114% | 120% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.17% | -0.95% | -1.31% | 108% | 109% |
| Class A2 | -0.88% | -0.98% | -1.05% | 108% | 107% |
| Class B | -0.79% | -0.94% | -0.80% | 108% | 107% |
| Class C | -0.66% | -0.57% | -0.62% | 111% | 114% |
| Class E |  |  |  |  |  |
| Overall | -0.85% | -0.85% | -0.90% | 109% | 109% |
| Class D | -0.47% | -0.31% | -0.46% | 111% | 115% |
| Class F | -0.66% | -0.69% | -0.72% | 110% | 107% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.82% | -0.64% | -0.70% | 108% | 111% |
| Class C | -0.71% | -0.50% | -0.98% | 109% | 112% |
| Class E | -0.79% | -1.09% | -0.37% | 104% | 103% |
| Overall | -0.78% | -0.71% | -0.71% | 107% | 109% |
| Class D | -0.42% | 0.07% | -0.42% | 113% | 120% |
| Class F | -0.87% | -1.15% | -1.22% | 106% | 107% |

# Adaptive loop filter (test 5.1)

### ALF simplification removal

In this proposal, ALF gradient subsampling and ALF virtual boundary processing are removed. Block size for classification is reduced from 4x4 to 2x2. Filter size for both luma and chroma, for which ALF coefficients are signalled, is increased to 9x9.

### ALF with fixed filters

To filter a luma sample, it is proposed to use three different classifiers (C0, C1, and C2) and three different sets of filters (F0, F1, and F2). Sets F0 and F1 contain fixed filters, with coefficients trained for classifiers C0 and C1. Coefficients of filters in F2 are signalled. Which filter from a set Fi is used for a given sample is decided by a class Ci assigned to this sample using classifier Ci.

#### Filtering

At first, two 13x13 diamond shape fixed filters F0 and F1 are applied to derive two intermediate samples R0x,y and R1x,y. After that, F2 is applied to R0x,y, R1x,y, and neighboring samples to derive a filtered sample as

Rx,y=Rx, y+i=019cifi,0+fi,1+i=2021cigi,

where fi,j  is the clipped difference between a neighboring sample and current sample Rx, y and gi is the clipped difference between Ri-20x,y and current sample. The filter coefficients ci, i=0,…21, are signalled.

#### Classification

Based on directionality Di and activity Ai, a class Ci is assigned to each 2x2 block:

Ci=Ai *MD,i+Di

where MD,i represents the total number of directionalities Di.

As in VVC, values of the horizontal, vertical, and two diagonal gradients are calculated for each sample using 1-D Laplacian. The sum of the sample gradients within a 4×4 window that covers the target 2×2 block is used for classifier C0 and the sum of sample gradients within a 12×12 window is used for classifiers C1 and C2. The sums of horizontal, vertical and two diagonal gradients are denoted, respectively, as ghi, gvi, gd1i and gd2i. The directionality Di is determined by comparing

rh,vi=maxghi, gviminghi, gvi ,  rd1,d2i=maxgd1i, gd2imingd1i, gd2i

with a set of thresholds. The directionality D2 is derived as in VVC using thresholds 2 and 4.5. For D0 and D1, horizontal/vertical edge strength EHVi and diagonal edge strength EDi are calculated first. Thresholds Th=[1.25, 1.5, 2, 3, 4.5, 8] are used. Edge strength EHVi is 0 if rh,vi≤ Th[0]; otherwise, EHVi is the maximum integer such that rh,vi>Th[EHVi -1]. Edge strength EDi is 0 if rd1,d2i≤Th[0]; otherwise, EDi is the maximum integer such that rd1,d2i>Th[EDi -1]. When rh,vi>rd1,d2i, i.e., horizontal/vertical edges are dominant, the Di is derived by using Table 3 (a); otherwise, diagonal edges are dominant, the Di is derived by using Table 3 (b).

Table 3. Mapping of EDi and EHVi to Di

(a)                                                                           (b)

| EDi EHVi | 0 | 1 | 2 | 3 | 4 | 5 | 6 |  | EHVi EDi | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 28 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 2 | 0 | 0 | 0 | 0 | 0 |  | 1 | 29 | 30 | 0 | 0 | 0 | 0 | 0 |
| 2 | 3 | 4 | 5 | 0 | 0 | 0 | 0 |  | 2 | 31 | 32 | 33 | 0 | 0 | 0 | 0 |
| 3 | 6 | 7 | 8 | 9 | 0 | 0 | 0 |  | 3 | 34 | 35 | 36 | 37 | 0 | 0 | 0 |
| 4 | 10 | 11 | 12 | 13 | 14 | 0 | 0 |  | 4 | 38 | 39 | 40 | 41 | 42 | 0 | 0 |
| 5 | 15 | 16 | 17 | 18 | 19 | 20 | 0 |  | 5 | 43 | 44 | 45 | 46 | 47 | 48 | 0 |
| 6 | 21 | 22 | 23 | 24 | 25 | 26 | 27 |  | 6 | 49 | 50 | 51 | 52 | 53 | 54 | 55 |

To obtain Ai, the sum of vertical and horizontal gradients Ai is mapped to the range of 0 to n, where n is equal to 4 for A2 and 15 for A0 and A1.

In an ALF_APS, up to 4 luma filter sets are signalled, each set may have up to 25 filters.

EE software is configured as BASE_ENCODER = 0, BASE_NORMATIVE = 0, TOOLS = 0, ALF_IMPROVEMENT = 1, config files are provided in cfg/tool-on/. Anchor is VTM-11.0.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.75% | -1.13% | -1.51% | 115% | 176% |
| Class A2 | -0.94% | -0.96% | -1.35% | 108% | 157% |
| Class B | -0.90% | -1.08% | -1.33% | 111% | 158% |
| Class C | -1.67% | -1.74% | -2.07% | 111% | 135% |
| Class E | -1.56% | -2.73% | -2.09% | 117% | 172% |
| Overall | -1.16% | -1.49% | -1.65% | 112% | 157% |
| Class D | -1.29% | -0.80% | -0.83% | 119% | 131% |
| Class F | -1.26% | -1.25% | -1.43% | 109% | 153% |
|  | Random access Main10 |  |  |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.17% | -0.86% | -1.15% | 106% | 185% |
| Class A2 | -1.51% | -1.38% | -2.29% | 107% | 167% |
| Class B | -1.54% | -1.47% | -1.23% | 111% | 161% |
| Class C | -1.60% | -1.84% | -2.02% | 117% | 155% |
| Class E |  |  |  |  |  |
| Overall | -1.48% | -1.43% | -1.64% | 111% | 165% |
| Class D | -1.37% | -1.01% | -0.88% | 139% | 154% |
| Class F | -1.10% | -0.98% | -1.11% | 118% | 138% |
|  | Low delay B Main10 |  |  |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.57% | -3.18% | -2.63% | 112% | 176% |
| Class C | -1.47% | -2.43% | -3.30% | 120% | 167% |
| Class E | -1.62% | -2.10% | -0.60% | 123% | 140% |
| Overall | -1.55% | -2.66% | -2.34% | 117% | 163% |
| Class D | -1.34% | -1.50% | -2.09% | 143% | 176% |
| Class F | -1.40% | -1.80% | -1.81% | 128% | 165% |

# Entropy coding

## Extended precision (test 6.1)

The intermediate precision used in the arithmetic coding engine is increased, including three elements. First, the precisions for two probability states are both increased to 15 bits, in comparison to 10 bits and 14 bits in VVC. Second, the LPS range update process is modified as below,

if q >= 16384

q = 215 – 1 – q

RLPS = ((range * (q>>6)) >>9) + 1,

if q >= 16384

q = 215 – 1 – q

RLPS = ((range * (q>>6)) >>9) + 1,

where range is a 9-bit variable representing the width of the current interval, q is a 15-bit variable representing the probability state of the current context model, and RLPS is the updated range for LPS. This operation can also be realized by looking up a 512×256-entry in 9-bit look-up table. Third, at the encoder side, the 256-entry look-up table used for bits estimation in VTM is extended to 512 entries.

EE software is configured as BASE_ENCODER = 0, BASE_NORMATIVE = 0, TOOLS = 0, EC_HIGH_PRECISION = 1, config files are provided in cfg/tool-on/. Anchor is VTM-11.0.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | -0.24% | -0.41% | 98% | 99% |
| Class A2 | -0.18% | -0.44% | -0.43% | 100% | 100% |
| Class B | -0.21% | -0.41% | -0.44% | 100% | 101% |
| Class C | -0.16% | -0.42% | -0.43% | 102% | 102% |
| Class E | -0.14% | -0.45% | -0.30% | 101% | 100% |
| Overall | -0.19% | -0.39% | -0.41% | 100% | 100% |
| Class D | -0.14% | -0.36% | -0.13% | 102% | 102% |
| Class F | -0.14% | -0.22% | -0.21% | 101% | 102% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.24% | 0.25% | -0.04% | 99% | 100% |
| Class A2 | -0.20% | 0.02% | -0.22% | 100% | 101% |
| Class B | -0.17% | -0.19% | 0.19% | 101% | 102% |
| Class C | -0.14% | -0.05% | 0.20% | 101% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.18% | -0.02% | 0.06% | 100% | 102% |
| Class D | -0.04% | -0.25% | 0.15% | 100% | 100% |
| Class F | -0.14% | -0.23% | -0.23% | 102% | 103% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.22% | 0.18% | 0.54% | 98% | 99% |
| Class C | -0.17% | -0.14% | 0.15% | 100% | 103% |
| Class E | -0.30% | 0.29% | 0.64% | 99% | 98% |
| Overall | -0.22% | 0.10% | 0.44% | 99% | 100% |
| Class D | -0.10% | 0.74% | 0.28% | 99% | 99% |
| Class F | -0.24% | -0.42% | 0.57% | 100% | 102% |

## Slice-type-based window size (test 6.2)

Since statistics are different with different slice types, it is beneficial to have a context’s probability state updated at a rate that is optimal under the given slice type. Therefore, for each context model, three window sizes are pre-defined for I-, B-, and P-slices, respectively, like the initialization parameters.

The context initialization parameters and window sizes are retrained.

This is tested on top of extended precision describe in the previous section. EE software is configured as BASE_ENCODER = 0, BASE_NORMATIVE = 0, TOOLS = 0, EC_HIGH_PRECISION = 1, SLICE_TYPE_WIN_SIZE = 1, config files are provided in cfg/tool-on/. Anchor is VTM-11.0.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | -0.43% | -0.61% | 99% | 98% |
| Class A2 | -0.22% | -0.43% | -0.47% | 99% | 99% |
| Class B | -0.25% | -0.51% | -0.60% | 101% | 100% |
| Class C | -0.19% | -0.53% | -0.55% | 102% | 103% |
| Class E | -0.17% | -0.47% | -0.59% | 101% | 100% |
| Overall | -0.23% | -0.48% | -0.56% | 101% | 101% |
| Class D | -0.12% | -0.50% | -0.42% | 101% | 103% |
| Class F | -0.20% | -0.33% | -0.47% | 101% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.30% | 0.03% | -0.20% | 100% | 100% |
| Class A2 | -0.20% | -0.28% | -0.18% | 100% | 101% |
| Class B | -0.22% | -0.25% | 0.07% | 101% | 102% |
| Class C | -0.18% | -0.05% | 0.20% | 101% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.22% | -0.15% | 0.00% | 101% | 102% |
| Class D | 0.00% | 0.18% | 0.16% | 99% | 99% |
| Class F | -0.12% | -0.23% | -0.14% | 102% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.27% | 0.19% | 0.15% | 99% | 100% |
| Class C | -0.15% | -0.01% | 0.15% | 100% | 105% |
| Class E | -0.28% | 0.19% | 0.60% | 100% | 99% |
| Overall | -0.23% | 0.12% | 0.26% | 100% | 101% |
| Class D | 0.01% | 0.22% | 1.20% | 99% | 101% |
| Class F | -0.29% | -0.40% | 0.17% | 99% | 99% |

# Combination tests

This section provides combined tests.

## All intra tools combined (test 7.1)

All intra tools described in the intra section are tested together.

EE software is configured as BASE_ENCODER = 0, BASE_NORMATIVE = 0, TOOLS = 0, MMLM = 1, GRAD_PDPC = 1, SECONDARY_MPM = 1, INTRA_6TAP = 1, ENABLE_DIMD = 1, config files are provided in cfg/tool-on/. Anchor is VTM-11.0.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.61% | -3.78% | -6.32% | 117% | 107% |
| Class A2 | -0.90% | -1.74% | -1.57% | 117% | 109% |
| Class B | -1.08% | -2.61% | -2.74% | 117% | 108% |
| Class C | -1.19% | -2.14% | -2.54% | 115% | 115% |
| Class E | -1.42% | -1.54% | -1.48% | 117% | 114% |
| Overall | -1.22% | -2.38% | -2.89% | 117% | 110% |
| Class D | -0.92% | -1.54% | -1.42% | 114% | 113% |
| Class F | -1.12% | -2.21% | -2.69% | 109% | 111% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.09% | -1.91% | -4.63% | 105% | 101% |
| Class A2 | -0.57% | -1.07% | -1.01% | 104% | 100% |
| Class B | -0.63% | -1.96% | -1.53% | 104% | 101% |
| Class C | -0.65% | -1.16% | -0.96% | 105% | 108% |
| Class E |  |  |  |  |  |
| Overall | -0.71% | -1.55% | -1.89% | 104% | 103% |
| Class D | -0.43% | -1.14% | -1.19% | 106% | 109% |
| Class F | -0.60% | -1.17% | -1.44% | 105% | 104% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.18% | -0.76% | -0.96% | 102% | 99% |
| Class C | -0.19% | -0.54% | -0.70% | 102% | 102% |
| Class E | -0.25% | 0.00% | -0.18% | 101% | 99% |
| Overall | -0.20% | -0.49% | -0.68% | 102% | 100% |
| Class D | -0.14% | -0.39% | 0.09% | 106% | 111% |
| Class F | -0.29% | -0.93% | -0.69% | 102% | 104% |

## Template and bilateral matching (test 7.2)

Templated matching is tested together with multi-pass DMVR and sample based BDOF.

EE software is configured as BASE_ENCODER = 0, BASE_NORMATIVE = 0, TOOLS = 0, TM_AMVP = 1, TM_MRG = 1, MULTI_PASS_DMVR = 1, SAMPLE_BASED_BDOF = 1, config files are provided in cfg/tool-on/. Anchor is VTM-11.0.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.58% | -3.73% | -4.39% | 140% | 190% |
| Class A2 | -5.41% | -5.26% | -5.33% | 139% | 248% |
| Class B | -3.35% | -3.45% | -3.41% | 141% | 218% |
| Class C | -3.63% | -3.62% | -3.82% | 141% | 229% |
| Class E |  |  |  |  |  |
| Overall | -3.88% | -3.91% | -4.10% | 141% | 220% |
| Class D | -3.91% | -3.70% | -3.76% | 137% | 234% |
| Class F | -1.45% | -1.64% | -1.90% | 126% | 158% |

## Overall results (test 7.3)

In this test, all described above changes enabled together and tested.

EE software is configured as BASE_ENCODER = 1, BASE_NORMATIVE = 1, TOOLS = 1, config files provided in cfg/ are used with classA.cfg applied to classes A1 and A2.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -5.37% | -9.93% | -10.84% | 208% | 211% |
| Class A2 | -5.02% | -8.10% | -6.15% | 201% | 193% |
| Class B | -4.60% | -7.49% | -7.97% | 225% | 191% |
| Class C | -5.41% | -7.68% | -8.02% | 224% | 180% |
| Class E | -5.64% | -8.17% | -6.86% | 230% | 213% |
| Overall | -5.15% | -8.15% | -7.97% | 218% | 196% |
| Class D | -4.31% | -5.95% | -5.72% | 237% | 188% |
| Class F | -8.17% | -10.63% | -11.09% | 177% | 182% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -11.82% | -13.54% | -16.14% | 275% | 445% |
| Class A2 | -13.10% | -14.71% | -14.45% | 260% | 533% |
| Class B | -11.13% | -12.38% | -11.82% | 258% | 444% |
| Class C | -12.91% | -13.25% | -13.36% | 270% | 438% |
| Class E |  |  |  |  |  |
| Overall | -12.14% | -13.31% | -13.62% | 265% | 459% |
| Class D | -14.02% | -13.65% | -13.47% | 286% | 464% |
| Class F | -10.77% | -12.74% | -12.90% | 231% | 319% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -9.30% | -10.48% | -10.21% | 230% | 279% |
| Class C | -10.86% | -10.71% | -11.98% | 248% | 268% |
| Class E | -9.05% | -10.57% | -8.34% | 203% | 244% |
| Overall | -9.76% | -10.58% | -10.37% | 229% | 266% |
| Class D | -12.53% | -12.18% | -13.05% | 264% | 314% |
| Class F | -11.29% | -11.53% | -11.38% | 216% | 223%! |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).