| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0134 |
| --- | --- |

| Title: | Crosscheck of CE-1.4 from JVET-V0052 (CE-1.1, CE-1.2 and CE-1.4: On the Rice parameter derivation for high bit-depth coding) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Adrian Browne | Email: | adrian.browne@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results for CE-1.4 described in JVET-V0052[1], a proposed modification to the Rice parameter derivation for regular residual coding. The code and description of the tests given in the CE documents JVET-V0022[2] matched. The simulation results gave an exact match with those supplied by the proponents. The draft specification text changes provided by the proponent included the significant functional changes.

# Introduction

CE-1.4 is a modification to regular residual coding (RRC) which changes the template function of 5 previous coded coefficients used to derive Rice parameters. The principle difference with other CE tests by the proponent is the addition of a history technique used to supply values for coefficients beyond the edge of the TU.

CE-1.4 contains two sub-tests, CE-1.4a resets the counters used to derive the history values to a default signalled at the slice level once per CTU. CE-1.4b maintains the counters over a complete slice.

Code for CE-1.4 was originally supplied inline with the CE timeline on 13th March 2021. The proponent identified an encoder side fix and requested a late change that was agreed by the CE-coordinators. The late change which consisted of a change to history counter storage  was uploaded on 23rd March 2021.

# Analysis

The code was found to be in agreement with the description in the CE[2]. The specification changes, included in the contribution document, included the functional changes but may require an update to handle counter storage.

# Results

The code[3] was compiled using gcc 8.2.1 and tested according to the high bit depth / high bit rate CTC[4] on Linux clusters running openSUSE Leap 15.2. The results were compared with a similarly prepared CE anchor consisting of VTM12.0 augmented with rice coding fixes (CE-1.1 and CE-2.1).

Results for CE-1.4a and CE-1.4b were found to be in agreement with the proponent’s across the low QP, standard QP and lossless test sets.

## CE-1.4a

### Low QP

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.32% | -0.22% | -0.21% | -0.34% | -0.22% | -0.21% | 101% | 102% |
| PQ422 | -0.34% | -0.25% | -0.26% | -0.36% | -0.25% | -0.27% | 101% | 102% |
| Overall | -0.33% | -0.24% | -0.24% | -0.35% | -0.24% | -0.24% | 101% | 102% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.07% | -0.06% | -0.07% | -0.07% | -0.06% | -0.07% | 102% | 103% |
| PQ422 | -0.05% | -0.09% | -0.10% | -0.05% | -0.08% | -0.09% | 102% | 102% |
| Overall | -0.06% | -0.07% | -0.08% | -0.06% | -0.07% | -0.08% | 102% | 103% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.08% | -0.07% | -0.09% | -0.07% | -0.07% | -0.09% | 101% | 101% |
| PQ422 | -0.09% | -0.07% | -0.09% | -0.09% | -0.07% | -0.09% | 101% | 102% |
| Overall | -0.08% | -0.07% | -0.09% | -0.08% | -0.07% | -0.09% | 101% | 102% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.48% | -0.28% | -0.29% | 103% | 103% |
| HLG422 | -0.52% | -0.33% | -0.34% | 102% | 102% |
| Overall | -0.50% | -0.30% | -0.32% | 102% | 103% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.02% | 0.00% | 0.01% | 101% | 102% |
| HLG422 | -0.02% | -0.02% | 0.02% | 101% | 101% |
| Overall | -0.02% | -0.01% | 0.01% | 101% | 101% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.04% | -0.01% | -0.02% | 101% | 101% |
| HLG422 | -0.04% | -0.01% | -0.01% | 101% | 101% |
| Overall | -0.04% | -0.01% | -0.01% | 101% | 101% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -2.72% | -2.09% | -2.08% | 105% | 120% |
| SVT12 | -1.64% | -1.32% | -1.34% | 104% | 108% |
| Overall | -2.18% | -1.71% | -1.71% | 104% | 114% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -1.51% | -1.58% | -1.57% | 113% | 111% |
| SVT12 | -0.71% | -0.63% | -0.63% | 101% | 104% |
| Overall | -1.11% | -1.10% | -1.10% | 107% | 108% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -1.43% | -1.46% | -1.45% | 109% | 111% |
| SVT12 | -0.66% | -0.53% | -0.54% | 101% | 106% |
| Overall | -1.04% | -0.99% | -0.99% | 105% | 108% |

### Standard QP

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.10% | -0.02% | -0.03% | -0.17% | -0.15% | -0.02% | -0.24% | -0.16% | 100% | 102% |
| Class H2 |  |  |  |  |  | 0.02% | -0.07% | 0.01% | 101% | 102% |
| Overall | 0.10% | -0.02% | -0.03% | -0.17% | -0.15% | 0.00% | -0.15% | -0.08% | 101% | 102% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.02% | -0.02% | -0.03% | 0.12% | -0.06% | -0.03% | 0.08% | -0.11% | 99% | 102% |
| Class H2 |  |  |  |  |  | -0.01% | -0.02% | 0.04% | 99% | 100% |
| Overall | 0.02% | -0.02% | -0.03% | 0.12% | -0.06% | -0.02% | 0.03% | -0.04% | 99% | 101% |

### Lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-1.4a_v2 |  | VTM_U_CE | CE-1.4a_v2 |  | VTM_U_CE | CE-1.4a_v2 |  |
| PQ444 | 2.5 | 2.6 | -2.26% | 3.0 | 3.1 | -3.07% | 3.0 | 3.1 | -2.98% |
| PQ422 | 2.3 | 2.4 | -2.36% | 2.8 | 2.9 | -3.06% | 2.8 | 2.9 | -2.97% |
| Overall | 2.4 | 2.5 | -2.31% | 2.9 | 3.0 | -3.06% | 2.9 | 3.0 | -2.98% |
| Enc Time[%] | 101% |  |  | 107% |  |  | 108% |  |  |
| Dec Time[%] | 113% |  |  | 106% |  |  | 104% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-1.4a_v2 |  | VTM_U_CE | CE-1.4a_v2 |  | VTM_U_CE | CE-1.4a_v2 |  |
| HLG444 | 1.7 | 1.8 | -3.54% | 1.9 | 2.0 | -4.40% | 1.9 | 2.0 | -4.32% |
| HLG422 | 1.6 | 1.7 | -3.44% | 1.9 | 1.9 | -4.21% | 1.9 | 1.9 | -4.12% |
| Overall | 1.7 | 1.7 | -3.49% | 1.9 | 1.9 | -4.30% | 1.9 | 2.0 | -4.22% |
| Enc Time[%] | 96% |  |  | 132% |  |  | 138% |  |  |
| Dec Time[%] | 125% |  |  | 113% |  |  | 113% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-1.4a_v2 |  | VTM_U_CE | CE-1.4a_v2 |  | VTM_U_CE | CE-1.4a_v2 |  |
| SVT16 | 1.2 | 1.2 | -1.38% | 1.2 | 1.2 | -0.39% | 1.2 | 1.2 | -0.37% |
| SVT12 | 1.2 | 1.3 | -2.88% | 1.3 | 1.3 | -1.51% | 1.3 | 1.3 | -1.42% |
| Overall | 1.2 | 1.2 | -2.13% | 1.2 | 1.3 | -0.95% | 1.2 | 1.3 | -0.89% |
| Enc Time[%] | 113% |  |  | 182% |  |  | 175% |  |  |
| Dec Time[%] | 150% |  |  | 119% |  |  | 117% |  |  |

## CE-1.4b

### Low QP

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.32% | -0.22% | -0.22% | -0.35% | -0.22% | -0.23% | 100% | 100% |
| PQ422 | -0.33% | -0.25% | -0.27% | -0.35% | -0.24% | -0.27% | 100% | 101% |
| Overall | -0.33% | -0.23% | -0.25% | -0.35% | -0.23% | -0.25% | 100% | 100% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.06% | -0.08% | -0.09% | -0.06% | -0.07% | -0.08% | 101% | 99% |
| PQ422 | -0.07% | -0.08% | -0.09% | -0.07% | -0.08% | -0.08% | 99% | 99% |
| Overall | -0.06% | -0.08% | -0.09% | -0.06% | -0.08% | -0.08% | 100% | 99% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.08% | -0.09% | -0.09% | -0.08% | -0.09% | -0.09% | 101% | 100% |
| PQ422 | -0.09% | -0.07% | -0.10% | -0.09% | -0.08% | -0.09% | 100% | 100% |
| Overall | -0.08% | -0.08% | -0.09% | -0.08% | -0.08% | -0.09% | 101% | 100% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.48% | -0.28% | -0.30% | 101% | 101% |
| HLG422 | -0.52% | -0.33% | -0.34% | 101% | 101% |
| Overall | -0.50% | -0.30% | -0.32% | 101% | 101% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.05% | 0.01% | 0.01% | 99% | 100% |
| HLG422 | -0.06% | -0.03% | -0.02% | 100% | 101% |
| Overall | -0.06% | -0.01% | 0.00% | 100% | 101% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.05% | -0.01% | -0.02% | 101% | 101% |
| HLG422 | -0.04% | -0.02% | -0.02% | 101% | 101% |
| Overall | -0.04% | -0.02% | -0.02% | 101% | 101% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -2.70% | -2.05% | -2.04% | 103% | 117% |
| SVT12 | -1.64% | -1.32% | -1.33% | 103% | 107% |
| Overall | -2.17% | -1.69% | -1.69% | 103% | 112% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -1.49% | -1.55% | -1.54% | 111% | 112% |
| SVT12 | -0.71% | -0.63% | -0.63% | 102% | 106% |
| Overall | -1.10% | -1.09% | -1.09% | 107% | 109% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -1.43% | -1.47% | -1.46% | 109% | 112% |
| SVT12 | -0.65% | -0.53% | -0.54% | 100% | 104% |
| Overall | -1.04% | -1.00% | -1.00% | 105% | 108% |

### Standard QP

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.01% | -0.03% | -0.05% | -0.29% | -0.25% | -0.05% | -0.38% | -0.25% | 101% | 101% |
| Class H2 |  |  |  |  |  | 0.00% | -0.02% | -0.09% | 101% | 101% |
| Overall | -0.01% | -0.03% | -0.05% | -0.29% | -0.25% | -0.02% | -0.20% | -0.17% | 101% | 101% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.00% | -0.02% | -0.02% | 0.06% | -0.07% | -0.03% | 0.01% | -0.06% | 100% | 100% |
| Class H2 |  |  |  |  |  | -0.01% | 0.00% | -0.04% | 99% | 100% |
| Overall | 0.00% | -0.02% | -0.02% | 0.06% | -0.07% | -0.02% | 0.00% | -0.05% | 100% | 100% |

### Lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-1.4b_v2 |  | VTM_U_CE | CE-1.4b_v2 |  | VTM_U_CE | CE-1.4b_v2 |  |
| PQ444 | 2.5 | 2.6 | -1.97% | 3.0 | 3.1 | -2.86% | 3.0 | 3.1 | -2.79% |
| PQ422 | 2.3 | 2.4 | -2.10% | 2.8 | 2.9 | -2.87% | 2.8 | 2.9 | -2.79% |
| Overall | 2.4 | 2.5 | -2.03% | 2.9 | 3.0 | -2.87% | 2.9 | 3.0 | -2.79% |
| Enc Time[%] | 101% |  |  | 108% |  |  | 109% |  |  |
| Dec Time[%] | 117% |  |  | 107% |  |  | 106% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-1.4b_v2 |  | VTM_U_CE | CE-1.4b_v2 |  | VTM_U_CE | CE-1.4b_v2 |  |
| HLG444 | 1.7 | 1.8 | -3.21% | 1.9 | 1.9 | -4.11% | 1.9 | 2.0 | -4.05% |
| HLG422 | 1.6 | 1.7 | -2.92% | 1.9 | 1.9 | -3.99% | 1.9 | 1.9 | -3.91% |
| Overall | 1.7 | 1.7 | -3.06% | 1.9 | 1.9 | -4.05% | 1.9 | 1.9 | -3.98% |
| Enc Time[%] | 97% |  |  | 131% |  |  | 139% |  |  |
| Dec Time[%] | 132% |  |  | 113% |  |  | 115% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-1.4b_v2 |  | VTM_U_CE | CE-1.4b_v2 |  | VTM_U_CE | CE-1.4b_v2 |  |
| SVT16 | 1.2 | 1.2 | -1.38% | 1.2 | 1.2 | -0.38% | 1.2 | 1.2 | -0.36% |
| SVT12 | 1.2 | 1.3 | -2.84% | 1.3 | 1.3 | -1.51% | 1.3 | 1.3 | -1.42% |
| Overall | 1.2 | 1.2 | -2.11% | 1.2 | 1.3 | -0.95% | 1.2 | 1.3 | -0.89% |
| Enc Time[%] | 114% |  |  | 184% |  |  | 176% |  |  |
| Dec Time[%] | 150% |  |  | 123% |  |  | 119% |  |  |

# References

[1] “CE-1.1, CE-1.2 and CE-1.4: On the Rice parameter derivation for high bit-depth coding”, JVET-V0052, D. Rusanovskyy, M. Karczewicz, L. P. Van, M. Coban

[2] “CE: Summary Report on Entropy Coding for High Bit Depth and High Bit Rate Coding”, JVET-V0022, A.Browne, T. Hashimoto, H. Jhu, D. Rusanovskyy, K. Kawamura, T. Zhou

[3] https://vcgit.hhi.fraunhofer.de/jvet-u-ce/VVCSoftware_VTM/-/tree/CE1.4

[4] “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018, A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu