| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0135 |
| --- | --- |

| Title: | Crosscheck of CE-4.5 and CE-4.7 from JVET-V0053 (CE-4.5, CE-4.6 and CE-4.7: Combinations of RRC tests CE-1.1, CE-1.2 and CE-1.4 with CE-3.1/CE-3.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Adrian Browne | Email: | adrian.browne@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results for CE-4.5 and CE-4.7 described in JVET-V0053 [1], a series of combinations of modifications to the Rice parameter derivation for regular residual coding, and to transform coefficient clipping. The simulation results matched those supplied by the proponents.

# Introduction

CE-4.5 is described [2] as a combination of CE-1.1 and CE-3.2. In addition, the proponent has also provided results for a combination of CE-1.1 and CE-3.1.

CE-4.7 is described [2] as two sub-tests, CE-4.7a combines CE-1.4a and CE-3.2 and CE-4.7b combines CE-1.4b and CE-3.2. In addition, the proponent has also provided results for combination of CE-1.4a and CE-1.4b with CE-3.1.

# Analysis

The code was found to be in agreement with the descriptions in the CE[3].

# Results

The code [4][5] was compiled using gcc 8.2.1 and tested according to the high bit depth / high bit rate CTC[6] on Linux clusters running openSUSE Leap 15.2. The results were compared with a similarly prepared CE anchor consisting of VTM12.0 augmented with rice coding fixes (CE-1.1 and CE-2.1).

Results for CE-4.5 and CE-4.7 were found to be in agreement with the proponent’s across the low QP, standard QP and lossless test sets.

## CE-4.5

### Low QP

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -1.23% | -1.74% | -1.87% | -1.24% | -1.76% | -1.88% | 103% | 95% |
| PQ422 | -1.04% | -1.27% | -1.38% | -1.05% | -1.28% | -1.37% | 103% | 94% |
| Overall | -1.14% | -1.51% | -1.62% | -1.14% | -1.52% | -1.63% | 103% | 95% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.69% | -0.74% | -0.79% | -0.69% | -0.74% | -0.80% | 101% | 98% |
| PQ422 | -0.61% | -0.53% | -0.59% | -0.61% | -0.53% | -0.59% | 101% | 98% |
| Overall | -0.65% | -0.63% | -0.69% | -0.65% | -0.63% | -0.69% | 101% | 98% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.60% | -0.69% | -0.74% | -0.60% | -0.68% | -0.74% | 101% | 98% |
| PQ422 | -0.56% | -0.52% | -0.61% | -0.55% | -0.52% | -0.60% | 101% | 98% |
| Overall | -0.58% | -0.61% | -0.68% | -0.58% | -0.60% | -0.67% | 101% | 98% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.49% | -0.71% | -0.73% | 110% | 95% |
| HLG422 | -0.41% | -0.62% | -0.64% | 106% | 96% |
| Overall | -0.45% | -0.66% | -0.69% | 108% | 95% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.64% | -0.87% | -0.92% | 103% | 95% |
| HLG422 | -0.55% | -0.64% | -0.69% | 102% | 95% |
| Overall | -0.59% | -0.76% | -0.80% | 102% | 95% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.53% | -0.77% | -0.82% | 102% | 96% |
| HLG422 | -0.43% | -0.59% | -0.65% | 101% | 96% |
| Overall | -0.48% | -0.68% | -0.73% | 102% | 96% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -1.39% | -1.12% | -1.13% | 107% | 99% |
| SVT12 | -0.83% | -1.00% | -1.03% | 97% | 95% |
| Overall | -1.11% | -1.06% | -1.08% | 102% | 97% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.78% | -0.82% | -0.81% | 107% | 98% |
| SVT12 | -0.78% | -0.65% | -0.66% | 100% | 94% |
| Overall | -0.78% | -0.73% | -0.73% | 104% | 96% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.59% | -0.86% | -0.85% | 104% | 98% |
| SVT12 | -0.62% | -0.66% | -0.68% | 99% | 94% |
| Overall | -0.60% | -0.76% | -0.76% | 102% | 96% |

### Standard QP

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM_U_CE-Benchmark-VTM-JVET-U2018-STDQP-Cluster2 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.14% | -0.07% | -0.05% | 0.02% | -0.29% | -0.04% | -0.14% | -0.30% | 100% | 100% |
| Class H2 |  |  |  |  |  | -0.03% | 0.13% | 0.17% | 100% | 101% |
| Overall | 0.14% | -0.07% | -0.05% | 0.02% | -0.29% | -0.04% | -0.01% | -0.06% | 100% | 100% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM_U_CE-Benchmark-VTM-JVET-U2018-STDQP-Cluster2 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.06% | -0.01% | -0.02% | 0.12% | -0.08% | -0.02% | 0.08% | -0.05% | 100% | 99% |
| Class H2 |  |  |  |  |  | -0.01% | -0.01% | -0.15% | 99% | 98% |
| Overall | -0.06% | -0.01% | -0.02% | 0.12% | -0.08% | -0.02% | 0.03% | -0.10% | 100% | 99% |

### Lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-4.5 |  | VTM_U_CE | CE-4.5 |  | VTM_U_CE | CE-4.5 |  |
| PQ444 | 2.5 | 2.5 | -0.97% | 3.0 | 3.0 | -1.55% | 3.0 | 3.0 | -1.62% |
| PQ422 | 2.3 | 2.3 | -0.98% | 2.8 | 2.8 | -1.55% | 2.8 | 2.8 | -1.64% |
| Overall | 2.4 | 2.4 | -0.98% | 2.9 | 2.9 | -1.55% | 2.9 | 2.9 | -1.63% |
| Enc Time[%] | 95% |  |  | 84% |  |  | 82% |  |  |
| Dec Time[%] | 102% |  |  | 97% |  |  | 93% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-4.5 |  | VTM_U_CE | CE-4.5 |  | VTM_U_CE | CE-4.5 |  |
| HLG444 | 1.7 | 1.7 | -1.65% | 1.9 | 1.9 | -2.67% | 1.9 | 1.9 | -2.74% |
| HLG422 | 1.6 | 1.7 | -1.35% | 1.9 | 1.9 | -2.35% | 1.9 | 1.9 | -2.42% |
| Overall | 1.7 | 1.7 | -1.50% | 1.9 | 1.9 | -2.51% | 1.9 | 1.9 | -2.58% |
| Enc Time[%] | 99% |  |  | 73% |  |  | 72% |  |  |
| Dec Time[%] | 97% |  |  | 92% |  |  | 93% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-4.5 |  | VTM_U_CE | CE-4.5 |  | VTM_U_CE | CE-4.5 |  |
| SVT16 | 1.2 | 1.2 | 0.05% | 1.2 | 1.2 | 0.11% | 1.2 | 1.2 | 0.12% |
| SVT12 | 1.2 | 1.3 | -1.38% | 1.3 | 1.3 | -0.77% | 1.3 | 1.3 | -0.71% |
| Overall | 1.2 | 1.2 | -0.66% | 1.2 | 1.2 | -0.33% | 1.2 | 1.2 | -0.29% |
| Enc Time[%] | 101% |  |  | 91% |  |  | 94% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  | 99% |  |  |

## CE-4.7a

### Low QP

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -1.51% | -1.96% | -2.11% | -1.52% | -1.99% | -2.12% | 104% | 96% |
| PQ422 | -1.33% | -1.54% | -1.64% | -1.34% | -1.54% | -1.63% | 104% | 96% |
| Overall | -1.42% | -1.75% | -1.87% | -1.43% | -1.76% | -1.88% | 104% | 96% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.72% | -0.76% | -0.82% | -0.72% | -0.76% | -0.84% | 102% | 100% |
| PQ422 | -0.65% | -0.57% | -0.66% | -0.66% | -0.56% | -0.64% | 102% | 100% |
| Overall | -0.69% | -0.67% | -0.74% | -0.69% | -0.66% | -0.74% | 102% | 100% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.66% | -0.72% | -0.81% | -0.66% | -0.72% | -0.81% | 101% | 98% |
| PQ422 | -0.63% | -0.59% | -0.64% | -0.62% | -0.58% | -0.64% | 101% | 99% |
| Overall | -0.64% | -0.66% | -0.73% | -0.64% | -0.65% | -0.72% | 101% | 98% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.91% | -1.04% | -1.07% | 113% | 96% |
| HLG422 | -0.90% | -1.00% | -1.04% | 106% | 97% |
| Overall | -0.91% | -1.02% | -1.05% | 109% | 96% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.64% | -0.85% | -0.89% | 103% | 97% |
| HLG422 | -0.55% | -0.62% | -0.68% | 102% | 97% |
| Overall | -0.59% | -0.74% | -0.79% | 103% | 97% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.53% | -0.77% | -0.82% | 103% | 99% |
| HLG422 | -0.45% | -0.59% | -0.65% | 102% | 99% |
| Overall | -0.49% | -0.68% | -0.74% | 102% | 99% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -2.90% | -2.27% | -2.26% | 111% | 119% |
| SVT12 | -1.88% | -1.88% | -1.90% | 99% | 100% |
| Overall | -2.39% | -2.08% | -2.08% | 105% | 109% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -1.60% | -1.64% | -1.63% | 114% | 112% |
| SVT12 | -1.15% | -1.01% | -1.02% | 103% | 101% |
| Overall | -1.37% | -1.32% | -1.32% | 109% | 106% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -1.41% | -1.58% | -1.57% | 110% | 112% |
| SVT12 | -0.98% | -0.93% | -0.95% | 103% | 100% |
| Overall | -1.19% | -1.26% | -1.26% | 106% | 106% |

### Standard QP

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.29% | -0.05% | -0.04% | -0.35% | 0.09% | -0.04% | -0.45% | 0.01% | 101% | 99% |
| Class H2 |  |  |  |  |  | -0.04% | 0.10% | 0.03% | 101% | 100% |
| Overall | 0.29% | -0.05% | -0.04% | -0.35% | 0.09% | -0.04% | -0.17% | 0.02% | 101% | 100% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.00% | -0.04% | -0.03% | 0.09% | -0.12% | -0.03% | 0.08% | -0.07% | 100% | 99% |
| Class H2 |  |  |  |  |  | -0.02% | -0.03% | 0.05% | 99% | 98% |
| Overall | 0.00% | -0.04% | -0.03% | 0.09% | -0.12% | -0.02% | 0.03% | -0.01% | 99% | 99% |

### Lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-4.7a |  | VTM_U_CE | CE-4.7a |  | VTM_U_CE | CE-4.7a |  |
| PQ444 | 2.5 | 2.6 | -2.26% | 3.0 | 3.1 | -3.07% | 3.0 | 3.1 | -2.98% |
| PQ422 | 2.3 | 2.4 | -2.36% | 2.8 | 2.9 | -3.06% | 2.8 | 2.9 | -2.97% |
| Overall | 2.4 | 2.5 | -2.31% | 2.9 | 3.0 | -3.06% | 2.9 | 3.0 | -2.98% |
| Enc Time[%] | 98% |  |  | 103% |  |  | 106% |  |  |
| Dec Time[%] | 112% |  |  | 105% |  |  | 105% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-4.7a |  | VTM_U_CE | CE-4.7a |  | VTM_U_CE | CE-4.7a |  |
| HLG444 | 1.7 | 1.8 | -3.54% | 1.9 | 2.0 | -4.40% | 1.9 | 2.0 | -4.32% |
| HLG422 | 1.6 | 1.7 | -3.44% | 1.9 | 1.9 | -4.21% | 1.9 | 1.9 | -4.12% |
| Overall | 1.7 | 1.7 | -3.49% | 1.9 | 1.9 | -4.30% | 1.9 | 2.0 | -4.22% |
| Enc Time[%] | 94% |  |  | 129% |  |  | 137% |  |  |
| Dec Time[%] | 125% |  |  | 114% |  |  | 115% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-4.7a |  | VTM_U_CE | CE-4.7a |  | VTM_U_CE | CE-4.7a |  |
| SVT16 | 1.2 | 1.2 | -1.39% | 1.2 | 1.2 | -0.39% | 1.2 | 1.2 | -0.35% |
| SVT12 | 1.2 | 1.3 | -2.88% | 1.3 | 1.3 | -1.51% | 1.3 | 1.3 | -1.42% |
| Overall | 1.2 | 1.2 | -2.14% | 1.2 | 1.3 | -0.95% | 1.2 | 1.3 | -0.88% |
| Enc Time[%] | 109% |  |  | 180% |  |  | 151% |  |  |
| Dec Time[%] | 145% |  |  | 124% |  |  | 120% |  |  |

## CE-4.7b

### Low QP

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -1.50% | -1.95% | -2.10% | -1.52% | -1.98% | -2.12% | 103% | 96% |
| PQ422 | -1.33% | -1.53% | -1.65% | -1.34% | -1.53% | -1.65% | 104% | 96% |
| Overall | -1.42% | -1.74% | -1.88% | -1.43% | -1.76% | -1.88% | 103% | 96% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.72% | -0.76% | -0.83% | -0.73% | -0.77% | -0.84% | 102% | 100% |
| PQ422 | -0.67% | -0.58% | -0.66% | -0.67% | -0.58% | -0.65% | 102% | 99% |
| Overall | -0.69% | -0.67% | -0.74% | -0.70% | -0.67% | -0.75% | 102% | 100% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.66% | -0.75% | -0.80% | -0.66% | -0.74% | -0.80% | 102% | 100% |
| PQ422 | -0.63% | -0.59% | -0.67% | -0.62% | -0.58% | -0.66% | 101% | 99% |
| Overall | -0.65% | -0.67% | -0.73% | -0.64% | -0.66% | -0.73% | 101% | 100% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.91% | -1.04% | -1.07% | 113% | 96% |
| HLG422 | -0.90% | -1.01% | -1.04% | 106% | 97% |
| Overall | -0.91% | -1.02% | -1.05% | 109% | 97% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.62% | -0.86% | -0.89% | 103% | 98% |
| HLG422 | -0.56% | -0.62% | -0.65% | 102% | 97% |
| Overall | -0.59% | -0.74% | -0.77% | 103% | 98% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.54% | -0.77% | -0.82% | 102% | 98% |
| HLG422 | -0.45% | -0.60% | -0.67% | 101% | 98% |
| Overall | -0.50% | -0.68% | -0.74% | 102% | 98% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -2.88% | -2.24% | -2.22% | 111% | 120% |
| SVT12 | -1.88% | -1.87% | -1.89% | 100% | 101% |
| Overall | -2.38% | -2.05% | -2.06% | 105% | 111% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -1.57% | -1.62% | -1.60% | 112% | 112% |
| SVT12 | -1.14% | -1.01% | -1.02% | 102% | 100% |
| Overall | -1.36% | -1.31% | -1.31% | 107% | 106% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -1.41% | -1.59% | -1.58% | 110% | 112% |
| SVT12 | -0.97% | -0.93% | -0.95% | 102% | 100% |
| Overall | -1.19% | -1.26% | -1.26% | 106% | 106% |

### Standard QP

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.16% | -0.02% | -0.01% | -0.23% | -0.21% | 0.00% | -0.30% | -0.25% | 99% | 99% |
| Class H2 |  |  |  |  |  | -0.01% | -0.03% | 0.19% | 101% | 100% |
| Overall | 0.16% | -0.02% | -0.01% | -0.23% | -0.21% | -0.01% | -0.16% | -0.03% | 100% | 100% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.03% | -0.03% | -0.03% | 0.11% | 0.00% | -0.03% | 0.09% | -0.02% | 98% | 98% |
| Class H2 |  |  |  |  |  | -0.01% | -0.04% | -0.14% | 97% | 97% |
| Overall | 0.03% | -0.03% | -0.03% | 0.11% | 0.00% | -0.02% | 0.03% | -0.08% | 98% | 98% |

### Lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-4.7b |  | VTM_U_CE | CE-4.7b |  | VTM_U_CE | CE-4.7b |  |
| PQ444 | 2.5 | 2.6 | -1.97% | 3.0 | 3.1 | -2.86% | 3.0 | 3.1 | -2.79% |
| PQ422 | 2.3 | 2.4 | -2.10% | 2.8 | 2.9 | -2.87% | 2.8 | 2.9 | -2.79% |
| Overall | 2.4 | 2.5 | -2.03% | 2.9 | 3.0 | -2.87% | 2.9 | 3.0 | -2.79% |
| Enc Time[%] | 102% |  |  | 109% |  |  | 110% |  |  |
| Dec Time[%] | 116% |  |  | 107% |  |  | 106% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-4.7b |  | VTM_U_CE | CE-4.7b |  | VTM_U_CE | CE-4.7b |  |
| HLG444 | 1.7 | 1.8 | -3.21% | 1.9 | 1.9 | -4.11% | 1.9 | 2.0 | -4.05% |
| HLG422 | 1.6 | 1.7 | -2.92% | 1.9 | 1.9 | -3.99% | 1.9 | 1.9 | -3.91% |
| Overall | 1.7 | 1.7 | -3.06% | 1.9 | 1.9 | -4.05% | 1.9 | 1.9 | -3.98% |
| Enc Time[%] | 98% |  |  | 133% |  |  | 142% |  |  |
| Dec Time[%] | 131% |  |  | 115% |  |  | 116% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM_U_CE | CE-4.7b |  | VTM_U_CE | CE-4.7b |  | VTM_U_CE | CE-4.7b |  |
| SVT16 | 1.2 | 1.2 | -1.39% | 1.2 | 1.2 | -0.39% | 1.2 | 1.2 | -0.36% |
| SVT12 | 1.2 | 1.3 | -2.84% | 1.3 | 1.3 | -1.51% | 1.3 | 1.3 | -1.42% |
| Overall | 1.2 | 1.2 | -2.11% | 1.2 | 1.3 | -0.95% | 1.2 | 1.3 | -0.89% |
| Enc Time[%] | 115% |  |  | 185% |  |  | 178% |  |  |
| Dec Time[%] | 147% |  |  | 122% |  |  | 119% |  |  |

# References

[1] “CE-4.5, CE-4.6 and CE-4.7: Combinations of RRC tests CE-1.1, CE-1.2 and CE-1.4 with CE-3.1/CE-3.2”, JVET-V0053, D. Rusanovskyy, M. Karczewicz, L. P. Van, M. Coban

[2] “CE on Entropy Coding for High Bit Depth and High Bit Rate Coding”, JVET-U2022, A. Browne, T. Hashimoto, H.-J. Jhu, D. Rusanovskyy, K. Kawamura, T. Zhou

[3] “CE: Summary Report on Entropy Coding for High Bit Depth and High Bit Rate Coding”, JVET-V0022, A.Browne, T. Hashimoto, H. Jhu, D. Rusanovskyy, K. Kawamura, T. Zhou

[4] https://vcgit.hhi.fraunhofer.de/jvet-u-ce/VVCSoftware_VTM/-/tree/CE4.5

[5] https://vcgit.hhi.fraunhofer.de/jvet-u-ce/VVCSoftware_VTM/-/tree/CE4.7

[6] “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018, A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu