| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0152 |
| --- | --- |

| Title: | Crosscheck of JVET-V0108 (AHG9: Colour Transform Information SEI message) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Fangjun Pu 432 Lakeside Drive, Sunnyvale, CA, 94085 US | Tel: Email: | +1 408 330 3281 Fangjun.Pu@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results for contribution JVET-V0108 on Colour Transform Information SEI message [1]. The cross-checking tests were performed for HDR PQ content under CTC conditions [2] for the second test which enables both CTI-SEI and LMCS. All results match the results provided in JVET-V0108.

# Introduction

JVET-V0108 is a follow-up of JVET-U0078 (Option 2), where a new SEI message, named Colour Transform Information (CTI) SEI message, was proposed. CTI can be used as a tool for out-of-loop content mapping. The mapping functions are based on piecewise linear models whose parameters are signalled in the CTI SEI message which is applied to the reconstructed pictures after decoding.

The signalling is summarized below.

For colour component 0:

- PWL0 coded

For colour components 1 and 2

- crossCompFlag coded

- If crossCompFlag = 1, chromaInferredFlag coded

- If crossCompFlag = 0 or chromaInferredFlag = 0, PWL1/PWL2 coded

- Else chromaOffset coded, PWL1/PWL2 inferred from PWL0

The colour transform process is summarized below. Mapping functions (noted below “LUT”) are derived from the PWL models.

For colour component 0:

- outC0 = LUT0[inC0]

For colour component x=1, 2

- crossCompFlag = 0	outCx = LUTx[inCx]

- crossCompFlag = 1	outCx = ( (inCx–2^(B–1)) * LUTx[inC0] + 2^(B–1) * 2^11 ) >> 11

In the proposed solution, CTI can be combined with LMCS which is in-loop luma mapping with chroma scaling applied in current VVC. The combined version provides the best coding performance for HDR-PQ content.

# Experimental Results

In the proposal, the CTI has been tested as a mandatory process for out-of-loop mapping for HDR-PQ content. Tests without and with LMCS have been made. The best results are obtained with CTI and LMCS both applied [1]. This report tests the combined version of CTI-SEI solution (Test 2).

In Test 2, CTI is enabled with the luma mapping function built from the LMCS luma mapping function used in VTM for HDR-PQ content. Cross-component scaling is applied, with cross-component scaling LUT inferred from the luma mapping function. In addition, LMCS is enabled using the SDR tuning algorithm. The corresponding configuration file is “sei_cti_v02.cfg”. Results are in Table 2.1.

Table 2.1 Coding performance for HDR-PQ over VTM12.0 (CTI enabled, LMCS enabled)

|  |  | wPSNR |  |  | PSNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| All Intra |  |  |  |  |  |  |  |  |  |
| -0.46% | -0.91% | -0.30% | -5.27% | -6.21% | 0.40% | -1.79% | -2.15% | 102% | 99% |
| Random Access Gop 32 |  |  |  |  |  |  |  |  |  |
| -2.30% | -3.31% | -1.74% | -5.65% | -7.35% | -1.06% | -1.32% | -1.99% | 99% | 101% |
| Low Delay B |  |  |  |  |  |  |  |  |  |
| -5.85% | -3.96% | -2.57% | -9.52% | -14.06% | -1.89% | 0.69% | -3.25% | 100% | 100% |
| Low Delay P |  |  |  |  |  |  |  |  |  |
| -6.68% | -3.83% | -2.49% | -9.90% | -14.50% | -1.86% | 1.18% | -3.40% | 99% | 98% |

All BD-rate match the results provided in JVET-V0108. Note that the encoding time is not reliable due to various machines in the cluster. Subjectively, no visual difference has been observed between the proposal and the VTM12.0 anchor.

# References

- E. François, M. Radosavljevic, P. de Lagrange, F. Leleannec, “AHG9: Colour Transform Information SEI message”, JVET-V0108. Apr. 2021

- A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-T2011. Oct. 2020