| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0162 |
| --- | --- |

| Title: | Crosscheck of JVET-V0066 (AHG8: Encoder improvements to palette coding for high bit depth) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu | Email: | jhuhong-jheng@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

Reported in this document is the cross-check results for AHG8: Encoder improvements to palette coding for high bit depth as reported in JVET-V0066. The cross-check partial results match the coding performance reported by the proponent.

# Results

The source code for JVET-V0066 was provided by the proponent. The software is built based on VTM-12.0. The crosscheck results are illustrated as following tables. The detailed results could be seen in the attached excel files.

Table 1. Test1 vs VTM12.0

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12 |  |  |
|  | Y / G | U / B | V / R | EncT | DecT |
| RGB 4:4:4 - TGM 1080p | 0.00% | 0.00% | 0.00% | 104% | 101% |
| RGB 4:4:4 - TGM 720p | 0.00% | 0.00% | 0.00% | 103% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 104% | 99% |

|  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12 |  |  |
|  | Y / G | U / B | V / R | EncT | DecT |
| RGB 4:4:4 - TGM 1080p | 0.00% | 0.00% | 0.00% | 105% | 96% |
| RGB 4:4:4 - TGM 720p | 0.00% | 0.00% | 0.00% | 117% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 111% | 99% |

Table 2. Test2 vs VTM12.0

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12 |  |  |
|  | Y / G | U / B | V / R | EncT | DecT |
| RGB 4:4:4 - TGM 1080p | -1.87% | -1.94% | -1.92% | 93% | 97% |
| RGB 4:4:4 - TGM 720p | -4.73% | -4.85% | -4.90% | 88% | 93% |
| Overall | -3.30% | -3.40% | -3.41% | 90% | 95% |

|  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12 |  |  |
|  | Y / G | U / B | V / R | EncT | DecT |
| RGB 4:4:4 - TGM 1080p | -1.51% | -1.52% | -1.52% | 92% | 97% |
| RGB 4:4:4 - TGM 720p | -2.69% | -3.32% | -3.01% | 92% | 95% |
| Overall | -2.10% | -2.42% | -2.27% | 92% | 96% |

# Conclusion

The partial results obtained in the cross-check match the results presented in JVET-V0066. Please note that the encoder and decoder time in the cross-check are not reliable.