| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V00177 |
| --- | --- |

| Title: | CE-related: Additional information for CE on HBD coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Adrian Browne Tomonori Hashimoto Hong-Jheng Jhu Dmytro Rusanovskyy Kei Kawamura Tianyang Zhou | Email: | adrian.browne@sony.com tomonori.hashimoto@sharp.co.jp jhuhong-jheng@kwai.com dmytror@qti.qualcomm.com ki-kawamura@kddi.com zhou.tianyang@sharp.co.jp |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This contribution presents additional results on the RRC methods for high bit depth video coding being studied in CE. Results include performance comparison for selected RRC methods against the HM16.23. as well as additional results on the performance of the Method 2 presented in JVET-T0105 [6]. Complexity increase compare to VVC is provided for selected RRC methods.

# Introduction

During the presentation of CE report, ther was inquiry to provide additional information on the performance of the RRC methods being tested in CE against the HM16.23 anchor. Additionally, there was a request to provide information on the performance of the Method 2 presented in JVET-T0105 [6].

# Additional simulation results

# Comparison against HM16.23

Results below have been produced by comparing reported results for CE tests against the HM16.23 anchor, provided by AhG8. Complete Excel tables are provided along this contribution.

Table 1 shows coding gain achieved by selected methods against the HM16.23. Impact of EGk invoking parameters change on top of selected tests is provided for SVT16 data separately in the CE3.1 (EGk) column.

Table 1. Summary of results (BD-rate gain) for selected RRC tests, CE CTC, LowQP test configuration.

|  |  | HBD CTC |  |  |  |  |  |  |  | CE3.1 (Egk) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test vs HM16.23 | HDR PQ |  |  | HDR HLG |  |  | SVT12 RGB | SVT16 RGB | SVT16 RGB |
|  |  | wY | wU | wV | Y | U | V | Aver. GBR | Aver. GBR | Aver. GBR |
| AI | VTM12 | -6.79% | -9.19% | -9.75% | -3.68% | -6.06% | -6.19% | 0.46% | 34.93% | 26.67% |
|  | AnchorCE | -7.24% | -9.64% | -10.23% | -4.09% | -6.39% | -6.53% | -2.17% | 0.77% | 0.24% |
|  | CE1.4-A | -7.56% | -9.85% | -10.44% | -4.57% | -6.67% | -6.82% | -3.56% | -1.55% | -1.74% |
| LDB | VTM12 | -6.61% | -7.67% | -8.77% | -5.14% | -8.18% | -7.01% | -1.35% | 21.09% | 17.88% |
|  | AnchorCE | -6.68% | -7.73% | -8.82% | -5.12% | -8.20% | -7.06% | -1.79% | 0.58% | 0.35% |
|  | CE1.4-A | -6.73% | -7.79% | -8.89% | -5.14% | -8.21% | -7.05% | -2.43% | -0.99% | -1.08% |
| RA | VTM12 | -6.81% | -7.91% | -8.64% | -5.72% | -8.64% | -7.32% | -1.64% | 19.55% | 16.42% |
|  | AnchorCE | -6.93% | -8.04% | -8.77% | -5.73% | -8.68% | -7.37% | -2.11% | 0.32% | 0.08% |
|  | CE1.4-A | -7.01% | -8.10% | -8.85% | -5.77% | -8.69% | -7.38% | -2.67% | -1.14% | -1.22% |

Table 2 shows coding gain achieved by enabling extended precission flag, as per CE3.1, for selected methods against the HM16.23 for 12 bits data.

Table 2. Summary of results (BD-rate gain) for selected RRC tests, CE CTC, LowQP test configuration.

|  |  | CE3.1 Extended precission flag + Test vs. HM16.23 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Testvs HM | HDR PQ |  |  | HDR HLG |  |  | SVT12 RGB |
|  |  | wY | wU | wV | Y | U | V | Aver.GBR |
| AI | VTM12+ce3.1 | -7.88% | -10.64% | -11.29% | -4.16% | -6.81% | -6.96% | -0.41% |
|  | AnchorCE+ce3.1 | -8.35% | -11.13% | -11.81% | -4.54% | -7.14% | -7.30% | -3.27% |
|  | CE1.4-A+ce3.1 | -8.63% | -11.31% | -11.99% | -5.00% | -7.38% | -7.54% | -4.06% |
| LDB | VTM12+ce3.1 | -7.21% | -8.24% | -9.37% | -5.69% | -8.85% | -7.78% | -2.07% |
|  | AnchorCE+ce3.1 | -7.27% | -8.30% | -9.44% | -5.69% | -8.88% | -7.81% | -2.48% |
|  | CE1.4-A+ce3.1 | -7.31% | -8.35% | -9.49% | -5.71% | -8.88% | -7.79% | -2.86% |
| RA | VTM12+ce3.1 | -7.32% | -8.44% | -9.22% | -6.17% | -9.25% | -8.01% | -2.30% |
|  | AnchorCE+ce3.1 | -7.45% | -8.58% | -9.37% | -6.19% | -9.29% | -8.05% | -2.74% |
|  | CE1.4-A+ce3.1 | -7.52% | -8.64% | -9.44% | -6.22% | -9.29% | -8.05% | -3.07% |

Table 3. Summary of results (BD-rate gain) for selected RRC tests, CE CTC, Lossless test configuration.

|  | Test | bit-rate saving |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | HDR PQ | HLG PQ | SVT12 | SVT16 |
| AI | VTM | 2.93% | 7.05% | 31.02% | 73.47% |
|  | AnchorCE | 0.36% | 2.82% | 3.12% | 1.46% |
|  | CE1.4-A | -1.97% | -0.77% | 0.15% | 0.05% |
| LDB | VTM | 3.67% | 2.09% | 29.88% | 80.20% |
|  | AnchorCE | 4.10% | 3.96% | 2.71% | 1.18% |
|  | CE1.4-A | 0.90% | -0.52% | 1.16% | 0.79% |
| RA | VTM | 3.30% | 1.85% | 29.30% | 80.25% |
|  | AnchorCE | 3.80% | 3.79% | 2.55% | 1.11% |
|  | CE1.4-A | 0.70% | -0.59% | 1.10% | 0.74% |

# Additional results for RRC method 2 from JVET-T0105

As per request on performance of RRC method 2 from JVET-T0105, including history-based extention, additional simulation results have been provided for this method in JVET-V0XXX.

Table below provide summary results reported for JVET-T0105 method (labled as T0105) and its history-based extension (labled as T0105+Hist) being tested under JVET-U2018 CTC, low QP configuration.

Current results have been produced by Qualcomm and being currently cross-checked by Kwai.

Table 4. Summary of results (BD-rate gain) for described RRC tests, CE CTC, LowQP test configuration.

| Test | HDR PQ |  |  | HDR HLG |  |  | SVT12 RGB | SVT16 RGB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | wY | wU | wV | Y | U | V | Aver.GBR | Aver.GBR |
| T0105 | 0.00% | 0.01% | 0.01% | 0.01% | 0.01% | 0.00% | 0.03% | 0.17% |
| T0105+Hist | -0.27% | -0.19% | -0.20% | -0.46% | -0.31% | -0.33% | -1.14% | -1.54% |
| T0105 | -0.01% | -0.01% | -0.02% | 0.00% | 0.00% | 0.02% | 0.00% |  |
| T0105+Hist | -0.08% | -0.09% | -0.11% | -0.06% | -0.05% | -0.04% | -0.54% | -1.52% |
| T0105 | 0.01% | 0.00% | 0.00% | 0.00% | 0.01% | 0.00% |  |  |
| T0105+Hist | -0.09% | -0.09% | -0.11% | -0.07% | -0.06% | -0.07% |  | -1.37% |

# Complexity assessment

Complexity assessment for RRC methods under investigation in CE have been conducted at the last U-meeting in the BoG. This table provides extract of the complexity assessment tables for selected methods with limmited changes and clarifications.

Additionally, complexity assessment for RRC method 2 from JVET-T0105 and CE Anchor method are provided.

## Method2 of JVET-T0105 Complexity increase vs. VVC

| Additional operations (spec text) vs VVCv1 | Operations Per Rice Call | Num Ops | Comments |
| --- | --- | --- | --- |
| Values RiceT and ShiftRice are initialized as following: RiceT = 32, Shift = sh_shift_rice |  |  | This is slice level. No additional process in TU. Bitdepth dependent syntax element sh_shift_rice. |
| Variable shift is derived as following for idx set equal to 0, if baseLevel is equal to 0, and set equal to 1, otherwise |  |  | Idx dependency on baseLevel is TU level. |
| shift = (localSumAbs < RiceT) ? 0 : Shift locSumAbs = locSumAbs >> shift | 1 cond., 1 shift | 1 | Fixed comparator combined with slice-level fixed shift. |
| cRiceParam is then is refined as: cRiceParam = cRiceParam + shift | 1 add | 1 |  |
| Total increase per Rice call |  | 2 |  |
| Total increase per 4x4 TU (critical pass): |  | 32 |  |
| Total increase per 4x4 TU: |  | 32 |  |
| Memory increase: | NO |  |  |
| Dependencies: | NO |  |  |

## CE Anchor Complexity increase vs. VVC

This test features no inter-block dependencies.

| Additional operations (spec text) vs VVCv1 | Operations Per Rice Call | Num Ops | Comments |
| --- | --- | --- | --- |
| Values RiceT are initialized as following: RiceT = {{32, 52}, {32 << sh_shift_rice, 52 << sh_shift_rice}} |  |  | This is slice level. No additional process in TU |
| Variable shift is derived as following for idx set equal to 0, if baseLevel is equal to 0, and set equal to 1, otherwise |  |  | Idx dependency on baseLevel is TU level. |
| shift = (localSumAbs < RiceT[0][idx]) ? 0 : (localSumAbs < RiceT[1][idx]) ? sh_shift_rice : (sh_shift_rice << 1) locSumAbs = locSumAbs >> shift | 1 cond., 1 shift | 1 | Parallel comparator with fixed thresholds and shifts. |
| cRiceParam is then is refined as: cRiceParam = cRiceParam + shift | 1 add | 1 |  |
| Total increase per Rice call |  | 2 |  |
| Total increase per 4x4 TU (critical pass): |  | 32 |  |
| Total increase per 4x4 TU: |  | 32 |  |
| Memory increase: | LUT1[4], LUT2[6] of ROM. |  |  |
| Dependencies: | NO |  |  |

## CE1.1 Complexity increase vs. VVC

This test features no inter-block dependencies.

| Additional operations (spec text) vs VVCv1 | Operations Per Rice Call | Num Ops | Comments |
| --- | --- | --- | --- |
| Values Tx and Rx are initialized as following: Tx[4] = { 32,128, 512, 2048 } Rx[5] = { 0, 2, 4, 6, 8} |  |  | This parameters are fixed. |
| shift = (localSumAbs < Tx[0]) ? Rx[0] : ( (localSumAbs < Tx[1]) ? Rx[1]: ((localSumAbs < Tx[2]) ? Rx[2]: ((localSumAbs < Tx[3]) ? Rx[3]: Rx[4]))) locSumAbs = locSumAbs >> shift | 1 cond., 1 shift in parallel | 1 | Parallel comparator can be used. Fixed shift integrated with parallel comparator. |
| cRiceParam is then is refined as: cRiceParam = cRiceParam + shift | 1 add | 1 |  |
| Total increase per Rice call |  | 2 |  |
| Total increase per 4x4 TU (critical pass): |  | 32 |  |
| Total increase per 4x4 TU: |  | 32 |  |
| Memory increase: | LUT1[4], LUT2[5] of ROM. |  |  |
| Dependencies: | NO |  |  |