| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 April 2021 | Document: JVET-V0178-v1 |
| --- | --- |

| Title: | AHG8: a combination of JVET-V0059 option 2 and JVET-V0122 for high bit depth and high bit rate extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fan Wang Zhihuang Xie Yue Yu Haoping Yu Dong Wang | Email: | wangfan6@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

In JVET-V0059, a method of using CABAC bypass alignment and a bypass coding of sb_coded_flag is proposed as option 2 to improve the throughput. It is asserted that with the proposed method, alignment is needed only once after the limit of the context coded bins has been reached for the TB. In JVET-V0122, a full-bypass mode in residual coding is proposed to improve the throughput and software encoding/decoding speed. It is asserted that with the proposed method, the coefficient or residual levels are only coded through bypass mode of the arithmetic coding engine without using any context-coded bin. It is observed that the simulation results showed over 15% reduction in average encoding and decoding time, even with a slight coding efficiency gain, from the 16-bit SVT sequences.

This contribution proposes a combination of JVET-V0059 option 2 and JVET-V0122. It is asserted that with the proposed method, all the syntax elements in residual coding, except for last significant coefficient position in RRC, are coded through bypass mode and alignment is needed only once after the last significant coefficient position in RRC and at the very beginning of a TB in TSRC. It is suggested to apply the proposed method in 16 bit high throughput configuration.

Following results for SVT 16-bit sequences are reported. It is believed that the proposed method in this contribution can truly enable decoding of all the coefficients in a TB simply by using a shift register and multiple coded bins can be decoded in parallel.

As compared to CE anchor (low QP):

- AI:  0.49 % (G), 0.45 % (B), 0.43 % (R),  86 % (EncT), 77 % (DecT)

- LDB: x.xx % (G), x.xx % (B), x.xx % (R),  xx % (EncT), xx % (DecT)

- RA:  x.xx % (G), x.xx % (B), x.xx % (R),  xx % (EncT), xx % (DecT)

For reference, below are the results reported in JVET-V0059 option 2:

- AI:  0.47 % (G), 0.49 % (B), 0.49 % (R),  99 % (EncT), 94 % (DecT)

- LDB: 0.55 % (G), 0.54 % (B), 0.54 % (R),  100 % (EncT), 94 % (DecT)

- RA:  0.57 % (G), 0.56 % (B), 0.56 % (R),  100 % (EncT), 94 % (DecT)

The full simulation results are included in the following of this document.

# Introduction

## JVET-V0059 option 2[1]: CABAC bypass alignment + context/bypass coding of sb_coded_flag

In order to improve the throughput of the CABAC engine, HEVC range extension (the High Throughput 4:4:4 16 Intra Profile) supports aligning the CABAC process prior to coding of bypass data, which allows easy, simultaneous decoding of multiple bypass bins[2]. JVET-U0069[3] proposed to use the same process as in HEVC range extension where the value of the ivlCurrRange of CABAC engine is set to 256 right before starting of the bypass coding of a coefficient group. After aligning the ivlCurrRange to 256, the decoding process of a bypass coded bin can be simplified. In order to guarantee that there is no context coded bins after reaching to the maximum limits, it was proposed in JVET-P0370 [4]that the sb_coded_flag (of both RRC and TSRC) be bypass coded after reaching to the number of context coded bins to the maximum limit. In order to reduce the number of bypass alignments, in option 2, the proposed alignment method (described in option 1) is combined with sb_coded_flag coding method proposed in JVET-P0370 when CABAC bypass alignment is enabled.

Following is the pseudo-code of option 2. The sps_cabac_bypass_alignment_enabled_flag is set to 1 to enable the proposed method. If the value of sps_cabac_bypass_alignment_enabled_flag is equal to 0, the proposed method is disabled and the behavior is same as current VVC. The variables remBinsPass1 and RemCcbs represent the remaining number of context coded bins available in the TB in regular and transform-skip residual coding, respectively. The red italic portions are the additional process as compared to the current VVC.

In case of regular residual coding:

- remBinsPass1 = ( ( 1  <<  ( log2TbWidth + log2TbHeight ) ) * 7 )  >>  2

- For each coefficient group

  - if (remBinsPass1  >=  4 || sps_cabac_bypass_alignment_enabled_flag == 0)

    - CABACBypasIntervalAlign = 0

    - context_coding of sb_coded_flag

  - else

    - bypass_coding of sb_coded_flag

  - if (sb_coded_flag ==1)

    - if (remBinsPass1  >=  4)

      - Pass 1:

        - sig_coeff_flag; remBinsPass1--;

        - abs_level_gtx_flag[ ][ 0 ] ; remBinsPass1--;

        - par_level_flag; remBinsPass1--;

        - abs_level_gtx_flag[ ][ 1 ] ; remBinsPass1--;

    - If (CABACBypasIntervalAlign == 0 && sps_cabac_bypass_alignment_enabled_flag)

      - CABAC bypass alignment (i.e.set ivlCurrRange to 256)

      - CABACBypasIntervalAlign = 1

    - Pass 2: abs_remainder, dec_abs_level

    - Pass 3: coeff_sign_flag

In case of transform-skip residual coding:

- RemCcbs = ( ( 1  <<  ( log2TbWidth + log2TbHeight ) ) * 7 )  >>  2

- For each coefficient group

  - if (RemCcbs >= 4 || sps_cabac_bypass_alignment_enabled_flag == 0)

    - CABACBypasIntervalAlign = 0

    - context_coding of sb_coded_flag

  - else

    - bypass_coding of sb_coded_flag

  - if (sb_coded_flag ==1)

    - if (RemCcbs >= 4)

      - Pass 1:

        - sig_coeff_flag; RemCcbs--;

        - coeff_sign_flag; RemCcbs--;

        - abs_level_gtx_flag[ ][ 0 ] ; RemCcbs--;

        - par_level_flag; RemCcbs--;

    - if (RemCcbs >= 4)

      - Pass 2:

        - abs_level_gtx_flag [ ][ 1 ]; RemCcbs--;

        - abs_level_gtx_flag [ ][ 2 ] ; RemCcbs--;

        - abs_level_gtx_flag [ ][ 3] ; RemCcbs--;

        - abs_level_gtx_flag [ ][ 4 ] ; RemCcbs--;

    - If (CABACBypasIntervalAlign == 0 && sps_cabac_bypass_alignment_enabled_flag)

      - CABAC-bypass alignment (i.e.set ivlCurrRange to 256)

      - CABACBypasIntervalAlign = 1

    - Pass 3: abs_remainder, coeff_sign_flag

Figure 1 shows an example of the bit-stream of option 2. It shows that after the number of context coded bins has reach the limit for the current TB, only bypass coding bins are processed and the bypass alignment is necessary at most once per TB.

Figure 1: JVET-V0059 option 2

## JVET-V0122[5]

This contribution proposes to use a full-bypass mode in residual coding for high bit depth, high bit rate coding. To implement this method, the initialization methods of remBinsPass1 in RRC and RemCcbs in TSRC are modified as follows, with changes highlighted in red color.

For RRC:

#### 7.3.11.11Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| … |  |
| if(sps_level_ccbs_disabled_flag) { |  |
| remBinsPass1 = 0 |  |
| }else{ |  |
| remBinsPass1 = ( ( 1 << ( log2TbWidth + log2TbHeight ) ) * 7 ) >> 2 |  |
| } |  |
| … |  |

The new syntax element sps_level_ccbs_disabled_flag is a SPS level flag which enables/disables the proposed method. If sps_level_ccbs_disabled_flag is equal to 1, the value of remBinsPass1 is set to 0. As a result, the context-adaptive coding of sig_coeff_flag, abs_level_gtx_flag and par_level_flag is always skipped, and the coefficient levels at all positions are always coded by dec_abs_level and coeff_sign_flag in the bypass mode. If sps_level_ccbs_disabled_flag is equal to 0, the current VVC RRC remains unchanged.

For TSRC:

| residual_ts_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| … |  |
| if(sps_level_ccbs_disabled_flag) { |  |
| RemCcbs = 0 |  |
| }else{ |  |
| RemCcbs = ( ( 1 << ( log2TbWidth + log2TbHeight ) ) * 7 ) >> 2 |  |
| } |  |
| … |  |

If sps_level_ccbs_disabled_flag is equal to 1, the value of RemCcbs is set to 0, all the context coded bins, including sig_coeff_flag, abs_level_gtx_flag par_level_flag and coeff_sign_flag, are always skipped. The residual levels at all positions are coded by abs_remainder and coeff_sign_flag in the bypass mode. If sps_level_ccbs_disabled_flag is equal to 0, the current VVC TSRC remains unchanged.

# Proposed method

This contribution proposes a combination of JVET-V0059 option 2 and JVET-V0122. It is asserted that with the proposed method, all the syntax elements in residual coding, except for last significant coefficient position in RRC, are coded through bypass mode and alignment is needed only once after the last significant coefficient position in RRC and at the very beginning of a TB in TSRC. It is suggested to apply the proposed method in 16 bit high throughput configuration.

A SPS level syntax element sps_high_throughput_flag is used to enable/disable the proposed method. If sps_high_throughput_flag is equal to 0, the current RRC and TSRC in VVC remains unchanged. If sps_high_throughput_flag is equal to 1, the following aspects are changed:

- The initial value of remBinsPass1 in RRC and RemCcbs in TSRC are set to 0, i.e. the maximum number of context coded bins is set to 0. As a result the coefficient or residual levels at all positions are coded only in the bypass mode.

- The alignment happens before going into sub-block, i.e. after last_sig_coeff_x_prefix, last_sig_coeff_y_prefix, last_sig_coeff_x_suffix and last_sig_coeff_y_suffix if any of residual_coding in RRC, and at the very beginning of residual_ts_coding in TSRC.

- All the syntax elements of sb_coded_flag are always coded in bypass mode.

Following is the pseudo-code.

In case of regular residual coding:

- For last significant coefficient position

  - context_coding of last_sig_coeff_x_prefix, last_sig_coeff_y_prefix, last_sig_coeff_x_suffix and last_sig_coeff_y_suffix

- For remBinsPass1 and alignment

  - if (sps_high_throughput_flag == 0)

    - remBinsPass1 = ( ( 1  <<  ( log2TbWidth + log2TbHeight ) ) * 7 )  >>  2

  - Else

    - remBinsPass1 = 0

    - CABAC bypass alignment (i.e.set ivlCurrRange to 256)

- For each coefficient group

  - if (sps_high_throughput_flag == 0)

    - context_coding of sb_coded_flag

  - else

    - bypass_coding of sb_coded_flag

  - if (sb_coded_flag ==1)

    - if (remBinsPass1  >=  4)

      - Pass 1:

        - sig_coeff_flag; remBinsPass1--;

        - abs_level_gtx_flag[ ][ 0 ] ; remBinsPass1--;

        - par_level_flag; remBinsPass1--;

        - abs_level_gtx_flag[ ][ 1 ] ; remBinsPass1--;

        - set firstPosMode1

    - Pass 2:

      - if (n > firstPosMode1 && abs_level_gtx_flag[ ][ 1 ] == 1) abs_remainder

      - if (n <= firstPosMode1) dec_abs_level

    - Pass 3: coeff_sign_flag

In case of transform-skip residual coding:

- For RemCcbs and alignment

  - if (sps_high_throughput_flag == 0)

    - RemCcbs = ( ( 1  <<  ( log2TbWidth + log2TbHeight ) ) * 7 )  >>  2

  - else

    - RemCcbs = 0

    - CABAC bypass alignment (i.e.set ivlCurrRange to 256)

- For each coefficient group

  - if (sps_high_throughput_flag == 0)

    - context_coding of sb_coded_flag

  - else

    - bypass_coding of sb_coded_flag

  - if (sb_coded_flag ==1)

    - if (RemCcbs >= 4)

      - Pass 1:

        - sig_coeff_flag; RemCcbs--;

        - coeff_sign_flag; RemCcbs--;

        - abs_level_gtx_flag[ ][ 0 ] ; RemCcbs--;

        - par_level_flag; RemCcbs--;

    - if (RemCcbs >= 4)

      - Pass 2:

        - abs_level_gtx_flag [ ][ 1 ]; RemCcbs--;

        - abs_level_gtx_flag [ ][ 2 ] ; RemCcbs--;

        - abs_level_gtx_flag [ ][ 3] ; RemCcbs--;

        - abs_level_gtx_flag [ ][ 4 ] ; RemCcbs--;

    - Pass 3: abs_remainder, coeff_sign_flag

Figure 2 shows an example of the bit-stream of the proposed method in RRC.

Figure 2: proposed method in RRC

Figure 3 shows an example of the bit-stream of the proposed method in TSRC.

Figure 3: proposed method in TSRC

# Simulation results

The test conditions in this contribution follow the CTC for high bit depth and high bit rate video coding [6]. The anchor is generated CE base [7], the test is produced by the CE base software with proposed method.

Low QP:

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE BASE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.49% | 0.45% | 0.43% | 86% | 77% |
| SVT12 | 2.07% | 2.05% | 2.02% | 83% | 82% |
| Overall | 1.28% | 1.25% | 1.22% | 85% | 79% |
|  |  |  | LDB |  |  |
|  |  |  | Over CE BASE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| SVT12 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  |  |  | RA |  |  |
|  |  |  | Over CE BASE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| SVT12 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall RGB | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE BASE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 3.50% | 5.17% | 5.65% | 89% | 91% |
| HLG422 | 3.42% | 4.79% | 5.21% | 84% | 92% |
| Overall | 3.46% | 4.98% | 5.43% | 86% | 92% |
|  |  |  | LDB |  |  |
|  |  |  | Over CE BASE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| HLG422 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  |  |  | RA |  |  |
|  |  |  | Over CE BASE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| HLG422 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall HLG | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE BASE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 7.59% | 10.61% | 10.91% | 7.42% | 10.35% | 10.74% | 92% | 92% |
| PQ422 | 6.03% | 8.04% | 7.95% | 5.87% | 7.87% | 7.82% | 88% | 91% |
| Overall | 6.81% | 9.32% | 9.43% | 6.64% | 9.11% | 9.28% | 90% | 92% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over CE BASE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| PQ422 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over CE BASE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| PQ422 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall PQ | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Lossless:

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | CE base | proposed |  | CE base | proposed |  | CE base | proposed |  |
| PQ444 | 2.5 | 2.4 | 3.02% | 0.0 | 0.0 | 0.00% | 0.0 | 0.0 | 0.00% |
| PQ422 | 2.3 | 2.3 | 2.61% | 0.0 | 0.0 | 0.00% | 0.0 | 0.0 | 0.00% |
| Overall | 2.4 | 2.3 | 2.82% | 0.0 | 0.0 | 0.00% | 0.0 | 0.0 | 0.00% |
| Enc Time[%] | 100% |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| Dec Time[%] | 69% |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | CE base | proposed |  | CE base | proposed |  | CE base | proposed |  |
| HLG444 | 1.7 | 1.7 | 0.71% | 0.0 | 0.0 | 0.00% | 0.0 | 0.0 | 0.00% |
| HLG422 | 1.6 | 1.6 | 0.84% | 0.0 | 0.0 | 0.00% | 0.0 | 0.0 | 0.00% |
| Overall | 1.7 | 1.7 | 0.78% | 0.0 | 0.0 | 0.00% | 0.0 | 0.0 | 0.00% |
| Enc Time[%] | 98% |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| Dec Time[%] | 59% |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | CE base | proposed |  | CE base | proposed |  | CE base | proposed |  |
| SVT16 | 1.2 | 1.2 | -0.20% | 0.0 | 0.0 | 0.00% | 0.0 | 0.0 | 0.00% |
| SVT12 | 1.2 | 1.2 | 0.16% | 0.0 | 0.0 | 0.00% | 0.0 | 0.0 | 0.00% |
| Overall | 1.2 | 1.2 | -0.02% | 0.0 | 0.0 | 0.00% | 0.0 | 0.0 | 0.00% |
| Enc Time[%] | 91% |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| Dec Time[%] | 67% |  |  | #DIV/0! |  |  | #DIV/0! |  |  |

## Analysis

For SVT 16-bit sequences, the proposed method can achieve over 15% reduction in average encoding and decoding time. For 12-bit SVT and PQ/HLG sequences, the proposed method also brings about 10% notable reduction in decoding time for low QP coding, but there is loss in coding efficiency. The main reason for this loss is that the coding bitrate is not high enough.

# Conclusion

This contribution proposes a combination of JVET-V0059 option 2 and JVET-V0122. It is asserted that with the proposed method, all the syntax elements in residual coding, except for last significant coefficient position in RRC, are coded through bypass mode and alignment is needed only once after the last significant coefficient position in RRC and at the very beginning of a TB in TSRC. It is believed the main benefit of the proposed method is to enable decoding of all the coefficients in a TB simply by using a shift register and multiple coded bins can be decoded in parallel. As a result, the goal of high coding throughput is achieved. It is suggested to apply the proposed method in 16-bit high throughput configuration.

# References

[2] M.G. Sarwer, J. Chen, Y. Ye, R. -L. Liao, “AHG8: CABAC-bypass alignment for high bit-depth coding”, JVET-V0059

[3]	K. Sharman, N. Saunders, J. Gamei, “AHG 5 and 18: Entropy Coding Throughput for High Bit Depths”, JCTVC-N0190

[3] M. G. Sarwer, J. Chen, Y. Ye, R. -L. Liao, “AHG8: CABAC-bypass alignment for high bit-depth coding”, JVET-U0069

[4] M. G. Sarwer, R. L. Liao, J. Luo, Y. Ye, “ Non-CE7: Signaling of coded_sub_block_flag”, JVET-P0370

[5] F. Wang, Z. Xie, Y. Yu, H. Yu, D. Wang, “AHG8: a full-bypass mode in residual coding for high bit depth and high bit rate extensions”, JVET-V0122

[6] A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018

[7] A. Browne, T. Hashimoto, H.-J. Jhu, D. Rusanovskyy, K. Kawamura, T. Zhou “CE on entropy coding for high bit depth and high bit rate coding”, JVET-U2022

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).