| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23nd Meeting, by teleconference, 7-16th July. 2021 | Document: JVET-W0091-v2 |
| --- | --- |

| Title: | AHG8: On constraining of bit depth of ALF classifier and CCLM derivation for coding of high bit-depth video data. |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy Marta Karczewicz | Tel: Email: | dmytror@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution presents results of a study on reusage of 10 bits implementations for ALF classifier and CCLM parameters derivation for higher bit-depth VVC coding. Tools under consideration has been tested under CTC for HBD/HBR video coding, JVET-T2018.

Under LowQP CTC, ALF reportedly provides around 0.4%/1.5%/1.4% of luma gain for AI/LDB/RA configuration of PQ12, respectively, and 0.05%/1.2%/1.2% of luma gain for AI/LDB/RA configurations of HLG12, respectively. It is reported that constraining operations of ALF classifiers to 10 bits input, no penalty on average is observed over PQ12/HLG12 and SVT classes in low QP configuration.

In normal QP configuration, ALF reportedly provides around 2.1% of luma gain and 28.4% of chroma gain on average across PQ12/HLG12 classes and AI/RA configurations. Constraining operations of ALF classifiers to 10 input bits, brings ~0.1% of penalty for luma and ~0.1% of chroma gain on average in PSNR metric.

CCLM gain in AI/LDB/RA configuration of HBD coding are 2.3%/0.2%/0.4% respectively, for PQ12, and 0.7%/0.1%/0.1%, respectively for HLG12. For SVT, reported CCLM gain is 1.2%/0.14%/0.5% on average over RGB, in AI/LDB/RA, respectively. It is reported that constraining operations of CCLM derivation to 10 bits, brings 0.01% penalty on average over PQ12/HLG12/SVT classes in low QP configuration over.

In normal QP, CCLM reportedly provides around 1.3% of luma gain and 24.2% of chroma gain on average across PQ12/HLG12 classes and AI/RA configurations. Constraining operations of CCLM derivation to 10 bits, brings ~0.1% of luma and ~0.5% of chroma penalty on average in PSNR metric.

It is proposed to consider constraining ALF classifier and CCLM derivation process to 10 bits video data to allow reusage of HW implementations of VVC Main10 for high bit-depth, high bitrate VVC coding.

# Introduction

At the previous meetings, a study on potential VVC extension for coding of high bit-depth video data was initiated [1]. It argued by the proponent of this contribution, that potential deployment of such high bit depth extension will benefit from ability to reuse HW implementations of certain VVC tools that are expected to be developed for 10 bits profiles and deployed earlier. In this contribution, elements of two VVC tools: ALF and CCLM, have been tested with 10bits constrained implementation following the AhG8 CTC proposed in [3].

## ALF classifier

Luma ALF processing in VVC consists of 3 stages: classifier, filter derivation and sample filtering. Two of these stages operate over the reconstructed samples: Classifier and Filtering. With coding of high bit-depth video data, there would be a need to redesign these two components to support higher bit depth.

To reduce the burden of ALF implementation for higher bit depth, it is suggested to constrain bit-depth of the ALF classifier operations to 10 bits, utilizing only 10 MSB of the reconstructed samples for classification as input to the clause 8.8.5.3. No changes to sample filtering process a proposed. Please see fragment of proposed VVC specification.

8.8.5.2.Coding tree block filtering process for luma samples Inputs of this process are: - a reconstructed luma picture sample array recPicture prior to the adaptive loop filtering process, - a filtered reconstructed luma picture sample array alfPictureL, - a luma location ( xCtb, yCtb ) specifying the top-left sample of the current luma coding tree block relative to the top-left sample of the current picture. Output of this process is the modified filtered reconstructed luma picture sample array alfPictureL. The variable numOperationalBits is set equal to 10. The array of luma picutre samples recPicture10Bits is derived from reconstructed luma picture sample of recPicture array by taking numOperationalBits most significant bits (MSB) of each luma sample. The derivation process for filter index subclause 8.8.5.3 is invoked with the location ( xCtb, yCtb ) and the reconstructed luma picture sample array recPicture10Bits as inputs, and filtIdx[ x ][ y ] and transposeIdx[ x ][ y ] with x, y = 0..CtbSizeY − 1 as outputs.

## CCLM

In CCLM of VVC, a predicted chroma block is generated based on a scaled version of the reconstructed luma block with use of linear model parameters. When LM prediction mode is selected, the following steps are invoked in order (as presented in Figure 2):

- Downsampling the neighboring luma samples, if needed, such as for 4:2:2 or 4:2:0 color format or access to neighboring samples in the case of 4:4:4.

- Derive the linear model parameters from neighboring luma and chroma samples.

- Downsampling the current luma block, if needed, and derive the prediction block for the chroma block from the luma block.

Figure 1 Decoding process of CCLM

To reduce the burden of CCLM implementation, it is suggested to constrain bit-depth of the LM parameter derivation and linear prediction processes to 10 bits. It is proposed to utilize only 10 MSB of the reconstructed luma samples for both derivation and prediction. For 4 chroma samples, Y’Cb’Cr representation, utilized in LM parameters derivation, a dynamical range shifting and limitation by clipping to the range 0..1023 is utilized.

Generated chroma samples as output of linear prediction are being reconstructed to the original dynamical range of the input bit depth.

Proposed changes to the VVC specification are limited to the clause 8.4.5.2.14, and extract of the specification text is shown below with introduced text marked with the green highlight.

##### 8.4.5.2.14 Specification of INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM intra prediction mode … Variable shift1 is set equal to max ( 0, (BitDepth – 10). If shift1 is greater than 0, variable offsetCh is set equal to ( (1 << ( BitDepth – 1) ) – ( 1 << 9). Otherwise, variable offsetCh are set equal to 0. ... - When numSampT is greater than 0, the selected neighbouring top chroma samples pSelC[ idx ] are set equal to Clip3( 0, 1023, p[ pickPosT[ idx ] ][ −1 ] – offsetCh ) with idx = 0..cntT − 1, and the down-sampled neighbouring top luma samples pSelDsY[ idx ] with idx = 0..cntT − 1 are specified as follows: - The variable x is set equal to pickPosT[ idx ]. - If both SubWidthC and SubHeightC are equal to 1, the following applies: pSelDsY[ idx ] = pY[ x ][ −1 ] >> shift1 (361) - Otherwise, the following applies: - If SubHeightC is not equal to 1 and bCTUboundary is equal to FALSE, the following applies: - If sps_chroma_vertical_collocated_flag is equal to 1, the following applies: pSelDsY[ idx ] = ( pY[ SubWidthC * x ][ −3 ] + pY[ SubWidthC * x − 1 ][ −2 ] + 4 * pY[ SubWidthC * x ][ −2 ] + (362) pY[ SubWidthC * x + 1 ][ −2 ] + pY[ SubWidthC * x ][ −1 ] + 4 ) >> ( 3 + shift1) - Otherwise (sps_chroma_vertical_collocated_flag is equal to 0), the following applies: pSelDsY[ idx ] = ( pY[ SubWidthC * x − 1 ][ −1 ] + pY[ SubWidthC * x − 1 ][ −2 ] + 2* pY[ SubWidthC * x ][ −1 ] + (363) 2 * pY[ SubWidthC * x ][ −2 ] + pY[ SubWidthC * x + 1 ][ −1 ] + pY[ SubWidthC * x + 1 ][ −2 ] + 4 ) >> ( 3 + shift1) - Otherwise (SubHeightC is equal to 1 or bCTUboundary is equal to TRUE), the following applies: pSelDsY[ idx ] = ( pY[ SubWidthC * x − 1 ][ −1 ] + 2 * pY[ SubWidthC * x ][ −1 ] + (364) pY[ SubWidthC * x + 1 ][ −1 ] + 2 ) >> ( 2 + shift1) … The prediction samples predSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0.. nTbH − 1 are derived as follows: predSamples[ x ][ y ] = Clip1( ( ( pDsY[ x ][ y ] * a ) >> k ) + b) + offsetCh (395)

# Simulation results

Both proposals have been implemented on top of the latest VTM-13.0 with HBD/HBR CTC configuration enabled [3]. Currently presented ALF OFF and CCLM OFF results have been tested with VTM-12.0.

## ALF

Table 1. ALF OFF test results against VTM12 anchor, under the HBR/HBR CTC, low QP.

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12+HBD CTC |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.37% | 0.97% | 1.63% | 0.32% | 0.96% | 1.61% | 99% | 95% |
| PQ422 | 0.34% | 0.78% | 1.38% | 0.29% | 0.74% | 1.37% | 99% | 91% |
| Overall | 0.35% | 0.88% | 1.51% | 0.30% | 0.85% | 1.49% | 99% | 93% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM12+HBD CTC |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.24% | 3.05% | 3.97% | 1.19% | 2.90% | 3.83% | 98% | 90% |
| PQ422 | 1.19% | 2.65% | 4.23% | 1.13% | 2.52% | 4.00% | 99% | 91% |
| Overall | 1.22% | 2.85% | 4.10% | 1.16% | 2.71% | 3.92% | 99% | 90% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM12+HBD CTC |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.19% | 2.98% | 3.90% | 1.11% | 2.80% | 3.79% | 100% | 88% |
| PQ422 | 1.11% | 2.68% | 4.43% | 1.02% | 2.53% | 4.19% | 100% | 92% |
| Overall | 1.15% | 2.83% | 4.16% | 1.06% | 2.67% | 3.99% | 100% | 90% |
| Overall PQ | 0.91% | 2.18% | 3.26% | 0.84% | 2.07% | 3.13% | 99% | 91% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12+HBD CTC |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.04% | 0.22% | 0.35% | 99% | 96% |
| HLG422 | 0.03% | 0.18% | 0.28% | 99% | 95% |
| Overall | 0.04% | 0.20% | 0.31% | 99% | 96% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12+HBD CTC |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.70% | 1.30% | 1.60% | 100% | 97% |
| HLG422 | 0.63% | 1.11% | 1.29% | 99% | 96% |
| Overall | 0.66% | 1.21% | 1.44% | 99% | 97% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12+HBD CTC |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.71% | 1.48% | 1.85% | 100% | 95% |
| HLG422 | 0.63% | 1.20% | 1.43% | 99% | 96% |
| Overall | 0.67% | 1.34% | 1.64% | 100% | 96% |
| Overall HLG | 0.46% | 0.92% | 1.13% | 99% | 96% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12+HBD CTC |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| SVT12 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12+HBD CTC |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 99% | 98% |
| SVT12 | 0.02% | 0.01% | 0.01% | 99% | 99% |
| Overall | 0.01% | 0.01% | 0.00% | 99% | 99% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12+HBD CTC |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| SVT12 | 0.03% | 0.01% | 0.01% | 100% | 99% |
| Overall | 0.01% | 0.01% | 0.01% | 100% | 99% |
| Overall RGB | 0.01% | 0.00% | 0.00% | 99% | 99% |

Table 2. Constrained ALF10 test results against VTM13 anchor, under the HBR/HBR CTC, low QP.

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13+HBD CTC |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| PQ422 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 101% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM13+HBD CTC |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.03% | -0.01% | -0.01% | -0.01% | 0.00% | -0.01% | 101% | 104% |
| PQ422 | 0.00% | 0.00% | 0.03% | 0.02% | 0.00% | 0.02% | 102% | 106% |
| Overall | -0.02% | 0.00% | 0.01% | 0.01% | 0.00% | 0.01% | 101% | 105% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM13+HBD CTC |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.04% | 0.01% | 0.01% | 0.05% | 0.01% | 0.01% | 101% | 104% |
| PQ422 | 0.02% | 0.01% | 0.01% | 0.04% | 0.01% | 0.01% | 101% | 102% |
| Overall | 0.03% | 0.01% | 0.01% | 0.05% | 0.01% | 0.01% | 101% | 103% |
| Overall PQ | 0.00% | 0.01% | 0.00% | 0.00% | 0.01% | 0.00% | 100% | 103% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13+HBD CTC |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | 0.00% | 0.01% | 99% | 100% |
| HLG422 | 0.00% | 0.00% | 0.01% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.01% | 99% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM13+HBD CTC |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.03% | -0.01% | 0.02% | 101% | 100% |
| HLG422 | 0.03% | -0.01% | -0.01% | 102% | 106% |
| Overall | 0.03% | -0.01% | 0.01% | 102% | 103% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM13+HBD CTC |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.02% | 0.00% | 0.02% | 101% | 103% |
| HLG422 | 0.02% | 0.02% | 0.03% | 100% | 102% |
| Overall | 0.02% | 0.01% | 0.02% | 101% | 103% |
| Overall HLG | 0.01% | 0.00% | 0.01% | 100% | 101% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM13+HBD CTC |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| SVT12 | 0.00% | 0.00% | 0.00% | 99% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM13+HBD CTC |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| SVT12 | 0.01% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 101% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM13+HBD CTC |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| SVT12 | 0.01% | 0.00% | 0.00% | 101% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Overall RGB | 0.00% | 0.00% | 0.00% | 100% | 101% |

Table 3. ALF OFF test results against VTM12 anchor, under the HBR/HBR CTC, normal QP.

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AnchorCE Over vtm12 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 18.64% | 1.97% | 1.52% | 20.49% | 68.32% | 1.46% | 21.83% | 60.20% | 98% | 88% |
| Class H2 |  |  |  |  |  | 2.67% | 29.00% | 17.07% | 98% | 91% |
| Overall | 18.64% | 1.97% | 1.52% | 20.49% | 68.32% | 2.07% | 25.41% | 38.64% | 98% | 90% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | AnchorCE Over vtm12 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 13.91% | 3.03% | 2.48% | 23.26% | 43.62% | 2.37% | 23.94% | 42.41% | 100% | 89% |
| Class H2 |  |  |  |  |  | 2.02% | 19.98% | 12.49% | 99% | 91% |
| Overall | 13.91% | 3.03% | 2.48% | 23.26% | 43.62% | 2.19% | 21.96% | 27.45% | 99% | 90% |

Table 4. Constrained ALF test results against VTM13 anchor, under the HBR/HBR CTC, normal QP.

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ALF10 Over vtm13 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.15% | 0.23% | 0.19% | -0.03% | 0.04% | 0.17% | -0.01% | 0.07% |  |  |
| Class H2 |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.15% | 0.23% | 0.19% | -0.03% | 0.04% |  |  |  |  |  |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | ALF10 Over vtm13 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.03% | 0.13% | 0.12% | -0.03% | -0.03% | 0.09% | -0.03% | -0.03% |  |  |
| Class H2 |  |  |  |  |  | 0.07% | -0.03% | -0.03% |  |  |
| Overall | 0.03% | 0.13% | 0.12% | -0.03% | -0.03% | 0.08% | -0.03% | -0.03% |  |  |

## CCLM

Table 1. CCLM OFF test results against VTM12 anchor, under the HBR/HBR CTC, low QP.

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12+HBD CTC |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 2.55% | 3.97% | 3.76% | 2.56% | 3.74% | 3.58% | 91% | 100% |
| PQ422 | 2.15% | 2.88% | 3.45% | 2.16% | 2.70% | 3.24% | 96% | 100% |
| Overall | 2.35% | 3.42% | 3.61% | 2.36% | 3.22% | 3.41% | 93% | 100% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM12+HBD CTC |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.73% | 1.46% | 1.90% | 0.73% | 1.22% | 1.62% | 98% | 98% |
| PQ422 | 0.48% | 1.09% | 1.92% | 0.49% | 0.89% | 1.62% | 98% | 99% |
| Overall | 0.61% | 1.28% | 1.91% | 0.61% | 1.05% | 1.62% | 98% | 99% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM12+HBD CTC |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.00% | 2.00% | 2.42% | 1.00% | 1.74% | 2.16% | 98% | 101% |
| PQ422 | 0.72% | 1.55% | 2.52% | 0.73% | 1.33% | 2.24% | 99% | 101% |
| Overall | 0.86% | 1.77% | 2.47% | 0.86% | 1.54% | 2.20% | 98% | 101% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12+HBD CTC |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.72% | 0.84% | 0.84% | 93% | 98% |
| HLG422 | 0.62% | 0.76% | 0.78% | 95% | 95% |
| Overall | 0.67% | 0.80% | 0.81% | 94% | 96% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12+HBD CTC |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.25% | 0.23% | 0.24% | 98% | 101% |
| HLG422 | 0.08% | 0.10% | 0.12% | 98% | 99% |
| Overall | 0.17% | 0.17% | 0.18% | 98% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12+HBD CTC |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.29% | 0.25% | 0.29% | 98% | 100% |
| HLG422 | 0.12% | 0.14% | 0.17% | 98% | 99% |
| Overall | 0.20% | 0.20% | 0.23% | 98% | 100% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12+HBD CTC |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.39% | 2.05% | 2.10% | 94% | 100% |
| SVT12 | -0.11% | 1.72% | 1.73% | 96% | 99% |
| Overall | -0.25% | 1.89% | 1.92% | 95% | 99% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12+HBD CTC |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.03% | 0.25% | 0.25% | 95% | 96% |
| SVT12 | -0.01% | 0.18% | 0.18% | 98% | 99% |
| Overall | -0.02% | 0.22% | 0.22% | 97% | 98% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12+HBD CTC |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.01% | 0.41% | 0.41% | 98% | 100% |
| SVT12 | -0.01% | 0.34% | 0.34% | 98% | 101% |
| Overall | -0.01% | 0.37% | 0.38% | 98% | 100% |

Table 2. Constrained CCLM10 test results against VTM13 anchor, under the HBR/HBR CTC, low QP.

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.02% | 0.05% | 0.08% | 0.02% | 0.05% | 0.07% | 99% | 98% |
| PQ422 | 0.05% | 0.07% | 0.10% | 0.05% | 0.07% | 0.09% | 100% | 99% |
| Overall | 0.03% | 0.06% | 0.09% | 0.03% | 0.06% | 0.08% | 100% | 98% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.00% | 0.00% | 0.03% | 0.00% | 0.00% | 0.02% | 101% | 98% |
| PQ422 | -0.01% | -0.01% | 0.03% | -0.01% | 0.00% | 0.02% | 101% | 98% |
| Overall | -0.01% | 0.00% | 0.03% | -0.01% | 0.00% | 0.02% | 101% | 98% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM-13.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.02% | 0.00% | 0.00% | 0.01% | 0.00% | 0.00% | 100% | 96% |
| PQ422 | 0.00% | 0.02% | 0.04% | 0.00% | 0.02% | 0.03% | 99% | 95% |
| Overall | 0.01% | 0.01% | 0.02% | 0.01% | 0.01% | 0.02% | 99% | 96% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.05% | 0.06% | 0.06% | 100% | 99% |
| HLG422 | 0.03% | 0.03% | 0.03% | 100% | 98% |
| Overall | 0.04% | 0.04% | 0.05% | 100% | 98% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.03% | 0.00% | 0.01% | 99% | 96% |
| HLG422 | 0.01% | 0.01% | 0.01% | 100% | 99% |
| Overall | 0.02% | 0.00% | 0.01% | 100% | 98% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | 0.00% | 0.00% | 99% | 97% |
| HLG422 | 0.01% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 98% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 101% | 102% |
| SVT12 | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.01% | 0.00% | 0.00% | 100% | 110% |
| SVT12 | 0.00% | 0.00% | 0.00% | 98% | 96% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 103% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-13.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 99% | 102% |
| SVT12 | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |

Table 7. CCLM OFF test results against VTM12 anchor, under the HBR/HBR CTC, normal QP.

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CCLM0 Over vtm12 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 15.28% | 0.84% | 0.71% | 19.54% | 21.45% | 0.71% | 18.42% | 18.94% | 101% | 102% |
| Class H2 |  |  |  |  |  | 1.01% | 29.09% | 22.10% | 101% | 101% |
| Overall | 15.28% | 0.84% | 0.71% | 19.54% | 21.45% | 0.71% | 18.42% | 18.94% | 101% | 102% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | CCLM0 Over vtm12 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 20.91% | 2.02% | 1.77% | 28.37% | 37.57% | 1.79% | 26.41% | 32.90% | 101% | 104% |
| Class H2 |  |  |  |  |  | 1.31% | 35.04% | 23.75% | 100% | 102% |
| Overall | 20.91% | 2.02% | 1.77% | 28.37% | 37.57% | 1.79% | 26.41% | 32.90% | 101% | 103% |

Table 8. Constrained CCLM 	test results against VTM12 anchor, under the HBR/HBR CTC, normal QP.

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CCLM10 Over vtm12 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.04% | 0.00% | 0.01% | -0.25% | -0.23% | 0.00% | -0.20% | -0.27% | 99% | 100% |
| Class H2 |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.04% | 0.00% | 0.01% | -0.25% | -0.23% |  |  |  |  |  |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | CCLM10 Over vtm12 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.02% | -0.01% | -0.01% | 0.04% | 0.06% | -0.02% | 0.06% | 0.03% | 99% | 100% |
| Class H2 |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.02% | -0.01% | -0.01% | 0.04% | 0.06% |  |  |  |  |  |

# Conclusion

This contribution present results of a study on reusage of 10 bits implementations of ALF classifier and CCLM for coding of video data at higher bit-depth. Based on provided results, it is proposed to consider constraining operations of ALF classifier and CCLM to 10 bits to allow reusage of HW implementations targeting VVC main10 for coding of high bit-depth video data.

# References

[1] A. Browne, T. Ikai, D.  Rusanovskyy, M.Sarwer, X. Xiu, “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018, JVET, 21th Meeting: by teleconference, 20-28 April. 2021

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).