| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0022-v1 |
| --- | --- |

| Title: | CE: Summary Report on Film Grain Synthesis |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Sean McCarthy Miloš Radosavljević Jay Shingala | Email: | sean.mccarthy@dolby.com milos.radosavljevic@interdigital.com jay.shingala@ittiam.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This contribution provides a summary report for the Core Experiment on film grain synthesis technology proposed at the W meeting of JVET. A total of 2 tests were completed within the CE between the JVET W and X meetings. In CE1, the use of the core 64x64 DCT-2 specified in VVC instead of the 64x64 DCT-2 defined in SMPTE RDD 5 specification was studied. In CE2, resolution-adaptive grain block size instead of fixed 8x8 grain block size was studied. CE1 results indicate that the selection of transform type does not affect subjective quality and does not change film grain processing time. CE1 also highlights additional implementation benefits. CE2 results indicate that resolution-adaptive grain block size does not affect subjective quality and reduces film grain processing time by approximately 20% for 4K content and 40-50% for 8K content. The software basis for the CE is VTM-13.2. JVET SDR-CTC Class A test sequences were use for complexity testing within the CE. Six test sequences from JVET FTP were used for subjective testing within the CE. All tests were conducted following CTC random access (RA) test conditions. Crosscheck results for the performed tests are submitted as separate documents and their summaries are integrated in this contribution.

# Methodology

The film grain synthesis (FGS) processes studied in this CE are based on SMPTE RDD 5 [1] with two independent modifications to test: CE1, DCT-2 transform type; and CE2, grain block size.

No change in the syntax of the film grain characteristics (FGC) SEI message is required.

The CE is described in JVET-W2022 [2].

## Software

CE test software is available on Git repo: https://vcgit.hhi.fraunhofer.de/jvet-w-ce/ce-fgs/-/tree/CE-FGS

CE test software is based on VTM-13.2 with extensions to be tested in CE proposed in JVET-V0093 [3] and JVET-W0095 [4].

CE software supports additional functionality as follows:

- Film grain analysis supporting multiple intensity intervals as proposed in JVET-W0072 [5]

- FGC SEI rewriter (described in section 1.1.1) enabling FGC SEI parameter values to be modified in existing bitstreams without re-encoding

### FGC SEI rewriter tool

In order to facilitate the FGS CE experiments, an FGC SEI rewriter tool based on VTM13.2 is developed and is released with the CE software. This tool allows FGC SEI parameter values to be modified in existing bitstreams without re-encoding. It also supports inserting FGC SEI messages in a VVC bitstream originally encoded without FGC SEI message. The bitstream with the inserted FGC SEI has identical MD5sum value to the bitstream having the FGC SEI enabled during encoding. The tool supports three operational modes, controlled by SEIFilmGrainOption: 0-no change; 1-FGC SEI removal; 2-FGC SEI insertion; 3-FGC SEI rewriter.

- FGC SEI removal	(applied on bitstreams with FGC SEI)

- FGC SEI insertion	(applied on bitstreams without FGC SEI)

- FGC SEI modification	(applied on bitstreams with FGC SEI)

The two typical use cases and examples are given below:

- A bitstream (test_fg1.bit) is encoded with a pre-configured FGC SEI (fg1.cfg), but the film grain parameters are not producing satisfactory film grain effects. One can use the tool to modify the values (fg2.cfg) of the existing FGC SEI. Example command line: 
./SEIFilmGrainAppStatic -c fg2.cfg -b test_fg1.bit -o test_fg2.bit --SEIFilmGrainOption=3

- A bitstream (test.bit) is encoded without FGC SEI so it cannot take advantage of benefit of the film grain synthesis. One can use the tool to easily insert the FGC SEI (fg.cfg) into the bitstream. Example command line:

./SEIFilmGrainAppStatic -c fg.cfg -b test.bit -o test_fg.bit --SEIFilmGrainOption=2

The tool manipulates the VVC bitstream at NALU level, so the processing time is extremely fast (e.g., to rewrite FGC SEI in a 4K/8K bitstream, the entire processing time is less than 0.1 sec). The basic workflow is illustrated in Figure 1.

Figure 1 Basic workflow of FGC SEI rewriter

## Complexity Tests

### Test sequences and conditions

CE complexity tests using 4K content were conducted with the JVET SDR-CTC [6] Class A test sequences. For tests using 8K content, JVET SDR-CTC Class A test sequences were upsampled to 8K using HDRTools-v0.19.1. All tests were conducted following CTC random access (RA) test conditions [6].

CE tests were conducted for both SIMD enabled and SIMD disabled.

The three combinations of FGC SEI parameter values listed in Table 1 were tested to investigate the impact of number of intensity intervals and number of colour components on processing times for grain blending.

Table 1 Film grain configuration settings for complexity test

| Film grain configuration | # of intensity intervals | Components |
| --- | --- | --- |
| FGC_SEI_I10C3 | 10 | Luma & Chroma |
| FGC_SEI_I10C1 | 10 | Luma only |
| FGC_SEI_I1C1 | 1 | Luma only |

### Calculation of film grain processing times

Absolute FGS processing time was measured with YUV output disabled. For each decoded bitstream, FGS processing time was calculated over all frames independently from decoding time. The reduction in FGS processing time is determined as follows:

Reduction in FGS processing time = (FT_CEx.y – FT_CEAnc ) / FT_CEAnc

where FT_CEAnc represents the FGS processing time for the anchor and FT_CEx.y represents the FGS processing time for CE1.1 (x = 1, y = 1), CE2.1 (x = 2, y = 1) and CE2.2 (x = 2, y = 2).

## Subjective Tests

For subjective tests, 6 test sequences with 1080p or 4K resolution (listed in Table 2) were selected from JVET FTP: ftp://ftp.ient.rwth-aachen.de. 4K test sequences were upsampled to 8K using HDRTools-v0.19.1. Random access test conditions were used with QP32 and QP37. Film grain configurations were adjusted for each clip. For CE2, decoded YUV files with and without FGS were viewed on a BenQ pro-level SW271C 27” 4K monitor.

Table 2 Test sequences (4K and 1080p) used in subjective test

| SeqName | Category | Resolution | FPS | Frames | BitDepth | MD5 of Source YUV |
| --- | --- | --- | --- | --- | --- | --- |
| MountainBay2 | Clean | 3840x2160 | 30 | 300 | 10 | f27b6b70244fb083baac546958fcf696 |
| OberbaumSpree | Clean | 3840x2160 | 60 | 600 | 10 | 6975b81c9e63c92b3bf4223796102da1 |
| Tango2 | Clean | 3840x2160 | 60 | 294 | 10 | 0471a59c423b7059c5c6c8b395e864a9 |
| RaceNight | Noisy | 3840x2160 | 50 | 600 | 10 | c06ed896df6e7dd90b4164df39652d50 |
| TrafficFlow | Noisy | 3840x2160 | 30 | 300 | 10 | 63135fc6fd7c73675f4ec1211b11dc93 |
| Crowdrun | Noisy | 1920x1080 | 50 | 500 | 8 | da34812b5b2c316d40481c7b6c841e41 |

## Crosscheck instructions

Detailed crosscheck instructions were provided to JVET before the beginning of the current meeting. They are available at: https://vcgit.hhi.fraunhofer.de/jvet-w-ce/ce-fgs/-/tree/CE-FGS within the “results” folder (see CEx_FGS_xCheck_Instruction.pptx). Bitstreams and additional supporting documents (per-sequence configurations, film grain configurations, md5sums) are also provided at the same location.

- Tests related to DCT2 transform type (CE1)

The goal of CE1.1 was to study the use of VVC DCT-2 in the context of film grain synthesis, compared to previously used SMPTE RDD 5 DCT-2 (anchor).

The DCT-2 transform is used to pre-compute a grain pattern database from which grain blocks are selected during the grain blending process.

Fixed 8x8 grain block size was used for both test and anchor in CE1.1.

  - Results for FGS processing times

FGS processing times are provided for CE2.1 in Table 3

Table 3: Results of the complexity test

|  | SIMD OFF |  |  | SIMD ON |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Configure | CEAnc FT (sec) | CE1.1 FT (sec) | Time Saving (%) | CEAnc FT (sec) | CE1.1 FT (sec) | Time Saving (%) |
| FGC_SEI_I10C3 | 8.57 | 8.67 | 1.21% | 3.99 | 3.87 | -2.96% |
| FGC_SEI_I10C1 | 5.43 | 5.53 | 1.89% | 2.50 | 2.49 | -0.21% |
| FGC_SEI_I1C1 | 5.93 | 6.01 | 1.29% | 2.66 | 2.59 | -2.96% |
| Average | 6.51 | 6.61 | 1.46% | 2.98 | 2.92 | -2.05% |

Time difference between anchor and CE1.1 test is shown to be negligible. Small difference is the result of testing error (the way we utilize our cluster and the fact that the time between two executions can slightly vary). Note that the absolute difference is relatively small. The observed performance was expected, since both versions are implemented in the same way by using matrix multiplication. Unification with VVC core transform leads to additional implementation cost savings and implementation simplifications, see Sec. 4.1.

  - Subjective test results

Subjectively, there is no difference between anchor and CE1.1. In addition, the average difference per pixel is 0.5905 over all sequences and QPs (mean squared error), computed on subjective test set. Such a small difference in pixel intensity is not visible to human eye.

It is to note that legacy SEI messages are not impacted by the proposed modifications, since the results are subjectively the same.

# Tests related to grain block size (CE2)

The goal of CE2 was to study resolution-adaptive grain block size. The anchor of CE2 was SMPTE RDD 5 with fixed 8x8 grain block size for all resolutions (HD, 4K and 8K). The test was use of resolution-adaptive BlockSize x BlockSize grain block size, where the variable BlockSize is determined as:

PicSizeInLumaSamples = PicHeightInLumaSamples * PicWidthInLumaSamples

if PicSizeInLumaSamples <= (1920 * 1080)
	BlockSize = 8
else if PicSizeInLumaSamples <= (3840 * 2160)
	BlockSize = 16
else
	BlockSize = 32

In addition, two sub-CEs tested different ways of pre-computing the grain database from which grain blocks are selected during grain blending. CE2.1 tested use of the transform specified in SMPTE RDD 5 to pre-compute the grain database. CE2.2 tested use of the DCT2 transform specified in VVC to pre-compute the grain database.

## Results for FGS processing times

FGS processing times are provided for CE2.1 in Table 4 and for CE2.2 in Table 5

Table 4 Average reduction in FGC processing time for CE2.1

|  |  | SIMD_OFF |  |  | SIMD_ON |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Content | Configure | CEAnc FT (sec) | CE2.1 FT (sec) | Reduction in FGS processing time (%) | CEAnc FT (sec) | CE2.1 FT (sec) | Reduction in FGS processing time (%) |
| CTC4K | FGC_SEI_I10C3 | 9.99 | 7.97 | -20.16% | 3.45 | 2.70 | -21.77% |
|  | FGC_SEI_I10C1 | 6.46 | 5.13 | -20.54% | 2.21 | 1.70 | -22.92% |
|  | FGC_SEI_I1C1 | 6.86 | 5.47 | -20.25% | 2.31 | 1.78 | -22.92% |
|  | Average | 7.62 | 6.07 | -20.32% | 2.60 | 2.01 | -22.54% |
|  |  | SIMD_OFF |  |  | SIMD_ON |  |  |
| Content | Configure | CEAnc FT (sec) | CE2.1 FT (sec) | Reduction in FGS processing time (%) | CEAnc FT (sec) | CE2.1 FT (sec) | Reduction in FGS processing time (%) |
| CTC8K | FGC_SEI_I10C3 | 39.78 | 17.41 | -56.25% | 13.33 | 7.47 | -43.98% |
|  | FGC_SEI_I10C1 | 25.83 | 12.88 | -50.14% | 8.61 | 5.17 | -40.04% |
|  | FGC_SEI_I1C1 | 27.39 | 13.15 | -52.01% | 9.09 | 5.14 | -43.43% |
|  | Average | 30.42 | 14.34 | -52.87% | 10.14 | 5.83 | -42.51% |

Table 5 Average reduction in FGC processing time for CE2.2

|  |  | SIMD_OFF |  |  | SIMD_ON |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Content | Configure | CEAnc FT (sec) | CE2.2 FT (sec) | Reduction in FGS processing time (%) | CEAnc FT (sec) | CE2.2 FT (sec) | Reduction in FGS processing time (%) |
| CTC4K | FGC_SEI_I10C3 | 9.99 | 7.94 | -20.46% | 3.45 | 2.68 | -22.15% |
|  | FGC_SEI_I10C1 | 6.46 | 5.14 | -20.45% | 2.21 | 1.70 | -22.93% |
|  | FGC_SEI_I1C1 | 6.86 | 5.47 | -20.26% | 2.31 | 1.76 | -24.01% |
|  | Average | 7.62 | 6.07 | -20.39% | 2.60 | 2.00 | -23.04% |
|  |  | SIMD_OFF |  |  | SIMD_ON |  |  |
| Content | Configure | CEAnc FT (sec) | CE2.2 FT (sec) | Reduction in FGS processing time (%) | CEAnc FT (sec) | CE2.2 FT (sec) | Reduction in FGS processing time (%) |
| CTC8K | FGC_SEI_I10C3 | 39.78 | 17.27 | -56.60% | 13.33 | 7.38 | -44.62% |
|  | FGC_SEI_I10C1 | 25.83 | 13.10 | -49.29% | 8.61 | 5.27 | -38.87% |
|  | FGC_SEI_I1C1 | 27.39 | 13.14 | -52.05% | 9.09 | 5.15 | -43.37% |
|  | Average | 30.42 | 14.38 | -52.74% | 10.14 | 5.85 | -42.34% |

The data in Table 4 and Table 5 indicate that resolution-adaptive grain block size results in a significant reduction in FGS processing time compared with the fixed 8x8 grain block size CE-Anchor. The average time reduction is around 20% for 4K clips (with 16x16 grain block size) and around 40~50% for 8K clips (with 32x32 grain block size).

It is noted that the FGC processing time is affected mainly by the number of colour components processed. The number of intensity intervals signalled in the FGC SEI message does not significantly affect FGS processing time. (Note, the number of intensity intervals does not affect how grain blocks are selected from the pre-computed grain pattern database. Thus, no effect on FGC processing time is expected.)

The average reduction in FGS processing time, measured as a percentage, is not strongly dependent on SIMD. However, the absolute FGS processing time for SIMD enabled tests is approximately 2.5 to 3 times faster than the FGS processing time tests in which SIMD was disabled.

The average reduction in FGS processing time is not affected by which transform is used to pre-compute the grain database from which grain blocks are selected during the FGS process, i.e., the FGS processing times for CE2.1 (SMPTE RDD 5 transform) and CE2.2 (VVC DCT-2) are very similar.

## Subjective test results

For subjective tests, similar visual quality was observed for CE2.1 and CE2.2 compared with the anchor for selected 1080p and 4K test sequences. For test sequences with grain or noise in the source, decoded pictures with grain blending appeared more similar to the source than decoded pictures without grain blending. For test sequences without grain or noise in the source, grain blending tended to mask compression artefacts (texture loss, basis patterns etc.) when coding at low bitrate.

# Complexity and memory requirements analysis

The FGS processes studied in this CE are based on SMPTE RDD 5 with two independent modifications to test: CE1, DCT-2 transform type; and CE2, grain block size.

The FGS process described in SMPTE RDD 5 consists of the following steps:

- Pre-compute a grain pattern database

- Calculate an average luma value for each BlockSize x BlockSize block in the decoded picture to determine the intensity interval to which the block belongs

- Calculate pseudo-random offsets to be used to select grain blocks from the pre-computed grain pattern database

- Deblock vertical edges across adjacent grain blocks

- Add grain blocks to the decoded picture

## Analysis of transform type (CE1)

The VVC DCT-2 transform is well studied, documented, and designed to ensure efficient implementation in both software and hardware [7,8]. The VVC transform may also be already available on decoders and may thus be considered implementation friendly, reducing the need for one additional transform. Supporting multiple transforms has several consequences related to complexity and implementation costs, e.g., additional burden to hardware implementation. It requires additional memory for storing the multiple transform cores. Additional kernel leads to increased area since hardware sharing is not possible. Thus, by omitting one additional transform the cost of implementation is reduced, e.g., on-chip implementation, memory requirement reduction. Even efficient software implementation such as partial butterfly implementation is simplified since it can be reused from the core transform. In other words, the transform implementation is simplified and unified by means of using only DCT-2 from VVC.

Historically, there was much effort put in the research how to reduce the memory requirements for transform cores, excluding some transforms from consideration, reducing the number of bits per transform coefficient, reducing total number of unique values, etc. This effort was mainly summarized in Ljubljana and Macao JVET meetings (K and L meetings). For example, in VVC it is proposed to use 8-bit primary transform cores, optimizing the memory for storing the primary transform cores and relaxing additional implementation cost [7].

For example, additional memory requirement for RDD 5 transform is: 44 unique coefficient (no sign counted) that can be stored with 6-bits precision (RDD5 design is a straightforward way of deriving transform cores using scaling and rounding – scaling by 2^8). In total there are 4096 coefficients requiring 6-bits for storing transform core. From the analysis of this CE, it can be considered that this leads to potentially unnecessary resource allocation, and that all process can be implemented with VVC DCT-2 without any loss of quality.

## Analysis of grain block size (CE2)

Estimates of theoretical computational complexity for the FGS process for the luma component are enumerated in Table 6. Memory requirements are listed in Table 7. The complexity and memory requirements for chroma components can be estimated by scaling the estimate for luma based on the chroma format idc.

Following conclusions can be drawn from the analysis of normalized operations per sample listed in Table 6:

- FGS deblocking filter complexity is reduced to half for 16x16 block size compared to 8x8 block size. The filter complexity is reduced to a quarter for 32x32 block size.

- The bit masking and bit extract operations are reduced by a factor of 6x for 16x16 and 24x for 32x32 compared to 8x8 block size

- The modulo(%) operations are reduced by a factor of 4x for 32x32 block size, while they remain the same for 16x16 block size

- Rest of the processing has similar computational complexity for all the block sizes but implementation specific benefits such as effective SIMD utilization and reduced process (function) overhead for larger blocks bring in additional benefits as explained in the additional section on software (SW) benefits below

Table 6 Theoretical computational complexity for different grain block sizes

| Operation | Total Operations |  |  | Operations Per Sample |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Block size |  |  | Block size |  |  |
|  | 8x8 | 16x16 | 32x32 | 8x8 | 16x16 | 32x32 |
| Block Average (for intensity interval selection) |  |  |  |  |  |  |
| Load | 64 | 256 | 1024 | 1 | 1 | 1 |
| Addition | 64 | 256 | 1024 | 1 | 1 | 1 |
| Shift | 1 | 1 | 1 | 1/64 | 1/256 | 1/1024 |
| Grain Block (fg_block) creation using Pseudo Random Number Generated offsets x(r,ec) |  |  |  |  |  |  |
| Modulo (%) | 2 (per 16x16) | 2 | 2 | 2/256 | 2/256 | 2/1024 |
| Load/Store | 64*2 | 256*2 | 1024*2 | 2 | 2 | 2 |
| Multiply | 64 | 256 | 1024 | 1 | 1 | 1 |
| BitMask / BitExtract | 6 | 4 | 4 | 6/64 | 4/256 | 4/1024 |
| 3-Tap Deblocking Filter for vertical edges of adjacent grain blocks |  |  |  |  |  |  |
| Load/Store | 32+16 | 64+32 | 128+64 | 3/4 | 3/8 | 3/16 |
| Additions | 32 | 64 | 128 | 1/2 | 1/4 | 1/8 |
| Shift | 32 | 64 | 128 | 1/2 | 1/4 | 1/8 |
| Grain Blending |  |  |  |  |  |  |
| Load/Store | 64*3 | 256*3 | 1024*3 | 3 | 3 | 3 |
| Additions | 64 | 256 | 1024 | 1 | 1 | 1 |
| Clip | 64 | 256 | 1024 | 1 | 1 | 1 |

Table 7 Memory requirements for different grain block sizes

| Buffer | Total Memory |  |  | Memory per sample |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Block Size |  |  | Block Size |  |  |
|  | 8x8 | 16x16 | 32x32 | 8x8 | 16x16 | 32x32 |
| FG Database | X * 4096 | X * 4096 | X * 4096 | X * 4096/64 | X * 4096/256 | X * 4096/1024 |
| Recon buffer* | 64 * 2 | 256 * 2 | 1024 * 2 | 2 | 2 | 2 |
| FG Block* | 64 * 2 | 256 * 2 | 1024 * 2 | 2 | 2 | 2 |

Notes:

- X can vary based on implementation:

  - If all the 13x13 combinations of cut-off frequencies allowed in SMPTE RDD 5 are used for the grain pattern database, X = 169

  - Otherwise, X is the number of component models with different cut-off frequencies as signalled in the SEI message of current access unit (e.g., max of 10 for SMPTE-RDD 5)

- Recon buffer and FG block buffer sizes are multiplied by a factor of 2 because one current block and one previous block are required for vertical edge deblocking

- The complexity for pre-computing the grain pattern database is negligible because the grain pattern database can be performed offline and stored as lookup.

Larger grain block sizes have additional benefits in SW due to following factors:

- SIMD benefits: Larger grain block sizes can utilize the SIMD processing more efficiently compared to smaller grain block sizes. For instance, 8x8 grain blocks use 128-bit SIMD parallelism which provides only 8x parallelism while 32x32 grain block sizes can be efficiently optimized for 256-bit (AVX2) SIMD width which provides 16x parallel processing for 16-bit input data. Even if 8x8 grain block sizes are optimized for 256-bit AVX2 SIMD, it will still need to load and store 128-bit data from different regions of memory which will increase the loads and stores by a factor of 2x compared to larger block sizes. It will also require additional overhead of scatter gather instructions and significant restructuring of code to process adjacent 8x8 grain blocks.

- Function overheads: For a given resolution, larger grain block sizes have fewer function calls compared to smaller block sizes because they process much larger data in one function call. ‘C’ calling function overheads are typically 15 cycles – 20 cycles for most x86 architectures. These function calls can be a considerable percentage of total cycles for simple operations related to block averaging and grain blending. Compared to 8x8 grain block sizes, function calling overhead is reduced by a factor of 4x for 16x16 grain block sizes and a factor of 16x for 32x32 grain block sizes which gives additional benefits beyond SIMD processing.

The benefits of efficient SIMD processing and reduced function calling overheads are reflected in the percentage FGS processing time reduction for 4k (16x16 block size) and 8k (32x32 block sizes) CE tests as shown in Table 3, Table 4 and Table 5.

- Crosschecks

Crosscheck reports are summarized in Table 8.

Table 8 Summary of crosscheck reports

| Test | Crosschecker | Crosscheck report |
| --- | --- | --- |
| CE-1.1 | Dolby, JVET-X0154 | results: code verification: . draft text verification: . further comments: |
| CE-2.1 | Alibaba, JVET-X0169 | results: code verification: . draft text verification: . further comments: |
| CE-2.2 | InterDigital, JVET-X01xx | results: code verification: draft text verification: . further comments: |

- Input contributions for CE

- Evaluation of VVC DCT-2 transform for Film Grain Synthesis (test CE1.1), M. Radosavljević, E. François (InterDigital), JVET-X0103.

- CE: Film Grain Synthesis (test CE2.1 and CE2.2), F. Pu, S. McCarthy, W. Husak, P. Yin, T. Lu, A. Arora, T. Chen (Dolby), V. G R, J. N. Shingala, K. Patankar, S. K., A. Shyam (Ittiam), JVET-X0048.

- CE-related contributions

No CE-related contributions

# Reference

- RDD 5:2006 - SMPTE Registered Disclosure Doc - Film Grain Technology — Specifications for H.264 | MPEG-4 AVC Bitstreams,” RDD 5:2006, pp. 1–18, Mar. 2006, doi: 10.5594/SMPTE.RDD5.2006.

- S. McCarthy, M. Radosavljević, J. Shingala, “Core Experiment on Film Grain Synthesis”, JVET-W2022, Teleconference, 7-16 July 2021.

- Miloš Radosavljević, Edouard François, Wassim Hamidouche, Thomas Amestoy, Guillaume Gautier, “AHG9: Film grain estimation and film grain synthesis for VVC – Film grain characteristics SEI message characteristics, film grain estimation and film grain synthesis modules”, JVET-V0093, Teleconference, 20-28 April, 2021.

- Sean McCarthy, Peng Yin, Walt Husak, Fangjun Pu, Taoran Lu, Tao Chen, Edouard François, Miloš Radosavljević, Vijayakumar G R, Kaustubh Patankar, Shireesh Kadaramandalgi, Ajayshyam, “AHG9: Fixed-point grain blending process for film grain characteristics SEI message”, JVET-W0095, Teleconference, 7-16 July, 2021

- Miloš Radosavljević, Edouard François, Wassim Hamidouche, Thomas Amestoy and Guillaume Gautier, “AHG9: Enhancement of film grain parameter estimation for different intensity intervals”, JVET-W0072, Teleconference, 7-16 July, 2021.

- Frank Bossen, Jill Boyce, Karsten Suehring, Xiang Li, Vadim Seregin, “VTM common test conditions and software reference configurations for SDR video”, JVET-T2010, Teleconference, 7-16 Oct. 2020.

- Xin Zhao, Xiang Li, Shan Liu, “CE6-related: On 8-bit primary transform core”, JVET-K0290, Ljubljana, SI, 10–18 July 2018.

- M. Budagavi, A. Fuldseth, G. Bjøntegaard, V. Sze and M. Sadafale, "Core Transform Design in the High Efficiency Video Coding (HEVC) Standard," in IEEE Journal of Selected Topics in Signal Processing, vol. 7, no. 6, pp. 1029-1041, Dec. 2013, doi: 10.1109/JSTSP.2013.2270429.