| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0023_r1 |
| --- | --- |

| Title: | EE1: Summary of Exploration Experiments on Neural Network-based Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina, S. Liu, W. Chen, F. Galpin, Y. Li, Z. Ma and H. Wang | Tel: Email: | elena.alshina@huawei.com, shanl@tencent.com, chenwei06@kwai.com, Franck.Galpin@InterDigital.com, yue.li@bytedance.com, mazhan@nju.edu.cn, hongtaow@qti.qualcomm.com |
| Source: | EE coordinators |  |  |

_____________________________

# Abstract

This contribution provides a summary report for the Exploration Experiment 1 on Neural Network-based Video Coding. In total 11 tests have been completed within theis EE between the JVET-W and JVET-X meetings to study and evaluate NNVC technologies, analyze their performance and complexity aspects. Several variants of NN-based in-loop filters with complexity 30...800 kMAC/pxl providing average bit-saving in a range 2...10% (in random access configuration) have been demonstrated in this round of Exploration Experiment. Several variants of super-resolution (from available in VVC standard functionality to NN-based with signaled parameters) demonstrate 1...6% bit-saving in random access configuration for 4K content.  An example of cross-platform bit-exact decodable implementation of NN-based Intra was demonstrated and cross-checked. This tool improves Intra coding by more than 3%.

- Test conditions and evaluation criteria

Tests in this Exploration Experiment follow AhG11 testing conditions and complexity assessment methodology [1]. Anchor is VTM11.0 with improved GOP-based temporal filter [2] enabled. In total 5 quality levels (corresponding QP=22, 27, 32, 37, 42) are used for objective performance measurement. It should be noticed that solutions presented in Super Resolution category are still far from target rates, and RD-curves often crossing. So, objective performance results for this category are less reliable than in other tests.

- Objective test results summary and observations

Performance test results in RA configuration for all proposals are shown in Table 1.

Table 1. Test results in Random Access cfg (all tests).

|  |  |  |  |  | Random Access (AVG, 5QPs) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | PSNR |  |  | MS-SSIM |  |  | Time |  |
| NN-filters |  |  |  |  |  |  |  |  |  |  |  |  |
| Doc# | Total CONV | Total memory, MB | kMAC /pxl | Pre-cision | Y | U | V | Y | U | V | ×Enc | ×Dec |
| JVET-X0110-1 | 6 | 3 | 151 | F | -2.5% | -6% | -4% | -1.9% | -7% | -4% | 1.2 | 392 |
| JVET-X0110-2 | 6 | 3 | 151 | F | -0.4% | 0% | -1% | -0.9% | -1.9% | -1.2% | 1 | 361 |
| JVET-X0065 | 37 | 146 | 520 | F | -10.1% | -24% | -23% | x | x | x | 1.8 | 390 |
| JVET-X0052 | 27 | 23 | 786 | F | -4.2% | -11% | -10% | -4.0% | -10% | -8% | 1.4 | 643 |
| JVET-X0140-1 | 35 | 2 | 34 | F | -5.8% | -7% | -5% | x | x | x | 1.2 | 41 |
| JVET-X0140-2 | 38 | 30 | 624 | F | -8.4% | -22% | -22% | x | x | x | 2.1 | 352 |
| JVET-X0053 | 13 | 0 | 29 | F | -1.1% | -5% | -4% | -1.2% | -5% | -3% | 1.1 | 12 |
| JVET-X0066 | 41 | 25 | 539 | F | -9.8% | -21% | -21% | x | x | x | 1.8 | 404 |
| Super Res |  |  |  |  |  |  |  |  |  |  |  |  |
| JVET-X0117 | 2 | 0.00 | 0.03 | int16 | -1.0% | 0.6% | 0.5% | -2.0% | -0.1% | -0.6% | 1.3 | 0.9 |
| JVET-X0064 | 36 | 22 | 764 | F | x | x | x | x | x | x | x | x |
| JVET-X0107 | 16 | 10 | 344 | F | 91% | 45% | 36% | x | x | x | 1.2 | 0.4 |
| JVET-X0074-1 | 10 | 1 | 85 | F | 58% | 71% | 66% | x | x | x | 0.8 | 0.5 |
| JVET-X0074-2 | 10 | 1 | 85 | F | 60% | 80% | 70% | x | x | x | x | x |
| NN-Intra |  |  |  |  |  |  |  |  |  |  |  |  |
| JVET-X0118-1 | 4 | 62 | 100 | F | -1.6% | -1% | -1% | -1.5% | 0% | 0% | 1.4 | 5 |
| JVET-X0118-3 | 4 | 31 | 100 | int16 | -1.6% | -1% | -1% | -1.5% | -0.1% | -0.1% | 1.1 | 3 |
| JVET-X0118-4 | 4 | 11 | 17 | int16 | -1.4% | -1% | -1% | -1.3% | 0% | 0% | 1.1 | 3 |

Following observations can be done.

Observation #1. As was suggested in [3] plot BD-rate gain vs kMAC/pxl and total memory size was made (Fig. 1). The dash line indicates capability of NVIDIA RTX3080 for 4K@60 processing. This graph is not directly comparable with one in [3], since average BD-rate gain was computed at slightly different conditions (not all data was available). But trend is very clear: proponents try to move their test point in this graph toward top-left corner. In [1] the solution with highest performance had ~1500 kMAC/pxl, currently the most complicated one is below 1000 kMAC/pxl, maintaining the gain at the level ~10% relatively to VTM.

Total memory size for all NN parameters is another important complexity measure. Almost all EE participants are switching between different NN models depending on situation (QP, block size or RDO choice). This adaptation is the source of additional performance improvements. Finding good balance between number of degrees of freedom and compression performance is an important aspect of AhG11/EE1 research. Fig. 2 shows compression performance gain vs total memory size needed for all NN parameters. For example, JVET-X0065 and JVET-X0066 are almost not distinguishable in Fig. 1, but form Fig. 2 one easily sees difference in terms of total memory size. Similar trend is also observed in Fig. 2, the test points are being pushed toward top-left corner so that less memory is required (the one with the largest memory size requires less than 150MB) but the majority of the gain is preserved.

Fig. 1. An average BD-rate gain in random access configuration vs kMAC/pxl.

Fig. 2. An average BD-rate gain in random access configuration vs totally memory size in MB.

Observation #2. There is strong correlation between kMAC/pxl number and decoder run-time increment factor relatively to VTM. Using fine-tuned model parameters leads to a slightly higher decoding run time, which is likely due to a more frequent use of the NN filter (Fig. 3).

Fig. 3. Decoding run-time increment factor (relatively to VTM) vs kMAC/pxl.

Observation #3. Filters with deeper neural network (higher total number of convolution layers) tend to have better complexity-performance trade-off (Fig. 4). Data shown only for NN-filter category, but NN-Intra show the same trend.

Fig. 4. An average BD-rate gain (for NN-filters) in random access configuration vs kMAC/pxl categorized depending on NN depth.

Observation #4. In NN-filter and NN-Intra categories BD-rate gain measured based on PSNR and MS-SSIM is very close. In super-resolution category MS-SSIM gain is almost twice higher than PSNR gain, indicating that MS-SSIM is less sensitive to re-sizing artifacts. It would be interesting to compare trend shown by BD-rate results in PSNR and MS-SSIM with visual tests results.

- Super resolution category specific

Not all proponents is this category provided results for all test sequences, so not all can appear in graph on Fig. 1. Proposals in super-resolution category mostly useful for high resolution sequence coding. Table 2 and Fig. 5 summarize performance for UHD classes (A1, A2) in test set only.

Still comparison is not very accurate due to curves overlap.  In Test 2.1 some efforts were taking to solve this problem. In order to avoid RD-curves overlap RPR was enabled only if it outperforms full-size coding (this is multi-pass encoding reflected in ×1.3 higher encoder run-time). In order to match target rate closer QP for down sampled sequence coding was selected per sequence, per parget rate. It should be discussed during the meeting if this type of solving RD-curves crossing problem can be recommended for further tests in Super-resolution category.

|  |  |  |  |  | Random Access (4K, 5QPs) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Labels on plot | Total CONV | Total memory, MB | kMAC /pxl | Pre-cision | PSNR |  |  | MS-SSIM |  |  | Time |  |
| JVET-X0117 | 2 | 0.00 | 0.03 | int16 | -2.4% | 2% | 1% | -5.1% | 0% | -2% | 1.3 | 0.8 |
| JVET-X0064 | 34-36 | 22 | 764 | F | -6.1% | 6% | -2% | x | x | x | 1.0 | 0.3 |
| JVET-X0107 | 16 | 10 | 344 | F | -0.8% | 50% | 28% | x | x | x | 1.1 | 0.4 |
| JVET-X0074-1 | 10 | 1 | 85 | F | -3.9% | 28% | 14% | x | x | x | 0.8 | 0.5 |
| JVET-X0074-2 | 10 | 1 | 85 | F | -0.9% | 51% | 24% | x | x | x | 0.5 | 0.3 |

Table 2. Test results in for Random Access cfg (UHD only).

Fig. 5. Only UHD BD-rate gain in random access configuration vs kMAC/pxl.

- NN-Intra  category specific

It makes sense to look closely onto test results in “all-intra” configuration while discussing Intra tools. The comparison of NN-filter and NN-Intra tests in this EE is shown in Table 3. With not that deep NN-structure and moderate kMAC/pxl (and decoding run time). NN-Intra tools shows performance competitive with NN-filters in all-intra configuration test (Fig. 5).

In opposite to enchantment filters NN-Intra operates on block level. It is unlikely that NN-dedicated HW would be used inside the picture (if the rest of the picture doesn’t require NN-operations). Proponent of this EE test provided  c++ compatible implementation of their NN algorithm (both float point and integer implementation are supported). This resulted in successful cross-check. It was observed that it is possible to get matched results only for integer implementation of NN-Intra. Also it was confirmed identical behavior of decoder under Linux and Windows for integer version of this EE test.  NN-library [4] provided by proponent of this EE test and integer implantation of NN algorithms definitely can be recommended for further wide use in this EE.

BD-rate gain in “all-intra” test vs kMAC/pxl for NN-solution is different categories is shown on Fig. 6.

Table 3. Test results in for All Intra cfg (all ).

|  |  |  |  |  | All Intra (AVG, 5QPs) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | PSNR |  |  | MS-SSIM |  |  | Time |  |
| NN-filters |  |  |  |  |  |  |  |  |  |  |  |  |
| Doc# | Total CONV | Total memory, MB | kMAC /pxl | Pre-cision | Y | U | V | Y | U | V | ×Enc | ×Dec |
| JVET-X0110-1 | 6 | 151 | 151 | F | x | x | x | x | x | x | x | x |
| JVET-X0110-2 | 6 | 151 | 151 | F | x | x | x | x | x | x | x | x |
| JVET-X0065 | 37 | 520 | 520 | F | -7.4% | -20% | -20% | x | x | x | 1.3 | 189 |
| JVET-X0052 | 27 | 786 | 786 | F | -5.3% | -10% | -10% | -4.8% | -9% | -9% | 1.5 | 387 |
| JVET-X0140-1 | 35 | 34 | 34 | F | -5.2% | -6% | -5% | x | x | x | 1.2 | 23 |
| JVET-X0140-2 | 38 | 624 | 624 | F | -7.6% | -18% | -20% | x | x | x | 1.8 | 203 |
| JVET-X0053 | 13 | 29 | 29 | F | -1.6% | -5% | -4% | -1.4% | -4% | -4% | 1.1 | 29 |
| JVET-X0066 | 41 | 539 | 539 | F | -7.4% | -19% | -19% | x | x | x | 1.4 | 238 |
| Super Res |  |  |  |  |  |  |  |  |  |  |  |  |
| JVET-X0117 | 2 | 0.03 | 0.03 | F | -0.5% | 0.9% | 0.7% | -1.5% | 0.6% | 0.3% | 1.0 | 0.9 |
| JVET-X0064 | 34-36 | 764 | 764 | F | x | x | x | x | x | x | x | x |
| JVET-X0107 | 16 | 344 | 344 | F | 29% | 20.3% | 2.4% | x | x | x | 1.3 | 0.5 |
| JVET-X0074-1 | 10 | 85 | 85 | F | 65% | 81% | 70% | x | x | x | 0.9 | 0.6 |
| JVET-X0074-2 | 10 | 85 | 85 | F | 65% | 95% | 73% | x | x | x | x | x |
| NN-Intra |  |  |  |  |  |  |  |  |  |  |  |  |
| JVET-X0118-1 | 4 | 100 | 100 | F | -3.3% | -3% | -3% | -3.3% | -2% | -2% | 3.7 | 40 |
| JVET-X0118-3 | 4 | 100 | 100 | int16 | -3.3% | -3% | -3% | -3.3% | -2% | -2% | 2.2 | 19 |
| JVET-X0118-4 | 4 | 17 | 17 | int16 | -2.9% | -2% | -2% | -2.9% | -2% | -2% | 2.2 | 18 |

# Fig. 6. BD-rate gain in all-intra access configuration vs kMAC/pxl.

- Recommendation

- Review all EE1 documents for more detailed information.

- Conduct BoG for preparing viewing test, better understanding test results and refining assessment methodology.

- Organize and conduct viewing during JVET-X meeting for prepared materials.

- References

[1] S. Liu, A. Segall, E. Alshina, R.-L. Liao Common Test Conditions and evaluation procedures for neural network-based video coding technology JVET-W2016.

[2] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/tree/master/Software%20Patches

[3] A. Segall, BoG Report: Neural Networks Video Coding Analysis and Planning JVET-W0182.

[4] F. Galpin, T. Dumas, P. Bordes, P. Nikitin, F. Le Leannec, E. Francois (InterDigital) AHG11: Small Ad-hoc Deep-Learning Library JVET-W0181.