| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0024-v1 |
| --- | --- |

| Title: | EE2: Summary Report on Enhanced Compression beyond VVC capability |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Chen Semih Esenlik Fabrice Le Leannec Ling Li Jacob Strom Martin Winken Xiaoyu Xiu Kai Zhang | Email: | vseregin@qti.qualcomm.com jiechen.cj@alibaba-inc.com semih.esenlik@huawei.com fabrice.leleannec@interdigital.com aurali@tencent.com jacob.strom@ericsson.com martin.winken@hhi.fraunhofer.de xiaoyuxiu@kwai.com zhangkai.video@bytedance.com |
| Source: | EE coordinators |  |  |

_____________________________

# Abstract

This document provides a summary report of Exploration Experiment on Enhanced Compression beyond VVC capability. The tests are categorized as partitioning, inter prediction, and in-loop filtering tests.

The software basis for this EE is ECM-2.0, released at https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-2.0. ECM-2.0 is used as an anchor in the tests.

Software for EE tests is released in the corresponding branches at https://vcgit.hhi.fraunhofer.de/ecm/jvet-w-ee2/ECM/-/branches.

Test and cross-check results can be found in the input JVET contributions and are located at https://vcgit.hhi.fraunhofer.de/ecm/jvet-w-ee2/simulation-results/-/tree/master.

# List of tests

|  | Tests | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 Partitioning |  |  |  |
| 1.1 | Encoder optimization for ECM | ByteDance Kai Zhang InterDigital Fabrice Le Léannec JVET-X0068 | Qualcomm Muhammed Coban JVET-X0144 Alibaba Jie Chen |
| 1.2 | ABT | ByteDance Kai Zhang InterDigital Fabrice Le Léannec JVET-X0068 | Alibaba Jie Chen |
| 1.3 | UQT | ByteDance Kai Zhang InterDigital Fabrice Le Léannec JVET-X0068 | Alibaba Jie Chen |
| 1.4 | ABT + UQT | ByteDance Kai Zhang InterDigital Fabrice Le Léannec JVET-X0068 | Alibaba Jie Chen |
| 3 Inter prediction |  |  |  |
| 3.1a | Combination of CIIP and DIMD | Alibaba Xinwei Li JVET-X0098 | ByteDance Yang Wang JVET-X0177 |
| 3.1b | Combination of CIIP and TIMD | Alibaba Xinwei Li JVET-X0098 | ByteDance Yang Wang JVET-X0177 |
| 3.2a | GPM with inter and intra prediction – method A | KDDI Yoshitaka Kidani JVET-X0077 |  |
| 3.2b | GPM with inter and intra prediction – method B | KDDI Yoshitaka Kidani JVET-X0077 |  |
| 3.3a | Bilateral matching AMVP-merge mode | Qualcomm Zhi Zhang JVET-X0083 | Kwai Wei Chen |
| 3.3b | Bilateral matching AMVP-merge mode with TM disabled | Qualcomm Zhi Zhang JVET-X0083 | Kwai Wei Chen |
| 3.3c | Template matching AMVP-merge mode | Qualcomm Zhi Zhang JVET-X0083 | Kwai Wei Chen |
| 3.4a | Adaptive decoder side motion vector refinement | Qualcomm Han Huang JVET-X0049 | Alibaba Ru-Ling Liao |
| 3.4b | Adaptive decoder side motion vector refinement with TM disabled | Qualcomm Han Huang JVET-X0049 | Alibaba Ru-Ling Liao |
| 4 In-loop filtering |  |  |  |
| 4.1 | Chroma bilateral filter | Bytedance Wenbin Yin JVET-X0067 | Alibaba Mohammed Golam Sarwer |
| 4.2a | CTB level filter shape selection of CCALF | Alibaba Mohammed Golam Sarwer JVET-X0045 | Qualcomm Nan Hu |
| 4.2b | CTB level filter shape selection of CCALF with removal of power of 2 constraint of filter coefficients | Alibaba Mohammed Golam Sarwer JVET-X0045 | Qualcomm Nan Hu |
| 4.3a | CCALF with larger filter size | Alibaba Mohammed Golam Sarwer JVET-X0045 | Qualcomm Nan Hu |
| 4.3b | CCALF with larger filter size with removal of power of 2 constraint of filter coefficients | Alibaba Mohammed Golam Sarwer JVET-X0045 | Qualcomm Nan Hu |
| 4.4 | Alternative 2x2 classifier for ALF | Qualcomm Nan Hu JVET-X0070 | Alibaba Mohammed Golam Sarwer |
| 4.5 | Alternative sample-based classifier for ALF | Qualcomm Nan Hu JVET-X0070 | Alibaba Mohammed Golam Sarwer |
| 4.6a | Combination of 4.2a and 4.4 (CTB level filter shape selection of CCALF and alternative 2x2 classifier for ALF) | Alibaba Mohammed Golam Sarwer Qualcomm Nan Hu JVET-X0046 | Bytedance Wenbin Yin |
| 4.6b | Combination of 4.2a and 4.5 (CTB level filter shape selection of CCALF and alternative sample-based classifier for ALF) | Alibaba Mohammed Golam Sarwer Qualcomm Nan Hu JVET-X0046 | Bytedance Wenbin Yin |
| 4.6c | Combination of 4.3a and 4.4 (CCALF with larger filter size and alternative 2x2 classifier for ALF) | Alibaba Mohammed Golam Sarwer Qualcomm Nan Hu JVET-X0046 | Bytedance Wenbin Yin |
| 4.6d | Combination of 4.3a and 4.5 (CCALF with larger filter size and alternative sample-based classifier for ALF) | Alibaba Mohammed Golam Sarwer Qualcomm Nan Hu JVET-X0046 | Bytedance Wenbin Yin |
| 4.7a | Combination of 4.2a and 4.1 (CTB level filter shape selection of CCALF and Chroma Bilateral) | Alibaba Mohammed Golam Sarwer Bytedance Wenbin Yin JVET-X0047 |  |
| 4.7b | Combination of 4.3a and 4.1 (CCALF with larger filter size and Chroma Bilateral) | Alibaba Mohammed Golam Sarwer Bytedance Wenbin Yin JVET-X0047 |  |
| 4.8a | Combination of 4.6a and 4.1 (Chroma Bilateral, CTB level filter shape selection of CCALF and alternative 2x2 classifier for ALF) | Alibaba Mohammed Golam Sarwer Bytedance Wenbin Yin Qualcomm Nan Hu JVET-X0071 |  |
| 4.8b | Combination of 4.6b and 4.1 (Chroma Bilateral, CTB level filter shape selection of CCALF and alternative sample-based classifier for ALF) | Alibaba Mohammed Golam Sarwer Bytedance Wenbin Yin Qualcomm Nan Hu JVET-X0071 |  |
| 4.8c | Combination of 4.6c and 4.1 (Chroma Bilateral, CCALF with larger filter size and alternative 2x2 classifier for ALF) | Alibaba Mohammed Golam Sarwer Bytedance Wenbin Yin Qualcomm Nan Hu JVET-X0071 |  |
| 4.8d | Combination of 4.6d and 4.1 (Chroma Bilateral, CCALF with larger filter size and alternative sample-based classifier for ALF) | Alibaba Mohammed Golam Sarwer Bytedance Wenbin Yin Qualcomm Nan Hu JVET-X0071 |  |
| 4.9a | Combination of 4.1 and 4.4 (Chroma Bilateral Filter and alternative 2x2 classifier for ALF) | Bytedance Wenbin Yin Qualcomm Nan Hu JVET-X0069 | Ericsson Jacob Strom |
| 4.9b | Combination of 4.1 and 4.5 (Chroma Bilateral Filter and alternative sample-based classifier for ALF) | Bytedance Wenbin Yin Qualcomm Nan Hu JVET-X0069 | Ericsson Jacob Strom |

# Description of tests

## Partitioning

### Test 1.1: Encoder optimization for ECM

In this test, maximum BT and TT depths are increased per class from 32 to up to 128, MTT hieratical depth is increased to 4 in I slices and MTT hieratical depth is temporal ID dependent for P- and B- slices varying between 2 and 3 as summarized in the following table.

| Class A | Class B | Classes C and D |
| --- | --- | --- |
| CTUSize: 256 MaxMTTHierarchyDepth: 3 MaxMTTHierarchyDepthI: 4 MaxMTTHierarchyDepthISliceL: 4 MaxMTTHierarchyDepthISliceC: 4 MaxBTLumaISlice: 128 MaxBTChromaISlice: 128 MaxBTNonISlice: 128 MaxTTLumaISlice: 128 MaxTTChromaISlice: 128 MaxTTNonISlice: 128 MaxABTLumaISlice: 128 MaxABTChromaISlice: 128 MaxABTNonISlice: 128 MaxMTTHierarchyDepthByTid: 333322 | CTUSize: 128 MaxMTTHierarchyDepth: 3 MaxMTTHierarchyDepthI: 4 MaxMTTHierarchyDepthISliceL: 4 MaxMTTHierarchyDepthISliceC: 4 MaxBTLumaISlice: 128 MaxBTChromaISlice: 128 MaxBTNonISlice: 128 MaxTTLumaISlice: 128 MaxTTChromaISlice: 128 MaxTTNonISlice: 128 MaxABTLumaISlice: 128 MaxABTChromaISlice: 128 MaxABTNonISlice: 128 MaxMTTHierarchyDepthByTid: 333322 | CTUSize: 128 MaxMTTHierarchyDepth: 3 MaxMTTHierarchyDepthI: 4 MaxMTTHierarchyDepthISliceL: 4 MaxMTTHierarchyDepthISliceC: 4 MaxBTLumaISlice: 64 MaxBTChromaISlice: 64 MaxBTNonISlice: 128 MaxTTLumaISlice: 32 MaxTTChromaISlice: 32 MaxTTNonISlice: 64 MaxABTLumaISlice: 64 MaxABTChromaISlice: 64 MaxABTNonISlice: 64 MaxMTTHierarchyDepthByTid: 333333 |

In cross-check report JVET-X0144, other variations of the configuration parameters are tested for classes A and B.

All provided tests result are summarized in the next table. In trade-off 0, normative MTT depth for temporal layers signalling in SPS is introduced with additional speed-up method is added, which allows to bypass certain mode checks at encoder. In all other trade-offs, which are encoder only changes for ECM-2.0, there is no normative SPS signalling and additional encoder speed-up method included.

|  | Class A |  | Class B |  |
| --- | --- | --- | --- | --- |
|  | MaxMTTHierarchyDepth | MaxMTTHierarchyDepthByTid (0-5) | MaxMTTHierarchyDepth | MaxMTTHierarchyDepthByTid (0-5) |
| Trade-off 0 (X0068) | 3 | 3 3 3 3 2 2 | 3 | 3 3 3 3 2 2 |
| Trade-off 1 (X0144) | 3 | 3 3 3 3 2 2 | 3 | 3 3 3 3 2 2 |
| Trade-off 2 (X0144) | 4 | 3 3 3 3 2 2 | 4 | 3 3 3 3 2 2 |
| Trade-off 3 (X0144) | 3 | 3 3 3 3 3 3 | 3 | 3 3 3 3 3 3 |

### Test 1.2: Asymmetric binary tree (ABT)

Four new asymmetric binary tree splitting modes are added to the multi-type tree structure of VVC shown on Figure 1.

| ABT-H1 | ABT-H2 |
| --- | --- |
| ABT-V1 | ABT-V2 |

Figure 1. ABT partitions

CU can be divided into 2 sub-CUs with 1/4 and 3/4 sizes. 6-point, 12-point, 24-point, and 48-point transforms/inverse-transforms are introduced. Intra prediction is also modified to accommodate non-power-2 block sizes.

Encoder optimization of the test 1.1along with the new partition specific speedups are applied.

### Test 1.2: Unsymmetric quad tree (UQT)

Four types of unsymmetric quad tree partitions with 1/8, 1/2, 1/4, and 1/8 sizes as shown on Figure 2 are tested.

Encoder optimization of the test 1.1along with the new partition specific speedups are applied.

| UQT-H1 | UQT-H2 |
| --- | --- |
| UQT-V1 | UQT-V2 |

Figure 2. UQT partitions

### Test 1.3: Asymmetric binary tree (ABT) and Unsymmetric quad tree (UQT)

This test is a combination of the tests 1.2 and 1.3.

## Inter prediction

### Tests 3.1: Combination of CIIP and DIMD/TIMD

In the tests, DIMD or TIMD methods are used to derive an intra prediction mode and replace the planar mode to generate the intra prediction signal in CIIP mode without extra signaling. The proposed method is only applied to coding blocks with an area less than or equal to 1024. And the propagated intra prediction mode of a CIIP coding block is always set to planar mode.

In the tests, the weights (wIntra, wInter) are modified if the derived intra prediction mode is an angular mode. For near-horizontal modes (2 <= angular mode index < 34), the current block is vertically divided as shown in Figure 3 (a); for near-vertical modes (34 <= angular mode index <= 66), the current block is horizontally divided as shown in Figure 3 (b).

Figure 3. The block division for angular modes

The (wIntra, wInter) for different sub-blocks are derived as follows.

| The sub-block index | (wIntra, wInter) |
| --- | --- |
| 0 | (6, 2) |
| 1 | (5, 3) |
| 2 | (3, 5) |
| 3 | (2, 6) |

In DIMD intra mode derivation, the intra prediction mode with the highest amplitude in the DIMD histogram is selected.

In TIMD intra mode derivation, the intra prediction mode with the smallest SATD value in the TIMD mode list is selected and mapped to one of the 67 regular intra prediction modes.

Test 3.1a: Combination of CIIP and DIMD

Test 3.1b: Combination of CIIP and TIMD

### Tests 3.2: GPM with inter and intra prediction

In the test, one of the GPM partitions can be inter or intra, a flag is signaled to indicate whether intra or inter prediction is used in a partition.

For the inter partition, a uni-directional predictor is generated by MVs from the merge candidate list. For the intra partition, an intra predictor is generated from the neighboring samples of the CU, pre-defined intra prediction directions considering GPM partitioning edge are selected and intra mode index is signalled. Finally, these two predictors are blended in the same way as in the existed GPM.

Test 3.2a: GPM with inter and intra prediction – method A.

Pre-defined intra prediction modes are restricted to the parallel and perpendicular angular modes relative to GPM partitioning edge.

Test 3.2b: GPM with inter and intra prediction – method B.

Pre-defined intra prediction modes are restricted to the parallel and perpendicular angular modes relative to GPM partitioning edge, and planar mode.

### Tests 3.3: AMVP-merge mode

AMVP-merge mode is tested where bi-directional predictor is composed of an AMVP predictor in one direction and a merge predictor in the other direction.

Uni directional AMVP predictor is signalled as in the regular AMVP mode. Merge index is not signalled, and merge predictor is selected from the candidate list with the smallest template or bilateral matching cost and both approaches are tested.

When the selected merge predictor and the AMVP predictor satisfy DMVR condition, which is there is at least one reference picture from the past and one reference picture from the future relatively to the current picture and the distances from two reference pictures to the current picture are the same, the bilateral matching MV refinement is applied for the merge MV candidate and AMVP MVP as a starting point. Otherwise, if template matching functionality is enabled, template matching MV refinement is applied to the merge predictor or the AMVP predictor which has a higher template matching cost.

The third pass which is 8x8 sub-PU BDOF refinement of the multi-pass DMVR is enabled to AMVP-merge mode coded block.

Additionally, the introduced AMVP-merge mode is tested when template matching is disabled in the anchor and the test.

Test 3.3a: Bilateral matching AMVP-merge mode

Test 3.3b: Bilateral matching AMVP-merge mode with TM disabled

Test 3.3c: Template matching AMVP-merge mode

### Tests 3.4: Adaptive decoder side motion vector refinement

In ECM, multi-pass DMVR refines both directions (L0 and L1) in a symmetrical manner as shown on Figure 4.

Figure 4. DMVR refinement

In the test, adaptive DMVR is introduced where it can refine only one either L0 or L1 direction of the bi-prediction. Those two additional modes are added to the existed DMVR and the choice of DMVR mode is signalled.

Like the regular merge mode, merge candidates for the proposed merge modes are derived from the spatial neighboring coded blocks, TMVPs, non-adjacent blocks, HMVPs, and pair-wise candidate.  The difference is that only those meet DMVR conditions are added into the candidate list. The same merge candidate list is used by the two proposed merge modes and merge index is coded as in regular merge mode.

Test 3.4a: Adaptive decoder side motion vector refinement

Test 3.4b: Adaptive decoder side motion vector refinement with TM disabled

## In-loop filtering

### Test 4.1: Chroma bilateral filter

Luma bilateral inloop filter (BIF) is extended to chroma component in parallel with SAO and CCSAO as shown on Figure 5.

Figure 5. Chroma bilateral filter

Chroma BIF can be switched on/off at CTU and slice levels. The filtering process of luma BIF, 5×5 diamond shape filter is used for generating the filtering offset. The difference between the central sample and each surrounding sample is calculated first. The coefficient for each reference sample is extracted from a pre-defined look-up-table based on the calculated difference directly. The coefficients used for chroma components are retrained, different from those from BIF-luma.

In luma BIF, the block-level filtering strength parameter c is determined based on luma TU size and CU mode. While in chroma BIF, the parameter for chroma components is determined based the chroma TU size and mode when dual-tree partitioning is enabled for the current slice, and based on the corresponding luma TU size and mode when dual-tree partitioning is disabled.

### Tests 4.2: CTB level filter shape selection of CCALF

In ECM-2.0, CCALF uses 8-tap hexagon shaped filter illustrated in Figure 6.

Figure 6. CCALF filter shape in ECM-2.0

In the EE, additional two filter shapes are tested as shown on Figure 7. Filter coefficients are carried in APS, up to 16 filters can be signalled, and for each filter shape index (shape_idx) is added. For each CTB, an index is signalled to the decoder to specify which filter shape and coefficients are used for that CTB.

Figure 7. Additional CCALF filter shapes

In ECM-2.0, CCALF filter coefficients are restricted to having only values in the form of power of 2, from the following set: {-64, -32, -16, -8, -4, -2, -1, 0, 1, 2, 4, 8, 16, 32, 64}. Additionally, removal of power-of-2 constraint on CCALF filter coefficients is tested.

Test 4.2a: CTB level filter shape selection of CCALF

Test 4.2b: CTB level filter shape selection of CCALF with removal of power of 2 constraint of filter coefficients

### Tests 4.3: CCALF with larger filter size

In this test, 25-tap large filter illustrated on Figure 7 is used replacing 8-tap hexagon filter and up to 16 filters are signalled in APS.

Figure 7. 25-tap CCALF filter shapes

Additionally, removal of power-of-2 constraint on CCALF filter coefficients is tested.

Test 4.3a: CCALF with larger filter size

Test 4.3b: CCALF with larger filter size with removal of power of 2 constraint of filter coefficients

### Tests 4.4 – 4.5: Alternative classifiers for ALF

In ECM-2.0, ALF uses Laplacian based classifier applied to each 2x2 luma block in a CTB. For each class, a specific filter is applied among the different filters in a filter set carried in APS. Based on the classification, geometric transformation, such as 90-degree rotation, diagonal or vertical flip, of the coefficients within a diamond filter shape can also be applied.

In the test, another band-based classifier is introduced. A flag is added to a luma filter set in APS to indicate whether the classifier of ECM-2.0 or the one of the alternative classifiers is applied. Geometrical transformation is not applied with the tested band classifier. The number of classes is equal to 25 as in the existed ECM ALF classifier.

In the 2x2 block based band-based classifier, the sum of sample values of a 2x2 luma block is calculated at first. Then the class index is calculated as below,

class_index = (sum * 25) >> (sample bit depth + 2).

In the sample band-based classifier, the class index of a luma sample is calculated as

class_index = (luma sample * 25) >> (sample bit depth).

In test 4.5, the frequency of filter switch using band classifier in 2x2 luma blocks is summarized in the table below. The ratio is calculated by dividing the number of 2x2 blocks having different filters by the total number of 2x2 blocks using band classifier.

|  | AI | RA | LDB |
| --- | --- | --- | --- |
| Class A1 | 8.96% | 3.15% |  |
| Class A2 | 19.09% | 5.70% |  |
| Class B | 11.35% | 5.72% | 5.47% |
| Class C | 8.47% | 2.24% | 3.07% |
| Class E | 5.33% |  | 3.17% |
| Overall | 10.49% | 4.27% | 3.98% |
| Class D | 4.20% | 4.45% | 5.17% |
| Class F | 19.90% | 11.42% | 6.16% |

Test 4.4: Alternative 2x2 classifier for ALF

Test 4.5: Alternative sample-based classifier for ALF

### Tests 4.6 – 4.9: Combination tests

The following combinations of the in-loop filtering methods are tested:

Test 4.6a: Combination of 4.2a and 4.4 (CTB level filter shape selection of CCALF and alternative 2x2 classifier for ALF)

Test 4.6b: Combination of 4.2a and 4.5 4 (CTB level filter shape selection of CCALF and alternative sample-based classifier for ALF)

Test 4.6c: Combination of 4.3a and 4.4 (CCALF with larger filter size and alternative 2x2 classifier for ALF)

Test 4.6d: Combination of 4.3a and 4.5 (CCALF with larger filter size and alternative sample-based classifier for ALF)

Test 4.7a: Combination of 4.2a and 4.1 (CTB level filter shape selection of CCALF and chroma bilateral filter)

Test 4.7b: Combination of 4.3a and 4.1 (CCALF with larger filter size and chroma bilateral filter)

Test 4.8a: Combination of 4.6a and 4.1 (CTB level filter shape selection of CCALF, alternative 2x2 classifier for ALF, and chroma bilateral filter)

Test 4.8b: Combination of 4.6b and 4.1 (CTB level filter shape selection of CCALF, alternative sample-based classifier for ALF, and chroma bilateral filter)

Test 4.8c: Combination of 4.6c and 4.1 (CCALF with larger filter size, alternative 2x2 classifier for ALF, and chroma bilateral filter)

Test 4.8d: Combination of 4.6d and 4.1 (CCALF with larger filter size, alternative sample-based classifiers for ALF, and chroma bilateral filter)

Test 4.9a: Combination of 4.1 and 4.4 (Chroma bilateral filter and alternative 2x2 classifier for ALF)

Test 4.9b: Combination of 4.1 and 4.5 (Chroma bilateral filter and alternative sample-based classifier for ALF)

# EE related documents

| JVET-X0056 | EE2-Related: Complexity reduction for decoder side motion derivation |
| --- | --- |
| JVET-X0072 | EE2-related: PDPC-skip scheme for angular intra modes |
| JVET-X0078 | EE2-related: Modified GPM with inter and intra prediction |
| JVET-X0085 | Non-EE2: Template Matching-based Reordering for Extended MMVD Design |
| JVET-X0086 | EE2-related: Adaptive Filter Shape Selection for ALF |
| JVET-X0087 | Non-EE2: Template Matching Based Merge Candidate List Construction (TM-MCLC) |
| JVET-X0088 | Non-EE2: History-based Affine Model Inheritance |
| JVET-X0089 | Non-EE2: Modifications of IBC Merge/AMVP List Construction |
| JVET-X0090 | Non-EE2: On combination of CIIP, OBMC and LMCS |
| JVET-X0091 | Non-EE2: On TMVP improvement |
| JVET-X0099 | Non-EE2: Extension of TIMD to intra chroma coding |
| JVET-X0100 | EE2-related: On propagating intra prediction mode for IBC |
| JVET-X0114 | EE2-related: Fix on issues of TIMD mode |
| JVET-X0115 | EE2-related: Optimization on the second mode derivation of DIMD blending mode |
| JVET-X0119 | Non-EE2: On pairwise merge candidate |
| JVET-X0121 | EE2-related: bug fixes for enabling RPR in ECM |
| JVET-X0122 | Non-EE2: Unification of negative modes processing in TIMD |
| JVET-X0131 | Non-EE2: Low-Complexity Improvements of Intra Coding for Screen Content |
| JVET-X0132 | EE2-Related: On MVD sign prediction |
| JVET-X0133 | Non-EE2: MV candidate type-based ARMC |
| JVET-X0134 | Non-EE2: On the number of TM merge candidates |
| JVET-X0135 | Non-EE2: Adaptive intra MTS |
| JVET-X0141 | EE2-3.1-related: CIIP with template matching |
| JVET-X0142 | Non-EE2: Extended MRL candidate list |
| JVET-X0145 | EE2-related: Template matching CIIP on top of EE2-3.1 |
| JVET-X0146 | Non-EE2: Decoder side motion derivation using sample's spatial correlation |
| JVET-X0147 | EE2-related: intra mode derivation based on TIMD for GPM inter/intra |
| JVET-X0155 | EE2-related: Crosscheck of JVET-X0114 (Fix on issues of TIMD mode) |
| JVET-X0156 | Non-EE2: Fix for histogram of gradients derivation in DIMD mode |
| JVET-X0166 | EE2-related: Combination of JVET-X0078 (Test 7/8), JVET-X0147 (Proposal-2), and GPM direct motion storage |

# AHG12 related documents

| JVET-X0058 | AHG12: Bilateral matching SMVD mode |
| --- | --- |
| JVET-X0105 | AHG12: Edge Classifier for Cross-component Sample Adaptive Offset (CCSAO) |
| JVET-X0120 | AHG12: On sign prediction |
| JVET-X0124 | AHG12: On signalling of intra template matching |
| JVET-X0139 | AHG12: removing a discontinuity in the discrete angle comparison in DIMD |
| JVET-X0148 | AHG12: On the PDPC handling in DIMD and TIMD |
| JVET-X0149 | AHG12: Removal of floating operations in DIMD and TIMD |
| JVET-X0150 | AHG12: Enhanced sign prediction |
| JVET-X0151 | AHG12: Non-adjacent spatial neighbors for affine merge mode |
| JVET-X0152 | AHG12: CCSAO classification with edge information |