| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0045-v1 |
| --- | --- |

| Title: | EE2-4.2, EE2-4.3: On CCALF filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer Ru-Ling Liao Jie Chen Yan Ye Xinwei Li 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | +1 (408) 858 6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group. |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2-4.2 and EE2-4.3. In EE2-4.2, a CTB can select one of two predefined filter shapes. In EE2-4.3, a single-shape large filter is used for all CTBs. This contribution also tested the impact of removing the power of 2 constraint of filter coefficient values. Following results are reported:

- Test 4.2.a: CTB level filter shape selection

  - AI: 0.04% (Y), -1.89%(U), -2.46%(V), 99%(EncTime), 101% (DecTime)

  - RA: 0.01% (Y), -1.79%(U), -2.04%(V), 99%(EncTime), 100% (DecTime)

  - LB: -0.02% (Y), -2.99%(U), -2.69%(V), 100%(EncTime), 101% (DecTime)

- Test 4.2.b: Test 4.2.a + removal of power of 2 constraint of filter coefficients

  - AI: 0.04% (Y),	-1.95%(U),	-2.55% (V),	99% (EncTime), 101%(DecTime)

  - RA: 0.03%(Y),	-1.75%(U),	-1.99%(V),	100%(EncTime), 101%(DecTime)

  - LB: 0.00%(Y), -3.00%(U), -2.99%(V), 101%(EncTime), 101%(DecTime)

- Test 4.3.a: CCALF with larger filter size

  - AI: 0.06% (Y),	-2.01% (U),	-2.61%(V),	100% (EncTime), 101% (DecTime)

  - RA: 0.01% (Y),	-1.69%(U),	-1.92%(V),	100%(EncTime), 100% (DecTime)

  - LB: -0.01% (Y), -2.58%(U),	-2.22%(V), 101%(EncTime), 101% (DecTime)

- Test 4.3.b: Test 4.3.a + removal of power of 2 constraint of filter coefficients

  - AI: 0.05% (Y), -2.08% (U),	-2.74% (V),	100% (EncTime), 102% (DecTime)

  - RA: 0.01% (Y),	-1.74% (U),	-2.03% (V),	100% (EncTime), 101% (DecTime)

  - LB: 0.05% (Y),	-2.86% (U),	-2.53% (V),	100% (EncTime), 100%(DecTime)

# Introduction

The cross component adaptive loop filter (CCALF) of ECM-2.0 is the same as in VVC except that in ECM-2.0 the constraint related to virtual boundary processing is removed. In ECM-2.0 (also in VVC), the CCALF process uses a linear filter to filter luma sample values and generate a residual correction for the chroma samples.  An 8-tap hexagon shaped filter is used in CCALF process, which is illustrated in Figure 1.

Figure 1: CCALF filter shape in ECM-2.0

The CCALF filter of ECM-2.0 may not achieve optimal compression performance. The compression performance of current CCALF design may improve using long tap filters and multiple filter shapes. The concept of multiple-shape CCALF filter was proposed in JVET-W0079 [1].

# Test 4.2: Multi-shape filters

In test 4.2, two filter shapes are proposed. The selection of shape is in the CTB level. In this test, for a given slice, the encoder can collect the statistics of the slice, analyze those and can signal up to 16 filters through APS. For each filter, the shape index (shape_idx in Figure 2) and associated filter coefficients are signalled. For each CTB, an index is signalled to the decoder to specify which filter shape and coefficients are used for that CTB.

Figure 2: CCALF filter shapes used in test 4.2

# Test 4.3: Long tap filter

In test 4.3, a 25-tap large filter is used. For a given slice, the encoder can collect the statistics of the slice, analyze those and can signal up to 16 filters through APS.

Figure 3: A 25-tap long tap filter

In ECM-2.0, CCALF filter coefficients are restricted to having only values in the form of power of 2, from the following set: {-64, -32, -16, -8, -4, -2, -1, 0, 1, 2, 4, 8, 16, 32, 64}. In test EE2.4.2b and EE2.4.3b, power of 2 constraints is removed. In test EE2.4.2b and EE2.4.3b, the value of the filter coefficient can be any integer value between -64 to +64, inclusive.

# Simulation results

The proposed methods are implemented on top of ECM-2.0. The performance is evaluated based on EE2 test conditions suggested in JVET-V2017 [2]. Following four tests are conducted.

- Test 4.2.a: CTB level filter shape selection of CCALF

- Test 4.2.b: CTB level filter shape selection of CCALF with removal of power of 2 constraint of filter coefficients

- Test 4.3.a: CCALF with larger filter size

- Test 4.3.b: CCALF with larger filter size with removal of power of 2 constraint of filter coefficients

.The simulation results are shown in Table 1, Table 2, Table 3, and Table 4.

Table 1: Results of test 4.2.a

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -1.23% | -3.23% | 99% | 100% |
| Class A2 | 0.08% | -2.71% | -4.14% | 99% | 100% |
| Class B | 0.08% | -3.42% | -2.48% | 99% | 100% |
| Class C | 0.00% | -0.61% | -0.62% | 100% | 102% |
| Class E | -0.02% | -0.91% | -2.42% | 100% | 101% |
| Overall | 0.04% | -1.89% | -2.46% | 99% | 101% |
| Class D | 0.02% | -0.90% | -1.05% | 100% | 103% |
| Class F | 0.06% | -1.99% | -1.16% | 100% | 101% |
| Class TGM |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.73% | -1.93% | 99% | 100% |
| Class A2 | 0.00% | -2.14% | -3.55% | 99% | 100% |
| Class B | 0.03% | -2.82% | -2.14% | 99% | 100% |
| Class C | 0.00% | -1.04% | -0.84% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -1.79% | -2.04% | 99% | 100% |
| Class D | 0.00% | -1.20% | -1.26% | 100% | 102% |
| Class F | 0.04% | -1.08% | -0.82% | 100% | 102% |
| Class TGM |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -3.77% | -2.87% | 99% | 100% |
| Class C | -0.10% | -2.47% | -1.49% | 101% | 102% |
| Class E | 0.01% | -2.39% | -4.00% | 101% | 103% |
| Overall | -0.02% | -2.99% | -2.69% | 100% | 101% |
| Class D | 0.04% | -2.67% | -1.79% | 101% | 102% |
| Class F | -0.01% | -2.50% | -1.53% | 101% | 102% |
| Class TGM |  |  |  |  |  |

Table 2: Results of test 4.2.b

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -1.25% | -3.26% | 99% | 101% |
| Class A2 | 0.09% | -2.68% | -4.25% | 99% | 101% |
| Class B | 0.07% | -3.39% | -2.54% | 99% | 101% |
| Class C | -0.01% | -0.74% | -0.71% | 100% | 102% |
| Class E | -0.02% | -1.13% | -2.60% | 100% | 102% |
| Overall | 0.04% | -1.95% | -2.55% | 99% | 101% |
| Class D | 0.01% | -1.09% | -1.26% | 100% | 103% |
| Class F | 0.04% | -2.02% | -1.22% | 100% | 101% |
| Class TGM |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.59% | -1.70% | 100% | 100% |
| Class A2 | 0.02% | -2.08% | -3.51% | 99% | 100% |
| Class B | 0.05% | -2.70% | -2.07% | 100% | 100% |
| Class C | 0.02% | -1.18% | -0.95% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -1.75% | -1.99% | 100% | 101% |
| Class D | -0.03% | -1.48% | -1.60% | 101% | 102% |
| Class F | 0.02% | -1.20% | -0.92% | 100% | 102% |
| Class TGM |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -3.78% | -3.03% | 100% | 100% |
| Class C | -0.05% | -2.59% | -1.86% | 101% | 101% |
| Class E | 0.03% | -2.25% | -4.42% | 102% | 100% |
| Overall | 0.00% | -3.00% | -2.99% | 101% | 101% |
| Class D | 0.03% | -3.38% | -2.57% | 102% | 103% |
| Class F | -0.12% | -2.33% | -1.28% | 100% | 101% |
| Class TGM |  |  |  |  |  |

Table 3: Results of test 4.3.a

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -1.19% | -3.52% | 99% | 101% |
| Class A2 | 0.11% | -3.01% | -4.86% | 99% | 101% |
| Class B | 0.10% | -3.46% | -2.42% | 99% | 101% |
| Class C | 0.01% | -0.58% | -0.69% | 100% | 102% |
| Class E | 0.00% | -1.30% | -2.35% | 100% | 102% |
| Overall | 0.06% | -2.01% | -2.61% | 100% | 101% |
| Class D | 0.03% | -0.81% | -0.90% | 99% | 103% |
| Class F | 0.07% | -1.79% | -0.82% | 101% | 101% |
| Class TGM |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.66% | -1.91% | 99% | 100% |
| Class A2 | -0.01% | -2.26% | -3.89% | 99% | 100% |
| Class B | 0.02% | -2.59% | -1.78% | 99% | 100% |
| Class C | 0.00% | -0.92% | -0.61% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -1.69% | -1.92% | 100% | 100% |
| Class D | 0.00% | -0.74% | -1.15% | 100% | 102% |
| Class F | 0.04% | -0.70% | -0.58% | 100% | 102% |
| Class TGM |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -3.46% | -2.44% | 99% | 100% |
| Class C | -0.06% | -2.25% | -1.14% | 102% | 102% |
| Class E | 0.05% | -1.57% | -3.28% | 102% | 102% |
| Overall | -0.01% | -2.58% | -2.22% | 101% | 101% |
| Class D | 0.12% | -2.28% | -1.96% | 101% | 102% |
| Class F | -0.26% | -1.83% | -1.42% | 101% | 102% |
| Class TGM |  |  |  |  |  |

Table 4: Results of test 4.3.b

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -1.22% | -3.51% | 100% | 101% |
| Class A2 | 0.12% | -2.96% | -4.90% | 100% | 101% |
| Class B | 0.09% | -3.52% | -2.54% | 100% | 101% |
| Class C | 0.00% | -0.77% | -0.85% | 100% | 103% |
| Class E | -0.01% | -1.41% | -2.63% | 100% | 102% |
| Overall | 0.05% | -2.08% | -2.74% | 100% | 102% |
| Class D | 0.02% | -1.10% | -1.22% | 100% | 103% |
| Class F | 0.05% | -1.88% | -1.16% | 101% | 102% |
| Class TGM |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.70% | -1.83% | 99% | 100% |
| Class A2 | 0.00% | -2.27% | -3.91% | 99% | 100% |
| Class B | 0.04% | -2.56% | -1.90% | 100% | 100% |
| Class C | -0.02% | -1.12% | -0.94% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -1.74% | -2.03% | 100% | 101% |
| Class D | 0.03% | -1.48% | -1.59% | 101% | 102% |
| Class F | 0.02% | -1.05% | -0.81% | 100% | 102% |
| Class TGM |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | -3.37% | -2.49% | 100% | 101% |
| Class C | -0.01% | -2.57% | -1.53% | 100% | 100% |
| Class E | 0.12% | -2.41% | -3.95% | 100% | 100% |
| Overall | 0.05% | -2.86% | -2.53% | 100% | 100% |
| Class D | 0.03% | -3.36% | -2.14% | 102% | 102% |
| Class F | 0.07% | -2.08% | -1.44% | 100% | 102% |
| Class TGM |  |  |  |  |  |

# Conclusion

This contribution reports the results of EE2.4.2 and EE2.4.3. In EE2.4.2, a CTB can select one of two predefined filter shapes. In EE2.4.3, a single-shape large filter is used for all CTBs. This contribution also tested the impact of removing the power of 2 constraint of filter coefficient values.

# References

M. G. Sarwer, R. -L. Liao, J. Chen, Y. Ye, X. Li, “AHG12: CTB level filter shape selection of CCALF”, JVET-W0079, July 2021.

V. Seregin, J. Chen, S. Esenlik, F. Le Léannec, L. Li, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-W2024, July 2021.

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).