| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0052 |
| --- | --- |

| Title: | EE1-1.3: neural network based in-loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liqiang Wang, Wei Jiang, Xiaozhong Xu, Shan Liu | Tel: Email: | {liqiangwang, vwjiang, xiaozhongxu, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports the EE results of JVET-W0113[], the neural network based in-loop filter is exploited to replace the Deblocking and SAO modules. Compared with the EE1 anchor, the proposed method can reportedly achieve {4.20%, 11.08, 10.16%}, {2.96%, 9.73%, 9.23%} and {5.34%, 10.03%, 10.04%} BD-rate savings with RA, LB and AI configurations, respectively.

Introduction

This contribution provides update results for the NN filter proposed in JVET-W0113[1]. The differences from the previous contribution are shown below:

- All the models are fine-tuned.

- The inference process is implemented by the libtorch.

- The inference process is adaptively skipped in the decoder, so the average decoder time decreases significantly.

Proposed method

The reconstructed image after LMCS is fed into the network. Then, the output of NN filter is processed by ALF and CCALF. In detail, as shown in Fig. 1, U and V channels are up-sampled and down-sampled in the pre-processing and post-processing, respectively.

Fig. 1 the pre-processing and post-processing units

Network architecture

Fig. 2 shows the network structure of the proposed NN filter. The first and last layers are regular convolutional layers with kernel size 3. The skip layer is a regular convolutional layer with kernel size 5. The structure of residual block (ResBlock) consists of 3 regular convolutional layers, where N denotes the number of ResBlock. N is set to 8 in this contribution. In the ResBlock, the first layer is a 1x1 convolutional layer followed by a ReLU activation function, the second layer is a 1x1 convolutional layer and the third layer is 3x3 convolutional layer. As for the internal convolutional layers, the number of feature maps is set as 64.

Fig. 2: Network architecture

Network information in training stage

Table 1: Network information for the NN-based video coding tool testing in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Tesla V100 SXM2 32GB |
|  | Framework: | Torch v1.4.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | ~1000 |
|  | Batch size: | 64 |
|  | Loss function: | L2 |
|  | Training time: | ~150h |
|  | Training data information: | DIV2K, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | Disable Deblock and SAO |
| Optional |  |  |
|  | Number of iterations | 600 |
|  | Patch size | 128x128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Mini-batch selection process: |  |
|  | Other information: |  |

Network information in inference stage

Table 2: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | Torch v1.4.0 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 596K |
|  | Total Number of Parameters (All Models) | 5.96M |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | 2.28M/model, 10 models in all |
|  | Multiplay Accumulate (MAC)/pixel | 786K |
| Optional |  |  |
|  | Total Conv. Layers | 27 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144x144 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

Implementation

- Deblock and SAO are disabled, and the proposed filter is placed before ALF.

- The proposed filter can be turned on/off at the CTU level and slice level.

- A scaling operation is carried out to refine the result of NN filter. The scaling factors are signaled in the slice header for each component.

Results

The anchor is VTM11+V0056[][], and the corresponding CTC results are provided in Table 3-5.

Table 3. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -3.67% | -6.32% | -6.56% | -5.04% | -7.63% | -6.72% | 149% | 68414% |
| Class A2 | -4.24% | -11.80% | -9.34% | -3.82% | -10.98% | -6.49% | 145% | 57584% |
| Class B | -4.17% | -11.49% | -10.20% | -3.86% | -10.29% | -8.44% | 153% | 70266% |
| Class C | -4.59% | -13.57% | -13.41% | -3.68% | -9.97% | -8.83% | 132% | 59806% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -4.20% | -11.08% | -10.16% | -4.04% | -9.81% | -7.81% | 145% | 64338% |
| Class D | -6.48% | -15.43% | -16.21% | -4.31% | -10.94% | -9.73% | 129% | 48325% |
| Class F | -1.64% | -7.91% | -7.70% | -0.87% | -6.27% | -6.06% | 185% | 34080% |

Table 4. Performance of the proposed method (LB)

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -2.64% | -8.05% | -7.97% | -2.20% | -7.58% | -7.65% | 149% | 73975% |
| Class C | -3.39% | -11.84% | -10.81% | -2.46% | -8.79% | -2.31% | 132% | 59721% |
| Class E | -2.90% | -5.97% | -7.04% | -1.57% | -2.76% | -2.50% | 222% | 62888% |
| Overall | -2.96% | -9.73% | -9.23% | -2.13% | -8.12% | -5.28% | 158% | 66141% |
| Class D | -4.88% | -13.98% | -12.36% | -3.47% | -10.79% | -4.53% | 129% | 52596% |
| Class F | -0.92% | -5.73% | -5.02% | -0.40% | -2.80% | -2.32% | 182% | 41951% |

Table 5. Performance of the proposed method (AI)

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -4.09% | -7.60% | -8.29% | -5.10% | -7.90% | -8.78% | 197% | 48010% |
| Class A2 | -4.58% | -10.55% | -10.03% | -4.28% | -9.64% | -7.03% | 151% | 37702% |
| Class B | -4.72% | -10.20% | -8.85% | -4.09% | -9.38% | -8.33% | 147% | 40598% |
| Class C | -6.20% | -11.90% | -12.86% | -5.07% | -10.79% | -10.71% | 123% | 27537% |
| Class E | -7.24% | -9.14% | -10.00% | -5.78% | -7.77% | -7.98% | 155% | 46574% |
| Overall | -5.34% | -10.03% | -10.04% | -4.79% | -9.22% | -8.66% | 150% | 38705% |
| Class D | -6.73% | -11.91% | -14.20% | -5.12% | -10.88% | -11.84% | 117% | 22763% |
| Class F | -2.87% | -6.97% | -6.52% | -2.19% | -6.17% | -6.44% | 122% | 29009% |

Conclusions

A neural network based in-loop filter is presented in this contribution, where Deblock and SAO are both replaced with the NN based filter. Good coding performance can be achieved by applying the proposed method.

References

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

[] L. Wang, W. Jiang, X. Xu, S. Liu. AHG11: neural network based in-loop filter. JVET-W0113, 23rd Meeting, by teleconference, 7–16 July 2021.

[] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc.

[] S. Liu, A. Segall, E. Alshina, R.-L. Liao. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-W2016, 23rd Meeting, by teleconference, 7–16 July 2021.