| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0053 |
| --- | --- |

| Title: | EE1-1.5: neural network based in-loop filter using depthwise separable convolution and regular convolution |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liqiang Wang, Sheng Lin, Xiaozhong Xu, Shan Liu and Xiang Li | Tel: Email: | {liqiangwang, barneylin, xiaozhongxu, shanl, xlxiangli} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports the EE results of JVET-W0151[1], the neural-network-based in loop filter using depthwise separable convolution (DSC) in EE1-1.5. With VTM11+ V0056[2][3] as anchor and test platform, it is reported that luma BD-Rate of 1.14%, 0.99%, and 1.62% can be achieved from RA, LB and AI configurations, respectively.

Introduction

This contribution provides update results for DSC neural network in JVET-W0151[1] with the same model. In the updated implementation, the inference process is implemented by the libtorch. Further, the inference process is adaptively skipped in the decoder, so the decoder time decreases significantly.

Depthwise Separable convolution with Pytorch

In depthwise separable convolution (DSC), the output value of the layer with tensor input size (N,Cin,H,W) and output with size (N,Cout,Hout,Wout) can be achieved in two steps with depthwise convolution. in the first step followed by pointwise convolution in the second step.

In the first step, DSC applies depthwise convolution using a single convolutional filter per each input channel k:

outNi,k=biask+weightk ⋆inputNi,k

where ⋆ is the valid 2-D cross-correlation operator, N is a batch size, C denotes a number of channels, H is a height of input planes in pixels, and W is width in pixels.

Then in the second step, it applies pointwise convolution to create the output of the DSC as a linear combination of the outputs of the depthwise convolution:

outNi, Coutj=biasCoutj+ k=0Cin-1weightCoutj,k ⋅outNi,k

When the kernel size of the 2-D convolution is (h,w), the number of weight is Cin⋅h⋅w+1+Cout⋅(Cin+1) and the number MACs is Hout⋅Wout⋅(Cin⋅h⋅w+Cin+Cin⋅Cout+Cout).

Description of the neural network based in-loop filter in JVET-U0060/ U0061 with depthwise separable convolution (DSC)

JVET-U0060 [4] reported the results of adding an additional filter called Dense Residual Convolutional Neural Network Based In-loop Filter (DRNLF) inserted between deblocking filter and SAO as shown in Figure 1. As shown in Figure 2, the DRNLF includes a QP Map Quantizer unit, a Pre-Processing unit, a Main-processing unit and a Post-Processing unit.

Figure 1: DRNLF in JVET-U0060 is an in-loop filter in between deblocking filter and SAO. The inputs to the DRMLF are the output of the deblocking filter and the QP map of the picture.

Figure 2: DRNLF in JVET-U0060 consists of a QP Map Quantizer, a Pre-Processing unit, a Main Processing unit and a Post-Processing unit.

In JVET-U0061[5], a clipping step is added to the Pre-Processing unit depicted in Figure 3 from JVET-U0060 [4] to obtain the Pre-Procssing unit in Figure 4 . The clipping is intended to impove the results in the training and the inference process of the neural network.

Figure 3: DRNLF Pre-Processing in JVET-U0060.

Figure 4:Clipping is added to the Pre-Processing to impove the results of the neural network.

Network information in training stage

Table 1: Network information for the NN-based video coding tool testing with DSC in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Intel Core i7-8700 3.20 GHz (6 cores), 32GB RAM and one 11 GB Nvidia GTX 1080Ti |
|  | Framework: | Torch v1.4.0 |
|  | Number of GPUs per Task | 1 Nvidia GTX 1080Ti was used for training |
|  | Epoch: | 300 |
|  | Batch size: | 16 |
|  | Training time: | 48h |
|  | Training data information: | DIV2K still images |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP = 22, 27, 32, 37 |
| Optional | Learning rate | 1e-3 |
|  | Number of iterations |  |
|  | Patch size | 64x64 |
|  | Learning rate: | 1.00E-03 |
|  | Optimizer: | ADAM |
|  | Loss function: | L1 |
|  | Preprocessing: | randomly cropped and rotated |
|  | Other information: |  |

Network information in inference stage

Table 2: Network information for the NN-based video coding tool testing in inference stage with DSC

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | Torch v1.4.0 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 23.43K |
|  | Total Number of Parameters (All Models) | 23.43K |
|  | Parameter Precision (Bits) in Float | 32 |
|  | Memory Parameter (MB) | 0.16MB |
|  | MAC (Giga) per pixel | 29K |
| Optional |  |  |
|  | Total Conv. Layers | 13 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: |  |
|  | Patch size | 144x144 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Results

The EE1-1.5 test was integrated with VTM11.0+V0056. The corresponding result is shown in Table 3 and 4. Compared with JVET-W0151[1], the decoder time decreases significantly with the similar performance.

Table 3. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -1.77% | -3.04% | -4.02% | -3.53% | -3.93% | -4.15% | 107% | 1693% |
| Class A2 | -1.17% | -5.05% | -3.11% | -0.78% | -3.85% | -1.76% | 106% | 910% |
| Class B | -0.83% | -5.89% | -4.25% | -0.42% | -5.34% | -3.06% | 107% | 986% |
| Class C | -1.03% | -6.25% | -5.03% | -0.58% | -5.85% | -4.42% | 104% | 1310% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -1.14% | -5.25% | -4.18% | -1.16% | -4.89% | -3.38% | 106% | 1166% |
| Class D | -1.33% | -4.14% | -4.67% | -0.54% | -4.31% | -4.45% | 104% | 1132% |
| Class F | -0.51% | -2.58% | -2.79% | -0.20% | -3.02% | -2.73% | 109% | 495% |

Table 4. Performance of the proposed method(LB)

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -1.43% | -6.23% | -5.83% | -3.22% | -10.67% | -6.25% | 105% | 2774% |
| Class A2 | -1.12% | -7.43% | -4.61% | -1.07% | -6.72% | -3.34% | 105% | 1510% |
| Class B | -0.65% | -8.31% | -5.98% | -0.43% | -7.88% | -5.93% | 106% | 1610% |
| Class C | -0.83% | -9.89% | -8.08% | -0.37% | -10.97% | -8.21% | 105% | 2209% |
| Class E | -1.75% | -3.73% | -4.46% | -0.88% | -2.68% | -3.20% | 115% | 525% |
| Overall | -0.99% | -9.01% | -6.91% | -0.52% | -9.26% | -6.94% | 108% | 1352% |
| Class D | -1.19% | -6.22% | -6.25% | -0.28% | -7.15% | -7.00% | 104% | 1931% |
| Class F | -0.40% | -4.03% | -3.94% | 0.14% | -5.80% | -5.27% | 110% | 723% |

Table 5. Performance of the proposed method (AI)

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -1.43% | -3.04% | -3.97% | -3.00% | -3.77% | -4.00% | 113% | 2900% |
| Class A2 | -1.35% | -3.94% | -2.98% | -1.11% | -3.16% | -2.00% | 106% | 2393% |
| Class B | -1.07% | -5.05% | -4.50% | -0.77% | -4.98% | -3.70% | 105% | 2731% |
| Class C | -2.03% | -6.61% | -5.57% | -1.33% | -6.29% | -5.29% | 103% | 2718% |
| Class E | -2.44% | -3.14% | -3.43% | -1.49% | -2.66% | -2.54% | 107% | 3819% |
| Overall | -1.62% | -4.56% | -4.22% | -1.44% | -4.38% | -3.63% | 106% | 2850% |
| Class D | -2.44% | -4.60% | -5.54% | -1.25% | -5.13% | -5.91% | 103% | 2354% |
| Class F | -1.03% | -3.05% | -3.16% | -0.66% | -3.20% | -3.34% | 102% | 1611% |

Conclusions

This contribution provides the EE results for the DSC loop filter tool from JVET-W0151 [1]. The results from this contribution demonstrated that the decoder runtime of DSC can be further decreased while the similar performance can be maintained.

References

- Liqiang Wang, Sheng Lin, Xiaozhong Xu,  Shan Liu, Cheung Auyeung and Xiang Li, “EE1: neural network based in-loop filter using depthwise separable convolution and regular convolution”. JVET-W0151, 23rd Meeting, by teleconference, 7–16 July 2021.

- https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc.

- S. Liu, A. Segall, E. Alshina, R.-L. Liao. “JVET common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-W2016, 23rd Meeting, by teleconference, 7–16 July 2021.

- Cheung Auyeung, Wei Wang, Wei Jiang, Xiang Li, Shan Liu,”EE1.1: A comparison of depthwise separable convolution and regular convolution with the JVET-T0057 neural network based in-loop filter”, JVET-U0060, 21st Meeting, by teleconference, 6–15 Jan. 2021.

- Cheung Auyeung, Xiang Li, Shan Liu, “EE-1.1-related: BD-Rate improvements to JVET-T0057 neural network based in-loop filter using depthwise separable convolution and regular convolution” JVET-U0061, 21st Meeting, by teleconference, 6–15 Jan. 2021.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).