| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0063-v1 |
| --- | --- |

| Title: | AHG10: Deblocking filter setting for VTM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Zhang, Xiang Li, Shan Liu | Tel: Email: | zhhanzhang@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In this contribution, a set of encoder only settings of deblocking filter are proposed for VTM. Basically, the deblocking filter control parameters BetaOffset_div2 and TcOffset_dive2 are adjusted based on temporal layer ID for both luma and chroma components. The proposed method is implemented by only modifying VTM configuration files. It is reported that under the CTC, compared with VTM-14.0, the overall performance are as follows:

- AI: -0.74%/-0.69%/-0.44% with 99%EncT/99%DecT

- RA: -0.32%/-0.58%/-0.50% with 100%EncT/100%DecT

- LDB: -0.62%/-0.41%/0.02% with 100%EncT/100%DecT

- LDP: -0.34%/-0.26%/-0.09% with 100%EncT/100%DecT

Introduction

In ECM [1], the deblocking filter (DBF) control parameters BetaOffset_div2 and TcOffset_div2 vary with the temporal level for better coding efficiency. Two new syntax elements  pps_beta_offset_div2_num_minus1 and pps_beta_Tc_div2_num_minus1 are also signaled in ECM bitstream. However, in VTM CTC, BetaOffset_div2 and TcOffset_div2 are set to the same value for all temporal layers. To align the deblocking settings of ECM and VTM while avoiding new syntax elements in VTM, this contribution proposes to use temporal layer dependent DBF control parameters in VTM.

Proposed Method

Considering the different temporal layer settings for different coding configurations, it is proposed to use the DBF control parameters in Table2.1, Table2.2, and Table2.3 for RA, LD, and AI, respectively. Those parameters are actually in line with ECM-2.0 code though no new syntax element is introduced.

Table2.1 – DBF control parameters for RA

| Tid Parameter | I slice | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BetaOffset_div2 | -2 | -2 | -2 | -2 | -2 | -2 | -2 |
| TcOffset_div2 | -5 | -4 | -3 | -2 | -1 | 0 | 0 |

Table2.2 – DBF control parameters for LD

| Tid Parameter | I slice | 0 |
| --- | --- | --- |
| BetaOffset_div2 | -3 | -3 |
| TcOffset_div2 | -3 | -3 |

Table2.3 – DBF control parameters for AI

| Tid Parameter | I slice |
| --- | --- |
| BetaOffset_div2 | -2 |
| TcOffset_div2 | -5 |

The proposed method is implemented by only modifying the DBF related parameters in the configuration file, as attached.

Test Results

Table3.1 summarizes the overall results of the proposed temporal layer dependent method compared to VTM-14.0 [2].

Table3.1- Test results of temporal layer-dependent DBF control parameters

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-14.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.87% | -0.76% | 0.26% | 100% | 101% |
| Class A2 | -0.77% | -0.83% | -0.72% | 97% | 98% |
| Class B | -0.89% | -0.66% | -0.58% | 100% | 100% |
| Class C | -0.56% | -0.99% | -1.01% | 100% | 97% |
| Class E | -0.56% | -0.15% | 0.15% | 100% | 101% |
| Overall | -0.74% | -0.69% | -0.44% | 99% | 99% |
| Class D | -0.48% | -0.73% | -0.71% | 100% | 101% |
| Class F | -0.39% | -1.06% | -1.49% | 100% | 98% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-14.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.19% | -0.01% | 101% | 101% |
| Class A2 | -0.42% | -0.27% | -0.36% | 100% | 101% |
| Class B | -0.39% | -0.67% | -0.54% | 100% | 100% |
| Class C | -0.23% | -1.00% | -0.94% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.32% | -0.58% | -0.50% | 100% | 100% |
| Class D | 0.03% | -0.91% | -0.62% | 101% | 104% |
| Class F | -0.19% | -1.37% | -1.74% | 101% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-14.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.52% | -0.27% | 0.14% | 100% | 99% |
| Class C | -0.38% | -0.65% | -0.05% | 100% | 100% |
| Class E | -1.11% | -0.30% | -0.06% | 100% | 102% |
| Overall | -0.62% | -0.41% | 0.02% | 100% | 100% |
| Class D | -0.21% | 0.17% | 0.44% | 100% | 101% |
| Class F | -0.70% | -0.89% | -0.69% | 100% | 96% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-14.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.36% | -0.34% | 0.01% | 100% | 100% |
| Class C | -0.23% | -0.48% | -0.07% | 100% | 98% |
| Class E | -0.45% | 0.17% | -0.29% | 100% | 101% |
| Overall | -0.34% | -0.26% | -0.09% | 100% | 100% |
| Class D | -0.22% | -0.29% | 0.67% | 100% | 101% |
| Class F | -0.44% | -0.64% | -0.73% | 100% | 97% |

Conclusion

This contribution proposes encoder only settings on DBF control parameters for VTM. It is reported that the proposed method obtains BD-rate change of -0.74% on AI, -0.32% on RA, -0.62% on LDB and -0.34% on LDP compared with VTM-14.0 with similar encoding and decoding time.

Reference

- M. Coban, F. Léannec, J. Ström, “Algorithm description of Enhanced Compression Model 2 (ECM 2)”, document JVET-W2025, 23rd JVET meeting: by teleconference, 7–16 July 2021.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/VTM-14.0

Patent rights declaration(s)

Tencent does not have any current or pending patent rights relating to the technology described in this contribution.