| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0079-v2 |
| --- | --- |

| Title: | Proposals on maximum bit rate for HEVC and VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba, Masaru Ikeda, Teruhiko Suzuki 2-10-1 Osaki, Shinagawa-ku Tokyo, JAPAN | Tel: Email: | +81-70-7598-7392 teruhiko.s@sony.com |
| Source: | Sony Group Corporation |  |  |

_____________________________

- Abstract

This contribution proposes to add new code points to support higher max bit rate for Main 4:2:2 and 4:4:4 Intra profiles for both HEVC and VVC. The authors believe that the current max bit rate is not sufficient to support high end production applications. Therefore, the additional code points to support higher max bit rate are proposed. The modifications to the spec text are summarized in the Annex. This proposal does not affect on the current existing code points.

Introduction

HEVC is used for high end camera for broadcast/movie production, e.g. XAVC. Currently, those camera/production systems support up to 8K@30p. ITU-R BT.2100 supports up to 8K@120p, therefore, those production systems will support such high resolution and high frame rate video format. For such very high-end applications, higher fidelity of video is required, e.g. visually lossless. It is identified that the current max bit rate for 4:2:2 and 4:4:4 Intra profile is not sufficient. JVET-X0109 reports bit rate exceeds the max bit rate constraint when low QP is used.

This contribution proposes to add new code points to support higher max bit rate and max cpb size for Intra profiles for 4:2:2 and 4:4:4. Since those are additional code points, the proposal does not affect on the existing code points of profile/levels.

Proposals

This contribution proposes to add new code points to support higher max bit rate for HEVC and VVC. Very high-end camera/production systems support 4:2:2, 4:4:4 and Intra coding are used. The current deployment is Main 422 10 profile. Therefore, this contribution proposes to add new code points on top of HEVC/VVC Main 422/444 Intra profiles, not High throughput profiles. There could be three options to extend max bit rate.

- Option 1: Modify the calculation to derive max bit rate

- Option 2: Add new “tier” above high tier

- Option 3: Add new level to support higher bit rate

The following sections explain more detail proposed changes. To simplify, the following sections explain for HEVC. VVC can be extended in the same way. The text changes are summarized in Annex A (HEVC) and Annex B (VVC).

Option 1: Add scaling factor for the calculation of maximum bit rate for 4:2:2 and 4:4:4 Intra

This option proposes to change the calculation to derive max bit rate and CPB size. Annex A specifies the equation to derive max bit rate as follows.

A.4.1 specifies max CPB size as follows.

For the VCL HRD parameters, CpbSize[ i ] shall be less than or equal to CpbVclFactor * MaxCPB

A.4.2 specifies max bit rate as follows.

The variable BrVclFactor, which represents the VCL bit rate scale factor, is set equal to CpbVclFactor * HbrFactor.

For the VCL HRD parameters, BitRate[ i ] shall be less than or equal to BrVclFactor * MaxBR

The above is the derivation for VCL HRD parameters, but NAL HRD parameters can be derived in similar way. In Option 1, a flag (general_br_factor_extension_flag) to identify higher max bit rate is sent at profile_tier_level().

- 7.3.3 Profile, tier and level syntax

| general_lower_bit_rate_constraint_flag | u(1) |
| --- | --- |
| if( general_profile_idc = = 4 ¦ ¦ general_profile_compatibility_flag[ 4 ]) { |  |
| or if( general_tier_flag && general_intra_constraint_flag && !general_max_420_chroma_constraint_flag && !general_lower_bit_rate_constraint_flag) { |  |
| general_br_factor_extension_flag | u(1) |
| general_reserved_zero_33bits | u(33) |
| else { |  |
| general_reserved_zero_34bits | u(34) |
| } |  |
| } |  |
| if( general_profile_idc = = 5 ¦ ¦ general_profile_compatibility_flag[ 5 ] ¦ ¦ general_profile_idc = = 9 ¦ ¦ general_profile_compatibility_flag[ 9 ] ¦ ¦ general_profile_idc = = 10 ¦ ¦ general_profile_compatibility_flag[ 10 ] ¦ ¦ general_profile_idc = = 11 ¦ ¦ general_profile_compatibility_flag[ 11 ] ) { |  |

If this flag is equal to 0, there is no change from the current spec. If this flag is equal to 1, max cpb size and max bit rate is doubled. Max cpb size can be derived as follows.

A.4.1   General tier and level limits

………

The variable CpbScaleFactor is defined as follows:

- If general_br_factor_extension_flag is equal to 1, CpbScaleFactor is set equal to 2.

- Otherwise, CpbScaleFactor is set equal to 1.

- For the VCL HRD parameters, CpbSize[ i ] shall be less than or equal to CpbVclFactor * CpbScaleFactor * MaxCPB for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where CpbSize[ i ] is specified in clause ‎E.3.3 based on parameters selected as specified in clause ‎C.1, CpbVclFactor is specified in Table A.10, and MaxCPB is specified in Table A.8 in units of CpbVclFactor bits.

- For the NAL HRD parameters, CpbSize[ i ] shall be less than or equal to CpbNalFactor * CpbScaleFactor * MaxCPB for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where CpbSize[ i ] is specified in clause ‎E.3.3 based on parameters selected as specified in clause ‎C.1, CpbNalFactor is specified in Table A.10, and MaxCPB is specified in Table A.8 in units of CpbNalFactor bits.

There could be two ways to double the max bit rate. Variant A is to modify HbrFactor, and Variant B is to modify the equation to derive max bit rate.

Variant A: Modify HbrFactor

In this case HbrFactor is doubled if general_br_factor_extension_flag is equal to 1. Otherwise, HbrFactor is unchanged.

A.4.2  Profile-specific level limits for the video profiles

…..

The variable HbrScaleFactor is defined as follows:

- If general_br_factor_extension_flag is equal to 1, HbrScaleFactor is set equal to 2.

- Otherwise, HbrScaleFactor is set equal to 1.

The variable HbrFactor is defined as follows:

- If the bitstream or sub-layer representation is indicated to conform to the Main profile or the Main 10 profile, HbrFactor is set equal to 1.

- Otherwise, if the bitstream or sub-layer representation is indicated to conform to the High Throughput 4:4:4, High Throughput 4:4:4 10, High Throughput 4:4:4 14, Screen-Extended High Throughput 4:4:4, Screen-Extended High Throughput 4:4:4 10, or Screen-Extended High Throughput 4:4:4 14  profile, HbrFactor is set equal to 6.

- Otherwise, if the bitstream or sub-layer representation is indicated to conform to the High Throughput 4:4:4 16 Intra profile, HbrFactor is set equal to 24 − ( 12 * general_lower_bit_rate_constraint_flag ) or 24 − ( 12 * sub_layer_lower_bit_rate_constraint_flag[ i ] ).

- Otherwise, HbrFactor is set equal to 2 − general_lower_bit_rate_constraint_flag or 2 − sub_layer_lower_bit_rate_constraint_flag[ i ].

- Then HbrFactor is set equal to HbrFactor * HbrScaleFactor.

Variant B: Modify BitRate

In this case max value of BitRate[i] is doubled if general_br_factor_extension_flag is equal to 1. Otherwise, max value of BitRate[i] is unchanged.

A.4.2  Profile-specific level limits for the video profiles

…..

The variable MaxbrScaleFactor is defined as follows:

- If general_br_factor_extension_flag is equal to 1, MaxbrScaleFactor is set equal to 2.

- Otherwise, MaxbrScaleFactor is set equal to 1.

e)  For the VCL HRD parameters, BitRate[ i ] shall be less than or equal to BrVclFactor * MaxBR * MaxbrScaleFactor for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where BitRate[ i ] is specified in clause ‎E.3.3 based on parameters selected as specified in clause ‎C.1 and MaxBR is specified in Table A.9 in units of BrVclFactor bits/s.

f) For the NAL HRD parameters, BitRate[ i ] shall be less than or equal to BrNalFactor * MaxBR * MaxbrScaleFactor for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where BitRate[ i ] is specified in clause ‎E.3.3 based on parameters selected as specified in clause ‎C.1 and MaxBR is specified in Table A.9 in units of BrNalFactor bits/s.

Option 2: Add new tier for 4:2:2 and 4:4:4 Intra

This option proposes to add new tier above high tier. Since general_tier_flag is one bit flag, another flag is necessary to signal higher tier. The syntax to indicate higher tier is the same as Option 1 (adding general_br_factor_extension_flag). If general_tier_flag is equal to 1 and general_br_factor_extension_flag is equal to 1, it indicates higher tier than high tier (Super high tier).

Table A.8 – General tier and level limits

| Level | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) |  |  | Max slice segments per picture MaxSliceSegmentsPerPicture | Max # of tile rows MaxTileRows | Max # of tile columns MaxTileCols |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Super high tier |  |  |  |
| 1 | 36 864 | 350 | - | - | 16 | 1 | 1 |
| 2 | 122 880 | 1 500 | - | - | 16 | 1 | 1 |
| 2.1 | 245 760 | 3 000 | - | - | 20 | 1 | 1 |
| 3 | 552 960 | 6 000 | - | - | 30 | 2 | 2 |
| 3.1 | 983 040 | 10 000 | - | - | 40 | 3 | 3 |
| 4 | 2 228 224 | 12 000 | 30 000 | - | 75 | 5 | 5 |
| 4.1 | 2 228 224 | 20 000 | 50 000 | - | 75 | 5 | 5 |
| 5 | 8 912 896 | 25 000 | 100 000 | - | 200 | 11 | 10 |
| 5.1 | 8 912 896 | 40 000 | 160 000 | - | 200 | 11 | 10 |
| 5.2 | 8 912 896 | 60 000 | 240 000 | - | 200 | 11 | 10 |
| 6 | 35 651 584 | 60 000 | 240 000 | 480 000 | 600 | 22 | 20 |
| 6.1 | 35 651 584 | 120 000 | 480 000 | 960 000 | 600 | 22 | 20 |
| 6.2 | 35 651 584 | 240 000 | 800 000 | 1 600 000 | 600 | 22 | 20 |

A.4.2  Profile-specific level limits for the video profiles

Table A.9 – Tier and level limits for the video profiles

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (BrVclFactor or BrNalFactor bits/s) |  |  | Min compression ratio MinCrBase |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Super high tier | Main tier | High tier | Super high tier |
| 1 | 552 960 | 128 | - | - | 2 | 2 | - |
| 2 | 3 686 400 | 1 500 | - | - | 2 | 2 | - |
| 2.1 | 7 372 800 | 3 000 | - | - | 2 | 2 | - |
| 3 | 16 588 800 | 6 000 | - | - | 2 | 2 | - |
| 3.1 | 33 177 600 | 10 000 | - | - | 2 | 2 | - |
| 4 | 66 846 720 | 12 000 | 30 000 | - | 4 | 4 | - |
| 4.1 | 133 693 440 | 20 000 | 50 000 | - | 4 | 4 | - |
| 5 | 267 386 880 | 25 000 | 100 000 | - | 6 | 4 | - |
| 5.1 | 534 773 760 | 40 000 | 160 000 | - | 8 | 4 | - |
| 5.2 | 1 069 547 520 | 60 000 | 240 000 | - | 8 | 4 | - |
| 6 | 1 069 547 520 | 60 000 | 240 000 | 480 000 | 8 | 4 | 4 |
| 6.1 | 2 139 095 040 | 120 000 | 480 000 | 960 000 | 8 | 4 | 4 |
| 6.2 | 4 278 190 080 | 240 000 | 800 000 | 1 600 000 | 6 | 4 | 4 |

Option 3: Add new levels for 4:2:2 and 4:4:4 Intra

This option proposes to add new level for higher bit rate. No syntax change is necessary. In case of VVC, level 6.3 was proposed and included in the JVET-W2005, VVC operation range extensions (Draft 4). This contribution proposes to include the same level (6.3) to HEVC to be consistent with VVC levels. New level is proposed on top of 6.3.

There could be two ways to extend max bit rate. Variant A is to add level 6.4, and Variant B is to modify high tier of level 6.3 Green highlighted parts are correspond with level 6.3 in VVC.

Variant A: Add level 6.4

A.4.1   General tier and level limits

………

Table A.8 – General tier and level limits

| Level | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) |  | Max slice segments per picture MaxSliceSegmentsPerPicture | Max # of tile rows MaxTileRows | Max # of tile columns MaxTileCols |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier |  |  |  |
| 1 | 36 864 | 350 | - | 16 | 1 | 1 |
| 2 | 122 880 | 1 500 | - | 16 | 1 | 1 |
| 2.1 | 245 760 | 3 000 | - | 20 | 1 | 1 |
| 3 | 552 960 | 6 000 | - | 30 | 2 | 2 |
| 3.1 | 983 040 | 10 000 | - | 40 | 3 | 3 |
| 4 | 2 228 224 | 12 000 | 30 000 | 75 | 5 | 5 |
| 4.1 | 2 228 224 | 20 000 | 50 000 | 75 | 5 | 5 |
| 5 | 8 912 896 | 25 000 | 100 000 | 200 | 11 | 10 |
| 5.1 | 8 912 896 | 40 000 | 160 000 | 200 | 11 | 10 |
| 5.2 | 8 912 896 | 60 000 | 240 000 | 200 | 11 | 10 |
| 6 | 35 651 584 | 60 000 | 240 000 | 600 | 22 | 20 |
| 6.1 | 35 651 584 | 120 000 | 480 000 | 600 | 22 | 20 |
| 6.2 | 35 651 584 | 240 000 | 800 000 | 600 | 22 | 20 |
| 6.3 | 80 216 064 | 240 000 | 800 000 | 600 | 22 | 20 |
| 6.4 | 80 216 064 | 240 000 | 1 600 000 | 600 | 22 | 20 |

A.4.2  Profile-specific level limits for the video profiles

Table A.9 – Tier and level limits for the video profiles

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (BrVclFactor or BrNalFactor bits/s) |  | Min compression ratio MinCrBase |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Main tier | High tier |
| 1 | 552 960 | 128 | - | 2 | 2 |
| 2 | 3 686 400 | 1 500 | - | 2 | 2 |
| 2.1 | 7 372 800 | 3 000 | - | 2 | 2 |
| 3 | 16 588 800 | 6 000 | - | 2 | 2 |
| 3.1 | 33 177 600 | 10 000 | - | 2 | 2 |
| 4 | 66 846 720 | 12 000 | 30 000 | 4 | 4 |
| 4.1 | 133 693 440 | 20 000 | 50 000 | 4 | 4 |
| 5 | 267 386 880 | 25 000 | 100 000 | 6 | 4 |
| 5.1 | 534 773 760 | 40 000 | 160 000 | 8 | 4 |
| 5.2 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6.1 | 2 139 095 040 | 120 000 | 480 000 | 8 | 4 |
| 6.2 | 4 278 190 080 | 240 000 | 800 000 | 6 | 4 |
| 6.3 | 4 812 963 840 | 320 000 | 800 000 | 6 | 4 |
| 6.4 | 4 812 963 840 | 320 000 | 1 600 000 | 6 | 4 |

Variant B: Modify high tier of level 6.3

A.4.1   General tier and level limits

………

Table A.8 – General tier and level limits

| Level | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) |  | Max slice segments per picture MaxSliceSegmentsPerPicture | Max # of tile rows MaxTileRows | Max # of tile columns MaxTileCols |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier |  |  |  |
| 1 | 36 864 | 350 | - | 16 | 1 | 1 |
| 2 | 122 880 | 1 500 | - | 16 | 1 | 1 |
| 2.1 | 245 760 | 3 000 | - | 20 | 1 | 1 |
| 3 | 552 960 | 6 000 | - | 30 | 2 | 2 |
| 3.1 | 983 040 | 10 000 | - | 40 | 3 | 3 |
| 4 | 2 228 224 | 12 000 | 30 000 | 75 | 5 | 5 |
| 4.1 | 2 228 224 | 20 000 | 50 000 | 75 | 5 | 5 |
| 5 | 8 912 896 | 25 000 | 100 000 | 200 | 11 | 10 |
| 5.1 | 8 912 896 | 40 000 | 160 000 | 200 | 11 | 10 |
| 5.2 | 8 912 896 | 60 000 | 240 000 | 200 | 11 | 10 |
| 6 | 35 651 584 | 60 000 | 240 000 | 600 | 22 | 20 |
| 6.1 | 35 651 584 | 120 000 | 480 000 | 600 | 22 | 20 |
| 6.2 | 35 651 584 | 240 000 | 800 000 | 600 | 22 | 20 |
| 6.3 | 80 216 064 | 240 000 | 1 600 000 | 600 | 22 | 20 |

A.4.2  Profile-specific level limits for the video profiles

Table A.9 – Tier and level limits for the video profiles

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (BrVclFactor or BrNalFactor bits/s) |  | Min compression ratio MinCrBase |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Main tier | High tier |
| 1 | 552 960 | 128 | - | 2 | 2 |
| 2 | 3 686 400 | 1 500 | - | 2 | 2 |
| 2.1 | 7 372 800 | 3 000 | - | 2 | 2 |
| 3 | 16 588 800 | 6 000 | - | 2 | 2 |
| 3.1 | 33 177 600 | 10 000 | - | 2 | 2 |
| 4 | 66 846 720 | 12 000 | 30 000 | 4 | 4 |
| 4.1 | 133 693 440 | 20 000 | 50 000 | 4 | 4 |
| 5 | 267 386 880 | 25 000 | 100 000 | 6 | 4 |
| 5.1 | 534 773 760 | 40 000 | 160 000 | 8 | 4 |
| 5.2 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6.1 | 2 139 095 040 | 120 000 | 480 000 | 8 | 4 |
| 6.2 | 4 278 190 080 | 240 000 | 800 000 | 6 | 4 |
| 6.3 | 4 812 963 840 | 320 000 | 1 600 000 | 6 | 4 |

Conclusions

This contribution proposes to add new code points to support higher max bit rate for Main 4:2:2 and 4:4:4 Intra profiles for both HEVC and VVC. The authors believe that the current max bit rate is not sufficient to support high end production applications. Therefore, the additional code points to support higher max bit rate are proposed. The modifications to the spec text are summarized in the Annex. This proposal does not affect on the current existing code points.

Reference

JVET-X0109 “On maximum bit-rates for high bit-depth profiles” [S. Keating, A. Browne, K. Sharman (Sony)]

Patent rights declaration(s)

Sony Group Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Annex A: Proposed text change for HEVC

Option 1: Add scaling factor for the calculation of maximum bit rate

1) Modify the profile tier level syntax as follows

- 7.3.3 Profile, tier and level syntax

Add the following yellow highlighted part:

| general_lower_bit_rate_constraint_flag | u(1) |
| --- | --- |
| if( general_profile_idc = = 4 ¦ ¦ general_profile_compatibility_flag[ 4 ]) { |  |
| if( general_tier_flag && general_intra_constraint_flag && !general_max_420_chroma_constraint_flag && !general_lower_bit_rate_constraint_flag) { |  |
| general_br_factor_extension_flag | u(1) |
| general_reserved_zero_33bits | u(33) |
| else { |  |
| general_reserved_zero_34bits | u(34) |
| } |  |
| } |  |
| if( general_profile_idc = = 5 ¦ ¦ general_profile_compatibility_flag[ 5 ] ¦ ¦ general_profile_idc = = 9 ¦ ¦ general_profile_compatibility_flag[ 9 ] ¦ ¦ general_profile_idc = = 10 ¦ ¦ general_profile_compatibility_flag[ 10 ] ¦ ¦ general_profile_idc = = 11 ¦ ¦ general_profile_compatibility_flag[ 11 ] ) { |  |

| sub_layer_lower_bit_rate_constraint_flag[ i ] | u(1) |
| --- | --- |
| if( general_tier_flag && general_intra_constraint_flag && !sub_layer_max_420_chroma_constraint_flag && !sub_layer_lower_bit_rate_constraint_flag) { |  |
| sub_layer_br_factor_extension_flag | u(1) |
| sub_layer_reserved_zero_33bits | u(33) |
| else { |  |
| sub_layer_reserved_zero_34bits | u(34) |
| } |  |
| } |  |

or

| general_lower_bit_rate_constraint_flag | u(1) |
| --- | --- |
| if( general_profile_idc = = 4 ¦ ¦ general_profile_compatibility_flag[ 4 ]) { |  |
| if( general_tier_flag && general_intra_constraint_flag && !general_lower_bit_rate_constraint_flag && (chroma_format_idc == 2 ¦¦ chroma_format_idc ==3)) { |  |
| general_br_factor_extension_flag | u(1) |
| general_reserved_zero_33bits | u(33) |
| else { |  |
| general_reserved_zero_34bits | u(34) |
| } |  |
| } |  |
| if( general_profile_idc = = 5 ¦ ¦ general_profile_compatibility_flag[ 5 ] ¦ ¦ general_profile_idc = = 9 ¦ ¦ general_profile_compatibility_flag[ 9 ] ¦ ¦ general_profile_idc = = 10 ¦ ¦ general_profile_compatibility_flag[ 10 ] ¦ ¦ general_profile_idc = = 11 ¦ ¦ general_profile_compatibility_flag[ 11 ] ) { |  |

| sub_layer_lower_bit_rate_constraint_flag[ i ] | u(1) |
| --- | --- |
| if( general_tier_flag && sub_layer_intra_constraint_flag && !general_lower_bit_rate_constraint_flag && (chroma_format_idc == 2 ¦¦ chroma_format_idc ==3)) { |  |
| sub_layer_br_factor_extension_flag | u(1) |
| sub_layer_reserved_zero_33bits | u(33) |
| else { |  |
| sub_layer_reserved_zero_34bits | u(34) |
| } |  |
| } |  |

2) Modify the profile tier level semantics as follows

Add the following yellow highlighted part:

- 7.4.4   Profile, tier and level semantics

general_max_12bit_‌constraint_flag, general_max_10bit_‌constraint_flag, general_max_8bit_‌constraint_flag, general_max_422chroma_‌constraint_flag, general_max_420chroma_‌constraint_flag, general_max_‌monochrome_‌constraint_flag, general_intra_‌constraint_flag, general_one_picture_only_‌constraint_flag general_lower_‌bit_rate_‌constraint_flag, general_max_14bit_‌constraint_flag and general_br_factor_extension_flag  when present, have semantics specified in Annex ‎A when the profile indicated by general_profile_idc or general_profile_compatibility_flag[ j ] is a profile specified in Annex ‎A.

Each of the following syntax elements:

–	sub_layer_profile_space[ i ],

–	sub_layer_tier_flag[ i ],

–	sub_layer_profile_idc[ i ],

–	sub_layer_profile_compatibility_flag[ i ][ j ],

–	sub_layer_progressive_source_flag[ i ],

–	sub_layer_interlaced_source_flag[ i ],

–	sub_layer_non_packed_constraint_flag[ i ],

–	sub_layer_frame_only_constraint_flag[ i ],

–	sub_layer_max_12bit_constraint_flag[ i ],

–	sub_layer_max_10bit_constraint_flag[ i ],

–	sub_layer_max_8bit_constraint_flag[ i ],

–	sub_layer_max_422chroma_constraint_flag[ i ],

–	sub_layer_max_420chroma_constraint_flag[ i ],

–	sub_layer_max_monochrome_constraint_flag[ i ],

–	sub_layer_intra_constraint_flag[ i ],

–	sub_layer_one_picture_only_constraint_flag[ i ],

–	sub_layer_lower_bit_rate_constraint_flag[ i ],

–	sub_layer_br_factor_extension_flag[ i ],

–	sub_layer_max_14bit_constraint_flag[ i ],

–	sub_layer_reserved_zero_33bits[ i ],

–	sub_layer_reserved_zero_34bits[ i ],

–	sub_layer_reserved_zero_7bits[ i ],

–	sub_layer_reserved_zero_35bits[ i ],

–	sub_layer_reserved_zero_43bits[ i ],

–	sub_layer_inbld_flag[ i ],

–	sub_layer_reserved_zero_bit[ i ], and

–	sub_layer_level_idc[ i ],

is referred to as the i-th corresponding sub-layer syntax element of each of the following syntax elements:

–	general_profile_space,

–	general_tier_flag,

–	general_profile_idc,

–	general_profile_compatibility_flag[ j ],

–	general_progressive_source_flag,

–	general_interlaced_source_flag,

–	general_non_packed_constraint_flag,

–	general_frame_only_constraint_flag,

–	general_max_12bit_constraint_flag,

–	general_max_10bit_constraint_flag,

–	general_max_8bit_constraint_flag,

–	general_max_422chroma_constraint_flag,

–	general_max_420chroma_constraint_flag,

–	general_max_monochrome_constraint_flag,

–	general_intra_constraint_flag,

–	general_one_picture_only_constraint_flag,

–	general_lower_bit_rate_constraint_flag,

–	general_br_factor_extension_flag,

–	general_max_14bit_constraint_flag,

–	general_reserved_zero_33bits,

–	general_reserved_zero_34bits,

–	general_reserved_zero_7bits,

–	general_reserved_zero_35bits,

–	general_reserved_zero_43bits,

–	general_inbld_flag,

–	general_reserved_zero_bit, and

–	general_level_idc,

respectively.

3) Modify the general tier and level limits as follows

Add the following yellow highlighted part:

A.4.1   General tier and level limits

………

The variable CpbScaleFactor is defined as follows:

- If general_br_factor_extension_flag is equal to 1, CpbScaleFactor is set equal to 2.

- Otherwise, CpbScaleFactor is set equal to 1.

- For the VCL HRD parameters, CpbSize[ i ] shall be less than or equal to CpbVclFactor * CpbScaleFactor * MaxCPB for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where CpbSize[ i ] is specified in clause ‎E.3.3 based on parameters selected as specified in clause ‎C.1, CpbVclFactor is specified in Table A.10, and MaxCPB is specified in Table A.8 in units of CpbVclFactor bits.

- For the NAL HRD parameters, CpbSize[ i ] shall be less than or equal to CpbNalFactor * CpbScaleFactor * MaxCPB for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where CpbSize[ i ] is specified in clause ‎E.3.3 based on parameters selected as specified in clause ‎C.1, CpbNalFactor is specified in Table A.10, and MaxCPB is specified in Table A.8 in units of CpbNalFactor bits.

4) Modify the profile-specific level limits as follows

There could be two ways to mofify maximum bit rate (Variant A and B)

Add the following yellow highlighted part:

Variant A : Modify HbrFactor

A.4.2  Profile-specific level limits for the video profiles

…..

The variable HbrScaleFactor is defined as follows:

- If general_br_factor_extension_flag is equal to 1, HbrScaleFactor is set equal to 2.

- Otherwise, HbrScaleFactor is set equal to 1.

The variable HbrFactor is defined as follows:

- If the bitstream or sub-layer representation is indicated to conform to the Main profile or the Main 10 profile, HbrFactor is set equal to 1.

- Otherwise, if the bitstream or sub-layer representation is indicated to conform to the High Throughput 4:4:4, High Throughput 4:4:4 10, High Throughput 4:4:4 14, Screen-Extended High Throughput 4:4:4, Screen-Extended High Throughput 4:4:4 10, or Screen-Extended High Throughput 4:4:4 14  profile, HbrFactor is set equal to 6.

- Otherwise, if the bitstream or sub-layer representation is indicated to conform to the High Throughput 4:4:4 16 Intra profile, HbrFactor is set equal to 24 − ( 12 * general_lower_bit_rate_constraint_flag ) or 24 − ( 12 * sub_layer_lower_bit_rate_constraint_flag[ i ] ).

- Otherwise, HbrFactor is set equal to 2 − general_lower_bit_rate_constraint_flag or 2 − sub_layer_lower_bit_rate_constraint_flag[ i ].

- Then HbrFactor is set equal to HbrFactor * HbrScaleFactor.

Variant B: Modify BitRate

A.4.2  Profile-specific level limits for the video profiles

…..

The variable MaxbrScaleFactor is defined as follows:

- If general_br_factor_extension_flag is equal to 1, MaxbrScaleFactor is set equal to 2.

- Otherwise, MaxbrScaleFactor is set equal to 1.

e)  For the VCL HRD parameters, BitRate[ i ] shall be less than or equal to BrVclFactor * MaxBR * MaxbrScaleFactor for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where BitRate[ i ] is specified in clause ‎E.3.3 based on parameters selected as specified in clause ‎C.1 and MaxBR is specified in Table A.9 in units of BrVclFactor bits/s.

f) For the NAL HRD parameters, BitRate[ i ] shall be less than or equal to BrNalFactor * MaxBR * MaxbrScaleFactor for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where BitRate[ i ] is specified in clause ‎E.3.3 based on parameters selected as specified in clause ‎C.1 and MaxBR is specified in Table A.9 in units of BrNalFactor bits/s.

E.3.3 Sub-layer HRD parameters semantics

The variable CpbCnt is set equal to cpb_cnt_minus1[ subLayerId ] + 1.

bit_rate_value_minus1[ i ] (together with bit_rate_scale) specifies the maximum input bit rate for the i-th CPB when the CPB operates at the access unit level. bit_rate_value_minus1[ i ] shall be in the range of 0 to 232 − 2, inclusive. For any i > 0, bit_rate_value_minus1[ i ] shall be greater than bit_rate_value_minus1[ i − 1 ].

When SubPicHrdFlag is equal to 0, the bit rate in bits per second is given by:

BitRate[ i ] = ( bit_rate_value_minus1[ i ] + 1 ) * 2( 6 + bit_rate_scale )	(E-77)

When SubPicHrdFlag is equal to 0 and the bit_rate_value_minus1[ i ] syntax element is not present, the value of BitRate[ i ] is inferred to be equal to BrVclFactor * MaxBR * MaxbrScaleFactor for VCL HRD parameters and to be equal to BrNalFactor * MaxBR * MaxbrScaleFactor for NAL HRD parameters, where MaxBR, BrVclFactor and BrNalFactor are specified in clause ‎A.4.

Option 2 : Add new tier

1) Modify the syntax and semantics

Syntax and semantics changes are the same as Option 1 (for 7.4.3 and 7.4.4)

2) Modify the general tier and level limits as follows

Add the following yellow highlighted part:

A.4.1   General tier and level limits

………

If general_br_extension_flag is presnet, a tier to which a bitstream conforms are indicated by the syntax elements general_tier_flag and general_br_factor_extension_flag, and a tier to which a sub-layer representation conforms are indicated by the syntax elements sub_layer_tier_flag[ i ] and general_br_factor_extension_flag, as follows:

–	If the specified level is not level 8.5, general_br_factor_extension_flag equal to 1 indicates conformance to the Super high tier, according to the tier constraints specified in A.9. general_tier_flag and sub_layer_tier_flag[ i ] shall be equal to 1, if general_br_factor_extension_flag is equal to 1.

Table A.8 – General tier and level limits

| Level | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) |  |  | Max slice segments per picture MaxSliceSegmentsPerPicture | Max # of tile rows MaxTileRows | Max # of tile columns MaxTileCols |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Super high tier |  |  |  |
| 1 | 36 864 | 350 | - | - | 16 | 1 | 1 |
| 2 | 122 880 | 1 500 | - | - | 16 | 1 | 1 |
| 2.1 | 245 760 | 3 000 | - | - | 20 | 1 | 1 |
| 3 | 552 960 | 6 000 | - | - | 30 | 2 | 2 |
| 3.1 | 983 040 | 10 000 | - | - | 40 | 3 | 3 |
| 4 | 2 228 224 | 12 000 | 30 000 | - | 75 | 5 | 5 |
| 4.1 | 2 228 224 | 20 000 | 50 000 | - | 75 | 5 | 5 |
| 5 | 8 912 896 | 25 000 | 100 000 | - | 200 | 11 | 10 |
| 5.1 | 8 912 896 | 40 000 | 160 000 | - | 200 | 11 | 10 |
| 5.2 | 8 912 896 | 60 000 | 240 000 | - | 200 | 11 | 10 |
| 6 | 35 651 584 | 60 000 | 240 000 | 480 000 | 600 | 22 | 20 |
| 6.1 | 35 651 584 | 120 000 | 480 000 | 960 000 | 600 | 22 | 20 |
| 6.2 | 35 651 584 | 240 000 | 800 000 | 1 600 000 | 600 | 22 | 20 |

A.4.2  Profile-specific level limits for the video profiles

Table A.9 – Tier and level limits for the video profiles

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (BrVclFactor or BrNalFactor bits/s) |  |  | Min compression ratio MinCrBase |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Super high tier | Main tier | High tier | Super high tier |
| 1 | 552 960 | 128 | - | - | 2 | 2 | - |
| 2 | 3 686 400 | 1 500 | - | - | 2 | 2 | - |
| 2.1 | 7 372 800 | 3 000 | - | - | 2 | 2 | - |
| 3 | 16 588 800 | 6 000 | - | - | 2 | 2 | - |
| 3.1 | 33 177 600 | 10 000 | - | - | 2 | 2 | - |
| 4 | 66 846 720 | 12 000 | 30 000 | - | 4 | 4 | - |
| 4.1 | 133 693 440 | 20 000 | 50 000 | - | 4 | 4 | - |
| 5 | 267 386 880 | 25 000 | 100 000 | - | 6 | 4 | - |
| 5.1 | 534 773 760 | 40 000 | 160 000 | - | 8 | 4 | - |
| 5.2 | 1 069 547 520 | 60 000 | 240 000 | - | 8 | 4 | - |
| 6 | 1 069 547 520 | 60 000 | 240 000 | 480 000 | 8 | 4 | 4 |
| 6.1 | 2 139 095 040 | 120 000 | 480 000 | 960 000 | 8 | 4 | 4 |
| 6.2 | 4 278 190 080 | 240 000 | 800 000 | 1 600 000 | 6 | 4 | 4 |

Option 3: Add new levels

Variant A

1) Modify the profile-specific level limits as follows

Add the following yellow highlighted part:

A.4.1   General tier and level limits

………

Table A.8 – General tier and level limits

| Level | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) |  | Max slice segments per picture MaxSliceSegmentsPerPicture | Max # of tile rows MaxTileRows | Max # of tile columns MaxTileCols |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier |  |  |  |
| 1 | 36 864 | 350 | - | 16 | 1 | 1 |
| 2 | 122 880 | 1 500 | - | 16 | 1 | 1 |
| 2.1 | 245 760 | 3 000 | - | 20 | 1 | 1 |
| 3 | 552 960 | 6 000 | - | 30 | 2 | 2 |
| 3.1 | 983 040 | 10 000 | - | 40 | 3 | 3 |
| 4 | 2 228 224 | 12 000 | 30 000 | 75 | 5 | 5 |
| 4.1 | 2 228 224 | 20 000 | 50 000 | 75 | 5 | 5 |
| 5 | 8 912 896 | 25 000 | 100 000 | 200 | 11 | 10 |
| 5.1 | 8 912 896 | 40 000 | 160 000 | 200 | 11 | 10 |
| 5.2 | 8 912 896 | 60 000 | 240 000 | 200 | 11 | 10 |
| 6 | 35 651 584 | 60 000 | 240 000 | 600 | 22 | 20 |
| 6.1 | 35 651 584 | 120 000 | 480 000 | 600 | 22 | 20 |
| 6.2 | 35 651 584 | 240 000 | 800 000 | 600 | 22 | 20 |
| 6.3 | 80 216 064 | 240 000 | 800 000 | 600 | 22 | 20 |
| 6.4 | 80 216 064 | 240 000 | 1 600 000 | 600 | 22 | 20 |

A.4.2  Profile-specific level limits for the video profiles

Table A.9 – Tier and level limits for the video profiles

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (BrVclFactor or BrNalFactor bits/s) |  | Min compression ratio MinCrBase |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Main tier | High tier |
| 1 | 552 960 | 128 | - | 2 | 2 |
| 2 | 3 686 400 | 1 500 | - | 2 | 2 |
| 2.1 | 7 372 800 | 3 000 | - | 2 | 2 |
| 3 | 16 588 800 | 6 000 | - | 2 | 2 |
| 3.1 | 33 177 600 | 10 000 | - | 2 | 2 |
| 4 | 66 846 720 | 12 000 | 30 000 | 4 | 4 |
| 4.1 | 133 693 440 | 20 000 | 50 000 | 4 | 4 |
| 5 | 267 386 880 | 25 000 | 100 000 | 6 | 4 |
| 5.1 | 534 773 760 | 40 000 | 160 000 | 8 | 4 |
| 5.2 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6.1 | 2 139 095 040 | 120 000 | 480 000 | 8 | 4 |
| 6.2 | 4 278 190 080 | 240 000 | 800 000 | 6 | 4 |
| 6.3 | 4 812 963 840 | 320 000 | 800 000 | 6 | 4 |
| 6.4 | 4 812 963 840 | 320 000 | 1 600 000 | 6 | 4 |

Variant B

1) Modify the profile-specific level limits as follows

Add the following yellow highlighted part:

A.4.1   General tier and level limits

………

Table A.8 – General tier and level limits

| Level | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) |  | Max slice segments per picture MaxSliceSegmentsPerPicture | Max # of tile rows MaxTileRows | Max # of tile columns MaxTileCols |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier |  |  |  |
| 1 | 36 864 | 350 | - | 16 | 1 | 1 |
| 2 | 122 880 | 1 500 | - | 16 | 1 | 1 |
| 2.1 | 245 760 | 3 000 | - | 20 | 1 | 1 |
| 3 | 552 960 | 6 000 | - | 30 | 2 | 2 |
| 3.1 | 983 040 | 10 000 | - | 40 | 3 | 3 |
| 4 | 2 228 224 | 12 000 | 30 000 | 75 | 5 | 5 |
| 4.1 | 2 228 224 | 20 000 | 50 000 | 75 | 5 | 5 |
| 5 | 8 912 896 | 25 000 | 100 000 | 200 | 11 | 10 |
| 5.1 | 8 912 896 | 40 000 | 160 000 | 200 | 11 | 10 |
| 5.2 | 8 912 896 | 60 000 | 240 000 | 200 | 11 | 10 |
| 6 | 35 651 584 | 60 000 | 240 000 | 600 | 22 | 20 |
| 6.1 | 35 651 584 | 120 000 | 480 000 | 600 | 22 | 20 |
| 6.2 | 35 651 584 | 240 000 | 800 000 | 600 | 22 | 20 |
| 6.3 | 80 216 064 | 240 000 | 1 600 000 | 600 | 22 | 20 |

A.4.2  Profile-specific level limits for the video profiles

Table A.9 – Tier and level limits for the video profiles

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (BrVclFactor or BrNalFactor bits/s) |  | Min compression ratio MinCrBase |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Main tier | High tier |
| 1 | 552 960 | 128 | - | 2 | 2 |
| 2 | 3 686 400 | 1 500 | - | 2 | 2 |
| 2.1 | 7 372 800 | 3 000 | - | 2 | 2 |
| 3 | 16 588 800 | 6 000 | - | 2 | 2 |
| 3.1 | 33 177 600 | 10 000 | - | 2 | 2 |
| 4 | 66 846 720 | 12 000 | 30 000 | 4 | 4 |
| 4.1 | 133 693 440 | 20 000 | 50 000 | 4 | 4 |
| 5 | 267 386 880 | 25 000 | 100 000 | 6 | 4 |
| 5.1 | 534 773 760 | 40 000 | 160 000 | 8 | 4 |
| 5.2 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6.1 | 2 139 095 040 | 120 000 | 480 000 | 8 | 4 |
| 6.2 | 4 278 190 080 | 240 000 | 800 000 | 6 | 4 |
| 6.3 | 4 812 963 840 | 320 000 | 1 600 000 | 6 | 4 |

Annex B: Proposed text change for VVC

Option 1: Add scaling factor for the calculation of maximum bit rate

1) Modify the general constraints information syntax as follows

Add the following yellow highlighted part:

- 7.3.3.2  General constraints information syntax

| gci_no_virtual_boundaries_constraint_flag | u(1) |
| --- | --- |
| gci_num_reserved_bits | u(8) |
| gci_num_additional_bits | u(8) |
| if( gci_num_additional_bits > 0 ) { |  |
| general_lower_bit_rate_constraint_flag | u(1) |
| if( general_tier_flag && gci_intra_only_constraint_flag && !general_lower_bit_rate_constraint_flag && ( general_profile_idc == 42 ¦¦ general_profile_idc == 44 ) { |  |
| general_br_factor_extension_flag | u(1) |
| numAdditionalBitsUsed = 2 |  |
| } else |  |
| numAdditionalBitsUsed = 1 |  |
| } else |  |
| numAdditionalBitsUsed = 0 |  |
| for( i = 0; i < gci_num_additional_bits − numAdditionalBitsUsed; i++ ) |  |
| gci_reserved_zero_bit[ i ] | u(1) |
| } |  |

2) Modify the general constraints information semantics as follows

Add the following yellow highlighted part:

- 7.4.4.2  General constraints information semantics

…..

general_lower_bit_rate_constraint_flag equal to 1 specifies that a lower maximum bit rate constraint may be imposed in a profile, tier and level specification in Annex A than would be imposed when general_lower_bit_rate_constraint_flag is equal to 0. general_lower_bit_rate_constraint_flag equal to 0 specifies that such a corresponding lower maximum bit rate constraint is not imposed in Annex A. [Ed. (MZ/YK/TI): During the editing of the DIS text, it was suggested that signalling this flag as an SPS extension flag instead of a GCI flag could be cleaner in terms of GCI mainly carrying tools on/off constraints and potential GCI skipping. On the other hand, the GCI flags are signalled in early position in SPS similar to tier information. Further study of this question is encouraged.]

general_br_factor_extension_flag equal to 1 specifies that higher maximum bit rate constraint may be imposed in a profile, tier and level specification in Annex A than would be imposed when general_br_factor_extension_flag is equal to 0. general_br_factor_extension_flag equal to 0 specifies that such a corresponding lower maximum bit rate constraint is not imposed in Annex A. if general_br_factor_extension_flag is not present in a bitstream, its value is inferred to be equal to 0.

3) Modify the general tier and level limits as follows

Add the following yellow highlighted part:

A.4.1  General tier and level limits

…

The variable CpbScaleFactor is defined as follows:

- If general_br_factor_extension_flag is equal to 1, CpbScaleFactor is set equal to 2.

- Otherwise, CpbScaleFactor is set equal to 1.

f) For the VCL HRD parameters, CpbSize[ Htid ][ i ] shall be less than or equal to CpbVclFactor * CpbScaleFactor * MaxCPB for at least one value of i in the range of 0 to hrd_cpb_cnt_minus1, inclusive, where CpbSize[ Htid ][ i ] is specified in clause 7.4.6.3 based on parameters selected as specified in clause C.1, CpbVclFactor is specified in Table A.3 and MaxCPB is specified in Table A.1 in units of CpbVclFactor bits.

- For the NAL HRD parameters, CpbSize[ Htid ][ i ] shall be less than or equal to CpbNalFactor * CpbScaleFactor * MaxCPB for at least one value of i in the range of 0 to hrd_cpb_cnt_minus1, inclusive, where CpbSize[ Htid ][ i ] is specified in clause 7.4.6.3 based on parameters selected as specified in clause C.1, CpbNalFactor is specified in Table A.3, and MaxCPB is specified in Table A.1 in units of CpbNalFactor bits.

4) Modify the profile-specific level limits as follows

There could be two ways to mofify maximum bit rate (Variant A and B)

Add the following yellow highlighted part:

Variant A : Modify HbrFactor

A.4.2  Profile-specific level limits

…

The variable HbrScaleFactor is defined as follows:

- If general_br_factor_extension_flag is equal to 1, HbrScaleFactor is set equal to 2.

- Otherwise, HbrScaleFactor is set equal to 1.

The variable HbrFactor is defined as follows:

- If the bitstream is indicated to conform to the Main 10, Main 10 4:4:4, Multilayer Main 10, or Multilayer Main 10 4:4:4 profile, HbrFactor is set equal to 1.

- Otherwise, HbrFactor is set equal to 21 −+ general_nonlower_bit_rate_constraint_flaggeneral_lower_bit_rate_constraint_flag.

The variable BrVclFactor, which represents the VCL bit rate scale factor, is set equal to CpbVclFactor * HbrFactor * HbrScaleFactor.

The variable BrNalFactor, which represents the NAL bit rate scale factor, is set equal to CpbNalFactor * HbrFactor * HbrScaleFactor.

The variable MinCr is set equal to MinCrBase * MinCrScaleFactor ÷ HbrFactor.

Variant B: Modify BitRate

A.4.2  Profile-specific level limits

…

The variable MaxbrScaleFactor is defined as follows:

- If general_br_factor_extension_flag is equal to 1, MaxbrScaleFactor is set equal to 2.

- Otherwise, MaxbrScaleFactor is set equal to 1.

……..

e) For the VCL HRD parameters, BitRate[ Htid ][ i ] shall be less than or equal to BrVclFactor * MaxbrScaleFactor * MaxBR for at least one value of i in the range of 0 to hrd_cpb_cnt_minus1, inclusive, where BitRate[ Htid ][ i ] is specified in clause 7.4.6.3 based on parameters selected as specified in clause C.1 and MaxBR is specified in Table A.2 in units of BrVclFactor bits/s.

- For the NAL HRD parameters, BitRate[ Htid ][ i ] shall be less than or equal to BrNalFactor * MaxbrScaleFactor * MaxBR for at least one value of i in the range of 0 to hrd_cpb_cnt_minus1, inclusive, where BitRate[ Htid ][ i ] is specified in clause 7.4.6.3 based on parameters selected as specified in clause C.1 and MaxBR is specified in Table A.2 in units of BrNalFactor bits/s.

Option 2 : Add new tier

1) Modify the syntax and semantics

Syntax and semantics changes are the same as Option 1 (for 7.3.32 and 7.4.4.2)

2) Modify the general tier and level limits as follows

Add the following yellow highlighted part:

A.4.1   General tier and level limits

………

A tier and level to which a bitstream conforms are indicated by the syntax elements general_tier_flag and general_level_idc, and a level to which a sublayer representation conforms are indicated by the syntax element sublayer_level_idc[ i ], as follows:

–	If the specified level is not level 15.5, general_tier_flag equal to 0 indicates conformance to the Main tier, general_tier_flag equal to 1 indicates conformance to the High tier, according to the tier constraints specified in Table 135 and general_tier_flag shall be equal to 0 for levels below level 4 (corresponding to the entries in Table 135 marked with "-"). Otherwise (the specified level is level 15.5), it is a requirement of bitstream conformance that general_tier_flag shall be equal to 1 and the value 0 for general_tier_flag is reserved for future use by ITU-T | ISO/IEC and decoders shall ignore the value of general_tier_flag.

–	general_level_idc and sublayer_level_idc[ i ] shall be set equal to a value of general_level_idc for the level number specified in Table 135.

If general_br_factor_extension_flag is present, a tier to which a bitstream conforms are indicated by the syntax elements general_tier_flag and general_br_extension_flag as follows:

–	If the specified level is not level 15.5 and general_br_factor_extension_flag is equal to 1, general_br_factor_extension_falg equal to 1 indicates conformance to the Super High tier, according to the tier constraints specified in Table 135 and general_tier_flag shall be equal to 1.

Table A.1 – General tier and level limits

| Level | general_level_idc value* |  | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) |  |  | Max slices per AU MaxSlicesPerAu | Max # of tiles MaxTilesPerAu | Max # of tile columns MaxTileCols |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Main tier | High tier | Super high tier |  |  |  |
| 1.0 | 16 |  | 36 864 | 350 | - | - | 16 | 1 | 1 |
| 2.0 | 32 |  | 122 880 | 1 500 | - | - | 16 | 1 | 1 |
| 2.1 | 35 |  | 245 760 | 3 000 | - | - | 20 | 1 | 1 |
| 3.0 | 48 |  | 552 960 | 6 000 | - | - | 30 | 4 | 2 |
| 3.1 | 51 |  | 983 040 | 10 000 | - | - | 40 | 9 | 3 |
| 4.0 | 64 |  | 2 228 224 | 12 000 | 30 000 | - | 75 | 25 | 5 |
| 4.1 | 67 |  | 2 228 224 | 20 000 | 50 000 | - | 75 | 25 | 5 |
| 5.0 | 80 |  | 8 912 896 | 25 000 | 100 000 | - | 200 | 110 | 10 |
| 5.1 | 83 |  | 8 912 896 | 40 000 | 160 000 | - | 200 | 110 | 10 |
| 5.2 | 86 |  | 8 912 896 | 60 000 | 240 000 | - | 200 | 110 | 10 |
| 6.0 | 96 |  | 35 651 584 | 80 000 | 240 000 | 480 000 | 600 | 440 | 20 |
| 6.1 | 99 |  | 35 651 584 | 120 000 | 480 000 | 960 000 | 600 | 440 | 20 |
| 6.2 | 102 |  | 35 651 584 | 180 000 | 800 000 | 1 600 000 | 600 | 440 | 20 |
|  |  | * The level numbers in this table are in the form of "majorNum.minorNum", and the value of general_level_idc for each of the levels is equal to majorNum * 16 + minorNum * 3. |  |  |  |  |  |  |  |

A.4.2  Profile-specific level limits

Table A.2 – Tier and level limits for the video profiles

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (BrVclFactor or BrNalFactor bits/s) |  |  | Min compression ratio MinCrBase |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Super high tier | Main tier | High tier | Super high tier |
| 1.0 | 552 960 | 128 | - | - | 2 | 2 | - |
| 2.0 | 3 686 400 | 1 500 | - | - | 2 | 2 | - |
| 2.1 | 7 372 800 | 3 000 | - | - | 2 | 2 | - |
| 3.0 | 16 588 800 | 6 000 | - | - | 2 | 2 | - |
| 3.1 | 33 177 600 | 10 000 | - | - | 2 | 2 | - |
| 4.0 | 66 846 720 | 12 000 | 30 000 | - | 4 | 4 | - |
| 4.1 | 133 693 440 | 20 000 | 50 000 | - | 4 | 4 | - |
| 5.0 | 267 386 880 | 25 000 | 100 000 | - | 6 | 4 | - |
| 5.1 | 534 773 760 | 40 000 | 160 000 | - | 8 | 4 | - |
| 5.2 | 1 069 547 520 | 60 000 | 240 000 | - | 8 | 4 | - |
| 6.0 | 1 069 547 520 | 60 000 | 240 000 | 480 000 | 8 | 4 | 4 |
| 6.1 | 2 139 095 040 | 120 000 | 480 000 | 960 000 | 8 | 4 | 4 |
| 6.2 | 4 278 190 080 | 240 000 | 800 000 | 1 600 000 | 8 | 4 | 4 |

Option 3: Add new levels

Variant A

1) Modify the profile-specific level limits as follows

Add the following yellow highlighted part:

- A.4.1	General tier and level limits

...

Table 135 – General tier and level limits

| Level | general_level_idc value* | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) |  | Max slices per AU MaxSlicesPerAu | Max # of tiles MaxTilesPerAu | Max # of tile columns MaxTileCols |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Main tier | High tier |  |  |  |
| 1.0 | 16 | 36 864 | 350 | - | 16 | 1 | 1 |
| 2.0 | 32 | 122 880 | 1 500 | - | 16 | 1 | 1 |
| 2.1 | 35 | 245 760 | 3 000 | - | 20 | 1 | 1 |
| 3.0 | 48 | 552 960 | 6 000 | - | 30 | 4 | 2 |
| 3.1 | 51 | 983 040 | 10 000 | - | 40 | 9 | 3 |
| 4.0 | 64 | 2 228 224 | 12 000 | 30 000 | 75 | 25 | 5 |
| 4.1 | 67 | 2 228 224 | 20 000 | 50 000 | 75 | 25 | 5 |
| 5.0 | 80 | 8 912 896 | 25 000 | 100 000 | 200 | 110 | 10 |
| 5.1 | 83 | 8 912 896 | 40 000 | 160 000 | 200 | 110 | 10 |
| 5.2 | 86 | 8 912 896 | 60 000 | 240 000 | 200 | 110 | 10 |
| 6.0 | 96 | 35 651 584 | 80 000 | 240 000 | 600 | 440 | 20 |
| 6.1 | 99 | 35 651 584 | 120 000 | 480 000 | 600 | 440 | 20 |
| 6.2 | 102 | 35 651 584 | 180 000 | 800 000 | 600 | 440 | 20 |
| 6.3 | 105 | 80 216 064 | 240 000 | 800 000 | 1 000 | 990 | 30 |
| 6.4 | 108 | 80 216 064 | 240 000 | 1 600 000 | 1 000 | 990 | 30 |
| * The level numbers in this table are in the form of "majorNum.minorNum", and the value of general_level_idc for each of the levels is equal to majorNum * 16 + minorNum * 3. |  |  |  |  |  |  |  |

- A.4.2	Profile-specific level limits

...

Table 136 – Tier and level limits for the video profiles

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (BrVclFactor or BrNalFactor bits/s) |  | Min compression ratio MinCrBase |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Main tier | High tier |
| 1.0 | 552 960 | 128 | - | 2 | 2 |
| 2.0 | 3 686 400 | 1 500 | - | 2 | 2 |
| 2.1 | 7 372 800 | 3 000 | - | 2 | 2 |
| 3.0 | 16 588 800 | 6 000 | - | 2 | 2 |
| 3.1 | 33 177 600 | 10 000 | - | 2 | 2 |
| 4.0 | 66 846 720 | 12 000 | 30 000 | 4 | 4 |
| 4.1 | 133 693 440 | 20 000 | 50 000 | 4 | 4 |
| 5.0 | 267 386 880 | 25 000 | 100 000 | 6 | 4 |
| 5.1 | 534 773 760 | 40 000 | 160 000 | 8 | 4 |
| 5.2 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6.0 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6.1 | 2 139 095 040 | 120 000 | 480 000 | 8 | 4 |
| 6.2 | 4 278 190 080 | 240 000 | 800 000 | 8 | 4 |
| 6.3 | 4 812 963 840 | 320 000 | 800 000 | 8 | 4 |
| 6.4 | 4 812 963 840 | 320 000 | 1 600 000 | 8 | 4 |

Variant B

1) Modify the profile-specific level limits as follows

Add the following yellow highlighted part:

- A.4.1	General tier and level limits

...

Table 135 – General tier and level limits

| Level | general_level_idc value* | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) |  | Max slices per AU MaxSlicesPerAu | Max # of tiles MaxTilesPerAu | Max # of tile columns MaxTileCols |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Main tier | High tier |  |  |  |
| 1.0 | 16 | 36 864 | 350 | - | 16 | 1 | 1 |
| 2.0 | 32 | 122 880 | 1 500 | - | 16 | 1 | 1 |
| 2.1 | 35 | 245 760 | 3 000 | - | 20 | 1 | 1 |
| 3.0 | 48 | 552 960 | 6 000 | - | 30 | 4 | 2 |
| 3.1 | 51 | 983 040 | 10 000 | - | 40 | 9 | 3 |
| 4.0 | 64 | 2 228 224 | 12 000 | 30 000 | 75 | 25 | 5 |
| 4.1 | 67 | 2 228 224 | 20 000 | 50 000 | 75 | 25 | 5 |
| 5.0 | 80 | 8 912 896 | 25 000 | 100 000 | 200 | 110 | 10 |
| 5.1 | 83 | 8 912 896 | 40 000 | 160 000 | 200 | 110 | 10 |
| 5.2 | 86 | 8 912 896 | 60 000 | 240 000 | 200 | 110 | 10 |
| 6.0 | 96 | 35 651 584 | 80 000 | 240 000 | 600 | 440 | 20 |
| 6.1 | 99 | 35 651 584 | 120 000 | 480 000 | 600 | 440 | 20 |
| 6.2 | 102 | 35 651 584 | 180 000 | 800 000 | 600 | 440 | 20 |
| 6.3 | 105 | 80 216 064 | 240 000 | 1 600 000 | 1 000 | 990 | 30 |
| * The level numbers in this table are in the form of "majorNum.minorNum", and the value of general_level_idc for each of the levels is equal to majorNum * 16 + minorNum * 3. |  |  |  |  |  |  |  |

- A.4.2	Profile-specific level limits

...

Table 136 – Tier and level limits for the video profiles

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (BrVclFactor or BrNalFactor bits/s) |  | Min compression ratio MinCrBase |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Main tier | High tier |
| 1.0 | 552 960 | 128 | - | 2 | 2 |
| 2.0 | 3 686 400 | 1 500 | - | 2 | 2 |
| 2.1 | 7 372 800 | 3 000 | - | 2 | 2 |
| 3.0 | 16 588 800 | 6 000 | - | 2 | 2 |
| 3.1 | 33 177 600 | 10 000 | - | 2 | 2 |
| 4.0 | 66 846 720 | 12 000 | 30 000 | 4 | 4 |
| 4.1 | 133 693 440 | 20 000 | 50 000 | 4 | 4 |
| 5.0 | 267 386 880 | 25 000 | 100 000 | 6 | 4 |
| 5.1 | 534 773 760 | 40 000 | 160 000 | 8 | 4 |
| 5.2 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6.0 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6.1 | 2 139 095 040 | 120 000 | 480 000 | 8 | 4 |
| 6.2 | 4 278 190 080 | 240 000 | 800 000 | 8 | 4 |
| 6.3 | 4 812 963 840 | 320 000 | 1 600 000 | 8 | 4 |