| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0083-v1 |
| --- | --- |

| Title: | EE2: Bilateral and template matching AMVP-merge mode (test 3.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi Zhang Han Huang Chun-Chi Chen Vadim Seregin Marta Karczewicz | Tel: Email: | zhizhang@qti.qualcomm.com hanhuang@qti.qualcomm.com chunchic@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports results for EE2-3.3 AMVP-merge mode, where AMVP predictor in one direction is combined with merge predictor in another direction to form a bi-directional predictor. Merge candidate is selected by using either template or bilateral matching cost. The proposed method was implemented on top of ECM-2.0, and it reports the simulation results compared to ECM-2.0 for random access configuration as follows:

EE2-3.3c (TM AMVP-merge mode):

-0.18% (Y), -0.12 (U), -0.09% (V), 102% (EncT), 95% (DecT)

EE2-3.3a (BM AMVP-merge mode):

-0.13% (Y), -0.09 (U), -0.10% (V), 101% (EncT), 95% (DecT)

EE2-3.3b (BM AMVP-merge mode with TM disabled in the anchor and the test):

-0.52% (Y), -0.49 (U), -0.52% (V), 105% (EncT), 98% (DecT)

# Introduction

In ECM-2.0, there are AMVP and merge modes. In JVET-W0106, it was proposed to combine uni-directional AMVP predictor and uni-directional merge predictor to form a bi-directional predictor.

To select merge predictor from the merge candidate list, either bilateral or template matching cost functions is used and both approaches are tested in EE.

# Proposed methods

In the proposed AMVP-merge mode, the bi-directional predictor is composed of an AMVP predictor in one direction and a merge predictor in the other direction.

AMVP part of the proposed mode is signaled as a regular uni-directional AMVP, i.e. reference index and MVD are signaled, and it has a derived MVP index if template matching is used (TM_AMVP) or MVP index is signaled when template matching is disabled. Merge index is not signalled, and merge predictor is selected from the candidate list with smallest template or bilateral matching cost.

When the selected merge predictor and the AMVP predictor satisfy DMVR condition, which is there is at least one reference picture from the past and one reference picture from the future relatively to the current picture and the distances from two reference pictures to the current picture are the same, the bilateral matching MV refinement is applied for the merge MV candidate and AMVP MVP as a starting point. Otherwise, if template matching functionality is enabled, template matching MV refinement is applied to the merge predictor or the AMVP predictor which has a higher template matching cost.

The third pass which is 8x8 sub-PU BDOF refinement of the multi-pass DMVR is enabled to AMVP-merge mode coded block.

# Simulation results

The proposed method was implemented on top of ECM-2.0. The performance was evaluated using the common test conditions as defined in [2]. The test includes a software optimization for bilateral matching DMVR. For each EE2-3.3 test, we report the test result by using ECM-2.0 as anchor and ECM-2.0 with software optimization (ECM-2.0 + SW OPT) as anchor.

EE2-3.3c: Template matching AMVP-merge mode

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EE2-3.3c Over ECM2.0 |  |  |  |  | EE2-3.3c Over ECM2.0 + SW OPT |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.06% | -0.21% | 103% | 97% | -0.11% | -0.06% | -0.21% | 104% | 100% |
| Class A2 | -0.18% | -0.11% | -0.09% | 103% | 95% | -0.18% | -0.11% | -0.09% | 105% | 100% |
| Class B | -0.19% | -0.14% | -0.04% | 101% | 95% | -0.19% | -0.14% | -0.04% | 103% | 99% |
| Class C | -0.22% | -0.14% | -0.06% | 104% | 95% | -0.22% | -0.14% | -0.06% | 104% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.18% | -0.12% | -0.09% | 102% | 95% | -0.18% | -0.12% | -0.09% | 104% | 100% |
| Class D | -0.21% | -0.35% | 0.12% | 104% | 95% | -0.21% | -0.35% | 0.12% | 106% | 100% |
| Class F (optional) | -0.12% | -0.11% | -0.24% | 102% | 98% | -0.12% | -0.11% | -0.24% | 103% | 100% |

EE2-3.3a: Bilateral matching AMVP-merge mode

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EE2-3.3a Over ECM2.0 |  |  |  |  | EE2-3.3a Over ECM2.0 + SW OPT |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | 0.02% | 0.00% | 102% | 97% | -0.08% | 0.02% | 0.00% | 104% | 100% |
| Class A2 | -0.12% | -0.02% | -0.07% | 102% | 95% | -0.12% | -0.02% | -0.07% | 104% | 100% |
| Class B | -0.13% | -0.17% | -0.13% | 100% | 95% | -0.13% | -0.17% | -0.13% | 102% | 99% |
| Class C | -0.16% | -0.12% | -0.15% | 103% | 95% | -0.16% | -0.12% | -0.15% | 103% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.13% | -0.09% | -0.10% | 101% | 95% | -0.13% | -0.09% | -0.10% | 103% | 100% |
| Class D | -0.16% | -0.08% | -0.01% | 103% | 94% | -0.16% | -0.08% | -0.01% | 104% | 100% |
| Class F (optional) | -0.09% | -0.03% | -0.13% | 101% | 98% | -0.09% | -0.03% | -0.13% | 102% | 100% |

EE2-3.3b: Bilateral matching AMVP-merge mode with template matching disabled in the anchor and the test

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EE2-3.3b Over ECM2.0-TM-OFF |  |  |  |  | EE2-3.3b Over ECM2.0-TM-OFF + SW OPT |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.40% | -0.25% | -0.38% | 106% | 99% | -0.40% | -0.25% | -0.38% | 107% | 102% |
| Class A2 | -0.60% | -0.40% | -0.47% | 106% | 97% | -0.60% | -0.40% | -0.47% | 107% | 102% |
| Class B | -0.49% | -0.46% | -0.35% | 105% | 97% | -0.49% | -0.46% | -0.35% | 106% | 101% |
| Class C | -0.59% | -0.56% | -0.59% | 106% | 98% | -0.59% | -0.56% | -0.59% | 107% | 103% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.52% | -0.43% | -0.44% | 105% | 98% | -0.52% | -0.43% | -0.44% | 107% | 102% |
| Class D | -0.72% | -0.49% | -0.52% | 105% | 96% | -0.72% | -0.49% | -0.52% | 106% | 102% |
| Class F (optional) | -0.22% | -0.13% | -0.21% | 104% | 99% | -0.22% | -0.13% | -0.21% | 104% | 101% |

# Conclusion

In this contribution, template matching AMVP-merge mode and bilateral matching AMVP-merge mode are proposed. When template matching is enabled, the trade-off between coding performance and complexity of the template matching AMVP-merge mode is better than bilateral matching AMVP-merge mode. It is suggested to adopt the proposed template-matching AMVP-merge mode into ECM.

When template matching is disabled, the gain of the proposed bilateral matching AMVP-merge mode is higher. It is suggested to adopt the proposed bilateral-matching AMVP-merge mode into ECM for template matching disabled experiments.

# References

[1] ECM-2.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-W2017, July. 2021.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).