| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0087-v1 |
| --- | --- |

| Title: | Non-EE2: Template Matching Based Merge Candidate List Construction (TM-MCLC) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Lei Zhao1, Kai Zhang2, Na Zhang1, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhaolei.0324@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

Abstract

In this contribution, a method of template matching based merge candidate list construction (TM-MCLC) is proposed. Instead of constructing the merge candidate list based on a predefined traversing order, TM-MCLC builds the merge candidate list by selecting more efficient candidates based on template matching costs. On top of ECM-2.0, simulation results of the proposed method are reported as below:

Tool-off test (ECM-2.0 as the anchor): RA: -0.xx%, 1xx%, 1xx%; LB: -0.xx%, 1xx%, 1xx%.

- Introduction

The quality of the merge candidate list fundamentally affects the inter coding performance. The merge candidate list construction method in VVC builds a merge list with a constant candidate number by selecting the available candidates in a predefined order. Adaptive reordering of merge candidates (ARMC) [1] was proposed to reorder merge candidates by template matching (TM) within a candidate list, which is constructed before performing TM. In this proposal, a method of template matching based merge candidate list construction (TM-MCLC) is further proposed to take advantage of TM to facilitate a better merge candidate list construction.

- Proposed methods

In ECM, non-adjacent and HMVP candidates are put into the merge candidate list based on a predefined traversing order after adjacent candidates and TMVP candidates. With TM-MCLC, non-adjacent and HMVP candidates are still put into the merge candidate list after adjacent candidates and TMVP candidates, but in an ascending order of template matching costs.

Figure 1. Non-adjacent positions used in this proposal.

More specifically, all available non-adjacent MVP and HMVP are collected in a group. Then TM cost associated with each candidate in the group is derived in a similar way to ARMC. Subsequently, all the candidates in the group are sorted in an ascending order regarding the corresponding TM costs. Finally, non-adjacent and HMVP candidates are put into the merge candidate list an ascending order of template matching costs.

In this proposal, non-adjacent MVPs in ECM software is further extended with more spatial and non-adjacent temporal positions, as shown in Figure 1. Besides the 18 positions for non-adjacent spatial MVPs in ECM-2.0, additional 32 spatial positions and 12 non-adjacent temporal positions are introduced, where non-adjacent temporal MVP positions locate in the same reference frame as the adjacent TMVP.

To reduce the complexity, integer precision is used for TM merge mode while 2-tap bilinear filter is used to generate the reference template for regular merge mode.

- Experimental Results

The proposed method has been implemented on top of the ECM-2.0 [2]. Results under RA/LDB configurations are presented in Table 1. The simulations were performed in accordance with the common test conditions [3]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of the TC-MCLC (tool-off test).

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.29% | -0.24% | -0.26% | 101% | 106% |  |  |  |  |  |
| Class C | -0.25% | -0.22% | -0.03% | 101% | 109% | -0.49% | -0.41% | -0.31% | 102% | 115% |
| Class E |  |  |  |  |  | -0.05% | -0.64% | -0.14% | 101% | 103% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.18% | -0.30% | 0.13% | 101% | 109% | -0.26% | -1.05% | -0.90% | 100% | 116% |
| Class F | -0.21% | -0.18% | -0.32% | 101% | 104% |  |  |  |  |  |

- Conclusions

TM-MCLC is presented in this proposal. Simulation results indicates 0.xx%/0.xx% coding gain in RA/LDB configurations with xxx%/xxx% and xxx%/xxx% complexity increase. It is recommended to adopt TM-MCLC in ECM.

- References

[1] N. Zhang, K. Zhang, L. Zhang, H. Liu, Z. Deng, Y. Wang, “AHG12: Adaptive Reordering of Merge

Candidates with Template Matching,” JVET-V0099, Apr. 2021.

[2] ECM-2.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-2.0.

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “VTM common test conditions and software reference configurations for SDR video,” JVET-T2010, Oct. 2020.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).