| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0090-v2 |
| --- | --- |

| Title: | Non-EE2: On combination of CIIP, OBMC and LMCS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao, Xinwei Li, Jie Chen, Yan Ye | Email: | ruling.lrl@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

In ECM-2.0, when CIIP, OBMC and LMCS are all applied to a CU, the samples generated by CIIP mode in the mapped domain are weighed with the samples generated by OBMC mode in the original domain. In this contribution, it is proposed to weigh the samples generated by CIIP and OBMC modes in the same domain. When performing weighting in the mapped domain, it is reported that on top of ECM-2.0, the overall coding performance impact for {Y, U, V, EncT, DecT} is {-0.01%, -0.02%, 0.01%, 100%, 100%} for RA and {-0.05%, 0.05%, -0.18%, 101%, 101%} for LDB. When performing weighting in the original domain, it is reported that on top of ECM-2.0, the overall coding performance impact for {Y, U, V, EncT, DecT} is {-0.01%, 0.09%, 0.05%, 100%, 100%} for RA and {-0.04%, 0.06%, -0.13%} for LDB. When performing OBMC before performing weighting with intra predicted samples, it is reported that on top of ECM-2.0, the overall coding performance impact for {Y, U, V, EncT, DecT} is {-0.01%, -0.02%, 0.02%, 100%, 100%} for RA and {-0.03%, -0.14%, -0.18%, 101%, 100%} for LDB.

Introduction

In ECM-2.0, when a CU is coded with CIIP, OBMC and LMCS enabled, the prediction samples of the CU are generated as follows:

PredY=128-w1×CIIPpredY+w1×OBMCpredY128

CIIPpredY=4-w0×FwdMapInterpredY+w0×IntrapredY4

, where InterpredY represents the samples predicted by the motion of current block in the original domain, IntrapredY represents the samples predicted using planar mode in the mapped domain, OBMCpredY represents the samples predicted by the motion of neighboring blocks in the original domain, and w0 and w1 are the weights. It is observed that CIIPpredY which is in the mapped domain is weighed with OBMCpredY which is in the original domain when CIIP, OBMC and LMCS are all applied to the CU.

Proposed methods

In this contribution, it is proposed to perform weighting of CIIPpredY and OBMCpredY in the same domain.

In the first method, it is proposed to perform weighting in the mapped domain. The forward mapping is performed to OBMCpredY before it’s weighed with CIIPpredY. The prediction samples are generated as follows:

PredY=128-w1×CIIPpredY+w1×FwdMap(OBMCpredY)128

In the second method, it is proposed to perform weighting in the original domain. The inverse mapping is performed to CIIPpredY before it’s weighed with OBMCpredY. The prediction samples are generated as follows:

PredY=FwdMap128-w1×InvMapCIIPpredY+w1×OBMCpredY128

In the third method, it is proposed to perform OBMC first, and then weigh with intra predicted samples.

InterpredY'=128-w1×InterpredY+w1×OBMCpredY128

PredY=4-w0×FwdMapInterpredY'+w0×IntrapredY4

Simulation results

The described methods are implemented on top of ECM-2.0 software and are tested under CTC for enhanced compression tool testing [1]. Table 1 and Table 2 show the results of performing weighting in the mapped domain and original domain, respectively. Table 3 shows the results of performing OBMC prior to weighting with intra predicted samples. It is observed that all three tests obtain minor coding gain with negligible impact on encoding and decoding time.

Table 1. Simulation results of performing weighting in the mapped domain

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.03% | -0.09% | 99% | 100% |
| Class A2 | 0.00% | -0.05% | 0.07% | 99% | 100% |
| Class B | -0.02% | -0.01% | 0.00% | 102% | 101% |
| Class C | 0.00% | -0.01% | 0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.02% | 0.01% | 100% | 100% |
| Class D | 0.03% | -0.03% | 0.00% | 99% | 100% |
| Class F | 0.00% | -0.04% | -0.01% | 101% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.17% | 0.00% | 101% | 101% |
| Class C | -0.09% | -0.39% | -0.08% | 99% | 100% |
| Class E | -0.03% | 0.46% | -0.60% | 101% | 102% |
| Overall | -0.05% | 0.05% | -0.18% | 101% | 101% |
| Class D | 0.12% | -0.55% | -0.56% | 100% | 99% |
| Class F | -0.02% | -0.05% | 0.03% | 101% | 101% |

Table 2. Simulation results of performing weighting in the original domain

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.18% | 0.08% | 99% | 100% |
| Class A2 | -0.01% | 0.06% | 0.08% | 99% | 100% |
| Class B | 0.00% | 0.05% | 0.00% | 100% | 100% |
| Class C | -0.02% | 0.08% | 0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.09% | 0.05% | 100% | 100% |
| Class D | -0.02% | -0.14% | 0.05% | 100% | 100% |
| Class F | 0.00% | -0.02% | -0.04% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 0.06% | 0.08% | 99% | 100% |
| Class C | -0.09% | 0.28% | 0.11% | 100% | 100% |
| Class E | -0.01% | -0.22% | -0.82% | 101% | 100% |
| Overall | -0.04% | 0.06% | -0.13% | 100% | 100% |
| Class D | 0.01% | -0.45% | -0.38% | 100% | 99% |
| Class F | -0.08% | -0.06% | -0.15% | 101% | 101% |

Table 3. Simulation results of performing OBMC before performing weighting with intra predicted samples

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.04% | 0.03% | 99% | 100% |
| Class A2 | -0.01% | -0.05% | 0.03% | 99% | 100% |
| Class B | -0.01% | -0.01% | -0.01% | 101% | 101% |
| Class C | -0.02% | -0.01% | 0.04% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.02% | 0.02% | 100% | 100% |
| Class D | 0.02% | -0.12% | 0.06% | 101% | 100% |
| Class F | 0.01% | 0.03% | 0.03% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.08% | 0.03% | 0.02% | 102% | 100% |
| Class C | -0.03% | -0.21% | 0.03% | 100% | 99% |
| Class E | 0.05% | -0.34% | -0.78% | 101% | 101% |
| Overall | -0.03% | -0.14% | -0.18% | 101% | 100% |
| Class D | 0.08% | -0.27% | 0.20% | 101% | 100% |
| Class F | -0.09% | -0.51% | -0.21% | 101% | 102% |

References

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-W2017, July 2021.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).