| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0091 |
| --- | --- |

| Title: | Non-EE2: On TMVP improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao, Jie Chen, Yan Ye, Xinwei Li | Email: | {ruling.lrl, jiechen.cj, yan.ye, sid.lxw}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution proposes to modify the derivation process of TMVP merge candidate. In the method 1, the reference picture of the TMVP merge candidate is selected from any one of reference pictures in the reference picture list instead of being fixed to the picture with index 0. The selected reference picture is the one whose scaling factor is the closet to 1. In the method 2, when constructing the merge candidate list of non-subblock merge modes, the TMVP is conditionally added to the list according to the motion information of neighboring spatial blocks. Moreover, after the list is constructed, the TMVP is refined using the template matching method. The template is used to decide the scaling factor and inter prediction direction of the TMVP. It is reported that on top of ECM-2.0, the overall coding performance impact for {Y, U, V, EncT, DecT} is:

Method 1:    RA{-0.04%, -0.01%, 0.00%, 100%, 101%}, LDB{-0.28%, -0.64%, -0.56%, 102%, 102%}

Method 1 + 2: RA{xxx%, xxx%, xxx%, xxx%, xxx%}, LDB{-0.34%, -0.44%, -0.50%, 102%, 103%}

Introduction

In the derivation of temporal merge candidate (TMVP), the temporal motion is scaled from the motion vector of collocated reference picture, which is explicitly signalled in slice header. The collocated block is selected from bottom-right position (C0) or center position (C1), as illustrated in Figure 1, according to predefined rules as follows:

- If position C0 is available, is inter-coded, and is in the same CTU row of current CU, the position C0 is used.

- Otherwise, if position C1 is inter-coded, the position C1 is used.

- Otherwise, TMVP is not available.

Figure 1. Position of spatial and temporal candidates

Besides, the temporal motion is derived according to an order for the MVs of different lists in the collocated blocks as follows:

- If the collocated block is bi-predicted and current picture is a low delay picture, the L0 and L1 MVs of the TMVP is scaled from the L0 and L1 MVs of the collocated block, respectively.

- Otherwise, if the collocated block is bi-predicted and current picture is a non-low delay picture,

  - if the collocated reference picture is from reference picture list 0, both the L0 and L1 MVs of TMVP is scaled from the L1 MV of the collocated block.

  - otherwise, both the L0 and L1 MVs of TMVP is scaled from the L0 MV of the collocated block.

- Otherwise (the collocated block is uni-predicted), both the L0 and L1 MVs of TMVP is scaled from the uni-predicted MV of the collocated block.

The TMVP is scaled from the selected MV of the collocated block according to the POC distances, tb and td, where tb is defined to be the POC difference between the reference picture of the current picture and the current picture and td is defined to be the POC difference between the reference picture of the co-located picture and the co-located picture. The target reference picture index of the TMVP is always fixed to 0 in ECM-2.0.

Figure 2. Scaling of TMVP

Proposed methods

This contribution proposes methods to improve the coding efficiency of TMVP, which is added to the merge candidate list of MMVD, CIIP, GPM, regular merge mode, SbTMVP and affine merge modes.

Method 1

When deriving the TMVP of merge modes from the collocated block, the selected MV is the one with the same target reference picture list. That is, the MVs of different lists in the collocated block are selected according to the rules as follows:

- If the collocated block is bi-predicted, the L0 and L1 MVs of the TMVP is scaled from the L0 and L1 MVs of the collocated block, respectively.

- Otherwise (the collocated block is uni-predicted), both the L0 and L1 MVs of TMVP is scaled from the uni-predicted MV of the collocated block.

Besides, the target reference picture can be selected from any one of reference picture in the list according to scaling factor. The scaling factor of each reference picture in the list is calculated, and the one whose scaling factor is the closest to 1 is used as the target reference picture of the TMVP.

Method 2

In the merge candidate list construction of a current CU coded using MMVD, CIIP, GPM, template matching and regular merge modes, it is proposed to conditionally add TMVP according to the motion information of spatial neighboring blocks, which are {A1, B1, B2} depicted in Figure 1. The conditions are illustrated in Figure 3, where cond_1, cond_2 and cond_3 are listed as follows:

- cond_1: position C0 is available, is inter-coded, and is in the same CTU row of the current CU

- cond_2: at least one of the spatial neighboring blocks has similar temporal motion with temporal motion C0 (or C1) and none of these spatial neighboring blocks uses TMVP

- cond_3: position C1 is inter-coded

Figure 3. Proposed conditions of adding TMVP candidate

In checking the similarity of the temporal motion, if the inter prediction direction of the two temporal motion are the same and the motion vector differences in horizontal and vertical direction are both smaller than a pre-defined threshold, the two temporal motion are treated as similar to each other. The pre-defined threshold is set to 5 for non-template matching coded CU, and is set to {16, 32, 64} for template matching coded block with size S equal to {S<64, 64≤S<256, S ≥256}, respectively.

Moreover, after the merge candidate list is constructed, if ARMC is applied to the current CU and the current picture is a non-low delay picture, the TMVP candidate is refined using the template matching method. The template is used to decide whether the TMVP is L0-predicted, L1-predicted or bi-prediction, and is used to decide which one of the three scaling factors {tbtd, tbtd×18, tbtd×98} is used to perform temporal motion vector scaling.

Simulation results

The described methods are implemented on top of ECM-2.0 software and are tested under CTC for enhanced compression tool testing [1]. Table 1 and Table 2 show the simulation results of method 1 and the combination of method 1 and method 2, respectively.

Table 1. Simulation results of method 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.03% | -0.04% | 99% | 100% |
| Class A2 | -0.03% | -0.09% | 0.00% | 99% | 100% |
| Class B | -0.03% | 0.02% | -0.01% | 101% | 102% |
| Class C | -0.09% | -0.03% | 0.04% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.01% | 0.00% | 100% | 101% |
| Class D | -0.09% | -0.05% | 0.12% | 100% | 103% |
| Class F | 0.00% | 0.07% | -0.09% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.21% | -0.20% | -0.42% | 102% | 101% |
| Class C | -0.71% | -1.06% | -0.97% | 101% | 104% |
| Class E | 0.17% | -0.82% | -0.24% | 103% | 103% |
| Overall | -0.28% | -0.64% | -0.56% | 102% | 102% |
| Class D | -0.75% | -1.08% | -1.69% | 103% | 106% |
| Class F | -0.36% | -0.67% | -0.75% | 100% | 103% |

Table 2. Simulation results of method 1 + method 2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.13% | -0.11% | -0.18% | 100% | 104% |
| Class C | -0.22% | -0.15% | -0.24% | 99% | 105% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.20% | -0.36% | -0.03% | 99% | 106% |
| Class F | -0.08% | -0.11% | -0.12% | 99% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.26% | -0.39% | -0.41% | 101% | 102% |
| Class C | -0.70% | -0.89% | -0.57% | 101% | 105% |
| Class E | -0.01% | 0.10% | -0.54% | 102% | 104% |
| Overall | -0.34% | -0.44% | -0.50% | 102% | 103% |
| Class D | -0.79% | -1.02% | -1.05% | 101% | 106% |
| Class F | -0.50% | -0.58% | -0.38% | 101% | 103% |

References

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-W2017, July 2021.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).