| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0103-v1 |
| --- | --- |

| Title: | [CE] Evaluation of VVC DCT-2 transform for Film Grain Synthesis (test CE1.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miloš Radosavljević, Edouard François (InterDigital) | Email: | {milos.radosavljevic,edouard.francois}@interdigital.com |
| Source: | Interdigital |  |  |

_____________________________

# Abstract

This is a summary report of the first Core Experiment (CE1.1) which is part of the Core Experiment on Film Grain Synthesis (JVET-W2022). The goal of CE1.1 is to study the use of core 64x64 DCT-2 specified in VVC instead of the 64x64 DCT-2 defined in SMPTE RDD 5 specification. The purpose of the CE is to show that the proposed modification has performance comparable to SMPTE RDD 5, however with additional implementation advantages.

It is recommended to adopt the film grain software into VTM, and also to incorporate film grain synthesis process into either VSEI or a technical report.

# Introduction

The goal of CE1.1 is to study the usage of VVC DCT-2 in the context of film grain synthesis, compared to previously used SMPTE RDD 5 DCT-2 [1].

The SMPTE RDD 5 specification [2] defines its own DCT-2 transform that is different from the one specified in VVC. To ensure consistency with the VVC specification, it is proposed instead to use the VVC core 64x64 DCT-2 transform within the film grain synthesis. The proposed modification leads to cost savings and simplified implementation. This change does not affect standardized Film Grain Characteristics (FGC) SEI message format.

JVET-V0093 proposed to use the VVC core 64x64 DCT-2 transform instead of the 64x64 transform specified in SMPTE RDD 5 in both, film grain analysis part and film grain synthesis part. Analysis part was improved in JVET-W0072, still using VVC DCT-2.

JVET-W0095 proposed an example of SMPTE RDD 5 Film Grain Technology, as a fixed-point process with support for 4K and 8K, and VVC DCT2. The process is identical to the SMPTE RDD 5 process already referenced in the FGC SEI message, except for two modifications, one of which is:

- The transformation process as specified in VVC (Rec. ITU-T H.266 | ISO/IEC 23090-3, clause 8.7.4.4, with trType inferred to equal 0) is used instead of the transform specified in SMPTE RDD 5. The VVC transform is more mature than the SMPTE RDD 5 transform. The VVC transform may also be already available on decoders and may thus be considered implementation friendly.

# Test conditions and guidelines

The complete film grain process is implemented within VTM 13.2. CE software is available on Git repo:

https://vcgit.hhi.fraunhofer.de/jvet-w-ce/ce-fgs/-/tree/CE-FGS

Commit SHA: e09283bcccb25992222cf7c7b0f67844cce0e2cb

It also supports additional features such as film grain analysis with multiple intensity intervals [3] and FGC SEI rewriter [4].

Several new macros are included in software:

#define FILM_GRAIN_ENABLE	1 // Plain SEI for film grain plus SMPTE RDD5 based film grain model

#if FILM_GRAIN_ENABLE

#define FILM_GRAIN_SYNTHESIS	1 // Enable film grain synthesis (SMPTE RDD5 based film grain synthesis enabled)

#define FILM_GRAIN_ANALYSIS	1 // Enable adaptive film grain parameter estimation (SMPTE RDD5 based film grain analysis enabled)

#define FILM_GRAIN_DCT	1 // 1 - DCT-2 from VVC; 0 -  RDD5 DCT

#define FILM_GRAIN_BLK_SIZE	1 // Adaptive block size for film grain blending

#if FILM_GRAIN_SYNTHESIS

#define CE_FGS_SIMD_OPT	1 // Enable SIMD optimization for film grain synthesis (only support 10bit, AVX2 for 32x32)

#define CE_FGS_TIME_CALC	1 // Calculate film grain synthesis time

#define FGC_SEI_REWRITER	1 // Enable film grain characteristics SEI message rewriter

#endif

#endif

CE1.1 anchor version follows RDD 5 specifications, meaning that it uses 8x8 block size for film grain blending and RDD 5 DCT-2 (64x64 transform). Therefore, for anchor version FILM_GRAIN_DCT is set to 0 and FILM_GRAIN_BLK_SIZE is set to 0.

CE1.1 test configuration still used 8x8 block size for film grain blending, but it utilizes DCT-2 as specified in VVC (64x64 transform). For CE1.1 version FILM_GRAIN_DCT is set to 1 and FILM_GRAIN_BLK_SIZE is still set to 0.

Both configurations are tested with SIMD option enabled and disabled (CE_FGS_SIMD_OPT set to 0 and to 1). Before building FGS executable, make sure your machine supports SIMD options (AVX/AVX2). CE software already adds the necessary changes in CMakeLists.txt.

There are two tests performed: complexity test and subjective test. Complexity test aims to analyze implementation complexity of the proposed VVC DCT-2 in the context of the film grain when compared to RDD 5 transform. Subjective test provides quality assessment of the synthesis process by using VVC DCT-2 compared to RDD 5 transform.

For tests we use 6 clips from JVET SDR-CTC testset Class A (JVET-T2010), and additional 6 sequences for subjective test, see Table 1. All sequences are 4K resolution except one that is FHD. Encoder configuration is random access, and QP is 22,27,32,37. For subjective test we use only QP 32 and 37.

Film grain characteristic SEI configurations are provided within CE software package. For complexity test there are three different configurations which are applied to all sequences in complexity test set. They reflect different complexity for film grain synthesis. One configuration applies film grain on all three components and uses 10 intensity intervals (FGC_SEI_I10C3.cfg). Another one uses 10 intensity intervals but applies film grain only on luma component (FGC_SEI_I10C1.cfg). The last one applies film grain only on luma component and uses one intensity interval to model film grain (FGC_SEI_I1C1.cfg). For subjective test, there is one FGC configuration adapted to each sequence in subjective test set.

Table 1: Test sequences used in subjective test

| SeqName-Subjective testset | Resolution | FPS | Frames | BitDepth |
| --- | --- | --- | --- | --- |
| MountainBay2 | 3840x2160 | 30 | 300 | 10 |
| OberbaumSpree | 3840x2160 | 60 | 600 | 10 |
| Tango2 | 3840x2160 | 60 | 294 | 10 |
| RaceNight | 3840x2160 | 50 | 600 | 10 |
| TrafficFlow | 3840x2160 | 30 | 300 | 10 |
| Crowdrun | 1920x1080 | 50 | 500 | 8 |

Result and detailed xCheck instructions were provided to JVET before the beginning of the meeting. They are available at: https://vcgit.hhi.fraunhofer.de/jvet-w-ce/ce-fgs/-/tree/CE-FGS within the “results” folder (see CE1_FGS_xCheck_Instruction.pptx).

Also, at the same repo, under the folder "results" bitstreams and additional supporting documents (per-sequence configurations, film grain configurations, md5sums) are provided.

Additional tool is provided called SEIFilmGrainApp [4]. It can be used to rewrite different FGC SEI configurations, hence no need to run encoder with different FGC parameters. Encoding can be performed only once and SEI can be re-written.

# CE 1.1 Test results and Results analyses

## Complexity Test results

We run tests on 4K content (Class A). Both anchor and CE1.1 are encoded using SIMD OFF and SIMD ON options. Encoded bitstreams are provided in the CE package on the git repo (with md5sum as well). Film grain synthesis runtime is collected after the decoding process. Film grain simulation time is the time necessary to simulate and blend film grain. Note that for this test we did not output reconstructed yuv files. The results of using VVC DCT-2 compared to anchor that utilizes RDD 5 DCT-2 are provided in the table below reporting film grain synthesis runtime.

Table 2: Results of the complexity test

|  | SIMD OFF |  |  | SIMD ON |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Configure | CEAnc FT (sec) | CE1.1 FT (sec) | Time Saving (%) | CEAnc FT (sec) | CE1.1 FT (sec) | Time Saving (%) |
| FGC_SEI_I10C3 | 8.57 | 8.67 | 1.21% | 3.99 | 3.87 | -2.96% |
| FGC_SEI_I10C1 | 5.43 | 5.53 | 1.89% | 2.50 | 2.49 | -0.21% |
| FGC_SEI_I1C1 | 5.93 | 6.01 | 1.29% | 2.66 | 2.59 | -2.96% |
| Average | 6.51 | 6.61 | 1.46% | 2.98 | 2.92 | -2.05% |

Time difference between anchor and CE1.1 test is shown to be negligible. Small difference is the result of testing error (the way we utilize our cluster and the fact that the time between two executions can slightly vary). Note that the absolute difference is relatively small. The observed performance was expected, since both versions are implemented in the same way by using matrix multiplication. This is also observed in CE2.1 vs. CE2.2 (see [4]).

It is noted that the FGC processing time is affected mainly by the number of colour components processed, and not on the number of the intensity intervals. Also, absolute FGS processing time for SIMD ON is about 2.5 times faster than with SIMD OFF test.

On the other side, we highlight some additional benefits of using VVC DCT-2 transform. There are many studies for VVC DCT-2 already presented in the literature (one of the most optimized transforms) presenting efficient hardware implementation, fast algorithms. The transform is well studied, documented, and designed to ensure efficient implementation in both software and hardware. The VVC transform may also be already available on decoders and may thus be considered implementation friendly, reducing the need for one additional transform. Supporting multiple transforms has several consequences related to complexity and implementation costs, e.g., additional burden to hardware implementation. It requires additional memory for storing the multiple transform cores. Additional kernel leads to increased area since hardware sharing is not possible. Thus, by omitting one additional transform the cost of implementation is reduced, e.g., on-chip implementation, memory requirement reduction. Even efficient software implementation such as partial butterfly implementation is simplified since it can be reused from the core transform. In other words, the transform implementation is simplified and unified by means of using only DCT-2 from VVC.

Historically, there was much effort put in the research how to reduce the memory requirements for transform cores, excluding some transforms from consideration, reducing the number of bits per transform coefficient, reducing total number of unique values, etc. This effort was mainly summarized in Ljubljana and Macao JVET meetings (K and L meetings). For example, in VVC it is proposed to use 8-bit primary transform cores, optimizing the memory for storing the primary transform cores and relaxing additional implementation cost [5].

For example, additional memory requirement for RDD 5 transform is: 44 unique coefficient (no sign counted) that can be stored with 6-bits precision (RDD5 design is a straightforward way of deriving transform cores using scaling and rounding – scaling by 2^8). In total there are 4096 coefficients requiring 6-bits for storing transform core. From the analysis of this CE, it can be considered that this leads to potentially unnecessary resource allocation, and that all process can be implemented with VVC DCT-2 without any loss of quality.

## Subjective Test results

Subjectively, there is no difference between anchor and CE1.1. In addition, the average difference per pixel is 0.5905 over all sequences and QPs (mean squared error), computed on subjective test set. Such a small difference in pixel intensity is not visible to human eye.

It is to note that legacy SEI messages are not impacted by the proposed modifications, since the results are subjectively the same.

In addition, it is observed that film grain can mask compression artifacts. Also, for sequences that originally possess film grain, reconstructed sequence with film grain is more similar to the source sequence than the one without synthetized film grain.

# Conclusions

By using the VVC DCT-2 core transform, the 64-point primary transform logic can be reused to implement film grain without any impact on the preformance. Totally, one kind of transform core can be used instead of two. There is no impact on the quality of the synthesized film grain when VVC transform is used. Computationaly wise, in worst case, performance is the same as with RDD 5 transform. The proposed modification does not affect complexity due to the similar implementation within the current software. In addition, other implementation benefits are introduced by this simple modification. Visual quality is virtually the same, and MSE is less then 1. That level of difference is not visible to the human eye. Legacy SEI messages are not impacted by the proposed modifications, since the results are subjectively the same, and the proposed modification does not change standardized SEI format.

Thus, we suggest accepting the proposed modification for film grain synthesis. It is recommended to adopt the film grain software into VTM, and also to incorporate film grain synthesis process into either VSEI or a technical report.

Thanks to Dolby and Tencent for xchecking our proposal.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Sean McCarthy, Miloš Radosavljević and Jay Shingala, “CE: Core Experiment on Film Grain Synthesis”, JVET-W2022, Teleconference, 7–16 July, 2021.

- “RDD 5:2006 - SMPTE Registered Disclosure Doc - Film Grain Technology — Specifications for H.264 | MPEG-4 AVC Bitstreams,” RDD 5:2006, pp. 1–18, Mar. 2006, doi: 10.5594/SMPTE.RDD5.2006.

- Miloš Radosavljević, Edouard François, Wassim Hamidouche, Thomas Amestoy and Guillaume Gautier, “AHG9: Enhancement of film grain parameter estimation for different intensity intervals”, JVET-W0072, Teleconference, 7-16 July, 2021.

- Fangjun Pu, Sean McCarthy, Walt Husak, Peng Yin, Taoran Lu, Arjun Arora, Tao Chen, Vijayakumar G R, Jay N Shingala, Kaustubh Patankar, Shireesh Kadaramandalgi, Ajayshyam, “CE: Film Grain Synthesis (test CE2.1 and CE2.2)”, JVET-X0048, Teleconference, 6-15 October, 2021.

- Xin Zhao, Xiang Li, Shan Liu, “CE6-related: On 8-bit primary transform core”, JVET-K0290, Ljubljana, SI, 10–18 July 2018.