| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0106-v1 |
| --- | --- |

| Title: | On constraints for intra profiles in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Steve Keating Takeshi Tsukuba | Email: | steve.keating@sony.com takeshi.tsukuba@sony.com |
| Source: | Sony |  |  |

_____________________________

# Abstract

This contribution proposes adding constraints on slice_type for Intra profiles in VVC.

# Problem

At the last meeting, the Main 12 Intra, Main 12 4:4:4 Intra and Main 16 4:4:4 Intra profiles were defined in VVC [1].

However, there is no constraint on slice type (or NAL unit type for VCL NAL units) for Intra profiles.

So, even if general_profile_idc indicates “Intra Profile”,

slice_type can take a value of I, B or P-slice

or nal_unit_type for VCL NAL units can take a value of Non-IRAP (Non-Intra random access point)

# Proposed Changes

It is proposed to add the following constraints in the Intra profile definition.

- Constrain slice types for Intra profiles using the existing syntax gci_intra_only_constraint_flag:

### A3.5	Format range extensions profiles

### …

Bitstreams conforming to the format range extensions profiles shall obey the following constraints:

In bitstreams conforming to the Main 12 Intra, Main 12 4:4:4 Intra, or Main 16 4:4:4 Intra profiles, gci_intra_only_constraint_flag in the referenced VPS (when available) and in the referenced SPSs shall be equal to 1.

- Similar constraint with HEVC:

### A3.5	Format range extensions profiles

### …

Bitstreams conforming to the format range extensions profiles shall obey the following constraints:

In bitstreams conforming to the Main 12 Intra, Main 12 4:4:4 Intra, or Main 16 4:4:4 Intra profiles, all pictures shall be IRAP pictures and the output order indicated in the bitstream among these pictures shall be the same as the decoding order.

Also, (editorial) change:

- In bitstreams conforming to the Main 12, Main 12 4:4:4, Main 16 4:4:4, Main 12 Intra, Main 12 4:4:4 Intra, or Main 16 4:4:4 Intra profiles, general_level_idc for all values of i in active SPSs shall not be equal to 255 (which indicates level 15.5).

To (from JVET-T2001-v1 Main 10 definition):

- In a bitstream conforming to the Main 12, Main 12 4:4:4, Main 16 4:4:4, Main 12 Intra, Main 12 4:4:4 Intra, or Main 16 4:4:4 Intra profiles, general_level_idc in the referenced VPS (when available) and in the referenced SPSs shall not be equal to 255 (which indicates level 15.5).

# Conclusion

A solution is described to constrain Intra profiles such that the slice_type cannot be that of a B or P-slice or the nal_unit_type cannot take a value of Non-IRAP.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “VVC operation range extensions (Draft 4)” JVET-W2005-v1, F Bossen, B Bross, T Ikai, D Rusanovskyy, G Sullivan, Y-K Wang