| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0115-v1 |
| --- | --- |

| Title: | EE2-related: Optimization on the second mode derivation of DIMD blending mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chuan Zhou Zhuoyi Lv Jinrong Zhang No.168 Weiwo Rd., Chang'an, Dongguan, Guangdong | Email: | chuan.zhou@vivo.com zhuoyi.lv@vivo.com jinrong.zhang@vivo.com |
| Source: | vivo Mobile Communication Co., Ltd. |  |  |

_____________________________

- Abstract

In this contribution, a method to optimize the second mode in the DIMD blending mode is proposed. The second mode is derived from the second and third largest histogram amplitude values by template-based method as in TIMD.

On top of ECM-2.0, the performance is reported:

AI: -0.02% (Y) 0.00% (U) 0.00% (V), runtimes 101% (enc) 102% (dec)
RA: -x.xx%	 (Y) x.xx% (U) -x.xx% (V), runtimes 10x% (enc) 10x% (dec)

Introduction

In EE2, two intra tools are included to further improve the coding efficiency by implicitly deriving the intra prediction mode and prediction blending.

In the decoder-side intra mode derivation (DIMD) mode [1], a Histogram of Gradients (HoG) is built by applying horizontal and vertical Sobel filters to reconstructed neighboring samples. One or two intra prediction modes are derived from the HoG with the tallest histogram bars. If the DIMD flag is false, the derived intra prediction mode can be used to construct the MPM list.

In the template-based intra mode derivation (TIMD) mode [2], for each candidate mode in the MPM list, the reference samples of the template are used to generate the prediction samples of the template. A cost is calculated as the sum of absolute transformed differences (SATD) between the prediction samples and the reconstruction samples of the template. One or two intra prediction modes with the smallest cost are selected to generate prediction samples of the current block.

Proposed method

The proposed method selects a third angular mode with the third largest non-zero amplitude value from the HoG. For the second and third angular prediction modes, a cost is calculated using SATD between the prediction and the reconstruction samples of the template. The best second mode is selected based on the cost.

In addition, a fast algorithm is included. The initial second mode is used to construct MPM list, rather than the derived second mode, besides the first mode.

Results

The proposed method is implemented on top of ECM-2.0 [3]. The simulation results are tested under ECM-2.0 test condition [4] with all intra and random access configurations.

Table 1. Simulation results of the proposed method.

|  |  |  |  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over ECM-2.0 |  |  |  |  |  |
|  | Y |  | U |  | V | EncT |  | DecT |  |
| Class A1 | 0.00% |  | 0.00% |  | -0.06% | 101% |  | 100% |  |
| Class A2 | -0.02% |  | 0.00% |  | 0.00% | 101% |  | 101% |  |
| Class B | -0.01% |  | 0.07% |  | -0.07% | 101% |  | 102% |  |
| Class C | -0.02% |  | -0.02% |  | 0.06% | 101% |  | 103% |  |
| Class E | -0.03% |  | -0.10% |  | 0.08% | 101% |  | 102% |  |
| Overall | -0.02% |  | 0.00% |  | 0.00% | 101% |  | 102% |  |
| Class D | 0.00% |  | 0.11% |  | 0.02% | 101% |  | 105% |  |
| Class F | -0.05% |  | -0.07% |  | -0.06% | 101% |  | 100% |  |
|  |  |  |  | Random access Main10 |  |  |  |  |  |
|  |  |  |  | Over ECM-2.0 |  |  |  |  |  |
|  | Y |  | U |  | V | EncT |  | DecT |  |
| Class A1 | -0.01% |  | 0.04% |  | -0.02% | 103% |  | 111% |  |
| Class A2 | #VALUE! |  | #VALUE! |  | #VALUE! | #NUM! |  | #NUM! |  |
| Class B | -0.03% |  | 0.01% |  | -0.05% | 101% |  | 100% |  |
| Class C | 0.00% |  | 0.00% |  | -0.01% | 149% |  | 155% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | #VALUE! |  | #VALUE! |  | #VALUE! | #NUM! |  | #NUM! |  |
| Class D | -0.03% |  | -0.35% |  | 0.29% | 151% |  | 174% |  |
| Class F | -0.01% |  | -0.04% |  | -0.48% | 120% |  | 126% |  |

Cross-checks

The proponents would like to thank Alibaba for cross-checking this proposal.

Conclusion

This contribution proposes a second mode derivation method for the DIMD blending mode. Compared with ECM2.0, it achieves % and % gain in AI and RA configurations, respectively.

References

- M. Abdoli, T. Guionnet, E. Mora, et. al, “Non-CE3: Decoder-side Intra Mode Derivation with Prediction Fusion Using Planar”, JVET-O0449, Gothenburg, July 2019.

- K. Cao, N. Hu, V. Seregin, “EE2-related Fusion for template-based intra mode derivation”, JVET-W0123, Jul. 2021.

- ECM-2.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM/-/tags/ECM-2.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-W2017, Jul. 2021.

Patent rights declaration(s)

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).