| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0129-v2 |
| --- | --- |

| Title: | AHG8: Independent Rice Parameter Derivation for high bit depth and high bit rate extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Yu Haoping Yu Zhihuang Xie Fan Wang Luhang Xu Dong Wang | Email: | yue.yu@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

In the history-based Rice parameter derivation method adopted in JVET-V meeting, a history value is transferred through a counter to update localSumAbs so that a more accurate Rice parameter can be derived to code remaining levels. The counter may be updated once in each TU based upon its value in the previous TU. This results in dependency among TUs. This contribution proposes an independent Rice parameter derivation method. If the last significant coefficient in a TU has abs_remainder[] part in its level coding, a fixed value is used as HistValue in the computation of localSumAbs. After the last significant coefficient is coded, the absolute value of last significant coefficient is used, as a history value, to compute localSumAbs for coding all remaining abs_remainder[ ] or dec_abs_level[ ] within the current TU. The part of the counter StatCoeff updating process that uses its value carried over from the previous TU is totally removed and therefore there is no any dependency between the current TU and the previous TUs. Under the high bit-depth common test condition (CTC), the simulation results show the following minor coding loss in BD-bitrate.

Low QP

PQ: AI: 0.08%, 0.04%, 0.04%; LDB: 0.01%, 0.00%, 0.02%; RA: 0.02%, 0.03%, 0.01%;

HLG: AI: 0.09%, 0.04%, 0.03%; LDB: 0.01%, 0.00%, 0.01%; RA: 0.02%, 0.02%, 0.02%;

SVT: AI: 0.15%, 0.10%, 0.10%; LDB: 0.07%, 0.07%, 0.07%; RA: 0.06%, 0.06%, 0.06%;

Lossless:

PQ: AI: 0.55%; LDB: 0.48%, RA: 0.45%;

HLG: AI: 0.53%; LDB: 0.51%, RA: 0.50%;

SVT: AI: 0.63%; LDB: 0.31%, RA: 0.30%;

# Introduction

A history-based Rice parameter derivation method JVET-V0106 [3] was adopted for VVC version 2 [2]. To improve coding efficiency, this method enhances the Rice parameter derivation process defined in VVC version 1 [1] by using a history derived value, instead of 0, for template positions outside the current TU in the computation of localSumAbs.

To maintain a history, a single counter per color component, StatCoeff[3] is utilized. It is being updated once per TU from the first, non-zero, Golomb-Rice coded transform coefficient (abs_remainder[ ] or dec_abs_level[ ]) through a process of exponential moving average.

When the first, non-zero, Golomb-Rice coded transform coefficient in TU is coded as abs_remainder, the history counter for color component cIdx is updated as the following:

StatCoeff[cIdx] = ( StatCoeff[cIdx] + Floor(Log2(abs_remainder[ ])) + 2 ) >> 1

When the first, non-zero, Golomb-Rice coded transform coefficient in TU is coded as dec_abs_level, the history counter for color component cIdx is updated as the following:

StatCoeff[cIdx] = ( StatCoeff[cIdx] + Floor(Log2(dec_abs_level[ ])) ) >> 1

Prior to TU decoding and history value update, the variable HistValue is being initialized as:

HistValue = 1 << StatCoeff[cIdx]

The HistValue is used for the template positions outside the current TU to update localSumAbs so that a more accurate Rice parameter will be used for coding abs_remainder[ ] or dec_abs_level[ ]. If the current CTU is the first CTU in a slice or tile, the StatCoeff[cIdx] is initialized as follows.

StatCoeff[ idx ] = 2 * Floor( Log2( BitDepth − 10 )

where BitDepth specifies the bit depth of the samples of the luma and chroma arrays, Floor( x ) represents the largest integer less than or equal to x and Log2( x ) is base-2 logarithm of x.

# Proposed Method

In the current method, updating StatCoeff needs its value carried over from the previous TUs and therefore it creates dependency among TUs. This contribution proposes an independent Rice parameter derivation method, which completely removes the TU dependency due to StatCoeff updating.

First, an initial value of statCoff_init[cIdx] for each color component cIdx is defined and specified at the slice header as follows.

# Change clause 7.3.7 Slice header syntax (additions are yellow-highlighted and the text in red color with strikethrough will be removed)

| slice_header( ) { | Descriptor |
| --- | --- |
| sh_picture_header_in_slice_header_flag | u(1) |
| ….. |  |
| if( sps_transform_skip_enabled_flag && !sh_dep_quant_used_flag && !sh_sign_data_hiding_used_flag ) |  |
| sh_ts_residual_coding_disabled_flag | u(1) |
| if( !sh_ts_residual_coding_disabled_flag && sps_ts_residual_coding_rice_present_in_sh_flag ) |  |
| sh_ts_residual_coding_rice_idx_minus1 | u(3) |
| if( sps_reverse_last_sig_coeff_enabled_flag ) |  |
| sh_reverse_last_sig_coeff_flag | u(1) |
| if(sps_persistent_rice_adaptation_enabled_flag ){ |  |
| for( i = 0; i < 3; i++ ) |  |
| statCoeff_init[ i ] | u(3) |
| } |  |
| if( pps_slice_header_extension_present_flag ) { |  |
| sh_slice_header_extension_length | ue(v) |
| for( i = 0; i < sh_slice_header_extension_length; i++) |  |
| sh_slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| if( NumEntryPoints > 0 ) { |  |
| sh_entry_offset_len_minus1 | ue(v) |
| for( i = 0; i < NumEntryPoints; i++ ) |  |
| sh_entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

statCoeff_init specifies the value used for calculating HistValue used for the residual_coding( ) syntax structure in the current slice. When not present, the value of statCoeff_init is inferred to be equal to 0.

If the last significant coefficient has abs_remainder[] part in its level coding, HistValue is calculated as follows.

HistValue = 1 << statCoeff_init[ idx ]

This HistValue is used to update localSumAbs for coding abs_remainder[] of the last significant coefficient. After the last significant coefficient is coded, HistValue is updated by the absolute value of the last significant coefficient and the updated HistValue is used for coding all remaining abs_remainder[ ] or dec_abs_level[ ] within the current TU. The part of the counter StatCoeff updating process in [3] that uses its value carried over from previous TU is totally removed and therefore there is no any dependency between the current TU and previous TUs due to StatCoeff updating.

Based upon the current VVC specifications [2], the specification changes are specified as follows.

# Change clause 7.3.11.11 (Residual coding syntax) as follows

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| … |  |  |
| } |  |
| … |  |
| numSbCoeff = 1 << ( log2SbW + log2SbH ) |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock = ( 1 << ( log2TbWidth + log2TbHeight − ( log2SbW + log2SbH ) ) ) − 1 |  |
| HistValue = sps_persistent_rice_adaptation_enabled_flag ? ( 1 << StatCoeff_init[ cIdx ] ) : 0 |  |
| updateHist = sps_persistent_rice_adaptation_enabled_flag ? 1 : 0 |  |
| … |  |
| for( n = firstPosMode0; n > firstPosMode1; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( abs_level_gtx_flag[ n ][ 1 ] ) { |  |
| abs_remainder[ n ] | ae(v) |
| if( updateHist && abs_remainder[ n ] > 0 ) { |  |
| StatCoeff[ cIdx ] = ( StatCoeff[ cIdx ] + Floor( Log2( abs_remainder[ n ] ) ) + 2 ) >> 1 |  |
| updateHist = 0 |  |
| } |  |
| } |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] +2 * abs_remainder[ n ] |  |
| if(updateHist){ |  |
| HistValue = AbsLevel[ xC ][ yC ] |  |
| updateHist = 0 |  |
| } |  |
| } |  |
| for( n = firstPosMode1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( sb_coded_flag[ xS ][ yS ] ) { |  |
| dec_abs_level[ n ] | ae(v) |
| if( updateHist && dec_abs_level[ n ] > 0 ) { |  |
| StatCoeff[ cIdx ] = ( StatCoeff[ cIdx ] + Floor( Log2( dec_abs_level[ n ] ) ) ) >> 1 |  |
| updateHist = 0 |  |
| } |  |
| } |  |
| if( AbsLevel[ xC ][ yC ] > 0 ) { |  |
| if( lastSigScanPosSb = = −1 ) |  |
| lastSigScanPosSb = n |  |
| firstSigScanPosSb = n |  |
| } |  |
| if( sh_dep_quant_used_flag ) |  |
| QState = QStateTransTable[ QState ][ AbsLevel[ xC ][ yC ] & 1 ] |  |
| } |  |
| ……. |  |

# Simulation results

The test conditions in this contribution follow the CTC for high bit depth and high bit rate video coding [4]. The anchor is generated with VTM-14.0, the test is the proposed method.

In the simulations presented below, the statCoff_init[cIdx] for each color component cIdx is defined as follows.

statCoeff_init[ idx ] = Clip(1, 8, (int) ((19 - QP) / 6))-1

where QP is the QP for current CU and Clip() is an operation defined as follows,

Clip( x, y, z ) =  x;z < xy;z > yz;otherwise

| HDR PQ |  |  |  |  |  | AI |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Over VTM14.0 |  |  |  |  |  |  |  |  |
|  |  | wPsnrY |  | wPsnrU |  | wPsnrV |  | psnrY |  | psnrU | psnrV |  | EncT | DecT |
| PQ444 |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | 0.07% |  | 0.04% | 0.03% |  | 108% | 110% |
| PQ422 |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | 0.09% |  | 0.04% | 0.04% |  | 105% | 104% |
| Overall |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | 0.08% |  | 0.04% | 0.04% |  | 106% | 107% |
|  |  |  |  |  |  | LDB |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Over VTM14.0 |  |  |  |  |  |  |  |  |
|  |  | wPsnrY |  | wPsnrU |  | wPsnrV |  | psnrY |  | psnrU | psnrV |  | EncT | DecT |
| PQ444 |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | 0.00% |  | -0.02% | 0.02% |  | 96% | 106% |
| PQ422 |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | 0.02% |  | 0.02% | 0.01% |  | 94% | 95% |
| Overall |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | 0.01% |  | 0.00% | 0.02% |  | 95% | 101% |
|  |  |  |  |  |  | RA |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Over VTM14.0 |  |  |  |  |  |  |  |  |
|  |  | wPsnrY |  | wPsnrU |  | wPsnrV |  | psnrY |  | psnrU | psnrV |  | EncT | DecT |
| PQ444 |  | 0.02% |  | 0.02% |  | 0.02% |  | 0.02% |  | 0.02% | 0.02% |  | 99% | 100% |
| PQ422 |  | 0.03% |  | 0.03% |  | 0.01% |  | 0.03% |  | 0.03% | 0.01% |  | 100% | 99% |
| Overall |  | 0.02% |  | 0.03% |  | 0.01% |  | 0.02% |  | 0.03% | 0.01% |  | 100% | 100% |
| Overall PQ |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | 0.04% |  | 0.02% | 0.02% |  | 100% | 102% |
| HDR HLG |  |  |  |  | AI |  |  |  |  |  |  |
|  |  |  |  |  | Over VTM14.0 |  |  |  |  |  |  |
|  | psnrY |  | psnrU |  | psnrV |  | EncT |  | DecT |  |  |
| HLG444 | 0.09% |  | 0.04% |  | 0.04% |  | 109% |  | 111% |  |  |
| HLG422 | 0.09% |  | 0.04% |  | 0.03% |  | 102% |  | 114% |  |  |
| Overall | 0.09% |  | 0.04% |  | 0.03% |  | 106% |  | 112% |  |  |
|  |  |  |  |  | LDB |  |  |  |  |  |  |
|  |  |  |  |  | Over VTM14.0_ |  |  |  |  |  |  |
|  | psnrY |  | psnrU |  | psnrV |  | EncT |  | DecT |  |  |
| HLG444 | 0.00% |  | -0.01% |  | -0.01% |  | 95% |  | 86% |  |  |
| HLG422 | 0.02% |  | 0.00% |  | 0.02% |  | 94% |  | 89% |  |  |
| Overall | 0.01% |  | 0.00% |  | 0.01% |  | 94% |  | 87% |  |  |
|  |  |  |  |  | RA |  |  |  |  |  |  |
|  |  |  |  |  | Over VTM14.0 |  |  |  |  |  |  |
|  | psnrY |  | psnrU |  | psnrV |  | EncT |  | DecT |  |  |
| HLG444 | 0.01% |  | 0.01% |  | 0.01% |  | 98% |  | 96% |  |  |
| HLG422 | 0.02% |  | 0.02% |  | 0.03% |  | 101% |  | 102% |  |  |
| Overall | 0.02% |  | 0.02% |  | 0.02% |  | 100% |  | 99% |  |  |
| Overall HLG | 0.04% |  | 0.02% |  | 0.02% |  | 100% |  | 100% |  |  |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.21% | 0.16% | 0.16% | 104% | 101% |
| SVT12 | 0.09% | 0.03% | 0.03% | 106% | 99% |
| Overall | 0.15% | 0.10% | 0.10% | 105% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.11% | 0.11% | 0.11% | 103% | 105% |
| SVT12 | 0.03% | 0.03% | 0.03% | 102% | 111% |
| Overall | 0.07% | 0.07% | 0.07% | 103% | 108% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM14.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.10% | 0.11% | 0.11% | 106% | 103% |
| SVT12 | 0.03% | 0.02% | 0.02% | 106% | 112% |
| Overall | 0.06% | 0.06% | 0.06% | 106% | 108% |
| Overall RGB | 0.09% | 0.08% | 0.08% | 104% | 105% |

Another comparison is made to modify the VTM-14.0 anchor without original history-based Rice parameter derivation (turning off persistent Rice adaption flag). There is coding gain when the proposed independent Rice parameter derivation method is over this new anchor.

| HDR PQ |  |  |  | AI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over VTM14.0 - PersistentRiceAdapt |  |  |  |
|  | psnrY |  | psnrU |  | psnrV | EncT | DecT |
| PQ444 | -0.12% |  | -0.07% |  | -0.09% | 105% | 104% |
| PQ422 | -0.13% |  | -0.09% |  | -0.10% | 100% | 98% |
| Overall | -0.13% |  | -0.08% |  | -0.10% | 102% | 101% |
| HDR PQ |  |  |  | LDB |  |  |  |
|  |  |  |  | Over VTM14.0 - PersistentRiceAdapt |  |  |  |
|  | psnrY |  | psnrU |  | psnrV | EncT | DecT |
| PQ444 | -0.06% |  | -0.07% |  | -0.05% | 105% | 117% |
| PQ422 | -0.07% |  | -0.05% |  | -0.09% | 103% | 111% |
| Overall | -0.06% |  | -0.06% |  | -0.07% | 104% | 114% |
| HDR PQ |  |  |  | RA |  |  |  |
|  |  |  |  | Over VTM14.0 - PersistentRiceAdapt |  |  |  |
|  | psnrY |  | psnrU |  | psnrV | EncT | DecT |
| PQ444 | -0.06% |  | -0.04% |  | -0.05% | 102% | 103% |
| PQ422 | -0.06% |  | -0.04% |  | -0.08% | 102% | 104% |
| Overall | -0.06% |  | -0.04% |  | -0.06% | 102% | 103% |

| HDR HLG |  |  |  | AI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over VTM14.0 - PersistentRiceAdapt |  |  |  |
|  | psnrY |  | psnrU |  | psnrV | EncT | DecT |
| HLG444 | -0.26% |  | -0.13% |  | -0.13% | 101% | 101% |
| HLG422 | -0.30% |  | -0.18% |  | -0.19% | 99% | 101% |
| Overall | -0.28% |  | -0.15% |  | -0.16% | 100% | 101% |
| HDR HLG |  |  |  | LDB |  |  |  |
|  |  |  |  | Over VTM14.0 - PersistentRiceAdapt |  |  |  |
|  | psnrY |  | psnrU |  | psnrV | EncT | DecT |
| HLG444 | -0.04% |  | -0.04% |  | -0.05% | 110% | 117% |
| HLG422 | -0.03% |  | -0.03% |  | -0.04% | 108% | 118% |
| Overall | -0.04% |  | -0.03% |  | -0.04% | 109% | 117% |
| HDR HLG |  |  |  | RA |  |  |  |
|  |  |  |  | Over VTM14.0 - PersistentRiceAdapt |  |  |  |
|  | psnrY |  | psnrU |  | psnrV | EncT | DecT |
| HLG444 | -0.04% |  | -0.02% |  | -0.03% | 106% | 116% |
| HLG422 | -0.04% |  | -0.04% |  | -0.03% | 106% | 119% |
| Overall | -0.04% |  | -0.03% |  | -0.03% | 106% | 118% |

| SVT RGB |  |  |  | AI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over VTM14.0 - PersistentRiceAdapt |  |  |  |
|  | psnrG |  | psnrB |  | psnrR | EncT | DecT |
| SVT16 | -1.48% |  | -1.21% |  | -1.19% | 109% | 128% |
| SVT12 | -0.71% |  | -0.43% |  | -0.43% | 105% | 106% |
| Overall | -1.09% |  | -0.82% |  | -0.81% | 107% | 117% |

| SVT RGB |  |  |  | LDB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over VTM14.0 - PersistentRiceAdapt |  |  |  |
|  | psnrG |  | psnrB |  | psnrR | EncT | DecT |
| SVT16 | -0.97% |  | -1.03% |  | -1.03% | 129% | 129% |
| SVT12 | -0.32% |  | -0.31% |  | -0.31% | 120% | 130% |
| Overall | -0.65% |  | -0.67% |  | -0.67% | 124% | 129% |
| SVT RGB |  |  |  | RA |  |  |  |
|  |  |  |  | Over VTM14.0 - PersistentRiceAdapt |  |  |  |
|  | psnrG |  | psnrB |  | psnrR | EncT | DecT |
| SVT16 | -0.96% |  | -0.93% |  | -0.93% | 131% | 130% |
| SVT12 | -0.31% |  | -0.27% |  | -0.27% | 119% | 129% |
| Overall | -0.63% |  | -0.60% |  | -0.60% | 125% | 129% |

# Conclusion

This contribution proposes an independent Rice parameter derivation method. The TU dependent StatCoeff counter updating process is totally removed. Simulation results show that the proposed method has minor loss in coding efficiency. This contribution suggests adopting the proposed modification.

# References

[1] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “Versatile Video Coding Editorial Refinements on Draft 10,” Document of Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-T2001_v2, 20th Meeting: by teleconference, 7–16 Oct. 2020.

[2] F. Bossen, B. Bross, T. Ikai, D. Rusanovskyy, G. J. Sullivan, Y.-K. Wang, “VVC operation range extensions (Draft 4),” Document of Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-W2005, 22nd Meeting, by teleconference, 7–16 July. 2021.

[3] Dmytro Rusanovskyy, Marta Karczewicz, Luong Pham Van, Muhammed Coban “CE-related: On history-enhanced method of Rice parameter derivation for regular residual coding (RRC) at high bit depths,” Document of Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-V0106, 22nd Meeting, by teleconference, 20–28 Apr. 2021.

[4] A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).