| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0145-v2 |
| --- | --- |

| Title: | EE2-related: Template matching CIIP on top of EE2-3.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang, Vadim Seregin, Marta Karczewicz | Email: | {yjchang, vseregin} @qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this contribution, a template matching CIIP (TM-CIIP) is proposed to refine the MV of inter part of CIIP. Compared to ECM-2.0 anchor, simulation results of the proposed method combined with EE2-3.1 are summarized as follows:

- TM-CIIP combined with EE2-3.1a

RA: -0.xx% (Y), -0.xx% (U), -0.xx% (V), 100% (EncT), 10x% (DecT)

- TM-CIIP combined with EE2-3.1b

RA: -0.xx% (Y), -0.xx% (U), -0.xx% (V), 100% (EncT), 10x% (DecT)

# Introduction

In current ECM, the CIIP is the prediction combination of inter and intra as follows:

- Planar + RegularMerge

- PDPC + RegularMerge

In EE2-3.1, the planar mode of intra part of CIIP is replaced with DIMD (EE2-3.1a) or TIMD (EE2-3.1a).

# Proposal

In this proposal, a template matching CIIP (TM-CIIP) is proposed that the MV of the inter part of CIIP is refined by template matching on top of EE2-3.1. An index coding is used as follows to derive which combination of intra modes and inter modes is used:

- Index 0: PDPC + RegularMerge

- Index 1: IntraMode + RegularMerge

- Index 2: PDPC + TMMerge

- Index 3: IntraMode + TMMerge

where IntraMode is planar if TM-CIIP applied to ECM, is DIMD if TM-CIIP applied to EE2-3.1a, and is TIMD if TM-CIIP applied to EE2-3.1b.

# Simulation results

The TM-CIIP combined with EE2-3.1 was tested using the common test conditions as defined in JVET-W2017 [3]. Table 1 and Table 2 shows the overall BD-rate performance.

Table 1. Performance of TM-CIIP combined with EE2-3.1a

|  |  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over ECM-2.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | -0.07% | -0.02% | 103% | 100% |
| Class C | -0.11% | 0.08% | 0.05% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.06% | -0.12% | 0.15% | 101% | 100% |
| Class F | -0.02% | -0.01% | -0.11% | 100% | 99% |
| Class TGM |  |  |  |  |  |
|  |  | Low delay B Main10 |  |  |  |  |  |
|  |  | Over ECM-2.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.25% | -0.26% | 0.04% | 106% | 103% |
| Class E | 0.00% | -0.60% | -0.86% | 104% | 100% |
| Overall |  |  |  |  |  |
| Class D | -0.09% | -0.38% | -0.51% | 104% | 104% |

Table 2. Performance of TM-CIIP combined with EE2-3.1b

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.12% | -0.06% | -0.10% | 103% | 99% |
| Class C | -0.11% | -0.07% | 0.00% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.06% | -0.29% | 0.04% | 103% | 100% |
| Class F | -0.05% | -0.07% | -0.10% | 101% | 100% |
| Class TGM | 0.00% | -0.03% | -0.01% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.28% | -0.09% | 0.07% | 102% | 100% |
| Class E | 0.05% | -0.65% | -0.68% | 101% | 100% |
| Overall |  |  |  |  |  |
| Class D | -0.10% | -0.20% | 0.11% | 100% | 97% |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).