| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021 | Document: JVET-X0202-v1 |
| --- | --- |

| Title: | AHG4: preparation of spatial scalability verification tests |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Information |  |  |  |
| Author(s) or Contact(s): | P. de Lagrange F. Urban E. François W. Hammidouche | Email: | philippe.delagrange@interdigital.com |
| Source: | Interdigital, IETR/INSA |  |  |  |

_____________________________

# Abstract

This contribution reports ongoing work to prepare visual tests for VVC in spatial scalability mode. The purpose of these tests is two-fold. Firstly, they aim at providing an evaluation report of VVC scalable profile in response to the TV 3.0 project Call for Proposals from the Brazilian Digital Terrestrial TV Forum (see JVET-U0128 and JVET-V0167). Secondly, these tests can be used as a basis for JVET verification tests of VVC in scalable mode.

# Introduction

The Brazilian Digital Terrestrial TV Forum (SBTVD Forum) has released in July 2020 a publicly available Call for Proposals [1, 2] for the specification of the next generation of the Brazilian Digital Terrestrial Television system, named "TV3.0" project. For the video coding part, VVC has been proposed by several organizations, namely DiBEG, Ateme, Fraunhofer HHI, and InterDigital, as a candidate addressing both single-layer coding and scalable dual-layer coding.

Objective and subjective tests results are requested in response to the CfP. Objective results based on the SBTVD CfP test conditions have already been reported. For visual tests, it is not requested to use the CFP test content, and JVET SDR and HDR VVC verification tests reports have been provided for the single-layer coding configuration. InterDigital and IETR/INSA are preparing visual tests to address the dual-layer scalable coding case. The target deadline for providing these results is October 29, 2021.

The focus in on spatial scalability since SBTVD requested performance tests for spatial scalability, but only functional tests for SNR and temporal scalability.

This work can be factorized and re-used as a basis for JVET verification tests of VVC in spatial scalability mode.

# Currently considered test settings

Considering the configurations used in the SBTVD TV3.0 CfP and the tight schedule, we have chosen to provide visual test results for the following settings.

## Layer settings

Test of spatial scalability, with horizontal/vertical spatial ratio 2x, with HD resolution as base layer, 4K resolution as enhancement layer.

## Anchors

As in SHVC verification tests, the main anchor considered is simulcast (bitrates of single layer at HD and 4K resolutions are added), with single layer at 4K resolution as a secondary anchor, as the overhead of multilayer vs single layer is the metric used by SBTVD in the TV3.0 CfP.

## Test content

Content from each category (SDR, HDR PQ, HDR HLG) is considered. The enable conducting those tests outside the scope of SBTVD, e.g. under the umbrella of JVET, the TV3.0 CfP specific test sequences will not be used.

It is planned to reuse the 4K content already selected for previous JVET VVC verification tests; this provides readily available anchors and saves content selection time as these contents have already been identified as relevant for visual tests.

Additional SDR content, not used in VVC verification tests, is also considered to be added.

- Reusing part of the SHVC verification test content may be desirable, in particular the two 4K clips (from NetFlix ElFuente) are being considered, so that the performance of SHVC and scalable VVC can be compared.

- More content with lower frame rate (e.g. 25fps) is also considered, to evaluate the impact of shorter IRAP periods on spatial scalability coding performance.

### Content preparation process

Content available in ProRes 422 format is converted to 10-bit YUV420 using ffmpeg v4.1.6 as detailed below.

Content from SHVC tests is actually re-extracted from the NetFlix ElFuente original “.mov” source (SDR, BT.709 color) to get a 10-bit test sequence (SHVC test was using 8-bit):

ffmpeg -y -ss 00:03:20.2 -i NETFLIX_ElFuente_4096x2160_28592frames_5994fps.mov -f rawvideo -pix_fmt yuv420p10le -sws_flags lanczos+accurate_rnd+full_chroma_inp -vsync 0 -vf crop=3840:2160:128:0,scale=out_v_chr_pos=128:out_h_chr_pos=0:3840:2160,setsar=1/1 -frames 600 ElFuente_12000.yuv

ffmpeg -y -ss 00:04:08.5817 -i NETFLIX_ElFuente_4096x2160_28592frames_5994fps.mov -f rawvideo -pix_fmt yuv420p10le -sws_flags lanczos+accurate_rnd+full_chroma_inp -vsync 0 -vf crop=3840:2160:128:0,scale=out_v_chr_pos=128:out_h_chr_pos=0:3840:2160,setsar=1/1 -frames 600 ElFuente_14900.yuv

Content from ITE (HLG, BT.2020 color) is converted as follows:

ffmpeg -y -ss 00:00:04.004 -i c07_Drama_sea_4K.mov -vf zscale=f=lanczos:c=topleft -frames 600 -pix_fmt yuv420p10le /home/videocandidates/dibeg/c07_DramaSea_3840x2160_60fps_10bit_420_BT2100HLG.yuv

ffmpeg -y -ss 00:00:05.005 -i c16_Paddock_follow_4K.mov -vf zscale=f=lanczos:c=topleft -frames 600 -pix_fmt yuv420p10le /home/videocandidates/c16_PaddockFollow_3840x2160_60fps_10bit_420_BT2100HLG.yuv

The base layer HD video source is then derived by downscaling the 4K source using the “TAppDownConvertStatic” software provided in the SHM. The downscale filters are exactly the same as those used in VVC RPR, and it is possible to control the chroma location.

Correctness of chroma location for both ffmpeg and TAppDownConvertStatic was carefully checked. Also, it was verified that TAppDownConvertStatic downscale and VVC upscale use the same luma phase (zero phase, meaning first pixel-center aligned, not image-centered), so that inter-layer reference (upscaled HD base layer) is exactly aligned with 4K layer.

For BT.709-color content (chroma location 0):

TAppDownConvertStatic 3840 2160 <input_4K.yuv> 1920 1080 <output_HD.yuv> 10 10

For BT.2020-color content (chroma location 2):

TAppDownConvertStatic 3840 2160 <input_4K.yuv> 1920 1080 <output_HD.yuv> 10 10 0 0 0 600 -phase 0 0 0 0

The downscale process described here does not change the colorspace, thus 4K BT.2020 content is downscaled to BT.2020 HD, which is not quite right from an industry standpoint, but allows to separate testing of spatial scalability and color gamut scalability. We do not plan to test color gamut scalability.

It is noted that all test content of TV3.0 CfP are in BT.2020 color space, even in HD resolution or lower, which may indicate that SBTVD is not interested by changing colorspace while performing spatial scalability.

The thumbnails shown below are generated from the 4K sources using ffmpeg and Gimp as follows:

ffmpeg -y -pix_fmt yuv420p10le -s 3840x2160 -framerate 59.94 -ss <timecode> -i <input.yuv> -vf zscale=f=lanczos:pin=2020:tin=arib-std-b67:min=2020_ncl:p=709:t=iec61966-2-1:m=709:npl=400 -s 480x270 -frames 1  <output.png>

with colorspace settings (pin, tin, min) and peak luminance (npl) depending on content (HLG shown here at 400 nits).

Then, the .png from HDR sources are opened in Gimp v2.10.8 and manually tone-mapped using the “Reinhard 2005” method, then saved to jpg. SDR thumbnails are untouched.

## Encoding settings

- As in SHVC visual tests, a balanced distribution between the base and the enhancement layers bitrate is targeted (EL/BL ratio equal to 1). [to be discussed: we may want to try lower EL/BL too]

- However, the 0.5s IRAP period used in SHVC visual tests is judged unrealistic in terms of real-life applications, so a 1s IRAP period is considered instead, as usual in random access configuration.

- Enabling MCTF or not is still an open issue, as its impact in scalable coding is unknown; simulations are running to assess the benefits of using this feature for the visual tests. It is observed from initial results that MCTF can save up to 30% bitrate at low QPs.

- The rate points selection is based on the following approach:

  - For content reused from SHVC tests, choose 2 or 3 QPs higher than those used for HEVC, targeting a luma PSNR slightly lower, as it has been observed on VVC visual tests that a slightly low PSNR compared to HEVC is judged slightly better by viewers due to the properties of VVC. An offset of -0.3dB to -0.5dB seems a good rule of thumb.

  - For content reused from VVC tests, select the same QPs for the enhancement layer.

  - The QP of the base layer is adjusted to be make its bitrate as close as possible to the enhancement layer. This requires a bit of trial and error. The QP offset is found to be very often equal to -2 or -3.

# Test logistics and method

## Test lab

As GBTech, Vabtech and RWTH labs are busy with other tests, IETR/INSA kindly accepts to perform the visual tests. This activity will be carried-out by IETR DIVAS platform, however with no guarantee to match the October 29 deadline.

## Test method

The tests are planned to be conducted using the degradation category rating (DCR) test method [3] with an 11-grade impairment scale as specified in Rec. ITU-R BT.500-14 [4], shown in Figure 1 below:

Figure 1: Meaning of the 11 grades numerical quality scale

# Candidate sequences and rate points

## SDR tests sequences

Currently, two sequences from SHVC visual tests are considered.

More sequences from JVET HDR verification tests or other sources are under investigation.

Table 1 – SDR test sequences under consideration (from SHVC VT)

| Test sequence & format | HD Base layer QPs | 4K Enhancement layer QPs | Thumbnail |
| --- | --- | --- | --- |
| ElFuente_12000 (Procession) 3840×2160, 60fps, 600 frames | 34 30 26 22 | 35 32 28 25 |  |
| ElFuente_14900 (RiverBoat) 3840×2160, 60fps, 600 frames | 34 30 26 22 | 36 32 28 25 |  |

Table 2 – Potential extra SDR test sequences from JVET VT

| Test sequence & format | HD Base layer QPs | 4K Enhancement layer QPs | Thumbnail |
| --- | --- | --- | --- |
| Marathon2 3840×2160, 30fps, 300 frames | 43 39 36 31 27 | 45 41 38 33 29 |  |
| MountainBay2 3840×2160, 30fps, 300 frames | 41 38 34 31 28 | 43 39 36 33 30 |  |
| TallBuildings2 3840×2160, 30fps, 300 frames | 42 38 33 29 25 | 44 40 36 32 28 |  |

Table 3 – Potential extra SDR test sequences from Arte

| Test sequence & format | HD Base layer QPs | 4K Enhancement layer QPs | Thumbnail |
| --- | --- | --- | --- |
| ChambordRoofs 3840×2160, 25fps, 250 frames | 42 38 34 30 25 | 44 40 36 32 28 |  |
| CorsaireJoy 3840×2160, 25fps, 250 frames | 42 38 34 30 26 | 44 40 36 32 28 |  |
| CorsaireNight 3840×2160, 25fps, 250 frames | 42 38 34 30 25 | 44 40 36 32 28 |  |
| TwoBirdsInDawn 3840×2160, 25fps, 250 frames | 42 38 34 30 26 | 44 40 36 32 28 |  |
| AngelInSunrise 3840×2160, 25fps, 250 frames | 42 38 34 30 26 | 44 40 36 32 28 |  |
| MarvelButtresses 3840×2160, 25fps, 250 frames | 41 37 33 29 25 | 44 40 36 32 28 |  |
| TravellingOut 3840×2160, 25fps, 250 frames | 42 38 34 29 25 | 44 40 36 32 28 |  |
| SeaMountVillage 3840×2160, 25fps, 238 frames | 41 37 33 29 25 | 44 40 36 32 28 |  |

The latter sequences from Arte need refinement of QPs after expert viewing (current QPs are arbitrary).

## HDR HLG tests sequences

Table 4 – HDR HLG test sequences under consideration

| Test sequence & format | HD Base layer QPs | 4K Enhancement layer QPs | Thumbnail |
| --- | --- | --- | --- |
| H3_AMS01_3840x2160_10bit_420_HLG (TramBridge) 3840×2160, 60fps, 600 frames | 40 36 32 28 24 | 43 39 35 31 27 |  |
| H3_AMS06_3840x2160_10bit_420_HLG (CanalWithTrees) 3840×2160, 60fps, 600 frames | 39 36 32 29 25 | 42 39 35 32 28 |  |
| RiverByBoat_3840x2160_60fps_10bit_HLG_420_type2 3840×2160, 60fps, 600 frames | 44 40 36 32 28 | 47 43 39 35 31 |  |
| C07_DramaSea 3840×2160, 60fps, 600 frames | 38 34 30 26 21 | 41 37 33 29 24 |  |
| C16_PaddockFollow 3840×2160, 60fps, 600 frames | 43 38 33 28 23 | 45 40 35 30 26 |  |

## HDR PQ tests sequences

Table 5 – HDR PQ test sequences under consideration

| Test sequence & format | HD Base layer QPs | 4K Enhancement layer QPs | Thumbnail |
| --- | --- | --- | --- |
| Chimera HDR5 (DrivingPOV_HDR) 3840×2160, 60fps, 600 frames | 42 36 30 26 19 | 45 38 32 28 22 |  |
| Meridian HDR2 (Car) 3840×2160, 60fps, 600 frames | 35 32 28 24 19 | 38 34 30 26 22 |  |
| Meridian HDR5 (Beach) 3840×2160, 60fps, 600 frames | 35 32 28 23 18 | 38 34 30 26 22 |  |
| Sparks DirtLot 3840×2160, 60fps, 600 frames | 41 35 30 24 18 | 44 38 33 27 22 |  |
| Sparks Welding 3840×2160, 60fps, 600 frames | 41 35 31 25 20 | 44 38 33 27 22 |  |

# Current results

## Bitrate balance between layers

The QP of enhancement layer (layer 1) is the same as 4K single layer, and is selected to span the range of MOS. When the sequence has been used for visual tests, the QPs are just reused.

The QP of base layer (layer 0) is select to get bitrates that are roughly equal for base layer and enhancement layer. This means that the QP difference between layers may not be constant.

Below is an example with ElFuente_12000 sequence (no MCTF, RAP 1s):

Table 6 – PSNR_Y results for ElFuente_12000 (no MCTF, RAP 1s)

| QP (L0) | Bitrate (L0) | PSNR_Y (L0) | QP (L1) | Bitrate (L1) | PSNR_Y (L1) | EL/BL | Bitrate (4K) | PSNR_Y (4K) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 22 | 10654 | 43.79 | 25 | 9200 | 42.12 | 0.86354 | 18458 | 42.33 |
| 26 | 5630 | 41.73 | 28 | 5298 | 40.99 | 0.94108 | 10136 | 41.25 |
| 30 | 3192 | 39.63 | 32 | 2606 | 39.41 | 0.81665 | 5469 | 39.79 |
| 34 | 1853 | 37.48 | 35 | 2156 | 38.22 | 1.16338 | 3690 | 38.62 |

Figure 2: EL/BL balance for ElFuente_12000 (no MCTF, RAP 1s)

It is noted that the enhanced layer bitrate is impacted in opposite direction by base layer bitrate (i.e. inter-layer prediction benefits from the quality of the base layer). Thus, considering layer 1 alone in comparison to single layer 4K is not sufficient; it is preferred to compare dual layer compared to simulcast. Speaking of simulcast, single layer HD QPs should be aligned with those of the base layer, which leads to almost identical coding results: base layer and single layer HD can be considered the same thing.

## Impact of MCTF

MCTF does have a positive impact on R/D performance for both single layer coding and dual layer coding, but this impact seems to be equivalent in both cases: very little difference in performance is seen when comparing dual layer coding to simulcast.

Table 7 – Dual-layer performance impact of MCTF (ElFuente sequences, RAP 1s)

| Name | BD-rate vs single layer (PSNR_Y) | BD-rate vs simulcast (PSNR_Y) | BD-rate vs single layer (SSIM_Y) | BD-rate vs simulcast (SSIM_Y) |
| --- | --- | --- | --- | --- |
| ElFuente_12000 (no MCTF) | +22.0% | -21.9% | +23.1% | -21.2% |
| ElFuente_12000 (MCTF) | +22.0% | -22.9% | +22.7% | -22.2% |
| ElFuente_14900 (no MCTF) | +21.5% | -22.3% | +20.8% | -22.7% |
| ElFuente_14900 (MCTF) | +21.5% | -22.6% | +20.6% | -23.0% |

Figure 3: Impact of MCTF on dual-layer coding performance (ElFuente_12000).
Solid lines: without MCFT, dashed lines: with MCTF

## Impact of Random-Access Period

When comparing the performance of dual-layer coding of VVC shown above to the SHVC verification test results where the same sequences were used, it is noted that VVC significantly underperforms SHVC (see JCTVC-V0069).

This may be partly due to the random-access period of 0.5s used in SHVC tests, which may favor dual layer coding (it is expected that inter-layer prediction benefit is highest in Intra/RAP pictures) [waiting for simulation results]. A significant quality drop is also observed globally for layer 1 compared to single layer, which may be abnormal.

Table 8 – Dual-layer performance comparison of VVC vs SHVC (ElFuente sequences)

| Name | BD-rate vs single layer (PSNR_Y) | BD-rate vs simulcast (PSNR_Y) |
| --- | --- | --- |
| ElFuente_12000 (SHVC) | +12.5% | -27.0% |
| ElFuente_12000 (VVC, RAP 1s) | +22.0% | -22.9% |
| ElFuente_12000 (VVC, RAP 0.5s) |  |  |
| ElFuente_14900 (SHVC) | +12.2% | -28.4% |
| ElFuente_14900 (VVC, RAP 1s) | +21.5% | -22.6% |
| ElFuente_14900 (VVC, RAP 0.5s) |  |  |

Figure 4: Comparison of VVC (left) and SHVC (right) R/D curves (ElFuente_12000)

## Bitrates and metrics for SDR sequences

Table 9 – Dual-layer BD-Rate performance of SDR sequences

| Name | MCTF | BD-rate vs single layer (PSNR_Y) | BD-rate vs simulcast (PSNR_Y) | BD-rate vs single layer (SSIM_Y) | BD-rate vs simulcast (SSIM_Y) |
| --- | --- | --- | --- | --- | --- |
| ElFuente_12000 |  | +22.0% | -21.9% | +23.1% | -21.2% |
| ElFuente_12000 | x | +22.0% | -22.9% | +22.7% | -22.2% |
| ElFuente_14900 |  | +21.5% | -22.3% | +20.8% | -22.7% |
| ElFuente_14900 | x | +21.5% | -22.6% | +20.6% | -23.0% |
| Marathon2 (incomplete) |  | +22.5% | -18.5% | +23.7% | -17.5% |
| Marathon2 (incomplete) | x | +22.6% | -19.6% | +23.6% | -18.8% |
| MountainBay2 |  | +32.4% | -12.6% | +27.9% | -15.3% |
| MountainBay2 | x | +32.5% | -13.0% | +27.9% | -15.8% |
| TallBuildings2 (EL/BL too high) |  | +44.7% | -5.2% | +39.1% | -8.2% |
| TallBuildings2 (EL/BL too high) | x | +44.0% | -5.9% | +39.1% | -8.5% |
| ChambordRoofs |  | +41.4% | -10.3% | +39.2% | -11.7% |
| ChambordRoofs | x | +41.7% | -10.6% | +39.1% | -12.0% |
| CorsaireJoy (EL/BL too low) |  | +18.2% | -24.3% | +18.9% | -23.7% |
| CorsaireJoy (EL/BL too low) | x | +19.1% | -24.0% | +20.4% | -23.0% |
| CorsaireNight (EL/BL a little low) |  | +29.0% | -17.5% | +29.2% | -17.4% |
| CorsaireNight (EL/BL a little low) | x | +29.2% | -17.7% | +29.2% | -17.7% |
| TwoBirdsInDawn |  | +37.2% | -11.5% | +36.1% | -12.2% |
| TwoBirdsInDawn | x | +37.1% | -11.7% | +35.8% | -12.4% |
| AngelInSunrise (EL/BL too low) |  | +31.1% | -19.2% | +30.8% | -19.4% |
| AngelInSunrise (EL/BL too low) | x | +29.8% | -20.2% | +29.5% | -20.4% |
| MarvelButtresses (EL/BL too high) |  | +43.5% | -4.3% | +37.8% | -8.4% |
| MarvelButtresses (EL/BL too high) | x | +42.9% | -5.6% | +37.0% | -9.6% |
| TravellingOut (EL/BL a little high) |  | +37.7% | -9.7% | +30.7% | -13.8% |
| TravellingOut (EL/BL a little high) | x | +37.0% | -10.5% | +29.3% | -15.0% |
| SeaMountVillage (EL/BL too high) |  | +51.6% | -0.7% | +46.7% | -3.9% |
| SeaMountVillage (EL/BL too high) | x | +51.6% | -0.7% | +45.8% | -4.3% |

The corresponding EL/BL balance and R/D curves are shown in the following figures:

Figure 5: R/D curves for ElFuente_12000

Figure 6: R/D curves for ElFuente_14900

Figure 7: R/D curves for Marathon2

Figure 8: R/D curves for MountainBay2

Figure 9: R/D curves for TallBuildings2

Figure 10: R/D curves for ChambordRoofs

Figure 11: R/D curves for CorsaireJoy

Figure 12: R/D curves for CorsaireNight

Figure 13: R/D curves for TwoBirdsInDawn

Figure 14: R/D curves for AngelInSunrise

Figure 15: R/D curves for MarvelButtresses

Figure 16: R/D curves for TravellingOut

Figure 17: R/D curves for SeaMountVillage

## Bitrates and metrics for HDR HLG sequences

Table 9 – Dual-layer BD-Rate performance of HDR HLG sequences

| Name | MCTF | BD-rate vs single layer (PSNR_Y) | BD-rate vs simulcast (PSNR_Y) | BD-rate vs single layer (SSIM_Y) | BD-rate vs simulcast (SSIM_Y) |
| --- | --- | --- | --- | --- | --- |
| TramBridge (EL/BL way too high) |  | +41.2% | -2.5% | +38.9% | -5.2% |
| CanalWithTrees (EL/BL way too high) |  | +28.5% | -7.2% | +28.2% | -8.0% |
| RiverByBoat (EL/BL too high) |  | +31.3% | -9.6% | +31.1% | -9.4% |
| DramaSea (EL/BL a little high) |  | +20.9% | -21.8% | +26.1% | -18.5% |
| PaddockFollow (EL/BL a little high) |  | +22.9% | -18.4% | +20.7% | -19.9% |

Figure 17: R/D curves for TramBridge

Figure 17: R/D curves for CanalWithTrees

Figure 17: R/D curves for RiverByBoat

Figure 17: R/D curves for DramaSea

Figure 17: R/D curves for PaddockFollow

## Bitrates and metrics for HDR PQ sequences

Table 9 – Dual-layer BD-Rate performance of HDR HLG sequences

| Name | MCTF | BD-rate vs single layer (PSNR_Y) | BD-rate vs simulcast (PSNR_Y) | BD-rate vs single layer (SSIM_Y) | BD-rate vs simulcast (SSIM_Y) |
| --- | --- | --- | --- | --- | --- |
| DrivingPOV_HDR |  | +28.1% | -16.6% | +28.8% | -16.1% |
| MeridianCar_HDR (EL/BL a little high) |  | +30.5% | -14.3% | +30.6% | -14.3% |
| MeridianBeach_HDR |  | +27.2% | -16.5% | +25.7% | -17.4% |
| SparksDirtLot |  | +26.7% | -18.2% | +23.8% | -20.6% |
| SparksWelding |  | +14.8% | -24.0% | +14.4% | -23.8% |

Figure 17: R/D curves for DrivingPOV_HDR

Figure 17: R/D curves for MeridianCar_HDR

Figure 17: R/D curves for MeridianBeach_HDR

Figure 17: R/D curves for SparksDirtLot

Figure 17: R/D curves for SparksDirtLot

# References

[1] Call for Proposals: TV 3.0 Project, Brazilian Digital Terrestrial Television System Forum, July 2020,  https://forumsbtvd.org.br/wp-content/uploads/2020/07/SBTVDTV-3-0-CfP.pdf

[2]	CfP Phase2 / Testing and Evaluation: TV3.0 Project, Brazilian Digital Terrestrial Television System Forum, March 2021, https://forumsbtvd.org.br/wp-content/uploads/2021/03/SBTVD-TV_3_0-P2_TE_2021-03-15.pdf

[3]	Recommendation ITU-T P.910 (2008), Subjective video quality assessment methods for multimedia applications.

[4]	Recommendation ITU-R BT.500-14 (2019), Methodologies for the subjective assessment of the quality of television images.

[5]	M. Wien, V. Baroncini, “VVC Verification Test Report for Ultra High Definition (UHD) Standard Dynamic Range (SDR) Video Content” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 document JVET-T2020, 20th JVET meeting, teleconference, October 2020.

[6]	M. Wien, V. Baroncini, “VVC Verification Test Report for High Dynamic Range Video Content” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 document JVET-W2020, 23rd JVET meeting, teleconference, July 2021.

[7]	Y. He, Y. Ye, F. Henry, Y.K. Wang, V. Baroncini, “Proposed SHVC verification test draft plan” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 document JCTVC-V0069, 22nd meeting, Geneva, CH, October 2015.

[8]	Y. He, V. Baroncini, Y.K. Wang, “SHVC verification test report” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 document JCTVC-W1004, 23rd meeting, San Diego, USA, February 2016.