| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0046-v1 |
| --- | --- |

| Title: | AHG11: ALF improvement for NNVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenjie Zou, Yi Zhou, Cheng Huang, and Yaxian, Bai | Email: | wjzou@xidian.edu.cn, yi.zhou@stu.xidian.edu.cn huang.cheng5@zte.com.cn bai.yaxian@zte.com.cn |
| Source: | Xidian University, ZTE Corporation |  |  |

_____________________________

- Abstract

This contribution presents an ALF modification in NNVC. When a neural network-based filter is used as in-loop filter, two ALF flags, which indicate ALF enabled/disable, are introduced for luma and chroma components, respectively. It is reported that, with JVET-X0065 as anchor and test platform, the overall performance are as follows:

- AI: 0.03%/-0.96%/-1.04% with 100%EncT/100%DecT,

- RA: 0.05%/-0.30%/-0.41% with 100%EncT/100%DecT.

Introduction

As shown in Figure 1b, in current solutions for NNVC, the NN filter generally replaces the DBF and SAO modules. ALF module still can improve the coding performance after the NN filter.

(a)

(b)

Figure 1: (a) Original in-loop filter process. (b) in-loop filter process after adding NN filter

In VVC [2], shown below and in Figure 2, there is an ALF Flag to control whether ALF is enable or disable. When this flag is on, luma ALF is always used, and then the chroma ALF and CCALF, are further be controlled by their own flags. In other words, if the AFL flag is off, the chroma ALF and CCALF will be forcibly closed.

Figure 2: Syntax structure in VVC

Benefitted from the advantage of ALF, the ALF Flag is almost always on using VVC encoder. So, subsequent chroma ALF and CCALF can obtain the most optimal decision. However, in the NN filter + ALF combination, due to a better performance provided by the NN filter than the DBF and SAO, a large number of ALF Flag determined by the luma component will be turned off. This process will limit the optimization of the chroma ALF and CCALF and thus reduce the overall performance.

Here, the related semantics of ALF in VVC is provided as follows.

sh_alf_enabled_flag equal to 1 specifies that ALF is enabled for the Y, Cb, or Cr colour component of the current slice. sh_alf_enabled_flag equal to 0 specifies that ALF is disabled for all colour components in the current slice.

sh_alf_cb_enabled_flag equal to 1 specifies that ALF is enabled for the Cb colour component of the current slice. sh_alf_cb_enabled_flag equal to 0 specifies that ALF is disabled for the Cb colour component of the current slice.

sh_alf_cr_enabled_flag equal to 1 specifies that ALF is enabled for the Cr colour component of the current slice. sh_alf_cr_enabled_flag equal to 0 specifies that ALF is disabled for the Cr colour component of the current slice.

sh_alf_cc_cb_enabled_flag equal to 1 specifies that CCALF is enabled for the Cb colour component. sh_alf_cc_cb_enabled_flag equal to 0 specifies that CCALF is disabled for the Cb colour component.

sh_alf_cc_cr_enabled_flag equal to 1 specifies that CCALF is enabled for the Cr colour component of the current slice. sh_alf_cc_cr_enabled_flag equal to 0 specifies that CCALF is disabled for the Cr colour component.

Proposed ALF-SPLIT method

A method called ALF-SPLIT is proposed to optimize the ALF module used in NNVC. As shown in Figure 3, the changes of the ALF flags are as follows：

- The existing ALF flag is only used to indicate whether luma ALF is on or off.

- An additional ALF flag is introduced to indicate whether chroma ALF and CCALF are on or off.

In this way, the ALF flag for the luma and chroma components can be determined independently.

Figure 3: Modified Syntax structure

Experimental results

First, the distribution of the ALF flag in the VTM11.0+MCTF [4], JVET-X0065, and JVET-X0065+AFL-SPLIT for AI configuration is shown in Table 1. The percentage of the ALF ON is calculated as:

R=NumALFon/NumSlice

where NumALFon represents the number of slices where the ALF flag is set to be ON, NumSlice is the number of slices.

In VTM11.0+MCTF, the ALF flag is 100% on and an extremely high percent for chroma components, respectively. When the NN filter is used, the percentage of ALF flag ON for all components significantly reduced. Here, we used the NN filter proposed in JVET-X0065 as an example. With the proposed ALF-SPLIT method, the ratio of ALF flag ON for chroma and CCALF increases. This phenomenon illustrates that the proposed method can let more ALF usage of the chroma components.

Table 1. The ratio of ALF ON(AI)

|  | VTM11.0+MCTF [4] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ALF |  | Cb ALF | Cr ALF | Cb CCALF | Cr CCALF |
| Class A1 | 100.00% |  | 100.00% | 100.00% | 46.55% | 57.88% |
| Class A2 | 100.00% |  | 100.00% | 100.00% | 60.00% | 60.00% |
| Class B | 100.00% |  | 95.78% | 95.78% | 60.00% | 56.01% |
| Class C | 100.00% |  | 97.24% | 97.24% | 57.57% | 53.56% |
| Class E | 100.00% |  | 100.00% | 100.00% | 60.00% | 51.02% |
| Overall | 100.00% |  | 94.53% | 94.53% | 52.57% | 49.05% |
| Class D | 100.00% |  | 74.39% | 74.39% | 32.80% | 24.27% |
| Class F | 100.00% |  | 99.75% | 99.75% | 48.70% | 47.87% |
|  | JVET-X0065 |  |  |  |  |  |
|  | ALF |  | Cb ALF | Cr ALF | Cb CCALF | Cr CCALF |
| Class A1 | 80.71% |  | 80.71% | 80.71% | 44.25% | 47.08% |
| Class A2 | 97.37% |  | 95.96% | 95.96% | 58.60% | 58.60% |
| Class B | 78.97% |  | 68.72% | 68.72% | 45.36% | 43.70% |
| Class C | 54.06% |  | 45.44% | 45.44% | 29.37% | 26.95% |
| Class E | 100.00% |  | 100.00% | 100.00% | 60.00% | 37.07% |
| Overall | 75.01% |  | 68.16% | 68.16% | 39.17% | 34.70% |
| Class D | 28.28% |  | 9.21% | 9.21% | 13.31% | 10.79% |
| Class F | 100.00% |  | 99.83% | 99.83% | 34.48% | 33.64% |
|  | JVET-X0065+ALF-SPLIT |  |  |  |  |  |
|  | ALF | Chroma ALF | Cb ALF | Cr ALF | Cb CCALF | Cr CCALF |
| Class A1 | 80.71% | 100.00% | 100.00% | 100.00% | 49.73% | 52.92% |
| Class A2 | 97.37% | 98.60% | 98.60% | 98.60% | 60.00% | 60.00% |
| Class B | 78.97% | 86.55% | 86.04% | 86.04% | 60.00% | 57.89% |
| Class C | 54.06% | 87.03% | 74.56% | 74.56% | 51.55% | 51.63% |
| Class E | 100.00% | 100.00% | 100.00% | 100.00% | 60.00% | 37.07% |
| Overall | 75.01% | 88.63% | 84.25% | 84.25% | 49.59% | 43.43% |
| Class D | 28.28% | 61.26% | 46.61% | 46.61% | 32.64% | 17.41% |
| Class F | 100.00% | 99.83% | 99.83% | 99.83% | 34.48% | 33.64% |

Based on the platform in JVET-X0065 [1], coding performance experiment was conducted following the common test conditions defined in [3]. Experimental results are shown in Table 2 and Table 3. The proposed method can obtain 0.03%, -0.96%, and -1.04% BD-rate for AI configuration on average for Y, Cb, and Cr components, respectively. For RA configurations, the proposed method can bring 0.05%, -0.30%, and -0.41% BD-rate on average for Y, Cb, and Cr components, respectively. At the same time, almost no increase in encoding and decoding time was led by the proposed ALF-SPLIT method.

Table 2. Performance of the proposed method on top of JVET-X0065 (AI)

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -0.72% | -2.75% | 100% | 101% |
| Class A2 | 0.00% | -0.17% | -0.21% | 100% | 101% |
| Class B | 0.04% | -1.68% | -1.02% | 101% | 100% |
| Class C | 0.06% | -1.55% | -1.17% | 100% | 101% |
| Class E | 0.01% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.03% | -0.96% | -1.04% | 100% | 100% |
| Class D | 0.12% | -1.53% | -1.27% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

Table 3. Performance of the proposed method on top of JVET-X0065 (RA)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | -0.46% | -0.96% | 100% | 100% |
| Class A2 | 0.05% | -0.08% | -0.56% | 100% | 100% |
| Class B | 0.02% | -0.39% | -0.07% | 100% | 100% |
| Class C | 0.04% | -0.23% | -0.33% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | -0.30% | -0.41% | 100% | 100% |
| Class D | 0.04% | -0.31% | -0.46% | 100% | 100% |
| Class F | 0.02% | -0.10% | -0.08% | 100% | 100% |

- 4 Conclusions

This contribution provides more flexibility for luma ALF and chroma ALF. The results from this contribution demonstrated that the proposed ALF-SPLIT shows coding gains on BD-Rate for RA/AI configurations compare with the anchor of JVET-X0065 [1].

- 5 References

- Y. Li, K. Zhang, L. Zhang, EE1-1.2: Test on Deep In-Loop Filter with Adaptive Model Selection and External Attention, Document JVET-X0065, online, October 2021.

- ITU-T Study Group, Series H: Audiovisual and multimedia systems: Infrastructure of audiovisual services–coding of moving video-versatile video coding, in General Secretariat and Telecom Radiocommunication (ITU-R) Standardization (ITU-T), sec. H 266, 2020.

- S. Liu, A. Segall, E. Alshina, R. -L. Liao, JVET common test conditions and evaluation procedures for neural network-based video coding technology, Document JVET-W2016, July 2021.

- https://vcgit.hhi.fraunhofer.de/jvet-w-ee1/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc

- 6 Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).