| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0048-v4 |
| --- | --- |

| Title: | AHG10: study of layer bitrate allocation for spatial scalability in VTM |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Information |  |  |  |
| Author(s) or Contact(s): | P. de Lagrange F. Urban G. Marquant | Email: | philippe.delagrange@interdigital.com |
| Source: | Interdigital |  |  |  |

_____________________________

# Abstract

First results of VVC spatial scalability visual performance tests have been reported in JVET-Y0047, with equal bitrate for HD base layer and UHD enhancement layer. However, it was noticed that the bitrate balance has a significant effect on coding performance, in favor of more bitrate in the base layer. This contribution reports experiments with various bitrate allocations, and reports gains and lower encoding time over single layer in some cases.

# Introduction

In JVET-Y0047, VVC spatial scalability was tested in conditions similar to SHVC verification test, to enable comparison. More specifically, base-layer (BL) and enhancement layer (EL) bitrates are matched (EL/BL ratio equals 1). However, exploring other bitrate allocation was welcome, as it was noted that it could have a significant impact on performance. This contribution attempts to explore the full range, between all bitrate on the HD base layer (i.e. RPR), and all bitrate on the UHD layer (i.e. single layer).

# Test settings

The same configuration based on CTCs as in JVET-Y0047 was used, with MCTF disabled, but varying bitrate allocation between HD base layer and UHD enhancement layer.

## Layer settings

Spatial scalability with horizontal/vertical spatial ratio 2x, with HD resolution as base layer (BL) and UHD resolution as enhancement layer (EL).

## Anchors

Contrary to JVET-Y0047, the only anchor considered here is UHD single layer, as we focus on spatial scalability as a coding performance tool, the two extremes of EL/BL bitrate allocation being single layer on one side (no bitrate on base layer), and pure upscale (RPR) on the other side (no bitrate on enhancement layer). Here we approximate RPR with maximum QP (63) on enhancement layer.

## Test content

The same test content as in JVET-Y0047 is used, so that results are consistent across the different tests.

## Encoding settings

VVC encodings were performed by the VTM in version 14.1, with an additional bug fix around Decoded Picture Buffer (DPB) management for correct dual-layer behaviour.

RA CTC configuration is used, except for MCTF which is disabled, as it has minor impact on coding performance of spatial scalability compared to single layer, as discussed in JVET-Y0047.

The EL/BL bitrate allocation is varied by keeping EL QP constant and equal to UHD single layer rate points, and varying BL QP. RPR is approximated by setting EL QP to 63, and varying BL QP on a wide scale.

The EL rate points are reused from JVET-Y0047, adding a lower bitrate point or two when relevant to show the benefit of spatial scalability.

# Sequences and rate points

As in JVET-Y0047, two sequences from SHVC verification tests and three from VVC SDR verification tests are used, plus three additional sequences from Arte.

Table 1 – SDR test sequences from SHVC VT (60fps)

| Test sequence & format | HD Base layer QPs | UHD Enhancement layer QPs | Thumbnail |
| --- | --- | --- | --- |
| ElFuente-12000 (Procession) 3840×2160, 60fps, 600 frames | 16 - 52 42 - 47 36 - 47 30 - 40 24 - 33 18 - 27 | 63 50 46 40 34 28 |  |
| ElFuente-14900 (RiverBoat) 3840×2160, 60fps, 600 frames | 16 - 52 35 - 47 37 - 48 31 - 41 25 - 34 19 - 28 | 63 50 47 41 35 29 |  |

Table 2 – SDR test sequences from VVC VT (30fps)

| Test sequence & format | HD Base layer QPs | UHD Enhancement layer QPs | Thumbnail |
| --- | --- | --- | --- |
| Marathon2 3840×2160, 30fps, 300 frames | 16 - 52 36 - 44 35 - 44 31 - 40 28 - 37 23 - 32 19 - 28 | 63 49 45 41 38 33 29 |  |
| MountainBay2 3840×2160, 30fps, 300 frames | 16 - 52 34 - 40 33 - 42 29 - 38 26 - 35 23 - 32 20 - 29 | 63 46 43 39 36 33 30 |  |
| TallBuildings2 3840×2160, 30fps, 300 frames | 16 - 52 36 - 47 32 - 43 28 - 39 20 - 35 20 - 30 16 - 25 | 63 48 44 40 36 32 28 |  |

Table 3 – SDR test sequences from Arte (25fps)

| Test sequence & format | HD Base layer QPs | UHD Enhancement layer QPs | Thumbnail |
| --- | --- | --- | --- |
| CorsaireJoy 3840×2160, 25fps, 250 frames | 16 - 48 38 - 53 38 - 49 38 - 60 34 - 40 30 - 36 26 - 32 22 - 28 | 63 52 48 44 40 36 32 28 |  |
| TwoBirdsInDawn 3840×2160, 25fps, 250 frames | 24 - 50 38 - 42 38 - 44 34 - 40 30 - 36 26 - 32 22 - 28 | 63 48 44 40 36 32 28 |  |
| AngelInSunrise 3840×2160, 25fps, 250 frames | 24 - 50 39 - 43 38 - 44 34 - 40 30 - 36 26 - 32 22 - 28 | 63 46 44 40 36 32 28 |  |

# Selection of QPs and bitrate balance between layers

In JVET-Y0047, it is noted that the enhanced layer bitrate is impacted in opposite direction by base layer bitrate (i.e. inter-layer prediction benefits from the quality of the base layer). For comparison with SHVC verification test, the same conditions (equal BL and EL bitrate) were used, and dual layer was compared with simulcast and single layer. However, it is noted that the bitrate allocation between layer has a significant impact on performance, and that exploring other settings is desirable. This is the purpose of the following.

The QP of enhancement layer (QPEL) is the same as UHD single layer (QPSL), and reused from JVET-Y0047, but adding one or two points in the low bitrate end when relevant.

Then, for each QPEL, the QP of base layer (QPBL) is varied to observe the impact on performance compared to single layer.

Figure 1 illustrates the balance of bitrates between layers explored for a specific sequence.

- The left part shows the EL/BL bitrate ratio depending on the QPBL sweep, for each QPEL.

- The right part shows EL bitrate vs BL bitrate, with a square root scale and grid lines that indicate constant sum (=constant dual layer bitrate) and constant ratio. Subdivision of constant sum grid is 2.5x and 5x; two extreme ratios made of single layer (all bitrate on UHD) and pseudo-RPR (all bitrate on base layer) are shown as separate curves.

Figure 1: EL & BL bitrates explored for CorsaireJoy

## Bitrate balance between layers: impact on quality

Figure 2 and Figure 3 illustrate the impact on dual-layer coding performance of the bitrate balance between layers: it shows PSNRY and SSIMY RD curves of a specific video sequence, where single layer is displayed as a continuous curve, and dual layer as “C” of “6”-shaped traces that follow the QPBL sweeps for each QPEL point (that matches QPSL). Additionally, the large QPBL sweep with QPEL=63, simulating RPR, is displayed as a dashed yellow curve. Each QPBL sweep is expected to start on the single layer curve (with high enough QPBL), and end like the pseudo-RPR curve (with low enough QPBL).

Figure 2: PSNRY for CorsaireJoy, with zoom on QPEL=44

Figure 3: SSIMY for CorsaireJoy

It can be seen that for low bitrates, dual-layer coding can outperform both single-layer and RPR. This is visible on SSIM even at higher bitrates, which can indicate that the visual benefit is significant.

In Figure 2 and Figure 3, it can be observed that for a specific bitrate, two points with different distortion can be obtained (with a difference as large as 0.7 dB in PSNRY), depending on layer bitrate allocation.

We can also notice that, once inter-layer prediction is effective (i.e. BL has a sufficient quality), the UHD layer quality increases with the quality of the base layer.

## Bitrates and metrics for SDR sequences

For each sequence, one QPBL point was selected for each QPEL, attempting to optimize PSNRY and/or SSIMY BD-rate. Figure 4 illustrate the process, where selected points are circled in black; Delta-rate here is the bitrate distance to the single-layer RD curve for each point. The values can be inaccurate on the extremes of the quality range, if points are projected on extrapolated single-layer RD-curve.

It can be noted that the lower the bitrate, the lower the delta-rate. Additional QPEL curves are shown to illustrate this property, even if not used to compute the final performance metrics.

By the way, we noted that the EL/BL balance selected in JVET-0047 (ratio=1) pretty reliably selects the worst case.

Figure 4: Delta-rate sweeps for each QPEL (CorsaireJoy)

Overall, we observe that, except for low-motion highly detailed sequences at high bitrate, dual-layer coding with optimized layer bitrate allocation can be on par or outperform RPR and single layer coding, while being sometimes faster to encode and providing the multi-layer flexibility (e.g. base layer compatibility with lower-profile decoders).

Table 3 summarizes performance metrics of dual layer vs single layer, for the selected sequences and well-chosen pairs of QPs. Full details are found in a spreadsheet attached to this contribution. Decoding times may be inaccurate. The performance is vastly improved compared to JVET-Y0047, for all cases.

We noted that performance highly depends on the bitrate range included to RD-rate computation: it would improve if including lower bitrates or rejecting highest bitrates.

| Name | QPBL | QPEL = QPSL | BD-rate vs single layer (PSNR_Y) | BD-rate vs single layer (SSIM_Y) | BD-rate vs single layer (VMAF) | Enc. time | Dec. time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Procession | 22, 28, 34, 40 | 28, 34, 40, 46 | +0.4% | -4.5% | -3.4% | 95% | 101% |
| RiverBoat | 23, 29, 34, 40 | 29, 35, 41, 47 | +0.0% | -6.4% | -5.1% | 101% | 111% |
| Marathon2 | 23, 27, 32, 35, 37 | 29, 33, 38, 41, 45 | +5.6% | +2.7% | +3.1% | 75% | 99% |
| MountainBay2 | 24, 27, 30, 33, 37 | 30, 33, 36, 39, 43 | +9.7% | +3.9% | +11.6% | 124% | 93% |
| TallBuildings2 | 25, 30, 35, 34, 38 | 28, 32, 36, 40, 44 | +16.6% | +9.7% | +9.3% | 173% | 112% |
| CorsaireJoy | 23, 26, 30, 34, 38 | 28, 32, 36, 40, 44 | -0.9% | -6.2% | -3.8% | 105% | 105% |
| TwoBirdsInDawn | 23, 26, 30, 34, 38 | 28, 32, 36, 40, 44 | +4.0% | -1.4% | +0.8% | 150% | 101% |
| AngelInSunrise | 23, 27, 30, 34, 38 | 28, 32, 36, 40, 44 | +10.2% | +6.3% | +11.7% | 163% | 105% |

Table 3: BD-rate and encoding/decoding time vs single layer

### Plots for Procession (ElFuente-12000)

Figure 5: EL/BL bitrate balance (Procession)

Figure 6: Delta-rates (Procession)

Figure 7: Rate-distortion curves (Procession)

### Plots for RiverBoat (ElFuente-14900)

Figure 8: EL/BL bitrate balance (RiverBoat)

Figure 9: Delta-rates (RiverBoat)

Figure 10: Rate-distortion curves (RiverBoat)

### Plots for Marathon2

Figure 11: EL/BL bitrate balance (Marathon2)

Figure 12: Delta-rates (Marathon2)

Figure 13: Rate-distortion curves (Marathon2)

### Plots for MountainBay2

Figure 14: EL/BL bitrate balance (MountainBay2)

Figure 15: Delta-rates (MountainBay2)

Figure 16: Rate-distortion curves (MountainBay2)

### Plots for TallBuildings2

Figure 17: EL/BL bitrate balance (TallBuildings2)

Figure 18: Delta-rates (TallBuildings2)

Figure 19: Rate-distortion curves (TallBuildings2)

### Plots for CorsaireJoy

Figure 20: EL/BL bitrate balance (CorsaireJoy)

Figure 21: Delta-rates (CorsaireJoy)

Figure 22: Rate-distortion curves (CorsaireJoy)

### Plots for TwoBirdsInDawn

Figure 23: EL/BL bitrate balance (TwoBirdsInDawn)

Figure 24: Delta-rates (TwoBirdsInDawn)

Figure 25: Rate-distortion curves (TwoBirdsInDawn)

### Plots for AngelInSunrise

Figure 26: EL/BL bitrate balance (AngelInSunrise)

Figure 27: Delta-rates (AngelInSunrise)

Figure 28: Rate-distortion curves (AngelInSunrise)