| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0088-v1 |
| --- | --- |

| Title: | EE2-related: IBC with Template Matching |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Robert K. Naser T. Poirier Y. Chen F. Galpin | Email: | antoine.robert@interdigital.com karam.naser@interdigital.com tangi.poirier@interdigital.com ya.chen@interdigital.com franck.galpin@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes to enable Template Matching with IBC. The reported simulation results of the proposed method on top of the ECM-3.1 in SCC test conditions show gains of {-0.53%, -0.44%, -0.43%}, {-0.36%, -0.34%, -0.37%} and {-0.20%, -0.44%, -0.43%} for YUV in AI, RA and LDB respectively with an unchanged complexity for both encoder and decoder. On top of the EE2-3.13 test, the reported results are slightly better especially in LDB with {-0.51%, -0.47%, -0.52%}, {-0.39%, -0.27%, -0.51%} and {-0.38%, -0.69%, -1.04%} for YUV in AI, RA and LDB respectively. The combination of the EE2-3.13 test and the proposed method can achieve, on top of the ECM-3.1, {-1.18%, -1.05%, -1.09%}, {-0.94%, -0.79%, -1.08%} and {-0.77%, -1.24%, -1.34%} for YUV in AI, RA and LDB respectively also with a similar complexity for both encoder and decoder.

# Introduction

The Template Matching has been introduced in the ECM [1] first for the regular merge and AMVP modes, then, more recently, in the GPM and CIIP merge modes.

# Proposal

It is proposed to also use Template Matching with IBC for both IBC merge mode and IBC AMVP mode.

In this proposal, the IBC-TM merge list has been modified compared to the one used by regular IBC merge mode such that the candidates are selected according to a pruning method with a motion distance between the candidates as in the regular TM merge mode. The ending zero motion fulfillment (which is a nonsense regarding Intra coding) has been replaced by motion vectors to the left (-W, 0), top (0, -H) and top-left (-W, -H) CUs, then, if necessary, the list is fulfilled with the left one without pruning.

In the IBC-TM merge mode, the selected candidates are refined with the Template Matching method prior to the RDO or decoding process. The IBC-TM merge mode has been put in competition with the regular IBC merge mode and a TM-merge flag is signaled.

In the IBC-TM AMVP mode, up to 3 candidates are selected from the IBC merge list. Each of those 3 selected candidates are refined using the Template Matching method and sorted according to their resulting Template Matching cost. Only the 2 first ones are then considered in the motion estimation process as usual.

The Template Matching refinement for both IBC-TM merge and AMVP modes is quite simple since IBC motion vectors are constrained (i) to be integer and (ii) within a reference region as shown in Figure 1. So, in IBC-TM merge mode, all refinements are performed at integer precision, and in IBC-TM AMVP mode, they are performed either at integer or 4-pel precision. In both cases, the refined motion vectors in each refinement step must respect the constraint of the reference region.

X

Curr

X

X

Curr

X

X

X

X

Curr

X

X

Curr

X

Figure 1: IBC reference region depending on current CU position

# Simulation results

The proposed method has been tested on top of the ECM-3.1 software [2] under the CTC conditions [3] and in SCC configuration. The simulation results are depicted in Table 1.

Table 1: Proposed IBC-TM on top of ECM-3.1

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.59% | -0.45% | -0.42% | 102% | 98% |
| Class TGM | -0.47% | -0.43% | -0.44% | 101% | 100% |
| Overall | -0.53% | -0.44% | -0.43% | 102% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.35% | -0.37% | -0.49% | 99% | 97% |
| Class TGM | -0.36% | -0.31% | -0.26% | 98% | 97% |
| Overall | -0.36% | -0.34% | -0.37% | 99% | 97% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.20% | -0.81% | -0.72% | 101% | 99% |
| Class TGM | -0.20% | -0.06% | -0.14% | 100% | 100% |
| Overall | -0.20% | -0.44% | -0.43% | 101% | 99% |

At last JVET meeting, a test 3.13 has been added into the EE2 [4]. It aims (i) at adding spatial neighboring motion vectors and a pairwise candidate in the IBC merge list, (ii) at selecting only candidates respecting the reference region condition, and (iii) at sorting these candidates by using ARMC in the IBC merge mode.

The proposed IBC-TM mode can be used on top or in combination with this EE2-3.13 proposal. Simulation results on top of the EE2-3.13 test are shown in Table 2. These results show a good synergy of the proposal with the EE2-3.13 test.

Table 2: Proposed IBC-TM on top of EE2-3.13 test

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 + EE2-3.13 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.59% | -0.58% | -0.60% | 101% | 101% |
| Class TGM | -0.42% | -0.36% | -0.43% | 99% | 101% |
| Overall | -0.51% | -0.47% | -0.52% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-3.1 + EE2-3.13 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.39% | -0.20% | -0.61% | 102% | 101% |
| Class TGM | -0.39% | -0.33% | -0.41% | 101% | 101% |
| Overall | -0.39% | -0.27% | -0.51% | 102% | 101% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-3.1 + EE2-3.13 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.36% | -1.03% | -1.68% | 101% | 100% |
| Class TGM | -0.39% | -0.35% | -0.40% | 100% | 99% |
| Overall | -0.38% | -0.69% | -1.04% | 100% | 100% |

The combination results of the proposed IBC-TM with the EE2-3.13 test on top of the ECM-3.1 are depicted in Table 3. These results confirm the good synergy when combining the proposal with EE2-3.13 test.

Table 3: Combination of EE2-3.13 test and proposed IBC-TM on top of ECM-3.1

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.83% | -0.74% | -0.77% | 101% | 98% |
| Class TGM | -1.53% | -1.37% | -1.41% | 99% | 97% |
| Overall | -1.18% | -1.05% | -1.09% | 100% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.58% | -0.38% | -1.00% | 99% | 98% |
| Class TGM | -1.29% | -1.20% | -1.16% | 98% | 99% |
| Overall | -0.94% | -0.79% | -1.08% | 99% | 98% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.41% | -1.45% | -1.63% | 101% | 100% |
| Class TGM | -1.13% | -1.04% | -1.05% | 100% | 99% |
| Overall | -0.77% | -1.24% | -1.34% | 100% | 100% |

# Conclusion

In this contribution, a simple Template Matching mode for IBC is proposed. A coding efficiency of {-0.53%, -0.36%, -0.20%} for Y in AI, RA and LDB has been achieved on top of the ECM-3.1 without additional encoding and decoding time increase. In combination with the EE2-3.13 test {-1.18%, -0.94%, -0.77%} for AI, RA and LDB is reported on top of the ECM-3.1 also without any additional encoding and decoding time increase. We therefore suggest investigating this proposal in the EE2 for the next meeting.

# References

- M. Coban, F. Le Léannec, M. Sarwer, J. Ström, “Algorithm description of Enhanced Compression Model 3 (ECM 3)”, JVET-X2025, October 2021.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-X2017, October. 2021.

- V. Seregin, J. Chen, L. Li, K. Naser, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-X2024, October 2021.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).