| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0090-v1 |
| --- | --- |

| Title: | AHG11: Neural Network Based Motion Compensation Enhancement for Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Changyue Ma Ru-Ling Liao Yan Ye | Tel: Email: | {changyue.mcy, ruling.lrl, yan.ye} @alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

This contribution proposes to enhance the motion compensated prediction of square coding blocks with neural network. The proposed method is mandatorily performed on all inter-coded square coding blocks. Experimental results demonstrate that, compared with VTM-11.0-nnvc-1.0, the proposed method achieves 1.23%, 1.20%, and 2.95% BD-rate reductions for Y component on Class B, C, and D under RA configuration, respectively.

Introduction

In this contribution, a neural network based motion compensation enhancement method is proposed. The proposed method is mandatorily performed on an inter-coded square coding block after it finishes motion compensation process. Currently, the proposed method is only applied on the luma component of the coding block with size 128x128, 64x64, 32x32, 16x16, and 8x8.

Proposed Method

Neural Network Architecture

Fig. 1 presents the structure of the proposed motion compensation enhancement neural network, where Fig. 1(a) presents the overall network structure and Fig. 1(b) presents the network structure of the residual unit. The proposed network takes the motion compensated coding block as input, where the coding block can be unidirectional predicted or bidirectional predicted. The coding block is first fed into a convolutional layer to extract the feature maps. After that, the feature maps pass through 10 residual units [1] and a convolutional layer. Finally, the neural network output is obtained by adding the input coding block and the learned residual information.

For all the convolutional layers in Fig. 1, kernel size of 3x3 is used. The number of the feature maps in each convolutional layer except the last layer is set to 64. The last convolutional layer produces 1 feature map. For activation function, ReLU is used.

- Overall network structure                          (b) Network structure of residual unit

Figure 1. The structure of the proposed motion compensation enhancement neural network.

Neural Network Inference

In inference stage, PyTorch 1.7.1 is used for performing the proposed neural network based motion compensation enhancement in VTM-11.0-nnvc-1.0. The inference information is summarized in Table 1 as suggested by [2].

Table 1. Inference information.

| Inference information |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | N/A |
|  | CPU Type | Intel(R) Xeon(R) CPU E5-2682 v4 @ 2.50GHz |
|  | Framework: | PyTorch 1.7.1 |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 776.71k/model (25 models in total) |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | 3.03M/model (25 models in total) |
|  | MACs | 783.1k/pixel |
| Optional | Total Conv. Layers | 22 |
|  | Total FC Layers | 0 |
|  | Batch size: | 1 block at a time |
|  | Patch size | Depending on the block (128x128, 64x64, 32x32, 16x16, 8x8) |

Neural Network Training

In training stage, PyTorch 1.7.0 is used for training the proposed neural network. For different coding block sizes (128x128, 64x64, 32x32, 16x16, and 8x8) and QPs (22, 27, 32, 37, and 42), the proposed network is trained independently. The BVI-DVC [3] dataset is used to train the neural network. The sequences are compressed by VTM-11.0-nnvc-1.0 with RA configurations. The training information is summarized in Table 2 as suggested by [2].

Table 2. Training information.

| Training information |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | NVIDIA Tesla V100-SXM2-32GB |
|  | CPU Type | Intel(R) Xeon(R) Platinum 8163 CPU @ 2.50GHz |
|  | Framework: | PyTorch 1.7.0 |
|  | Number of GPUs per Task: | 1 |
|  | Epoch: | 50 |
|  | Batch size: | 64 |
|  | Total Parameter Number: | 776.71k/model (25 models in total) |
|  | Parameter Precision (Bits): | 32 (F) |
|  | Training data information: | BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP {22, 27, 32, 37, 42} |
| Optional | Patch size | 128x128, 64x64, 32x32, 16x16, 8x8 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Loss function: | L2 |
|  | Preprocessing: | Normalize to 0~1 |

Experimental Results

The common test conditions defined in [2] are used to evaluate the proposed method. The experimental results on VTM-11.0-nnvc-1.0 under RA configuration are shown in Table 3. It is observed that 1.23%, 1.20%, and 2.95% BD-rate reductions are achieved for Y component on Class B, C, and D under RA configuration, respectively.

Table 3. Experimental results on VTM-11.0-nnvc-1.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0-nnv-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.23% | 0.12% | 0.11% | 2029% | 22315% |
| Class C | -1.20% | -0.09% | -0.14% | 1804% | 19404% |
| Overall |  |  |  |  |  |
| Class D | -2.95% | -1.36% | -1.05% | 1700% | 29635% |

Conclusions

This contribution presents a neural network based motion compensation enhancement method. Experimental results demonstrate that the proposed method achieves coding performance improvement compared with VTM-11.0-nnvc-1.0.

References

- Kaiming He, Xiangyu Zhang, Shaoqing Ren, and Jian Sun, “Deep residual learning for image recognition," in Proceedings of the IEEE conference on computer vision and pattern recognition, 2016, pp. 770-778.

- S. Liu, A. Segall, E. Alshina, and R.-L. Liao, “JVET common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-X2016, 24th Meeting: by teleconference, 6 – 15 October 2021.

- Ma, Di, Fan Zhang, and David R. Bull. "BVI-DVC: A Training Database for Deep Video Compression." arXiv preprint arXiv:2003.13552 (2020).

- Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).