| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0094-v1 |
| --- | --- |

| Title: | EE2-4.1: Test Results on Sign Prediction Improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen Yan Ye Ru-Ling Liao Xinwei Li | Tel: Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba Group. |  |  |

_____________________________

- Abstract

This contribution reports the results of test 4.1 of EE2. In test 4.1.a, an adaptive selection of transform coefficients of which the signs to be predicted based on qIdx was tested. In test 4.1.c, on top of test 4.1.a, the sign prediction area was extended from 4x4 block to maximum 32x32 block within a TB. Following results are reported:

- Test 4.1.a: adaptive select signs to be predicted based on qIdx

  - AI: -0.06% (Y), -0.08%(U), -0.04%(V), 103%(EncTime), 101% (DecTime)

  - RA: -0.07% (Y), 0.05%(U), 0.01%(V), 101%(EncTime), 100% (DecTime)

  - LB: -0.07% (Y), -0.07%(U), -0.34%(V), 100%(EncTime), 101% (DecTime)

- Test 4.1.c: Test 4.1.a + extend sign prediction area to maximum 32x32

  - AI: -0.11% (Y),	-0.09%(U),	-0.09% (V),	106% (EncTime), 104%(DecTime)

  - RA: -0.13%(Y),	-0.03%(U),	-0.02%(V),	101%(EncTime), 108%(DecTime)

  - LB: -0.15%(Y), -0.09%(U), -0.43%(V), 100%(EncTime), 105%(DecTime)

Introduction

In ECM2.0, signs of some transform coefficient levels are encoded using a system of prediction. A sign that has been predicted is no longer equiprobable (EP) signaled in the bitstream, but is replaced by a “residue” (signaled using an associated CABAC context) which indicates if the prediction was correct or not. In ECM2.0, at most the first K (which is set to 8) non-zero coefficients in the raster-scan order of the top-left 4x4 area of a TB are selected for the sign prediction.

In last meeting, JVET-X0120[1] and JVET-X0150[2] were proposed to improve the sign prediction in ECM2.0 and both of these two contributions were included EE2 for further study. JVET-X0120 proposed to 1) adaptively select the signs to be predicted based on the absolute value of qIdx (qIdx is the transform coefficient level after compensating the impact of the multiple quantizers in DQ) and 2) extend the area for sign prediction from 4x4 to maximum 32x32. JVET-X0150 proposed to 1) adaptively select the signs to be predicted based on their impacts on the quality of the reconstructed border samples of one TB and 2) also applied sign prediction to LFNST blocks.

Test 4.1.a: Adaptive select the signs to be predicted based on qIdx

In test 4.1.a, instead of always selecting the first K transform coefficients in raster-scan order of a top-left 4x4 area for sign prediction, the transform coefficients with the largest K qIdx value of the top-left 4x4 area were selected. qIdx value is the transform coefficient level after compensating the impact from the multiple quantizers in DQ. A larger qIdx value will produce a larger de-quantized transform coefficient level. qIdx is derived as follows

qIdx = (abs(level) << 1) - (state & 1);

where level is the transform coefficient level parsed from the bitstream and state is a variable maintained by the encoder and decoder in DQ.

Test 4.1.c: Test 4.1.a + Extend sign prediction area

On top of test 4.1.a, the sign prediction area was extended to maximum 32x32 in test 4.1.c. In this test, signs of top-left MxN block are predicted. The value of M and N is computed as follows:

  - M=minw, 32

  - N=min⁡(h, 32)

where, w and h are the width and height of the transform block.

Please note that only the area for the sign prediction is extended, the maximum number of signs to be predicted does not increase and is still restricted by maxNumPredSigns which is set to 8 as in ECM CTC. So the worst case is the same as the anchor.

Simulation results

The above two tests are implemented on top of ECM3.1. The performance is evaluated based on EE2 test conditions suggested in JVET-X2017[3].

- Test 4.1.a: adaptive selection of signs to be predicted based on qIdx

- Test 4.1.c: test 4.1.a + extend sign prediction area to maximum 32x32

The simulation results are shown in Table 1, Table 2

Table 1: Results of test 4.1.a

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.05% | -0.02% | 103% | 100% |
| Class A2 | -0.08% | -0.05% | -0.01% | 103% | 101% |
| Class B | -0.07% | -0.05% | -0.08% | 103% | 102% |
| Class C | -0.05% | -0.12% | -0.03% | 104% | 103% |
| Class E | -0.07% | -0.14% | -0.03% | 102% | 100% |
| Overall | -0.06% | -0.08% | -0.04% | 103% | 101% |
| Class D | -0.05% | 0.07% | -0.18% | 104% | 103% |
| Class F | -0.01% | 0.12% | -0.09% | 102% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.05% | -0.22% | 101% | 100% |
| Class A2 | -0.09% | 0.01% | -0.01% | 101% | 101% |
| Class B | -0.06% | 0.01% | -0.02% | 101% | 100% |
| Class C | -0.05% | 0.20% | 0.23% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | 0.05% | 0.01% | 101% | 100% |
| Class D | -0.08% | 0.34% | -0.13% | 101% | 100% |
| Class F | 0.02% | 0.12% | -0.06% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -0.21% | -0.08% | 102% | 102% |
| Class C | -0.09% | 0.25% | 0.09% | 101% | 101% |
| Class E | -0.10% | -0.26% | -1.36% | 99% | 99% |
| Overall | -0.07% | -0.07% | -0.34% | 101% | 101% |
| Class D | -0.03% | 0.62% | -0.66% | 101% | 101% |
| Class F | -0.22% | -0.84% | 0.23% | 100% | 99% |

Table 2: Results of test 4.1.c

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.11% | -0.03% | 105% | 103% |
| Class A2 | -0.11% | -0.01% | -0.05% | 106% | 104% |
| Class B | -0.12% | -0.11% | -0.13% | 106% | 105% |
| Class C | -0.10% | -0.08% | -0.07% | 107% | 106% |
| Class E | -0.14% | -0.15% | -0.14% | 106% | 103% |
| Overall | -0.11% | -0.09% | -0.09% | 106% | 104% |
| Class D | -0.10% | -0.01% | -0.31% | 108% | 113% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.12% | -0.17% | 102% | 105% |
| Class A2 | -0.16% | 0.20% | -0.08% | 102% | 103% |
| Class B | -0.13% | -0.04% | -0.04% | 101% | 110% |
| Class C | -0.06% | -0.12% | 0.18% | 102% | 110% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -0.03% | -0.02% | 101% | 108% |
| Class D | -0.07% | 0.01% | -0.09% | 102% | 114% |
| Class F | -0.05% | -0.21% | -0.28% | 102% | 115% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.14% | -0.19% | -0.02% | 101% | 106% |
| Class C | -0.11% | 0.12% | 0.07% | 101% | 105% |
| Class E | -0.22% | -0.22% | -1.79% | 100% | 102% |
| Overall | -0.15% | -0.09% | -0.43% | 101% | 105% |
| Class D | -0.06% | -0.51% | -0.38% | 100% | 111% |
| Class F | -0.11% | -0.16% | -0.02% | 102% | 107% |

Conclusion

This contribution reports the results of test 4.1 of EE2. Test 4.1.a proposed to adaptively select the signs to be predicted based on qIdx and test 4.1.c additionally extended sign prediction area to maximum 32x32. With these improvements, it achieves {-0.11%, -0.09%, -0.09%} AI, {-0.13%, -0.03%, -0.02%} in RA and { -0.15%, -0.09%, -0.43%} in LB with no significant running time increase.

References

M. G. Sarwer, Y. Ye, J. Chen, R.-L, Liao, “AHG12: On sign prediction”, JVET-X0120, Oct 2021.

X. Xiu, J.-H. Jhu, W. Chen, C.-W. Kuo, N. Yan, Y.-W. Chen, X. Wang, “AHG12: Enhanced sign prediction”, JVET-X0150, Oct. 2021.

M. Karczewice, Y. Yan, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-X2017, Oct. 2021

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).