| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0095-v2 |
| --- | --- |

| Title: | Non-EE2: RPR with luma-only re-scaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | P.Bordes, F.Galpin, E.Francois | Email: | philippe.bordes@interdigital.com franck.galpin@interdigital.com edouard.francois@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes extending RPR feature of picture size adaptation to chroma format adaptation. Doing so, one can re-scale luma only but not chroma. It reports this luma-only RPR provides BD-rate gains while chroma qp-offset alignment is more consistent across sequences and configurations.

# Introduction / Problem Statement

The purpose of RPR tool is to enable resizing of picture size dynamically, without need for inserting an IRAP picture. This feature is useful for UHD content since it reduces significantly blocking artefacts at low bit rate (see §7) and the encoding/decoding times (complexity) are decreased.

However, RPR may show PSNR drop for chroma component because RPR performs an additional down-scaling filtering on samples that were already filtered from the original canonical 4:4:4 content to create the 4:2:0 format.

We tested RPR in ECM-3.0 [1] with method cfg-3 proposed in AHG-11 [2] for comparison with ECM anchors. The cfg-3 is a combination of full size and quarter size coding (RPR) adaptively per sequences and target rate (RPR is used at low bit rate if it improves anchors).

The results are reported with 5 QPs corresponding to {22,…42} and {17,…37} for anchors and RPR streams respectively, using CTC on classes A and B, plus 7 sequences 8K1. The BD-rate curves are obtained by replacing anchor points {qp42 and/or qp37} with {qp37 and/or qp32} RPR points respectively (cfg-3).

Table 1: ECM-3 coding performance with regular RPR enabled compared with full size coding (cfg-3).

|  | All Intra (ECM-3.0) |  |  |  |  |  | Random-Access (ECM-3.0) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | EncT | DecT | Y | U | V |  | EncT | DecT |
| Class 8K | -1,77% | 0,21% | 0,78% |  | 88% | 63% |  |  |  |  |  |  |
| Class A1 | -2,03% | 7,86% | 5,93% |  | 80% | 61% | -1,85% | 2,75% | 4,22% |  | 61% | 59% |
| Class A2 | -2,51% | 5,69% | 4,35% |  | 80% | 67% | -0,69% | 14,22% | 9,52% |  | 66% | 66% |
| Class B | -0,07% | 0,48% | 0,52% |  | 97% | 91% | -0,02% | 0,20% | 0,20% |  | 96% | 95% |
| Overall | -1,60% | 3,56% | 2,90% |  | 86% | 71% | -0,85% | 5,72% | 4,65% |  | 74% | 73% |

# Proposal

One main motivation is to try mitigating the PSNR chroma drop due to implicit double down-scaling stages for chroma when regular RPR is applied on 4:2:0 format. One proposes to increase the flexibility of RPR by allowing down-scaling luma only. Then the codec may switch dynamically resolution and/or chroma format which is signaled in PPS. To still benefit from the cross-component tools (intra prediction, CC-SAO, CC-ALF), this down-scaling luma only feature is limited to down-scaling by 2 both horizontally and vertically, so that the chroma format of the reconstructed pictures may be 4:2:0 (no-RPR or regular RPR) or 4:4:4 (luma-only RPR).

# Tests

For simulations (cfg-3) with regular RPR, one used negative chroma qp-offset correction (qpC) to cope with the double chroma down-scale filtering. For simulations with luma-only RPR, since the chroma is not re-scaled, one used positive chroma qp-offset correction (qpC) to align the chroma qp with anchors only.

Table 2: ECM-3 coding performance with regular RPR enabled and proposed luma-only RPR, using chroma qp-offset (qpC) and compared with full size coding (AI).

| AI | Regular RPR – AI (ECM-3.0) |  |  |  |  |  | Luma-only RPR - AI (ECM-3.0) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | qpC | EncT | DecT | Y | U | V | qpC | EncT | DecT |
| Class 8K | -1,77% | 0,21% | 0,78% | 0 | 88% | 63% | -1,75% | -0,82% | -0,67% | +6 | 95% | 69% |
| Class A1 | -0,47% | -3,04% | -2,42% | -4 | 91% | 78% | -0,98% | -1,00% | 0,04% | +4 | 91% | 72% |
| Class A2 | -0,02% | -0,32% | -0,25% | -4 | 93% | 88% | -0,34% | -0,01% | -0,03% | +4 | 92% | 81% |
| Class B | 0,00% | 0,00% | 0,00% | -4 | 100% | 100% | -0,03% | 0,00% | 0,02% | +4 | 99% | 97% |
| Overall | -0,57% | -0,79% | -0,47% |  | 93% | 83% | -0,78% | -0,46% | -0,16% |  | 94% | 79% |

Table 3: ECM-3 coding performance with regular RPR enabled and proposed luma-only RPR, using chroma qp-offset (qpC) and compared with full size coding (RA).

| RA | Regular RPR – RA (ECM-3.0) |  |  |  |  |  | Luma-only RPR - RA (ECM-3.1) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | qpC | EncT | DecT | Y | U | V | qpC | EncT | DecT |
| Class 8K | -0,44% | -5,29% | -3,46% | -2 | 84% | 77% | -0,47% | -5,95% | -4,24% | +5 | 86% | 77% |
| Class A1 | -0,63% | -3,07% | -1,82% | -2 | 79% | 75% | -0,96% | -2,83% | -1,27% | +5 | 75% | 70% |
| Class A2 | -0,08% | 0,38% | 0,24% | -2 | 92% | 92% | -0,11% | 0,54% | 0,35% | +5 | 87% | 84% |
| Class B | -0,01% | -0,03% | -0,07% | -2 | 96% | 95% | -0,02% | 0,09% | 0,08% | +5 | 97% | 95% |
| Overall | -0,24% | -0,91% | -0,55% |  | 89% | 87% | -0,39% | -2,04% | -1,27% |  | 86% | 82% |

Table 4: ECM-3 coding performance with the proposed luma-only RPR, using chroma qp-offset (qpC) equal to +5 for all configurations, compared with full size coding (AI,RA).

|  | Luma-only RPR - AI (ECM-3.1) |  |  |  |  |  | Luma-only RPR - RA (ECM-3.1) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | qpC | EncT | DecT | Y | U | V | qpC | EncT | DecT |
| Class 8K | -1,31% | -2,42% | -1,60% | +5 | 97% | 74% | -0,47% | -5,95% | -4,24% | +5 | 86% | 77% |
| Class A1 | -1,43% | 2,06% | 2,58% | +5 | 90% | 73% | -0,96% | -2,83% | -1,27% | +5 | 75% | 70% |
| Class A2 | -0,93% | 0,82% | -1,27% | +5 | 94% | 82% | -0,11% | 0,54% | 0,35% | +5 | 87% | 84% |
| Class B | -0,05% | 0,33% | 0,36% | +5 | 104% | 101% | -0,02% | 0,09% | 0,08% | +5 | 97% | 95% |
| Overall | -0,93% | 0,20% | -0,02% |  | 96% | 83% | -0,39% | -2,04% | -1,27% |  | 86% | 82% |

Table 5: ECM-3 coding performance with the proposed luma-only RPR, using chroma qp-offset (qpC) equal to +6 for all configurations, compared with full size coding (AI,RA).

|  | Luma-only RPR - AI (ECM-3.1) |  |  |  |  |  | Luma-only RPR - RA (ECM-3.1) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | qpC | EncT | DecT | Y | U | V | qpC | EncT | DecT |
| Class 8K | -1,75% | -0,82% | -0,67% | +6 | 95% | 69% | -0,70% | -4,43% | -2,75% | +6 | 83% | 72% |
| Class A1 | -1,84% | 4,50% | 4,76% | +6 | 90% | 73% | -1,40% | 0,25% | 1,69% | +6 | 69% | 65% |
| Class A2 | -1,62% | 2,73% | 0,20% | +6 | 93% | 81% | -0,30% | 0,34% | -2,03% | +6 | 81% | 78% |
| Class B | -0,07% | 0,47% | 0,52% | +6 | 104% | 102% | -0,03% | 0,19% | 0,19% | +6 | 96% | 95% |
| Overall | -1,32% | 1,72% | 1,20% |  | 96% | 81% | -0.61% | -0.91% | -0.73% |  | 82% | 78% |

For regular RPR, the choice of chroma qp-offset (qpC) should be adjusted per sequence type and depends on coding configuration, whereas for luma only RPR the chroma qp-offset value is more consistent across sequences and configs.

Table 2: In AI configuration, the BD-rate gain of luma-only RPR compared to regular RPR is -0.21%/0.33%/0.31% in YUV respectively.

Table 3: In RA configuration, the BD-rate gain of luma-only RPR compared to regular RPR is -0.15%/-1,13%/-0,72% in YUV respectively.

Table 4 and Table 5: one obtains consistent BD-rate gains when using same chroma qp-offset (qpC equal to 5 or 6) with luma-only RPR, for all classes and all configurations. The chroma qp-offset being chosen to get chroma qp close to the qp of the chroma anchors.

A tentative explanation is that RPR is very efficient when the input sequence has few high spatial frequencies (ex: for 8K content, the regular RPR does not require chroma qp-offset correction in AI). For regular RPR, the down-scaling of chroma removes the high frequencies that can be partially recovered at expense of using more bits for chroma (negative chroma qp-offset), then the qp-offset value depends on input signal characteristics. In case of luma-only RPR, the purpose of the chroma qp-offset is only to align with the CTC chroma qp anchors, and it remains consistent with the RPR versus non-RPR streams QP alignment whatever sequence type or configuration.

# Conclusion

RPR improves the coding efficiency for low bit-rates which improves the overall BD-rate coding gains and reduce the encoding/decoding times.

The luma-only RPR tool provides at least same BD-rate gains as regular RPR. However, the regular RPR requires adjusting the chroma qp-offset per sequence and depends on coding configuration to align with BD-rate anchor curves.

The luma-only RPR is a simple extension of RPR which overcomes the issue of additional down-filtering of chroma in 4:2:0. It also facilitates the selection of QP to be consistent across sequence types and configurations.

# References

- https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- S. Liu, A.Segall, E.Alshina, R.Liao, “JVET common test conditions and evaluation procedures for neural network-based video coding technology,” JVET-X2016.

- Version 11.0_nnvc-1.0 of the VTM NNVC software is available at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc

- P.Bordes, F.Galpin, F.Leleannec, “ALF parameters for RPR,” JVET-W0073.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Examples

Figure 1: anchor ECM-3.0 qp-42 905kpbs (left) and regular RPR 971 kbps qp-37 (right).

Figure 2: regular RPR (left) and luma-only RPR (right), qp-37.

Figure 2: regular RPR (left) and luma-only RPR (right), qp-37, (bottom: U component).

Figure 3: regular RPR (left) and luma-only RPR (right), qp-37.

1 frame in AI, 17 frames in RA, {BodeMuseum, NeptuneFountain2, NeptuneFountain3, OberbaumSpree, QuadrigaTree, SubwayTree, TiergartenParkway}.