| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0098-v1 |
| --- | --- |

| Title: | EE1-related: Combination of VVC deblocking and NN loop filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, J. Ström, D. Liu, R. Sjöberg | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson .com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

NN loop filtering as in EE1-1.6 has shown to be capable of promising bitrate reductions while also mostly producing good visual quality. One concern is that when NN loop filter is not selected, no deblocking filtering at all is performed.

This contribution enables the use of VVC deblocking in combination with NN loop filtering. The NN loop filtering is from EE1-1.6 but with some notable differences. Instead of signalling which QP to use for the model from a fixed set of QPs, only one QP is used. Also, the maximum block size (excluding border extension for input) for luma is restricted to 128x128 for inter pictures for resolutions below 4K. The output is a convex combination of the VVC deblocked samples and the NN-processed samples. Since both VVC deblocking and NN loop filtering provide a deblocking effect, the output samples will always have seen some form of deblocking. The VVC deblocking is performed in RDO and a deblocking beta offset of -2 is also used.

The BDR effect and the impact on encoding and decoding time of the proposal compared to the EE1-1.6 is as follows:

AI:   0.12% Y, -0.79% U, -1.05% V, EncTime: 105%, DecTime: 96%

RA: -0.27% Y, -0.66% U, -0.31% V, EncTime: 76%, DecTime: 90%

LDB: -0.69% Y, -2.13% U, -1.60% V, EncTime: 71% DecTime: 84%

A notable encoder speedup can be seen for RA and LDB due to omitting multiple runs of the NN model with different QPs as in EE1-1.6. It is also asserted, by visual inspection, that the proposal makes sure that sufficient deblocking always is applied.

# Introduction

NN loop filtering from EE1-1.6 [1] can provide nice bitrate savings while also mostly produce good visual quality. One issue in EE1-1.6 is that when NN is not selected for a frame or a CTU, no deblocking filtering is used at all which can result in bad visual quality. Another issue with EE1-1.6 is that encoder complexity is high since different QP inputs are tried.

# Proposal

In this proposal we only use a single QP input to the NN model but instead make NN filtering with reduced strength available by using a weighted combination of NN filtering and VVC deblocking filtering:

dfnn=df+w*(nn-df)

where df is a deblocked sample, w is a weight between 0 and 1, and nn is an NN filtered sample. This can be written as a convex combination

dffnn=(1-w)*df + w*nn

which means that an output sample dffnn will always be somewhere between the deblocked samples (df) and the NN filtered sample (nn).

Instead of mode 1 (QP+offset1) and mode 2 (QP+offset2) in EE-1.6 we use a default weighted combination of NN filtering and VVC deblocking filtering with weight 0.75 and 0.5 respectively. In the block-based slice mode we use either VVC deblocking only or a weighted combination of NN filtering and VVC deblocking filtering. We also add two additional block-based slice modes for the case of weighted combination of NN filtering and VVC deblocking filtering based on default weight of 0.75 and 0.5 respectively.

We also reduce the max block size to 128x128 for luma in inter coded pictures for resolutions below 4K to enable more local control of the NN filtering in inter coded pictures.

# Complexity

The decoder complexity of the proposal is similar as in EE1-1.6 since we use the unchanged NN model. The complexity of VVC deblocking is marginal compared to NN filtering. The encoder complexity is reduced for inter pictures since we only apply the NN model once per picture instead of 3 times as in EE1-1.6. In addition to EE1-1.6 we apply VVC deblocking in RDO but the complexity of that is relatively small compared to NN loop filtering.

# Results

We provide results for EE1 CTC [2]. The encoding and decoding time numbers indicate a speedup of the proposed method for RA and LDB mainly due to the use of a single QP mode. Only Class C and Class D in random access configuration have been run with anchor and test on same machine, so reported numbers for other classes and configurations are not completely reliable. It can also be noted an improvement of BDR for RA and LDB. Visual assessment reveals a significant improvement on class E in LDB and mainly similar visual quality for other cases.

Table 1: Random access: Anchor is EE-1.6

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.56% | -0.74% | -0.17% | 73% | 86% |
| Class A2 | -0.40% | -0.28% | 0.03% | 76% | 87% |
| Class B | -0.22% | -0.68% | -0.53% | 70% | 91% |
| Class C | -0.03% | -0.85% | -0.38% | 85% | 95% |
| Class E |  |  |  |  |  |
| Overall | -0.27% | -0.66% | -0.31% | 76% | 90% |
| Class D | -0.09% | -0.29% | 0.08% | 85% | 94% |
| Class F | -0.04% | -0.31% | 0.05% | 67% | 85% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 2: Low-delay B: Anchor is EE1-1.6:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.52% | -2.57% | -2.16% | 74% | 86% |
| Class C | -0.21% | -1.59% | -0.89% | 75% | 82% |
| Class E | -1.62% | -4.27% | -3.89% | 62% | 81% |
| Overall | -0.69% | -2.13% | -1.60% | 71% | 84% |
| Class D | -0.14% | -1.42% | -1.11% | 82% | 93% |
| Class F | -0.45% | -2.15% | -1.30% | 64% | 73% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 3: All intra: Anchor is EE1-1.6:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.08% | -0.01% | -0.71% | 100% | 97% |
| Class A2 | 0.16% | -0.67% | -0.61% | 108% | 100% |
| Class B | 0.15% | -1.29% | -1.16% | 108% | 99% |
| Class C | 0.14% | -0.76% | -0.87% | 104% | 94% |
| Class E | 0.05% | -0.92% | -1.90% | 104% | 96% |
| Overall | 0.12% | -0.79% | -1.05% | 105% | 96% |
| Class D | 0.23% | -0.89% | -0.57% | 100% | 92% |
| Class F | 0.06% | -0.63% | -0.19% | 107% | 98% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Conclusion

The contribution indicates:

- It can be beneficial to combine VVC deblocking with NN loop filtering to make sure that some form of deblocking processing (VVC deblocking or NN loop filtering) is always used to ensure robust visual quality.

- Instead of multiple QP modes a default weighted combination of NN filtering and VVC deblocking can be used.

- It can be good to have the max block size of the NN filtering controllable.

# References

[1] EE1-1.6: Combined Test of EE1-1.2 and EE1-1.4, Y. Li, K. Zhang, L. Zhang, H. Wang, J. Chen, K. Reuze, A.M. Kotra, M. Karczewicz, JVET-X0066, October 2021

[2] Common Test Conditions and evaluation procedures for neural network-based video coding technology, E. Alshina, R.-L. Liao, S. Liu, A. Segall, JVET-X2016, October 2021

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).