| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0103-v1 |
| --- | --- |

| Title: | AHG9: Down-sample phase indication (SEI message) |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | P. Bordes, P. de Lagrange, E. Francois | Email: | philippe.bordes@interdigital.com philippe.delegrange@interdigital.com edouard.francois@interdigital.com |
| Source: | Interdigital |  |  |  |

_____________________________

# Abstract

Down-sampling pictures before video coding is a tool commonly used for seamless adapting bit-streams to available bandwidth. For example, this feature is supported by MPEG-DASH clients that switches between representations of different resolutions and by VVC that implements per picture resolution adaptation (RPR).

To ensure that the receivers and players appropriately display (up-sample) the decoded video, they need to know the phases that have been used when creating the down-sampled video.

This contribution proposes a new SEI message indicating the phases and other information necessary to correctly interpret the reconstructed video for resampling to the original size. This SEI message can apply to AVC, HEVC and VVC video streams and it may overcome some limitations of previous proposal JVET-X0092 discussed in last JVET meeting and on the reflector.

# Problem Statement

The indication of the phases used when creating the down-sampled video allows the receivers and players to appropriately up-sample the decoded pictures to the original size.

To cover all use cases, and given the discussions in last JVET meeting and on reflector, this contribution tries to address the following points:

- The luma and chroma can have a phase shift in down-sampling, and they may be different.

- The chroma location type would also have impact on the correct down-sampling filter phase to be used for chroma.

- Higher precision in specifying the phase value (the syntax “co-sited/centered” which is well adapted to 2:1 down-sampling ratio doesn’t handle cases with other phases such as ¼ phase, useful for other down-sampling ratios typically).

- The syntax should indicate (or allow to derive) the down-sampling ratio so that the decoder can inversely up-sample with the right ratio.

- The syntax should be applicable to AVC, HEVC and VVC. In the case of VVC, several PPS may be present in bit-stream, with potentially different picture sizes and down-sampling phases.

# Proposal

The following syntax for the SEI message is proposed:

Table 1: proposed syntax of SEI message down-sampling phase indication.

| phase_indication( payloadSize ) { | Descriptor |
| --- | --- |
| orig_pic_size_present_flag | u(1) |
| if ( orig_pic_size_present_flag ) { |  |
| orig_pic_width_in_luma_samples | ue(v) |
| orig_pic_height_in_luma_samples | ue(v) |
| orig_chroma_collocated_present_flag | u(1) |
| if ( chroma_collocated_present_flag ) { |  |
| orig_chroma_horizontal_collocated_flag | u(1) |
| orig_chroma_vertical_collocated_flag | u(1) |
| } |  |
| } |  |
| num_pps_down_sampling_phase_minus1 | u(8) |
| for ( i=0 ; i<=num_pps_down_sampling_phase_minus1 ; i++ ) |  |
| pps_id[ i ] | u(8) |
| phase_horizontal_idc[ i ] | u(8) |
| phase_vertical_idc[ i ] | u(8) |
| downsampled_chroma_collocated_present_flag | u(1) |
| if ( downsampled_chroma_collocated_present_flag ) { |  |
| downsampled_chroma_horizontal_collocated_flag[ i ] | u(1) |
| downsampled_chroma_vertical_collocated_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |

orig_pic_size_present_flag equal to 1 indicates that the orig_pic_width_in_luma_samples and orig_pic_height_in_luma_samples syntax elements are present. orig_pic_size_present_flag equal to 0 indicates that the orig_pic_width_in_luma_samples and orig_pic_height_in_luma_samples syntax elements are not present.

orig_pic_width_in_luma_samples specifies the width, in units of luma samples, of the original pictures. orig_pic_width_in_luma_samples shall not be equal to 0.

orig_pic_height_in_luma_samples specifies the height, in units of luma samples, of the original pictures. orig_pic_height_in_luma_samples shall not be equal to 0.

NOTE –If orig_pic_size_present_flag is equal to 0, then the original picture size for up-sampling reconstructed pictures may be inferred to the size specified in the SPS by sps_pic_width_max_in_luma_samples, sps_pic_height_max_in_luma_samples for VVC, and 2 x pic_width_in_luma_samples and 2 x pic_height_in_luma_samples for AVC and HEVC.

orig_chroma_collocated_present_flag equal to 1 indicates that the syntax elements orig_chroma_horizontal_collocated_flag  and  orig_chroma_vertical_collocated_flag are present. orig_chroma_collocated_present_flag equal to 0 indicates that the syntax elements orig_chroma_horizontal_collocated_flag  and  orig_chroma_vertical_collocated_flag are not present.

orig_chroma_horizontal_collocated_flag equal to 1 indicates the chroma sample positions are not horizontally shifted relative to corresponding luma sample positions, for the original pictures. orig_chroma_horizontal_collocated_flag equal to 0 indicates the chroma sample positions are shifted to the right by 0.5 in units of luma samples relative to corresponding luma sample positions, for the original pictures. When not present, orig_chroma_horizontal_collocated_flag is inferred to 1.

orig_chroma_vertical_collocated_flag equal to 1 indicates the chroma sample positions are not vertically shifted relative to corresponding luma sample positions, for the original pictures. orig_chroma_vertical_collocated_flag equal to 0 indicates the chroma sample positions are shifted downward by 0.5 in units of luma samples relative to corresponding luma sample positions, for the original pictures. When not present, orig_chroma_vertical_collocated_flag is inferred to 0.

num_pps_down_sampling_phase_minus1 plus 1 indicates the number of PPS for which the down-sampling phase is specified.

pps_id[ i ] specifies the value of the pps_pic_parameter_set_id of the ith PPS.

phase_horizontal_idc[ i ] indicates the horizontal sampling grid alignment (also known as down-sampling phase of the top-left sample) of the reconstructed pictures with respect to the original picture width in a grid resolution of 8 phase positions, of the pictures using PPS with pps_pic_parameter_set_id equal to pps_id[ i ].

phase_vertical_idc[ i ] indicates the vertical sampling grid alignment (a.k.a. down-sampling phase of the top-left sample) of the reconstructed pictures height with respect to the original picture height in a grid resolution of 8 phase positions, of the pictures using PPS with pps_pic_parameter_set_id equal to pps_id[ i ].

NOTE – For AVC, HEVC, the reconstructed picture size is specified in SPS and is same whatever PPS is used. Then, it is a requirement that num_pps_down_sampling_phase_minus1 is equal to 0 and pps_pic_parameter_set_id may be not equal to pps_id[ i ].

downsampled_chroma_collocated_present_flag equal to 1 indicates that the syntax elements downsampled_chroma_horizontal_collocated_flag[ i ]  and  downsampled_chroma_vertical_collocated_flag [ i ] are present. downsampled_chroma_collocated_present_flag equal to 0 indicates that the syntax elements downsampled_chroma_horizontal_collocated_flag [ i ]  and  downsampled_chroma_vertical_collocated_flag [ i ] are not present.

downsampled_chroma_horizontal_collocated_flag [ i ] equal to 1 indicates the chroma sample positions are not horizontally shifted relative to corresponding luma sample positions, for the pictures using PPS with pps_pic_parameter_set_id equal to pps_id[ i ]. downsampled_chroma_horizontal_collocated_flag [ i ] equal to 0 indicates the chroma sample positions are shifted to the right by 0.5 in units of luma samples relative to corresponding luma sample positions, for the pictures using PPS with pps_pic_parameter_set_id equal to pps_id[ i ]. When not present, downsampled_chroma_horizontal_collocated_flag [ i ] is inferred to 1.

downsampled_chroma_vertical_collocated_flag [ i ] equal to 1 indicates the chroma sample positions are not vertically shifted relative to corresponding luma sample positions, for the pictures using PPS with pps_pic_parameter_set_id equal to pps_id[ i ]. downsampled_chroma_vertical_collocated_flag [ i ] equal to 0 indicates the chroma sample positions are shifted downward by 0.5 in units of luma samples relative to corresponding luma sample positions, for the pictures using PPS with pps_pic_parameter_set_id equal to pps_id[ i ]. When not present, downsampled_chroma_vertical_collocated_flag [ i ] is inferred to 0.

# Conclusion

It is proposed to signal the phase indication in a new SEI message added to Versatile supplemental enhancement information messages for coded video bitstreams (ITU-T H.274 | ISO/IEC 23002-7).

# References

- J.Samuelsson, A.Tourapis, D.Podborski, K.Rapaka, D.Singer, “Down-sample phase indication (SEI message),” JVET-X0092, October 2021.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-W2017, July. 2021.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).