| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0103-v3 |
| --- | --- |

| Title: | AHG9: Re-scaling phase indication |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | P. Bordes, P. de Lagrange, E. Francois | Email: | philippe.bordes@interdigital.com philippe.delegrange@interdigital.com edouard.francois@interdigital.com |
| Source: | Interdigital |  |  |  |

_____________________________

# Abstract

Down-sampling pictures before video coding is a tool commonly used for seamless adapting bit-streams to available bandwidth. For example, this feature is supported by MPEG-DASH clients that switches between representations of different resolutions and by VVC that implements per picture resolution adaptation (RPR).

To ensure that the receivers and players appropriately display (up-sample) the decoded video, they need to know the phases that have been used when creating the down-sampled video.

This contribution proposes a new SEI message indicating the phases and other information necessary to correctly interpret the reconstructed video for resampling to the original size. This SEI message can apply to AVC, HEVC and VVC video streams and it may overcome some limitations of previous proposal JVET-X0092 discussed in last JVET meeting and on the reflector.

# Problem Statement

The indication of the phases used when creating the down-sampled video allows the receivers and players to appropriately up-sample the decoded pictures to the original size.

To cover all use cases, and given the discussions in last JVET meeting and on reflector, this contribution tries to address the following points:

- The luma and chroma can have a phase shift in down-sampling, and they may be different.

- The chroma location type would also have impact on the correct down-sampling filter phase to be used for chroma.

- Higher precision in specifying the phase value (the syntax “co-sited/centered” which is well adapted to 2:1 down-sampling ratio doesn’t handle cases with other phases such as ¼ phase, useful for other down-sampling ratios typically).

- The syntax should indicate (or allow to derive) the down-sampling ratio so that the decoder can inversely up-sample with the right ratio.

- The syntax should be applicable to AVC, HEVC and VVC. In the case of VVC, several PPS may be present in bit-stream, with potentially different picture sizes and down-sampling phases.

# Proposal

The following syntax for the re-scaling phase indication (RSPI) SEI message is proposed:

Table 1: proposed syntax of SEI message re-scaling phase indication.

| phase_indication( payloadSize ) { | Descriptor |
| --- | --- |
| orig_pic_size_present_flag | u(1) |
| if ( orig_pic_size_present_flag ) { |  |
| orig_pic_width_in_luma_samples | ue(v) |
| orig_pic_height_in_luma_samples | ue(v) |
| orig_chroma_collocated_present_flag | u(1) |
| if ( chroma_collocated_present_flag ) { |  |
| orig_chroma_horizontal_collocated_flag | u(1) |
| orig_chroma_vertical_collocated_flag | u(1) |
| } |  |
| } |  |
| phase_horizontal_idc | u(8) |
| phase_vertical_idc | u(8) |
| chroma_collocated_present_flag | u(1) |
| if (chroma_collocated_present_flag ) { |  |
| chroma_horizontal_collocated_flag | u(1) |
| chroma_vertical_collocated_flag | u(1) |
| } |  |
| } |  |

The following semantics for the RSPI SEI message is proposed:

The RSPI SEI message provides information for deriving the horizontal and vertical phase shift values of the reconstructed top-left luma and chroma samples into a hypothetical original picture. The size of the hypothetical original picture should be larger than the reconstructed pictures. This information can be used by the receiver to enable more consistent rendering, when rendering at a higher resolution and when switching between multiple resolutions.

orig_pic_size_present_flag equal to 1 indicates that the orig_pic_width_in_luma_samples and orig_pic_height_in_luma_samples syntax elements are present. orig_pic_size_present_flag equal to 0 indicates that the orig_pic_width_in_luma_samples and orig_pic_height_in_luma_samples syntax elements are not present.

orig_pic_width_in_luma_samples specifies the width, in units of luma samples, of an hypothetical original picture. orig_pic_width_in_luma_samples shall not be equal to 0.

orig_pic_height_in_luma_samples specifies the height, in units of luma samples, of an hypothetical original picture. orig_pic_height_in_luma_samples shall not be equal to 0.

If orig_pic_size_present_flag is equal to 0, then orig_pic_width_in_luma_samples and orig_pic_height_in_luma_samples are inferred to be equal to sps_pic_width_max_in_luma_samples, sps_pic_height_max_in_luma_samples of the CLVS respectively for VVC (Rec. ITU-T H.266 | ISO/IEC 23090-3), are inferred to be equal to 2 x pic_width_in_luma_samples and 2 x pic_height_in_luma_samples of the CLVS respectively for HEVC (Rec. ITU-T H.265 | ISO/IEC 23008-2), are inferred to be equal to 2 x PicWidthInSamplesL and 2 x PicHeightInSamplesL of the CLVS respectively for AVC (Rec. ITU-T H.264 | ISO/IEC 14496-10).

orig_chroma_collocated_present_flag equal to 1 indicates that the syntax elements orig_chroma_horizontal_collocated_flag  and  orig_chroma_vertical_collocated_flag are present. orig_chroma_collocated_present_flag equal to 0 indicates that the syntax elements orig_chroma_horizontal_collocated_flag  and  orig_chroma_vertical_collocated_flag are not present.

orig_chroma_horizontal_collocated_flag equal to 1 indicates the chroma sample positions are not horizontally shifted relative to corresponding luma sample positions, for the original pictures. orig_chroma_horizontal_collocated_flag equal to 0 indicates the chroma sample positions are shifted to the right by 0.5 in units of luma samples relative to corresponding luma sample positions, for the original pictures. When not present, orig_chroma_horizontal_collocated_flag is inferred to 1.

orig_chroma_vertical_collocated_flag equal to 1 indicates the chroma sample positions are not vertically shifted relative to corresponding luma sample positions, for the original pictures. orig_chroma_vertical_collocated_flag equal to 0 indicates the chroma sample positions are shifted downward by 0.5 in units of luma samples relative to corresponding luma sample positions, for the original pictures. When not present, orig_chroma_vertical_collocated_flag is inferred to 0.

phase_horizontal_idc indicates the horizontal sampling grid alignment  of the reconstructed pictures with respect to the original picture width in a grid resolution of 8 phase positions, of the reconstructed pictures.

phase_vertical_idc indicates the vertical sampling grid alignment  of the reconstructed pictures height with respect to the original picture height in a grid resolution of 8 phase positions, of the reconstructed pictures.

chroma_collocated_present_flag equal to 1 indicates that the syntax elements chroma_horizontal_collocated_flag  and  chroma_vertical_collocated_flag are present. chroma_collocated_present_flag equal to 0 indicates that the syntax elements chroma_horizontal_collocated_flag   and  chroma_vertical_collocated_flag are not present.

chroma_horizontal_collocated_flag equal to 1 indicates the chroma sample positions are not horizontally shifted relative to corresponding luma sample positions, for the reconstructed pictures. chroma_horizontal_collocated_flag equal to 0 indicates the chroma sample positions are shifted to the right by 0.5 in units of luma samples relative to corresponding luma sample positions, for the reconstructed pictures. When not present, chroma_horizontal_collocated_flag is inferred to 1.

chroma_vertical_collocated_flag equal to 1 indicates the chroma sample positions are not vertically shifted relative to corresponding luma sample positions, for the reconstructed pictures. chroma_vertical_collocated_flag equal to 0 indicates the chroma sample positions are shifted downward by 0.5 in units of luma samples relative to corresponding luma sample positions, for the reconstructed pictures. When not present, chroma_vertical_collocated_flag is inferred to 0.

For VVC (Rec. ITU-T H.266 | ISO/IEC 23090-3), it is a requirement of bitstream conformance that chroma_collocated_present_flag is equal to 0, and the values of chroma_horizontal_collocated_flag and chroma_vertical_collocated_flag are inferred to sps_chroma_horizontal_collocated_flag and sps_chroma_vertical_collocated_flag respectively.

For AVC (Rec. ITU-T H.264 | ISO/IEC 14496-10) and HEVC (Rec. ITU-T H.265 | ISO/IEC 23008-2), if VUI are present, it is a requirement of bitstream conformance that chroma_collocated_present_flag is equal to 0.

Table 2 – Persistence scope of RSPI SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| Re-scaling phase indication (RSPI) | The CLVS containing the RSPI SEI message and up to but not including the next RSPI SEI message |

One RSPI SEI is associated with each PPS in the CLVS. The RSPI SEI is associated with all PPS of the CLVS containing the RSPI SEI message up to the next RSPI SEI message. The subsequent RSPI SEI messages will be associated with the subsequent PPSs up to but not including the next RSPI SEI message. If a RSPI SEI message is associated with several PPS, the size of the reconstructed pictures specified by the PPS should be the same. The size of the hypothetical original picture should be the same for all the RSPI SEI in the CLVS.

# Conclusion

It is proposed to signal the phase indication in a new SEI message added to Versatile supplemental enhancement information messages for coded video bitstreams (ITU-T H.274 | ISO/IEC 23002-7).

# References

- J.Samuelsson, A.Tourapis, D.Podborski, K.Rapaka, D.Singer, “Down-sample phase indication (SEI message),” JVET-X0092, October 2021.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-W2017, July. 2021.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).