| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0106-v1 |
| --- | --- |

| Title: | EE2-5.1: Edge-based classifier for Cross-component Sample Adaptive Offset (CCSAO) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra Nan Hu Vadim Seregin Marta Karczewicz C.-W. Kuo, Xiaoyu Xiu, Yi-Wen Chen, Hong-Jheng Jhu, W. Chen, Ning Yan, Xianglin Wang | Tel: Email: | akotra@qti.qualcomm.com cheweikuo@kwai.com |
| Source: | Qualcomm Inc., Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of EE-5.1. EE-5.1 studied the performance of introducing an “Edge-Based” classifier to the cross-component sample adaptive offset (CCSAO). CCSAO design in ECM-3.1 only uses the band classifier to classify a given sample. This contribution reports the results of extending the CCSAO design by adding the “Edge-based” classifier.

The objective results, over ECM-3.1 Anchor for CTC configuration are as follows:

| Config. | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| AI | -0.01% | -0.57% | -0.55% | 101% | 103% |
| RA | -0.04% | -0.62% | -0.43% | 100% | 101% |
| LDB | -0.22% | -0.67% | -0.07% | 101% | 100% |

# Introduction

In the VVC standard [3], Sample Adaptive Offset (SAO) filter uses either the “edge” or “band” classifier during sample classification. Cross-component Sample Adaptive offset (CCSAO) in ECM-3.1 [4] leverages the correlation between the different components to improve the reconstructed quality of the signal. However, the CCSAO in ECM-3.1 only uses the “band” classifier for sample classification. 
To achieve a better complexity/performance trade-off for the whole CCSAO filter, JVET-X0105 [1] proposed an extension to the CCSAO design by also using the edge-based classifier during sample classification.

# Proposal

Similar to the edge classifier of SAO in VVC [1], the proposed edge-based classifier of CCSAO also uses the four 1-D directional patterns for sample classification: horizontal, vertical, 135° diagonal and 45° diagonal, as shown in the figure below.

Figure1. Four 1-D directional patterns for CCSAO EO sample classification: horizontal (EO class = 0), vertical (EO class =1), 135° diagonal and 45° diagonal

For every 1-D pattern, each sample is classified based on the sample difference between the luma sample value labeled as “c” and its two neighbor luma samples labeled as “a” and “b” along the selected 1-D pattern.

Similar to SAO, the encoder may decide the best 1-D directional pattern using the rate-distortion optimization (RDO) and signal this additional information in each classifier/set. Both the sample differences “a-c” and “b-c” are compared against a pre-defined threshold value (Th) to derive the final “class_idx” information.

The encoder selects the best “Th” value from an array of pre-defined threshold values based on RDO and the index into the “Th” array is signalled.

Furthermore, an additional difference between CCSAO edge-based classifier and the SAO edge classifier in VVC [1] is that, in the former, Chroma samples use the co-located Luma samples for deriving the edge information (samples “a”, “c” and “b” are the co-located luma samples) whereas, in the later Chroma samples use its own neighboring samples for deriving the edge information.

The Edge-based classifier process is formulated as follows:

Ea=(a-c<0)? (a-c<(-Th)? 0:1) : (a-c<(Th)? 2:3) 				– (1)

Eb=(b-c<0)? (b-c<(-Th)? 0:1) : (b-c<(Th)? 2:3) 				– (2)

class_idx = iB * 16 + Ea * 4 + Eb 					– (3)

C'rec=Clip1Crec+σCCSAOclass_idx                        – (4)

variable “iB” in equation (3) is derived as follows.

iB=cur ∙Ncur≫BD  (or) iB=col1 ∙Ncol1≫BD (or) iB=col2 ∙Ncol2≫BD,   – (5)

wherein, sample “cur” is the current sample being processed, col1 and col2 are the co-located samples. When Luma samples are processed, col1 and col2 are the co-located Cb and Cr samples respectively. When Chroma(Cb) samples are processed, col1 and col2 are the co-located Y andCrsamples respectively. Similarly When Chroma(Cr)samples are processed, col1 and col2 are the co-located Y and Cb samples respectively.

Based on RDO, encoder signals one among the samples “cur”, “col1", "col2" used in deriving the band information.

# Simulation results

The proposed edge-based classifier for CCSAO is implemented on top of ECM-3.1 [4] and the simulation is performed under common test conditions as described in JVET-X2017 [6]. SIMD optimizations were not used in the implementations. The BD-Rate impact is summarized in Table below.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.76% | -0.55% | 103% | 104% |
| Class A2 | -0.03% | -0.86% | -3.39% | 99% | 104% |
| Class B | -0.01% | -0.99% | 0.42% | 101% | 104% |
| Class C | 0.03% | 0.22% | 0.18% | 100% | 99% |
| Class E | -0.03% | -0.45% | -0.31% | 102% | 103% |
| Overall | -0.01% | -0.57% | -0.55% | 101% | 103% |
| Class D | 0.02% | -0.14% | 0.22% | 100% | 99% |
| Class F | -0.08% | -0.50% | -0.14% | 101% | 102% |
| Class TGM | -0.28% | -1.13% | -1.01% | 99% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.57% | -0.05% | 100% | 102% |
| Class A2 | -0.13% | -1.05% | -3.00% | 100% | 101% |
| Class B | -0.10% | -1.03% | 0.42% | 101% | 100% |
| Class C | 0.05% | 0.16% | 0.13% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.62% | -0.43% | 100% | 101% |
| Class D | 0.05% | -0.04% | -0.28% | 101% | 100% |
| Class F | -0.17% | -0.27% | -0.15% | 102% | 101% |
| Class TGM | -0.32% | -0.50% | -0.44% | 101% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.36% | -1.37% | -0.06% | 100% | 101% |
| Class C | -0.12% | 0.02% | 0.11% | 100% | 101% |
| Class E | -0.13% | -0.41% | -0.33% | 105% | 98% |
| Overall | -0.22% | -0.67% | -0.07% | 101% | 100% |
| Class D | -0.02% | -0.91% | -0.24% | 101% | 101% |
| Class F | -0.22% | -0.71% | 0.07% | 102% | 101% |
| Class TGM | -0.55% | -0.57% | -0.33% | 101% | 101% |

# References

- A. M. Kotra et al., AHG12: Edge Classifier for Cross-component Sample Adaptive Offset (CCSAO), JVET-X0105, November 2021, Teleconference

- C.-W. Kuo et al., AHG12: CCSAO classification with edge information, JVET-X0152, November 2021, Teleconference

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 10)”, JVET-S2001, Jun. 2020, Teleconference

- ECM-3.1, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-3.1

- C.-W. Kuo, X. Xiu, Y.-W. Chen, H.-J. Jhu, W. Chen, X. Wang, “EE2-5.1: Cross-component Sample Adaptive offset”, JVET-W0066, July 2021, Teleconference

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-X2017, November 2021, Teleconference.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).