| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0116-v1 |
| --- | --- |

| Title: | EE2-2.1: Extended MRL candidate list |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Keming Cao, Yao-Jen Chang, Bappaditya Ray, Vadim Seregin, Marta Karczewicz | Tel: Email: | kemicao@qti.qualcomm.com yjchang@qti.qualcomm.com bray@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports test results of EE2 test 2.1a and 2.1b for extended MRL candidate list. The test was conducted on top of ECM-3.1.

EE2-2.1a Extended MRL candidate list:

Extended MRL candidate list {1, 3, 5, 7, 12} is applied in EE2-2.1a, and it reports -0.09%, -0.05%, -0.09% BD rate for luma and chroma components, respectively, with 101% encoder and 100% decoder run time in all intra configurations of common test condition.

EE2-2.1b Extended MRL with reduced number candidates:

Extended MRL candidate list {1, 3, 5, 7} and {1, 3, 5} are applied in EE2-2.1b and it reports -0.07%, -0.07%, -0.04% BD rate for luma and chroma components, respectively, with 100% encoder and 99% decoder run time in all intra configurations for {1, 3, 5, 7}.

It reports -0.06%, -0.06%, -0.02% BD rate for luma and chroma components, respectively, with 100% encoder and 100% decoder run time in all intra configurations for {1, 3, 5}.

# Introduction

Multiple reference line (MRL) is used in VVC and ECM, MRL candidate list consists of 2 additional reference lines, i.e., {1, 2}.

# Proposed Method

This contribution proposes to extend MRL candidate lists to include more reference lines. Instead of {1, 2}, extended MRL candidate list {1, 3, 5, 7, 12}, {1, 3, 5, 7} or {1, 3, 5} is used as shown in Fig.1. For template-based intra mode derivation (TIMD), instead of the full MRL candidate list, only the first two line candidates, i.e., {1, 3}, are used.

Figure 1. Extended MRL candidate list

# Results

EE2-2.1a was conducted on top of ECM-3.1, and it reports -0.09%, -0.05%, -0.09% BD rate for luma and chroma components, respectively, with 101% encoder and 100% decoder run time in all intra configurations of common test condition.

EE2-2.1b was conducted on top of ECM-3.1, and it reports -0.07%, -0.07%, -0.04% BD rate for luma and chroma components, respectively, with 100% encoder and 99% decoder run time in all intra configurations of common test condition for MRL list {1, 3, 5, 7} and -0.06%, -0.06%, -0.02% BD rate for luma and chroma components, respectively, with 100% encoder and 100% decoder run time in all intra configurations of common test condition for MRL list {1, 3, 5}

Details of the test results are shown in Table 1, Table 3 and Table 4, respectively.

Table 2 shows the frequency for each line of MRL candidate list in EE2-2.1a for all intra configuration when MRL candidate list {1, 3, 5, 7, 12} is used.

Table 1 Test results for 2.1a, MRL candidate list is {1, 3, 5, 7, 12}

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.07% | 0.03% | 101% | 100% |
| Class A2 | -0.07% | 0.00% | -0.08% | 101% | 100% |
| Class B | -0.13% | -0.14% | -0.19% | 103% | 106% |
| Class C | -0.10% | -0.04% | -0.06% | 99% | 96% |
| Class E | -0.10% | -0.11% | -0.06% | 100% | 96% |
| Overall | -0.09% | -0.05% | -0.09% | 101% | 100% |
| Class D | -0.03% | 0.09% | -0.07% | 99% | 93% |
| Class F | -0.09% | -0.11% | 0.03% | 98% | 95% |
| Class TGM | -0.16% | -0.05% | -0.07% | 99% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | -0.18% | 100% | 99% |
| Class A2 | -0.06% | 0.04% | 0.01% | 99% | 99% |
| Class B | -0.05% | 0.01% | 0.18% | 96% | 96% |
| Class C | -0.05% | -0.07% | 0.14% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.01% | 0.06% | 98% | 98% |
| Class D | 0.03% | 0.15% | -0.18% | 98% | 97% |
| Class F | -0.05% | 0.17% | 0.02% | 97% | 97% |
| Class TGM | -0.08% | -0.05% | 0.05% | 96% | 96% |

Table 2 Frequency for each line in MRL candidate list {1, 3, 5, 7, 12}

|  | line 1 | line 3 | line 5 | line 7 | line 12 |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 36.24% | 14.96% | 14.75% | 13.19% | 20.86% |
| Class A2 | 53.81% | 18.86% | 10.88% | 7.10% | 9.35% |
| Class B | 42.57% | 20.90% | 16.12% | 9.53% | 10.87% |
| Class C | 53.28% | 18.69% | 11.29% | 7.54% | 9.20% |
| Class E | 46.31% | 21.08% | 12.36% | 8.45% | 11.80% |
| Overall | 46.37% | 19.29% | 13.70% | 9.04% | 11.60% |
| Class D | 44.80% | 19.43% | 13.46% | 9.19% | 13.11% |
| Class F | 57.17% | 18.00% | 9.55% | 6.62% | 8.66% |
| Class TGM | 37.74% | 17.43% | 19.89% | 10.41% | 14.53% |

Table 3 Test results for 2.1b, MRL candidate list is {1, 3, 5, 7}

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.01% | 0.03% | 100% | 96% |
| Class A2 | -0.06% | -0.09% | 0.01% | 100% | 97% |
| Class B | -0.10% | -0.14% | -0.13% | 101% | 101% |
| Class C | -0.08% | -0.04% | -0.12% | 100% | 102% |
| Class E | -0.07% | -0.04% | 0.09% | 100% | 99% |
| Overall | -0.07% | -0.07% | -0.04% | 100% | 99% |
| Class D | -0.01% | 0.07% | -0.07% | 98% | 92% |
| Class F | -0.07% | -0.13% | -0.03% | 99% | 101% |
| Class TGM | -0.14% | -0.07% | -0.07% | 100% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.04% | -0.07% | 101% | 100% |
| Class A2 | -0.07% | 0.05% | -0.01% | 100% | 101% |
| Class B | -0.03% | 0.02% | 0.08% | 99% | 98% |
| Class C | -0.03% | 0.14% | 0.05% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | 0.05% | 0.03% | 100% | 100% |
| Class D | -0.01% | 0.29% | -0.19% | 96% | 97% |
| Class F | -0.04% | 0.00% | -0.13% | 99% | 99% |
| Class TGM | -0.06% | -0.02% | -0.03% | 97% | 98% |

Table 4 Test results for 2.1b, MRL candidate list is {1, 3, 5}

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.01% | 0.04% | 100% | 100% |
| Class A2 | -0.07% | 0.04% | 0.03% | 100% | 100% |
| Class B | -0.08% | -0.15% | -0.05% | 102% | 105% |
| Class C | -0.06% | -0.04% | -0.09% | 98% | 97% |
| Class E | -0.06% | -0.22% | 0.03% | 99% | 96% |
| Overall | -0.06% | -0.08% | -0.02% | 100% | 100% |
| Class D | -0.02% | 0.05% | -0.17% | 99% | 95% |
| Class F | -0.03% | 0.04% | 0.06% | 98% | 98% |
| Class TGM | -0.09% | -0.03% | -0.05% | 100% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.16% | -0.17% | 100% | 99% |
| Class A2 | -0.06% | 0.11% | 0.06% | 98% | 98% |
| Class B | -0.04% | 0.09% | 0.10% | 99% | 99% |
| Class C | 0.01% | 0.12% | 0.27% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.12% | 0.09% | 99% | 99% |
| Class D | -0.02% | 0.33% | -0.23% | 98% | 98% |
| Class F | -0.05% | -0.12% | -0.33% | 97% | 98% |
| Class TGM | -0.06% | -0.09% | 0.00% | 98% | 99% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).