| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0145 |
| --- | --- |

| Title: | EE2-3.12a: History-parameter-based affine model inheritance |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Zhipin Deng Na Zhang, Yang Wang 8910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution presents the EE2-3.12a test results for history-parameter-based affine model inheritance (HAMI) proposed in JVET-X0088. With HAMI, affine parameters of previously affine-coded block have been stored in a History-Parameter Table (HPT). New affine merge and AMVP candidates as well as new regular merge candidates can be constructed with affine parameters fetched from HPT and base MVs fetched from neighbouring blocks, and put into the sub-block-based merge candidate list, the affine AMVP candidate list and the regular merge candidate list, respectively. On top of ECM-3.1, simulation results of the proposed method are reported as below:

RA: {-0.17%, -0.12%, -0.11%, 99%, 100%}; LB: { -0.11%, -0.21%, 0.02%, 100%, 100%}.

# Introduction

In VVC, the affine model can be inherited from a neighbouring block. In JVET-M0266 [1] and JVET-N0263 [2], history-parameter-based affine model inheritance (HAMI) was proposed to allow the affine model to be inherited from a previously affine-coded block which may not be neighboring to the current block. A further study of HAMI with new aspects introduced was proposed in JVET-X0088 [3] and studied in EE2-3.12a.

# Proposed methods

A history-parameter table (HPT) is established. An entry of HPT stores a set of affine parameters: a, b, c and d, each of which is represented by a 16-bit signed integer. Entries in HPT is categorized by reference list and reference index. Five reference indices are supported for each reference list in HPT. In a formular way, the category of HPT (denoted as HPTCat) is calculated as

HPTCat (RefList, RefIdx) = 5×RefList + min (RefIdx, 4),

wherein RefList and RefIdx represents a reference picture list (0 or 1) and a reference index, respectively. For each category, at most seven entries can be stored, resulting in 70 entries totally in HPT. At the beginning of each CTU row, the number of entries for each category is initialized as zero. After decoding an affine-coded CU with reference list RefListcur and RefIdxcur, the affine parameters are utilized to update entries in the category HPTCat(RefListcur, RefIdxcur) in a way similar to HMVP table updating.

A history-affine-parameter-based candidate (HAPC) is derived from a neighbouring 4×4 block denoted as A0, A1, B0, B1 or B2 in Fig. 1 and a set of affine parameters stored in a corresponding entry in HPT. The MV of a neighbouring 4×4 block served as the base MV. In a formulating way, the MV of the current block at position (x, y) is calculated as:

,

where (mvhbase, mvvbase) represents the MV of the neighbouring 4×4 block, (xbase, ybase) represents the center position of the neighbouring 4×4 block. (x, y) can be the top-left, top-right and bottom-left corner of the current block to obtain the corner-position MVs (CPMVs) for the current block, or it can be the center of the current block to obtain a regular MV for the current block.

Fig. 1 shows an example of how to derive an HAPC from block A0. The affine parameters {a0, b0, c0, d0} are directly fetched from one entry of category HPTIdx(RefListA0, refIdx0A0) in HPT. The affine parameters from HPT, with the center position of A0 as the base position, and the MV of block A0 as the base MV, are used together to derive the CPMVs for an affine merge HAPC, or an affine AMVP HAPC. They can also be used to derive MVs located at the center of the current block, as regular merge candidates. A HAPC can be put into the sub-block-based merge candidate list, the affine AMVP candidate list or the regular merge candidate list. As a response to new HAPCs being introduced, the size of sub-block-based merge candidate list is increased from five to ten and twelve for random access and low-delay B configurations, respectively. Besides, the size of regular merge candidate list is increased from ten to eleven for random access configurations to accommodate the newly added regular merge candidates.

Fig. 1 An example of generating an HAPC.

# Experimental results

The proposed method is tested on top of ECM-3.1 [4], according to the common test conditions [5]. The simulation results are summarized in table 1.

Table 1: Performance of the proposed method

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.25% | -0.27% | 102% | 101% |  |  |  |  |  |
| Class A2 | -0.29% | -0.20% | -0.23% | 101% | 102% |  |  |  |  |  |
| Class B | -0.18% | -0.04% | -0.07% | 100% | 102% | -0.15% | -0.34% | 0.11% | 102% | 101% |
| Class C | -0.09% | -0.08% | 0.06% | 96% | 96% | -0.15% | -0.05% | -0.02% | 98% | 100% |
| Class E |  |  |  |  |  | 0.01% | -0.19% | -0.06% | 98% | 98% |
| Overall | -0.17% | -0.12% | -0.11% | 99% | 100% | -0.11% | -0.21% | 0.02% | 100% | 100% |
| Class D | -0.14% | -0.14% | -0.01% | 98% | 95% | -0.18% | -0.57% | -0.05% | 96% | 95% |
| Class F | -0.09% | -0.01% | -0.11% | 97% | 97% | 0.03% | 0.26% | -0.14% | 98% | 98% |

# Conclusions

This contribution presents EE test results for history-parameter-based affine model inheritance. The proposed method contributes promising coding gains with negligible complexity overhead. It is recommended to adopt it into ECM.

# References

- K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, “CE2-5.5: History-parameter-based affine model inheritance,” JVET-N0263, Mar. 2019.

- K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong, “CE2-related: History-based affine merge candidates,” JVET-M0266, Jan. 2019.

- K. Zhang, L. Zhang, Z. Deng, N. Zhang, Y. Wang, “Non-EE2: History-parameter-based Affine Model Inheritance,” JVET-X0088, Oct. 2021.

- ECM-3.1, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-3.1.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-X2017, Oct. 2021.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).