| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0147-v1 |
| --- | --- |

| Title: | EE2-5.2: Adaptive Filter Shape Selection for ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenbin Yin1, Kai Zhang2, Li Zhang2 Nan Hu3, Vadim Seregin3, Marta Karczewicz3 Mohammed Golam Sarwer4, Ru-Ling Liao4, Jie Chen4, Yan Ye4, Xinwei Li4 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA 35775 Morehouse Drive, San Diego, CA 92121, USA 4525 Almanor Ave, Sunnyvale, CA, USA | Tel: Email: | yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com nanh@qti.qualcomm.com m.sarwer@alibaba-inc.com |
| Source: | Bytedance Inc. Qualcomm Incorporated Alibaba Group |  |  |

_____________________________

Abstract

In the current ALF design, the online-trained filters only have one fixed shape. In this contribution, a CTB-level adaptive filter shape selection (AFSS) method for ALF is proposed. With AFSS, a filter shape index that is associated with luma filters is signalled for each APS. Each CTB could select the best luma filter shape from the available APSs.

On top of ECM-3.1, simulation results (BD-rate changes, encoding time and decoding time) are reported as below:

EE2-5.2 a:

AI: -0.05%, -0.00%, -0.00%, 102%, 100%;

RA: -0.13%, 0.02%, 0.04%, 102%, 101%;

LB: %, %, %, %, %;

EE2-5.2 b:

AI: -0.04%, 0.00%, 0.00%, 103%, 101%;

RA: -0.10%, -0.01%, 0.00%, 102%, 100%;

LB: %, %, %, %, %;

- Introduction

In ALF of ECM 3.1, only a fixed diamond filter shape is used for the online-trained luma filters. The single shape design may not achieve the optimal compression performance. In this contribution, an adaptive filter shape selection (AFSS) method for ALF is proposed.

- Proposed methods

  - AFSS

With AFSS [1], two candidate filter shapes: a diamond shape candidate as shown in Figure 2-1(a), which is currently applied in ECM, and a cross shape candidate as shown in Figure 2-1(b)/(c), which is newly introduced, can be adaptively selected by the luma filters in ALF. The number of coefficients that need to be trained and signaled for a luma filter is 20 with all the filter shapes. In test EE2-5.2a, filter shape selection is performed between Figure 2-1(a) and Figure 2-1(b) while in test EE2-5.2b Figure 2-1(a) and Figure 2-1(c) are used as the shape candidates. The only difference between the two shapes in Figure 2-1(b) and Figure 2-1(c) is the maximum filter length.

| (a) | (b) | (c) |
| --- | --- | --- |

Figure 2-1 Filter shapes in AFSS.

In each APS, a shape index for the online-trained luma filters is signaled to decoder. Each APS contains the luma filters that are associated with the filter shape index.

For each CTB, an APS index is signaled to indicate which luma filter shape is used to filter the current CTB. When filtering a luma sample, the coefficients and clip indices are also rearranged according to the corresponding filter shape.

- Simulation results

The proposed method is implemented on top of ECM-3.1 [2]. The simulation results are tested under common test condition [3] and the anchor is ECM-3.1.

Table 3-1 shows the simulation results of test EE2-5.2a.

Table 3-2 shows the simulation results of test EE2-5.2b.

It is observed that for both tests, the performance on Luma component is promising. Besides, the AFSS does not introduce any decoder complexity and only slightly increases the encoding time.

TABLE 3-1 Performance of the EE2-5.2a

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.01% | 0.01% | 105% | 100% | -0.08% | -0.12% | -0.07% | 101% | 101% |  |  |  |  |  |
| Class A2 | -0.13% | 0.00% | 0.00% | 102% | 100% | -0.16% | 0.02% | 0.02% | 101% | 101% |  |  |  |  |  |
| Class B | -0.01% | 0.00% | 0.00% | 102% | 100% | -0.06% | 0.11% | 0.05% | 102% | 100% |  |  |  |  |  |
| Class C | -0.05% | 0.00% | 0.00% | 100% | 100% | -0.22% | 0.02% | 0.10% | 102% | 101% |  |  |  |  |  |
| Class E | 0.00% | 0.00% | 0.00% | 103% | 101% |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.05% | 0.00% | 0.00% | 102% | 100% | -0.13% | 0.02% | 0.04% | 102% | 101% |  |  |  |  |  |
| Class D | -0.03% | 0.00% | 0.00% | 101% | 97% | -0.33% | 0.11% | 0.17% | 102% | 101% | -0.22% | -0.23% | 0.20% | 105% | 98% |

TABLE 3-2 Performance of the EE2-5.2b

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.01% | 0.01% | 105% | 100% | -0.04% | 0.03% | -0.06% | 101% | 101% |  |  |  |  |  |
| Class A2 | -0.10% | 0.00% | 0.00% | 103% | 99% | -0.11% | -0.05% | -0.03% | 101% | 100% |  |  |  |  |  |
| Class B | -0.02% | 0.00% | 0.00% | 103% | 100% | -0.05% | -0.03% | 0.02% | 102% | 100% |  |  |  |  |  |
| Class C | -0.05% | 0.00% | 0.00% | 102% | 104% | -0.20% | 0.03% | 0.03% | 102% | 101% |  |  |  |  |  |
| Class E | -0.02% | 0.01% | 0.01% | 103% | 100% |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.04% | 0.00% | 0.00% | 103% | 101% | -0.10% | -0.01% | 0.00% | 102% | 100% |  |  |  |  |  |
| Class D | -0.03% | 0.00% | 0.00% | 101% | 104% | -0.32% | 0.02% | 0.19% | 103% | 101% | -0.24% | 0.35% | 0.09% | 105% | 100% |

- Conclusion

Two tests of adaptive filter shape selection for ALF are reported in this contribution. Both tests can achieve promising coding performance with limited encoding time increase and no decoding time changes. It is recommended to adopt one of the tests to the next version of ECM software.

- References

- W. Yin, K. Zhang, L. Zhang and et.al, “EE2-related: Adaptive Filter Shape Selection for ALF,” JVET-X0086, October 2021.

- ECM-3.1, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-3.1.

- M. Karczewicz, Y.Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-X2017, October. 2021.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).